3GPP TSG-RAN5 Meeting #105	R5-246454r1

# Orlando, US, 18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3024 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding transmitter requirements for UL-MIMO bands n13, n26, n85, n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing transmitter requirements for UL-MIMO bands n13, n26, n85, n105 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added transmitter requirements for UL-MIMO bands n13, n26, n85, n105 in clauses - 6.2D.1 UE maximum output power for UL MIMO - 6.2D.2 UE maximum output power reduction for UL MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmitter requirements for UL-MIMO bands n13, n26, n85, n105 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1, 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Editorial correction in Table 6.2D.2.5-1. Point changed to comma |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

Editor’s Note:

- No test points are defined for 2-layer UL MIMO since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- No test points are defined for Power Class 1.5 non-FWA UEs supporting UFPTx since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- TP analysis needs to be updated regarding Note 3 in Table 6.2D.1.4.1-2

6.2D.1.1	Test purpose

To verify that the error of the UE maximum output power for UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UEs, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n2 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n5 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n8 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n13 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n24 |  |  |  |  |  |  | 23 | +2/-41 |  |  |
| n25 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n26 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n28 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n38 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n41 |  |  | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n71 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n85 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n97 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n105 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n5 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n8 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n13 |  |  |  |  |  |  |  | +2+TT/-3-TT |  |  |
| n24 |  |  |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n26 |  |  |  |  |  |  |  | +2+TT/-31-TT |  |  |
| n28 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n85 |  |  |  |  |  |  |  | +2+TT/-31-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n105 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 NOTE 3: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

-------- Skipped unchanged clauses --------

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n85, n97, n105) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n13, n24, n25, n26, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79, n85, n105) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （14.02 - TT） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------