3GPP TSG-RAN5 Meeting #105	R5-246593r1

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4723 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS broadcast test case 14.1.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Table 14.1.1.4.1.3.3-1 RRCReconfiguration is also used for step 14. - The condition for Table 14.1.1.4.1.3.3-14:TMGI is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Step14 in Table 14.1.1.4.1.3.3-1. - Add the condition in Table 14.1.1.4.1.3.3-14:TMGI, plmn-Index-r17=1 is used for Step 4 while plmn-Index-r17=1 is used for Step 13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration is not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: 1. Steps changes in Table 14.1.1.4.1.3.3-12 MBSBroadcastConfiguration and Table 14.1.1.4.1.3.3-13: MBS-SessionInfoList are not correct, so the changes are deleted. 2. Steps changes in Table 14.1.1.4.1.3.3-14: TMGI is not correct, update Step3 to Step4 and Step14 to Step13. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.1.4	MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling

14.1.1.4.1	MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling / Intra-band Contiguous CA

14.1.1.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC CONNECTED state with SCell configured and interested to receive MBS broadcast services }

ensure that {

when { UE is receiving SIB20 for the Scell via dedicated signalling and plmn-IdentityInfoList-r17 is not present in the SCellConfig }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH in the Scell at the next repetition period and starts MBS reception }

}

(2)

with { UE in NR RRC CONNECTED state with SCell configured and interested to receive MBS broadcast services }

ensure that {

when { UE is receiving SIB20 for the Scell via dedicated signalling and plmn-IdentityInfoList-r17 is present in the SCellConfig }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH in the Scell at the next repetition period and starts MBS reception }

}

14.1.1.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 6.3.2, 5.9.2.2 and 5.9.2.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.9.2.2]

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s).

[TS 38.331, clause 5.9.2.3]

An MBS capable UE interested to receive or receiving an MBS broadcast service shall:

…

1>	if the UE enters a cell broadcasting SIB20; or

1>	if the UE receives sCellSIB20:

2>	acquire the MBSBroadcastConfiguration message on MCCH in the concerned cell at the next repetition period.

[TS 38.331, clause 6.3.2]

CellGroupConfig field descriptions

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field and plmn-IdentityInfoList are both absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field and npn-IdentityInfoList are both absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

14.1.1.4.1.3	Test description

14.1.1.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 is the SCell to be added.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	 None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	UE is made interested in receiving MBS Broadcast service with MBS Service ID ‘000001’H.

14.1.1.4.1.3.2	Test procedure sequence

Table 14.1.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing a sCellToAddModList with SCell NR Cell 3 addition. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 3 | The SS transmits Activation MAC control element to activate NR SCell 3. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 4 | The SS starts to transmit MBSBroadcastConfiguration message on NR Cell 3 | - | - | - | - |
| 5 | The SS transmits an RRCReconfiguration message containing SIB20 of NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 7 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 3. | - | - | - | - |
| 8a1-8a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C on NR Cell 1. | - | - | - | - |
| - | Exception: Step 9 is repeated 5 times. | - | - | - | - |
| 9 | The SS transmits an MBS Packet on the MTCH on NR Cell 3. | <-- | MBS Packet. | - | - |
| 10 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message on NR Cell 1. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 11 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 12 | Check: Is the number of reported MBS Packets received on the MTCH in step 11 greater than zero? | - | - | 1 | P |
| 13 | The SS update MBSBroadcastConfiguration message on NR Cell 3 | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message to update SIB20 of NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 16 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 3. | - | - | - | - |
| 17 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 18 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 19a1-19a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C on NR Cell 1. | - | - | - | - |
| - | Exception: Step 20 is repeated 5 times. | - | - | - | - |
| 20 | The SS transmits an MBS Packet on the MTCH on NR Cell 3. | <-- | MBS Packet. | - | - |
| 21 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message on NR Cell 1. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 22 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 23 | Check: Is the number of reported MBS Packets received on the MTCH in step 22 greater than zero? | - | - | 2 | P |

14.1.1.4.3.3	Specific message contents

Table 14.1.1.4.1.3.3-1: RRCReconfiguration (steps 1 ,5 and 14, Table 14.1.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 14.1.1.4.1.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-2: CellGroupConfig (Table 14.1.1.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon | ServingCellConfigCommon | Table 14.1.1.4.1.3.3-5 | Step 1 |
|  | Not present |  | Step 5, Step 14 |
| sCellConfigDedicated | ServingCellConfig with condition No_UL and Scell_Add |  | Step 1 |
|  | Not present |  | Step 5, Step 14 |
| sCellSIB20-r17 | Not present |  | Step 1 |
| sCellSIB20-r17 CHOICE { |  |  | Step 5, Step 14 |
| setup | SCellSIB20-r17 | OCTET STRING (CONTAINING SystemInformation) |  |
| } |  |  |  |
| plmn-IdentityInfoList-r17 | Not present |  | Step 5 |
| plmn-IdentityInfoList-r17 CHOICE { |  |  | Step 14 |
| Setup SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { |  |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { | 2 entries |  |  |
| plmn-Identity[1] | PLMN for NR Cell 3 | entry 1 TS 38.508-1 [4] Table 4.4.2-3 |  |
| plmn-Identity[2] | PLMN 15 | entry 2 TS 36.523-1 [42], Table 6.0.1-1 |  |
| } |  |  |  |
| trackingAreaCode | TrackingAreaCode for NR Cell 3 | TS 38.508-1 [4] Table 4.4.2-3 |  |
| ranac | RAN-AreaCode | TS 38.508-1[4] Table 4.6.3-135 |  |
| cellIdentity | CellIdentity for NR Cell 3 | TS 38.508-1 [4]Table 4.4.2-2 |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-3: SystemInformation (Table 14.1.1.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-IEs SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { | 1 entry |  |  |
| sib20-v1700 | SIB20-r17 | Table 14.1.1.4.1.3.3-4 |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-4: SIB20-r17 (Table 14.1.1.4.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB20-r17 ::= SEQUENCE { |  |  |  |
| cfr-ConfigMCCH-MTCH-r17 | CFR-ConfigMCCH-MTCH-r17 | Table 14.1.1.4.1.3.3-4A |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-4A: CFR-ConfigMCCH-MTCH-r17 (Table 14.1.1.4.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE { |  |  |  |
| locationAndBandwidthBroadcast-r17 | Not present |  |  |
| pdsch-ConfigMCCH-r17 | PDSCH-ConfigBroadcast | TS 38.508-1 [4] Table 4.6.7-8 |  |
| commonControlResourceSetExt-r17 | Not present |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-5: ServingCellConfigCommon (Table 14.1.1.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168, condition SCell_add and No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 14.1.1.4.1.3.3-6 |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-6: DownlinkConfigCommon (Table 14.1.1.4.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-52, condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.1.4.1.3.3-7 |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-7: BWP-DownlinkCommon (Table 14.1.1.4.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10, condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon | Table 14.1.1.4.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-8: PDCCH-ConfigCommon (Table 14.1.1.4.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96, condition SCell_Add and MBS_Broadcast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| searchSpaceMCCH-r17 | SearchSpaceId with condition CSS |  |  |
| commonSearchSpaceListExt2-r17 SEQUENCE(SIZE (1..4)) OF SearchSpaceExt-v1700 { | 1 entry |  |  |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-9: ACTIVATE TEST MODE (preamble, Table 14.1.1.4.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.1.4.1.3.3-10: CLOSE UE TEST LOOP (step 8a1 and step 19a1, Table 14.1.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.7A.2, condition UE TEST LOOP MODE C and Broadcast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| UE test loop mode C LB setup |  | MRB ID |  |  |
| MRB ID | ‘0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0’B | Bit1 of Octet1 = 1: Broadcast MRB. Bit4 – bit1 of Octet2 = 0 0 0 0 and bit8 of Octet3 = 1: Identity of the logical channel of broadcast MTCH is 2. | Step19a1 |  |

Table 14.1.1.4.1.3.3-11: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 10, Table 14.1.1.4.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-9

Table 14.1.1.4.1.3.3-12: MBSBroadcastConfiguration (step 4 and step 13, Table 14.1.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList | Table 14.1.1.4.1.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-13: MBS-SessionInfoList (Table 14.1.1.4.1.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (0..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 1 entry |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  |  |  |
| mbs-SessionId-r17 | TMGI | Table 14.1.1.4.1.3.3-14 |  |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 1 |  | Step 4 |
|  | 2 |  | Step 13 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.4.1.3.3-14: TMGI (Table 14.1.1.4.1.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  | Step 4 |
|  | 2 |  | Step 13 |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>