3GPP TSG-RAN5 Meeting #105	R5-246604r2

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4727 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new eRedCap test case 7.1.1.1.26 - Contention free random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new eRedCap test case is added to test that eRedCap UE use the set of RA resource for RedCap for CFRA when the set of RA resource for eRedCap is not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new eRedCap test case 7.1.1.1.26 Random access procedure / eRedCap UE identification / Msg1-based / Contention free random access procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_redcap_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.26(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0525 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Delete the redundant condition “NR” in the Table 7.1.1.1.26.3.3-1 2nd revision: - Update coversheet. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.1.26	Random access procedure / eRedCap UE identification / Msg1-based / Contention free random access procedure

7.1.1.1.26.1	Test Purpose (TP)

(1)

with { UE supporting eRedCap and in NR RRC_CONNECTED state }

ensure that {

when { UE performs handover to the target cell and starts random Access procedure, and contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with eRedCap indication }

then { the UE selects the set of Random Access resources configured with eRedCap indication for this Random Access procedure}

}

(2)

with { UE supporting eRedCap and in NR RRC_CONNECTED state }

ensure that {

when { UE performs handover to the target cell and starts random Access procedure, and contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is no set of Random Access resources available that is only configured with eRedCap indication and there is one set of Random Access resources available that is only configured with RedCap indication }

then { the UE selects the set of Random Access resources configured with RedCap indication for this Random Access procedure }

}

7.1.1.1.26.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.1.1b. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.1b]

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure in rach-ConfigDedicated:

2>	if RedCap is applicable for this Random Access procedure:

…

2>	else if eRedCap is applicable for this Random Access procedure:

3>	if Msg1 repetition number is indicated in rach-ConfigDedicated:

…

3>	else:

4>	if there is one set of Random Access resources available that is only configured with eRedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else if there is one set of Random Access resources available that is only configured with RedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else:

5>	select the set of Random Access resources that not is associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

[TS 38.321, clause 5.1.1]

1>	else if contention-free Random Access Resources have not been explicitly provided by an LTM Cell Switch Command MAC CE, the Random Access procedure was not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3, contention-free Random Access Resources associated with CSI-RSs have been explicitly provided in rach-ConfigDedicated and at least one CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

…

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

7.1.1.1.26.3	Test description

7.1.1.1.26.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured.

-	Test loop function (Off)

7.1.1.1.26.3.2	Test procedure sequence

Table 7.1.1.1.26.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to handover NR Cell 1 to target NR Cell 2, including RACH-ConfigDedicated, and two FeatureCombinationPreambles-r17 in RACH-ConfigCommon. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | Check: Does the UE transmit Preamble on PRACH corresponding to ra-PreambleIndex in RACH-ConfigDedicated configured in step 1 on NR Cell 2? | --> | PRACH Preamble | 1 | P |
| 3 | The SS transmits Random Access Response on NR cell 2, with RAPID corresponding to ra-PreambleIndex in RACH-ConfigDedicated in step 1. | <-- | Random Access Response | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 5 | The SS transmits an RRCReconfiguration message to handover NR Cell 2 to target NR Cell 1, including RACH-ConfigDedicated, and one FeatureCombinationPreambles-r17 in RACH-ConfigCommon. (Note 2) | <-- | RRCReconfiguration | - | - |
| 6 | Check: Does the UE transmit Preamble on PRACH corresponding to ra-PreambleIndex in RACH-ConfigDedicated configured in step 5 on NR Cell 1? | --> | PRACH Preamble | 2 | P |
| 7 | The SS transmits Random Access Response on NR cell 1, with RAPID corresponding to ra-PreambleIndex in RACH-ConfigDedicated in step 5. | <-- | Random Access Response | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| Note 1: one FeatureCombinationPreambles-r17 only configured with eRedCap indication, the other FeatureCombinationPreambles-r17 only configured with RedCap indication. Note 2: one FeatureCombinationPreambles-r17 only configured with RedCap indication. |  |  |  |  |  |

7.1.1.1.26.3.3	Specific message contents

Table 7.1.1.1.26.3.3-1: RRCReconfiguration (step 1 and step 5, Table 7.1.1.1.26.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig as per TS 38.508-1[4] Table 4.6.3-132 with conditions DRBn and Recover_PDCP | n set to the default DRB of the first PDU session |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 7.1.1.1.26.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.26.3.3-2: CellGroupConfig (Table 7.1.1.1.26.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.1.26.3.3-4 |  |
| newUE-Identity | UE identity different from NR cell 1 UE identity |  | Step 1 |
|  | UE identity different from NR cell 2 UE identity |  | Step 5 |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated | Table 7.1.1.1.26.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.26.3.3-3: RACH-ConfigDedicated (Table 7.1.1.1.26.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  | entry 1 |  |
| ra-PreambleIndex | 52 |  | Step 1 |
|  | 53 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.26.3.3-4: ServingCellConfigCommon (Table 7.1.1.1.26.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.1.26.3.3-5 |  |
| } |  |  |  |
| featurePriorities-r17 SEQUENCE { |  |  |  |
| redCapPriority-r17 | 0 |  |  |
| } |  |  |  |
| featurePriorities-v1800 | Not present |  | Step 5 |
| featurePriorities-v1800 SEQUENCE { |  |  | Step 1 |
| eRedCapPriority-r18 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.26.3.3-5: BWP-UplinkCommon (Table 7.1.1.1.26.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.1.26.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.26.3.3-6: RACH-ConfigCommon (Table 7.1.1.1.26.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdSSB | 96 | -60dBm |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 1 entry |  | Step 5 |
|  | 2 entries |  | Step 1 |
| FeatureCombinationPreambles-r17[1] SEQUENCE { |  | entry 1 | Step 1 and Step 5 |
| featureCombination-r17 SEQUENCE { |  |  |  |
| redCap-r17 | True |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 |  |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 4 |  |  |
| rsrp-ThresholdSSB-r17 | 96 | -60dBm | Step 1 |
|  | 0 | -156dBm | Step 5 |
| } |  |  |  |
| FeatureCombinationPreambles-r17[2] SEQUENCE { |  | entry 2 | Step 1 |
| featureCombination-r17 SEQUENCE { |  |  |  |
| eRedCap-r18 | True |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 12 |  |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 4 |  |  |
| rsrp-ThresholdSSB-r17 | 0 | -156dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>