3GPP TSG-RAN5 Meeting #105	R5-246619r1

# Orlando, Florida, USA, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1105 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to UL-256QAM requirements in 6.2A.2.0.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 6.2A.2.0 missing 256QAM of minimum conformance requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.2A.2.0.1 and add 256QAM of minimum conformance requirements in 6.2A.2.0.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: in Table 6.2A.2.0.2-1, keep “Note 1” from core spec. |  |  |  |  |  |  |  |  |

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.0	Minimum conformance requirements

< Start of Change >

##### 6.2A.2.0.1	General

The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA. In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2, shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2A.4.0 apply.

The requirements in the following subclauses are only applicable to the following CA configurations:

-	intra-band contiguous uplink CA, with the aggregated channel bandwidth up to 800 MHz.

-	intra-band non-contiguous uplink CA with UL frequency separation no greater than 1400 MHz, and no more than 3 sub-blocks. A sub-block may consist of single CC or multiple contiguous CCs.

-	In case the CA configuration consists of a single UL CC, MPR for contiguous UL CA applies and where necessary, BWchannel shall be used as BWchannel_CA.

##### 6.2A.2.0.2	Maximum output power reduction for power class 1

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA+∆MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Table 6.2A.2.0.2-1.

∆MPR for 256 QAM as specified in Table 6.2.2.3.1-3 applies.

Table 6.2A.2.0.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.3.1-1 if CABW  200 MHz, from Table 6.2.2.3.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3.1-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3.1-1 and Table 6.2.2.3.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A + 14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

For intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA+∆MPR, -10*A +  14.4)

Where:

MPRNC_CA is derived from table 6.2A.2.0.2-2

∆MPR as specified in Table 6.2.2.3.1-3 applies.

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

Table 6.2A.2.0.2-2: MPRNC_CA for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRNC_CA.

< End of Change >