# Title		Test Tolerances for NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA when serving cell under CCA

Source:			Qualcomm

# 1.	Introduction

This document relates to the inter frequency NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA when serving cell under CCA test case in TS 38.533.

The test cases in question, from TS 38.533, are:

- 11.1.4.2, NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.21.0 Annex A

#### A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA

##### A.11.1.4.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR cell subject to CCA to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2A.2.5 when the E-UTRAN cell is of lower priority.

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.11.1.4.2.1-1, A.11.1.4.2.1-2, A.11.1.4.2.1-3 and A.11.1.4.2.1-4. The test consists of three successive time periods, with time duration of T1 and T2 respectively. Both NR Cell 1 and E-UTRAN Cell 2 are already identified by the UE prior to the start of the test. E-UTRAN Cell 2 is of lower priority than Cell 1.

Table A.11.1.4.2.1-1: Supported test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.11.1.4.2.1-2: General test parameters for NR cell subject to CCA to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell 1 | The UE camps on Cell 1 in the initial phase. |
| T1 end condition | Active cell |  | 1, 2 | Cell 2 | The UE shall perform reselection to Cell 2 during T1. |
|  | Neighbour cells |  | 1, 2 | Cell 1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell 1 | The UE shall perform reselection to Cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2 | Cell 2 |  |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DBT Window Configuration |  |  | 1, 2 | DBT.1 | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.2 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH |  |  | 1 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
| configuration index |  |  | 2 | 4 |  |
| T1 |  | s | 1, 2 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.4.2.1-3: Cell specific test parameters for NR Cell 1 subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.1.1.CCA |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2 | 0.9 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2 | 0.75 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2 | 0.5 |  |
| UL CCA probability PCCA_UL |  | 1, 2 | 1 |  |
| Md,max |  | 1, 2 | 16 |  |
| Mm,max |  | 1, 2 | 4 |  |
| Me,max |  | 1, 2 | 8 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 CCA |  |
| RMSI CORESET RMC Configuration |  | 1, 2 | CR.1.1 CCA |  |
| Dedicated CORESET RMC |  | 1, 2 | CCR.1.1 CCA |  |
| Configuration |  |  |  |  |
| SSB configuration |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0 |  |
| RLM-RS |  | 1, 2 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |
|  | dBm/SCS | 1, 2 | -95 |  |
|  | dBm/15 kHz | 1, 2 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 2 | -99 | -83 |
|  | dB | 1, 2 | -4 | 12 |
|  | dB | 1, 2 | -4 | 12 |
| Io | dBm/38.16 MHz | 1, 2 | -62.50 | -51.69 |
| Treselection | s | 1, 2 | 0 |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |
| Threshx, high (Note 2) | dB | 1, 2 | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell Note 3: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |

Table A.11.1.4.2.1-4: Cell specific test parameters for E-UTRA Cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | s | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.11.1.4.2.2	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

-	Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

-	TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 7.68 s, allow 8 s for the cell re-selection delay to a lower priority E-UTRAN cell.

# 3. Discussion

The test case has 2 time periods: T1 and T2. To analyse the test case it is helpful to show the value of Srxlev for each cell versus time, together with the thresholds, as shown below:

3.1 Selection criterion for NR

For Cell 1 (NR), the selection criterion is satisfied according to the equation from TS 38.304 v18.3.0 clause 5.2.3.2:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp

The values in the equation for Cell 1 NR Srxlev are derived as follows:

- Srxlev is evaluated at T1 and T2

- Qrxlevmeas is the Measured cell RX level value (SS-RSRP) as given in table A.11.1.4.2.2-3

- Qrxlevmin for the NR cell is -137dBm/30kHz as given in tables A.11.1.4.2.2-3

- Qrxlevminoffset is only taken into account as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, and is therefore not applicable here.

- Pcompensation = 0 according to the table in TS 38.304 clause 5.2.3.2.

- Qoffsettemp is the additional offset to be used for cell selection and re-selection, the value is set to 0 in this test case.

The equation therefore reduces to:

Srxlev = Qrxlevmeas – (Qrxlevmin) which is drawn for each cell at T1 and T2 in the diagram above.

Cell 1 at T1 has RSRP -99dBm/30kHz, so Srxlev = -99 – (-137) = 38.

Cell 1 at T2 has RSRP -83dBm/30kHz, so Srxlev = -83 – (-137) = 54.

3.2 Selection criterion for E-UTRA

For cell 2 (E-UTRA), the selection criterion is satisfied according to the equation from TS 36.304 clause 5.2.3.2:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp

The values in the equation for Cell 2 EUTRA Srxlev are derived as follows:

- Srxlev is evaluated for cell 2 at T1 and T2.

- Qrxlevmeas is the Measured cell RX level value (RSRP) as given in table A.11.1.4.2.2-4.

- Qrxlevmin for EUTRA neighbour cell is -140dBm as given in A.11.1.4.2.2-4.

- Qrxlevminoffset is only taken into account as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, and is therefore not applicable here.

- Pcompensation =0 according to the table in TS 36.304 v18.2.0 clause 5.2.3.2.

- Qoffsettemp is the additional offset to be used for cell selection and re-selection, the value is set to 0 in this test case.

The equation therefore reduces to:

Srxlev = Qrxlevmeas – (Qrxlevmin) which is drawn for cell 2 at T1 and T2 in the diagram above.

Cell 2 at T1, T2 and T3 has RSRP -84dBm/15kHz so Srxlev = -84 – (-140) = 56

3.3 Thresholds

The thresholds for NR are defined in TS 38.304 v15.3.0 clause 5.2.4.7:

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for NR inter-frequency and inter-RAT measurements.

More detail about SnonIntraSearchP is given in TS 38.133 clause 4.2A.2.5:

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The thresholds for EUTRA are defined in TS 36.304 clause 5.2.4.7:

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

3.4 Test case verdict

The design of the test case relies on the UE being able to measure RSRP during T1 and T2, and correctly compare them to the signalled thresholds.

A detailed analysis of the test case is given in section 4 of this document.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.11.1.4.2.2-3 and A.11.1.4.2.2-4 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Some signalled parameters Qrxlevmin, SnonIntraSearch, ThreshX, HighP, ThreshServing, LowP, and ThreshX, LowP are also copied, although these are not subject to uncertainty.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet. The UE compares Srxlev values to thresholds, so the RSRP values need to be converted to Srxlev.

In this test case the UE compares Srxlev values to thresholds, so a further step is done to calculate the difference between Srxlev and the thresholds:

- (Cell 1 Srxlev - Thresh_Serving_LowP)

- (Cell 1 Srxlev - Thresh_X_HighP)

- (Cell 2 Srxlev - Thresh_X_LowP)

c) Uncertainties

The SS provides AWGN, 1 E-UTRA Cell and 1 NR intra-frequency cell to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB

- Ratio of NR Cell signal / noise, Ês1 / Noc ±0.3 dB

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB

- Ratio of E-UTRA Cell signal / noise, Ês2 / Noc ±0.3 dB

This choice of each cell forms a minimum set, so the superposition principle can be applied if necessary.

For inter-frequency reselection, the UE ranking requirement points to the side conditions for reselection measurements in the following extract from TS 38.133:

#### 4.2.2.5	Measurements of inter-RAT E-UTRAN cells

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NEUTRA_carrier is the total number of configured E-UTRA carriers in the neighbour frequency list. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN/2.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 within (NEUTRA_carrier) * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every (NEUTRA_carrier) * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within (NEUTRA_carrier) * Tevaluate,EUTRAN when Treselection = 0 as speficied in table 4.2.2.5-1 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

<< clauses skipped >>

In TS 38.133, clause B.1.9 for Inter-frequency refers to Table B.1.8-1 Intra-frequency.

## B.1.8	Conditions for measurements on NR intra-frequency cells for cell re-selection under CCA

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.8-1 for FR1 NR cells under CCA.

Table B.1.8-1: Conditions for intra-frequency cell re-selection in FR1 under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -120 | -117 | -4 |
|  | NR_CCA_FR1_J | -119.5 | -116.5 |  |
| NOTE 1: NR operating band groups are as defined in clause 3A.4.1. |  |  |  |  |

..and from TS 36.133:

##### 4.2.2.5.6	Measurements of NR cells

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT NR layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT NR layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT NR layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT NR measurements. When the measurement rules indicate that inter-RAT NR cells are to be measured, the UE shall measure SS-RSRP and SS-RSRQ of detected NR cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NNR_carrier is the total number of configured NR carriers in the neighbour frequency list. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured NR cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

The UE shall be able to evaluate whether a newly detectable inter-RAT NR cell meets the reselection criteria defined in TS 36.304 [1] within (NNR_carrier) * TdetectNR when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or [6.5] dB in FR2 for reselections based on ranking or 6 dB in FR1 or [7.5] dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and [4] dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT NR cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

Cells which have been detected shall be measured at least every (NNR_carrier) * TmeasureNR when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT NR cell has met reselection criterion defined in TS 36.304 [1] within (NNR_carrier) * TevaluateNR when Treselection = 0 as specified in Table 4.2.2.5.6-1 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or [6.5] dB in FR2 for reselections based on ranking or 6 dB in FR1 or [7.5] dB in FR2 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 and [4] in FR2 for SS-RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT NR cell is satisfied with the reselection criteria which are defined in TS 36.304 [1], the UE shall evaluate this NR cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.5.6-1: Tdetect,NR, TmeasureNR, and Tevaluate,NR

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x 1.5 x N1 (36 x 1.5 x N1) | 1.28 x 1.5 x N1 (4 x 1.5 x N1) | 5.12 x 1.5 x N1 (16 x 1.5 x N1) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. |  |  |  |  |  |

<< clauses skipped >>

In TS 36.133, clause B.1.2 for Inter-frequency refers to Table B.1.1-1 Intra-frequency.

## B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

This clause defines the E-UTRAN intra-frequency RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection are defined in Table B.1.1-1.

Table B.1.1-1: Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection

| Parameter | E-UTRA operating band groups Note 3 | Minimum RSRP Note 1 | Minimum SCH_RP Note 1 | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | FDD_A, TDD_A | -124 | -124 | -4 | -4 |
|  | FDD_B1, FDD_B2 | -123.5 | -123.5 |  |  |
|  | FDD_C, TDD_C | -123 | -123 |  |  |
|  | FDD_D | -122.5 | -122.5 |  |  |
|  | FDD_E, TDD_E | -122 | -122 |  |  |
|  | FDD_F | -121.5 Note 2 | -121.5 Note 2 |  |  |
|  | FDD_G | -121 | -121 |  |  |
|  | FDD_H | -120.5 | -120.5 |  |  |
|  | FDD_N | -117.5 | -117.5 |  |  |
| NOTE 1: This condition level is increased by ∆>0, when applicable, as described in Section B.4.2. NOTE 2: The condition is -122 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

### A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss

For a UE supporting inter-band carrier aggregation configuration with uplink in one E-UTRA band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 36.101 [5], 7.3.1-1A, there is no adjustment of test parameters in the tests specified in TS 38.133 when ΔRIB,c  1 dB.

In summary the SSB Es/Iot of NR Cell 1 and the RSRP Es/Iot of E-UTRAN Cell 2 shall be above -4dB, the cell should have at least 6 dB better ranked and the tightest requirement for SSB_RP shall be above -116.5dBm/30kHz for band group NR_CCA_FR1_J, for RSRP and SCH_RP shall be above -117.5 + 1dB for band group FDD_N.

d) Controlled parameters critical to verdict

A diagram giving the Cell 1 and Cell 2 Srxlev values during T1 and T2 is provided in section 3 of this document. It also includes the relevant thresholds. This is the most appropriate view to analyse the critical parameters for this test.

From TS 38.133 clause A.11.1.4.2.1, Cell 2 (EUTRA) is of higher priority than NR Cell 1.

- Before the start of T1 the UE has selected Cell 1 (low priority).

- During T1 the UE is required to reselect to EUTRA Cell 2 (low priority), The conditions for the reselection towards lower priority are:

- ThreshX, HighP is not applicable during T1 (only applies when reselecting towards higher priority Cell 1)

- The low priority Cell 2 must be good enough, expressed as Srxlev ≥ ThreshX, LowP with a margin of at least 6dB (from TS 38.133 clause 4.2A.2.5)

- The high priority Cell 1 must be bad enough, expressed as Srxlev ≤ ThreshServing, LowP interpreting TS 38.133 clause 4.2A.2.5, to require a margin of at least 6dB.

- Cell 1 must meet the SSB_RP and SSB Es/Iot side conditions in TS 38.133 clause 4.2A.2.5 and Cell 2 must meet the RSRP and SCH side conditions (level, Es/Iot) in TS 36.133 clause 4.2A.2.5.

- During T2 the UE is required to reselect to NR Cell 1 (high priority). The conditions for the reselection towards higher priority are:

- The high priority Cell 1 must be good enough, expressed as Srxlev ≥ ThreshX, HighP, with a margin of at least 6dB (from TS 38.133 clause 4.2A.2.5).

- ThreshX, LowP is not applicable during T2 (only applies when reselecting towards frequency X from a higher priority frequency)

- ThreshServing, LowP is not applicable during T2 (only applies when reselecting towards lower priority frequency)

- Cell 1 must meet the SSB_RP and SSB Es/Iot side conditions in TS 38.133 clause 4.2A.2.5 and Cell 2 must meet the RSRP and SCH side conditions (level, Es/Iot) in TS 36.133 clause 4.2A.2.5.

The critical conditions are related to the 6 controlled parameters listed in section d) of the accompanying spreadsheet. They have been derived by study of the test case diagram and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. The sensitivity factors are the ratio (effect on a critical parameter y / a test equipment uncertainty x), and are usually in dB/dB. In this RRM test case for cell 1 (NR), there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero. For cell 2 (EUTRA), there is a simple one-to-one relationship between the parameters that can be set by the test equipment.

For example, for cell 1 an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 SS-RSRP, so the sensitivity factor is 1.000 during T1 and T2. It would also cause 1dB error in the derived parameter (Cell 1 Srxlev - Thresh_X_HighP), so the sensitivity factor is also 1.000 during T3. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 1 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

For cell 2, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 2 RSRP, so the sensitivity factor is 1.000 during T2 and T3. It would also cause 1dB error in the derived parameter (Cell 2 Srxlev - Thresh_X_LowP), so the sensitivity factor is also 1.000 during T2. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

The normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

The strategy for determining the parameters to offset is given below.

The order of determining offsets (Test Tolerances) is chosen to derive the minimum values in a logical sequence.

- During T1 NR Cell 1 Ês/Iot just meeting the core requirement of Ês/Iot   -4 dB in TS 38.133 clause 4.2A.2.5. The Es/Noc of cell 1 is therefore increased by the amount of its uncertainty, to ensure it does not fall below the minimum value of -4dB. Also, the Srxlev of high priority Cell 1 is set to be below ThreshServing, LowP with a margin of 6dB. The Noc1 of cell 1 is therefore decreased by the (Cell 1 Srxlev - Thresh_Serving_LowP) uncertainty. This ensures that the margin of 6dB is maintained.

- During T1 the Srxlev of low priority EUTRA Cell 2 is set to be above ThreshX, LowP with a margin of 6dB, just meeting the core requirement of  6 dB in TS 36.133 clause 4.2A.2.5. To maintain the 6dB margin the Es/Ioc of cell 2 is increased by the (Cell 2 Srxlev - Thresh_X_LowP) uncertainty. This ensures that the margin of 6dB is maintained. The Ês/Iot   -4 dB side condition in TS 36.133 clause 4.2A.2.5 is easily met by Cell 1.

- During T2, the Srxlev of high priority NR Cell 1 is set to be above ThreshX, HighP with a margin of 6dB. To maintain the 6dB margin the Es/Noc of cell 1 is increased by the (Cell 1 Srxlev - Thresh_X_HighP) uncertainty. The Ês/Iot   -4 dB side condition in TS 38.133 clause 4.2A.2.5 is easily met by Cell 1.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

For Cell 1 during T1, The Es/Noc of cell 1 is increased by the amount of its uncertainty, to ensure it does not fall below the minimum value of -4dB. The actual offset required is 0.4 dB. Also, the Noc1 of cell 1 is tdecreased by the (Cell 1 Srxlev - Thresh_Serving_LowP) uncertainty. This ensures that the margin of 6dB is maintained. The actual offset required is -2 dB. The values are found empirically by observing the minimum/maximum parameter values in step g).

For Cell 1 during T2, to maintain the 6dB margin, the Es/Noc of cell 1 is increased by the (Cell 1 Srxlev - Thresh_X_HighP) uncertainty. The actual offset required is 1.6 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

For Cell 2 during T1, to maintain the 6dB margin, the Es/Ioc of cell 2 is increased by the (Cell 2 Srxlev - Thresh_X_LowP) uncertainty. The actual offset required is 1.6 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Required margin relative to threshold, Es/Iot range, RSRP level.). It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 11.1.4.2 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- NR Cell 1 SSB_RP and E-UTRAN Cell 2 RSRP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference