3GPP TSG-RAN5 Meeting #105	R5-247014r1

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3083 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test requirement in REFSENS for 2DL CA testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The REFSENS test requirement for 8 Rx antenna port need to be updated in test case 7.3A.1 and 7.3A.1_1. For reducing the burden of maintenance in the future, the format of table 7.3A.1_1.5-1 and 7.3A.1_1.5-1a could be updated. And NOTE for 8Rx testing is updated in test configuration tables |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating notes in test configuration tables for UE supporting 8Rx - Updating test requirement in 7.3A.1.5 for 8Rx antenna ports. - Updating the format of table 7.3A.1_1.5-1 and Table 7.3A.1_1.5-1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirement of REFSENS for 8Rx is absent for 2DL CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1, 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8Rx are introduced in R5-247012 (CR 3081) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: remove unnecessary words after <End of Change> in this CR Merge changes to table 7.3A.1_1.5-1 in R5-246276 from Nokia Merge changes to table 7.3A.1_1.5-1 in R5-246136 from KDDI Merge changes to table 7.3A.1_1.5-1 in R5-247383 from ZTE As per comments, using “symbol ‘REFSENS2Rx’ + MSD” value as the test requirements for victim bands for easier maintenance in the future. And keep the original numbers of notes in test requirement tables, meanwhile VOID unnecessary notes and introducing new notes below them. |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 7.3A.1	Reference sensitivity power level for 2DL CA without exception

### <Unchanged Skipped>

7.3A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1.4.1-1: Test Configuration Table for intra-band contiguous 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters CA Configurations |  |  |  |  |  |  |
| CA Configuration /NRB |  | DL Allocation |  | UL Allocation |  |  |
| PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| Lowest NRB_agg (NOTE 4) | Lowest NRB_agg (NOTE 4) | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | - |
| Highest NRB_agg (NOTE 4) | Highest NRB_agg (NOTE 4) | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. Note 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 4: In CA_n66B configuration with the same NRB_agg CC combination, PCC shall be selected as the lower CH BW Note 5: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. |  |  |  |  |  |  |

Table 7.3A.1.4.1-2: Test Configuration Table for inter-band 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Mid range for PCC and SCC with exceptions for CA configurations containing the following band combinations (Note 8): CA_n1-n77: Mid in band n1 and Low in band n77 CA_n2-n77: UL and DL Mid in band n2 and band n77 at 3850MHz CA_n3-n77: Mid in band 3 and High in band 77. CA_n8-nX: Low range for PCC in Band 8 CA_n14-n77: Mid in band n14 and Low in band n77 CA_n70-n71: High range for PCC in band 71. CA_n3-n78: Mid in band 3 and High in band 78. CA_n5-n78: Mid in band 5 and High in band 78 CA_n29-n71: Low in band 29 and High in band 71. CA_n41-n71: Low in band 41 and Low in band 71. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  | PCC NRB | SCC NRB | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | nY | default | Highest (Note 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | nX | default | Highest (Note 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the PCC bands and PCC NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 5: For band combinations including operating band without uplink band (as noted in Table 5.2-1), only the CA configuration where PCC band has uplink band shall be tested. Note 6: Each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.1. DL channel bandwidth shall be selected first. Note 7: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 8: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. Note 9: CA_n29A-n71A testpoint 2, is tested according to reference sensitivity levels specified in Clause 7.3A.1_1.5 due to cross band isolation exception specified in Table 7.3A.0.6-3. Testpoint 1 is not applicable due to SDL. Note 10: CA_n41A-n66A, testpoint 1, is tested according to reference sensitivity levels specified in Clause 7.3A.1_1.5 due to cross band isolation exception specified in Table 7.3A.0.6-3. Testpoint 2 (UL on n66) shall be tested in this testcase. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1.4.1-3: Test Configuration Table for intra-band non-contiguous 2DL CA without exception

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  |  | PCC | Wgap / [MHz] | SCC |  | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 |  | nX | CC2 | Highest | Max (NOTE 4) | Lowest |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nX | CC1 |  | nX | CC2 | Highest NRB_agg (NOTE 5) | Max (NOTE 4) | Highest NRB_agg (NOTE 5) |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Test Settings for a CA_n2(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | CC1 |  | n2 | CC2 | 5MHz | 30.0 (NOTE 7) | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 25@0 | - |
| Test Settings for a CA_n71(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | CC1 |  | n71 | CC2 | 5MHz | 5.0 (NOTE 8) | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 20@0 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration Note 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 6: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected Note 7: The Wgap is centered in the middle of the band. Specific frequency parameters are defined in Table 7.3A.1.4.1-3a. Note 8: The Wgap is centered in the middle of the band. Specific frequency parameters are defined in Table 7.3A.1.4.1-3b. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Skipped>

7.3A.1.5	Test requirement

For 2DL carrier aggregation, test parameters are specified in table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. For the CA configurations listed in table 7.3A.1.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1a and table 7.3.2.5-1b for non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2a and 7.3.2.5-2b for non-SDL carrier for 4 Rx antenna port, in table 7.3.2.5-2e and table 7.3.2.5-2f for non-SDL carrier for 8 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous 2 DL CA, the test requirement for shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1 for the SCC. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
|  | CA_n5B | - |
|  | CA_n41C | - |
|  | CA_n48B | - |
|  | CA_n48C |  |
| Intra-band contiguous 2DL CA | CA_n66B | - |
|  | CA_n77C | - |
|  | CA_n78B | - |
|  | CA_n78C | - |
|  | CA_n79C |  |
|  | CA_n48(2A) | - |
|  | CA_n2(2A) | - |
|  | CA_n66(2A) | - |
| Intra-band non-contiguous 2DL CA | CA_n77(2A) | - |
|  | CA_n78(2A) | - |
|  | CA_n71(2A) | - |
|  | CA_n1A-n3A |  |
|  | CA_n1A-n8A | - |
|  | CA_n1A-n28A | - |
|  | CA_n1A-n41A |  |
|  | CA_n1A-n77A | - |
|  | CA_n1A-n78A | - |
|  | CA_n2A-n5A |  |
|  | CA_n2A-n14A |  |
|  | CA_n2A-n30A | - |
|  | CA_n2A-n48A | - |
|  | CA_n2A-n66A | - |
|  | CA_n2A-n77A | - |
|  | CA_n3A-n5A |  |
|  | CA_n3A-n8A | - |
|  | CA_n3A-n28A | - |
|  | CA_n3A-n41A |  |
|  | CA-n3A-n77A | - |
| Inter-band 2DL CA | CA_n3A-n78A | - |
|  | CA_n5A-n30A | - |
|  | CA_n5A-n48A |  |
|  | CA_n5A-n66A | - |
|  | CA_n5A-n77A | - |
|  | CA_n5A-n78A |  |
|  | CA_n8A-n78A | - |
|  | CA_n14A-n30A |  |
|  | CA_n14A-n66A |  |
|  | CA_n14A-n77A |  |
|  | CA_n20A-n78A | - |
|  | CA_n24A-n41A |  |
|  | CA_n24A-n48A |  |
|  | CA_n24A-n77A |  |
|  | CA_n25A-n77A | - |
|  | CA_n25A-n78A | - |
|  | CA_n28A-n40A | - |
|  | CA_n28A-n41A | - |
|  | CA_n28A-n77A | - |
|  | CA_n28A-n78A | - |
|  | CA_n28A-n79A | - |
|  | CA_n30A-n66A | - |
|  | CA_n30A-n77A | - |
|  | CA_n39A-n41A |  |
|  | CA_n41A-n66A (Note 1) | - |
|  | CA_n41A-n71A | - |
|  | CA_n41A-n77A |  |
|  | CA_n41A-n79A | - |
|  | CA_n48A-n77A | - |
|  | CA_n66A-n70A | - |
|  | CA_n66A-n71A | - |
|  | CA_n66A-n77A |  |
|  | CA_n66A-n78A | - |
|  | CA_n70A-n71A | - |
|  | CA_n71A-n77A | - |
|  | CA_n71A-n78A | - |
| SDL configuration | CA_n29A-n66A | - |
|  | CA_n29A-n70A | - |
|  | CA_n29A-n71A (NOTE 1) | - |
| Note 1: Configuration has sensitivity exceptions according to notes in table 7.3A.1.4.1-2. Other Rx cases shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1 Note 2: CA_n41A-66A testpoint 1, reference sensitivity requirement is tested in Clause 7.3A.1_1 due to cross band isolation exception specified in Table 7.3A.0.6-3. Other Rx cases using configuration for testpoint 1 shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1.5. |  |  |

Table 7.3A.1.5-2: Reference sensitivity for SDL bands

| NR band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n29 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, Table 7.3.2.4.1-3, Table 7.3.2.5-1a, Table 7.3.2.5-1b, Table 7.3.2.5-2a, Table 7.3.2.5-2b, Table 7.3.2.5-2c, Table 7.3.2.5-2d, Table 7.3.2.5-2e and Table 7.3.2.5-2f and Table 7.3A.1.4-1 with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.0.2.2-1 for the SCC(s). For aggregation of two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.0.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

For band combinations including operating bands without uplink band (as noted in Table 5.2-1), the requirements are specified in Table 7.3A.1.5-1 and for any band with uplink the uplink configuration specified in Table 7.3.2.4.1-2. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels, as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one-sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal, as described in Annex A.5.1.1/A.5.2.1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

### <Unchanged Skipped>

### 7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A IMD2 and IMD4 is currently missing in TR 38.905.

### <Unchanged Skipped>

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC”and “SCC” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n3 | 1760 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | Low | n3 | High | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n1 | Low | n3 | High | 50MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1965 MHz (UL) | n8 | 887.5 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 2139 MHz | n28 | 713 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n1 | 2139 MHz | n28 | 713 MHz (UL) | 50 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n1A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1955 MHz (UL) | n41 | 2501 MHz | 50 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 2 | n1 | 1970 MHz (UL) | n41 | 2546 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n41 | 2546 MHz | n1 | 2167.5 MHz (UL) | 30 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Mid | n77 | 3900 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n1 | Mid | n77 | 3900 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n1 | Mid | n77 | 3875 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n78 | 3710 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1860 MHz (UL) | n48 | 3690 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | UL 1852.5/DL 1932.5 | n48 | 3625 MHz | 5 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/DL 1935 | n66 | UL 1775/DL 2175 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n2 | UL 1883.3/DL 1963.3 | n66 | UL 1750/DL 2150 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1880 MHz (UL) | n77 | 3760 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | 1880 MHz (UL) | n77 | 3760 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n2 | 1880 MHz (UL) | n77 | 3735 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n77 | 3920 MHz | n2 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 5 | n77 | 3920 MHz | n2 | DL Mid | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 6 | n2 | UL 1855/DL 1935 | n77 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 7 | n2 | UL 1900/DL 1980 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 8 | n2 | UL 1885/DL 1965 | n77 | 3810 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 95 | n2 | 1987.5 MHz (DL) | n77 | 3305 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_4 |
| Test Settings for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | TBD | n77 | TBD | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n3 | 1721 MHz (UL) | n5 | 838 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1755MHz | n8 | 900MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | Mid | n8 | Mid | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1740 MHz (UL) | n41 | 2657.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n41 | 2546 MHz (UL) | n3 | 1877.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n77 | 3495 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n77 | 3495 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | Mid | n77 | 3470 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n3 | 1740 MHz (UL) | n77 | 3575 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n3 | 1765 MHz (UL) | n77 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 65 | n77 | 3685 MHz | n3 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 75 | n77 | 3685 MHz | n3 | DL Mid | 40 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n78 | 3495 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n78 | 3495 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | Mid | n78 | 3470 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n3 | 1740 MHz (UL) | n78 | 3575 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n3 | 1765 MHz (UL) | n78 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 65 | n78 | 3685 MHz | n3 | DL Mid | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 75 | n78 | 3685 MHz | n3 | DL Mid | 40 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for a CA_n5A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 838/DL 883 | n66 | UL 1721/DL 2121 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for a CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | 836.5 MHz (UL) | n77 | 3346 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | 846.5 MHz (UL) | n77 | 3386 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n5 | 826.5 MHz (UL) | n77 | 4132.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n5 | 826.5 MHz (UL) | n77 | 4132.5 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 5 | n77 | 3526 MHz | n5 | DL Mid | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 6 | n77 | 3526 MHz | n5 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 7 | n5 | UL 844/ DL 889 | n77 | 3421 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 8 | n5 | UL 829/ DL 874 | n77 | 4190 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | Mid | n78 | 3346 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | 846.5 MHz (UL) | n78 | 3386 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n7A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3350 MHz | n7 | 2687.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n14A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | Mid | n77 | 3965 MHz | 5 Mhz | 10 Mhz | CP-OFDM QPSK | 10@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n14 | Mid | n77 | 3965 MHz | 5 Mhz | 100 Mhz | CP-OFDM QPSK | 20@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n14 | 793 MHz (UL) | n77 | 3935 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 4 | n77 | 3815 MHz | n14 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | 25@0 |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n77 | 3815 MHz | n14 | Mid | 10 MHz | 10 MHz | CP-OFDM QPSK | 50@0 |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| Test Settings for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n20 | Mid | n78 | High | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n20 | Mid | n78 | 3388 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 3 | n20 | 850 MHz (UL) | n78 | 3359 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 |  |
| Test Settings for CA_n25A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | 1882.5 MHz (UL) | n77 | 3765 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n25 | 1882.5 MHz (UL) | n77 | 3765 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n25 | 1852.5 MHz (UL) | n77 | 3680 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n25 | 1962.5 MHz | n77 | 3925 MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n25 | 1962.5 MHz | n77 | 3925 MHz (UL) | 40 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| Test Settings for CA_n25A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | 1882.5 MHz (UL) | n78 | 3765 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n25 | 1882.5 MHz (UL) | n78 | 3765 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n25 | 1852.5 MHz (UL) | n78 | 3680 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1721 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | 1707.5 MHz (UL) / 2007.5 MHz (DL) | n26 | 831 MHz (UL) / 876 MHz (DL) | 5 MHz UL / 5 MHz DL | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n28A-n40A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | 705.5 MHz (UL) | n77 | 3527.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n28 | 705.5 MHz (UL) | n77 | 3527.5 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 3 | n28 | 705.5 MHz (UL) | n77 | 3587.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | 705.5 MHz (UL) | n78 | 3527.5 MHz | 5 MHz | 10MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n28 | 705.5 MHz (UL) | n78 | 3527.5 MHz | 5 MHz | 100MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n28 | 705.5 MHz | n78 | 3527.5 MHz (UL) | 5 MHz | 10MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| Test Settings for CA_n29A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | High | n29 | Low | 20 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| Test Settings for CA_n30A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | 2310 MHz (UL) | n66 | 2197.5 MHz | 10 MHz | 15 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| Test Settings for CA_n30A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n77 | 3532.5 MHz | n30 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK |  | REFSENS_CA_2 |
| 2 | n77 | 3487.5 MHz | n30 | Mid | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK |  | REFSENS_CA_2 |
| 3 | n30 | Mid | n77 | 3487.5 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | 1912.5 MHz | n41 | 2550 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 2 | n39 | 1912.5 MHz | n41 | 2550 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 3 | n41 | 2530 MHz | n39 | 1897.5 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 4 | n41 | 2530 MHz | n39 | 1897.5 MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 5 | n39 | 1900 MHz | n41 | 2501 MHz | 40 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| 6 | n41 | 2546 MHz | n39 | 1917.5 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for a CA_n41A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2546 MHz | n66 | 2197.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n41A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | Low | n41 | 2662 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | Low | n41 | High | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n77 | 3960 MHz | n41 | 2640 MHz | 20 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n77 | 3960 MHz | n41 | 2640 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 36 | n41 | 2545 MHz | n77 | 3305 MHz | 60 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 46 | n41 | 2625 MHz | n77 | 3305 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n41 | 2640 MHz | n77 | 3305 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 6 | n77 | 3350 MHz | n41 | 2685 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 7 | n77 | 3350 MHz | n41 | 2640 MHz | 100 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 85 | n41 | 2545 MHz | n77 | 3305 MHz | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 95 | n41 | 2625 MHz | n77 | 3305 MHz | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | n41 | 2640 MHz | n79 | 4405 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 25 | n79 | 4450 MHz | n41 | 2685 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1730 MHz (UL) | n48 | 3660 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n66 | High | n48 | 3555 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | High | n48 | Low | 40 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | 1777.5 MHz | n48 | 3580 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3695 MHz (UL) | n70 | 1697.5 MHz (UL) | 10 MHz | 15 MHz UL / 25 MHz DL | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n71 | 675 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | 1712.5 MHz (UL) | n77 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | UL 1775/ DL 2175 | n77 | 3950 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n66 | UL 1760/ DL 2160 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n78 | 3500 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1750 MHz (UL) | n78 | 3500 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | 1712.5 MHz (UL) | n78 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |

| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n71 | Low | n70 | Low | 5 MHz | 25 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | Low | n70 | Low | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n70 | 1697.5 MHz (UL) | n71 | 695.5 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | 671 MHz (UL) | n77 | 3309 MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n71A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | 680.5 MHz (UL) | n78 | 3402.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | 681.5 MHz (UL) | n78 | 3361.5MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n78A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3750 | n79 | 4420 | 100 | 40 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 2 | n79 | 4450 | n78 | 3795 | 100 | 10 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n79 | 4450 | n78 | 3750 | 100 | 100 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 45 | n78 | 3750 | n79 | 4450 | 100 | 100 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3A.0.4-1. REFSENS_CA_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3A.0.4-4b for PC3 and Table 7.3A.0.4-4c for PC2. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference due to 2UL CA according to table 7.3A.0.5-1 for PC3 and table 7.3A.0.5-1a for PC2. REFSENS_CA_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation for NR CA FR1 according to table 7.3A.0.6-1 for PC3 and Table 7.3A.0.6-3a for PC2. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2A.4.1-3 of TS 38.508-2. Note 5: This test ID is for UL PC2 only. Note 6: This test ID is for UL PC3 only. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1_1.4.1-2: Test Configuration Table for intra-band non-contiguous 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  |  | PCC | Wgap / [MHz] | SCC |  | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n71(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | CC1 |  | n71 | CC2 | 5MHz | 25.0 | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@0 | - |
| 2 | n71 | CC1 |  | n71 | CC2 | 15MHz | 10.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@2 | - |
| 3 | n71 | CC1 |  | n71 | CC2 | 15MHz | 5.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 20@19 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions according to table 7.3A.0.2.2-1 Note 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration Note 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 6: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Skipped>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 9, Note 10) |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | 1 | n1 | 5 | REFSENS2Rx +23 |
|  |  | n3 | 5 | REFSENS |
|  | 2 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS2Rx +3 |
|  | 3 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS2Rx +19.7 |
| CA_n1A-n8A | 1 | n1 | 5 | REFSENS2Rx +6 |
|  |  | n8 | 5 | REFSENS |
| CA_n1A-n28A | 1 | n1 | 5 | REFSENS2Rx +10.2 |
|  |  | n28 | 5 | REFSENS |
|  | 2 | n1 | 50 | REFSENS2Rx +1.1 |
|  |  | n28 | 5 | REFSENS |
| CA_n1A-n41A | 1 | n1 | 50 | REFSENS |
|  |  | n41 | 10 | REFSENS2Rx +6.1 |
|  | 2 | n1 | 20 | REFSENS |
|  |  | n41 | 100 | REFSENS2Rx +0.7 |
|  | 3 | n1 | 5 | REFSENS2Rx +18.1 |
|  |  | n41 | 100 | REFSENS |
| CA_n1A-n77A | 1 | n1 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +27.1 |
|  | 2 | n1 | 20 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n1 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +1.9 |
| CA_n1A-n78A | 1 | n1 | 5 | REFSENS2Rx +8 |
|  |  | n78 | 10 | REFSENS |
| CA_n2A-n48A | 1 | n2 | 5 | REFSENS |
|  |  | n48 | 10 | REFSENS2Rx +1.9 |
|  | 2 | n2 | 5 | REFSENS2Rx +12 |
|  |  | n48 | 20 | REFSENS |
| CA_n2A-n66A | 1 | n2 | 5 | REFSENS2Rx +20 |
|  |  | n66 | 5 | REFSENS |
|  | 2 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +4 |
| CA_n2A-n77A | 1 | n2 | 5 | REFSENS |
|  |  | n775 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n2 | 10 | REFSENS |
|  |  | n775 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n2 | 5 | REFSENS |
|  |  | n775 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n2 | 5 | REFSENS2Rx +6.7 |
|  |  | n775 | 10 | REFSENS |
|  | 5 | n2 | 20 | REFSENS2Rx +3.7 |
|  |  | n775 | 20 | REFSENS |
|  | 6 | n2 | 5 | REFSENS2Rx +26 |
|  |  | n775 | 10 | REFSENS |
|  | 7 | n2 | 5 | REFSENS2Rx +8 |
|  |  | n775 | 10 | REFSENS |
|  | 8 | n2 | 5 | REFSENS2Rx +5 |
|  |  | n775 | 10 | REFSENS |
| CA_n3A-n8A | 1 | n3 | 10 | REFSENS |
|  |  | n8 | 5 | REFSENS2Rx +8 |
|  | 2 | n3 | 10 | REFSENS2Rx +6.4 |
|  |  | n8 | 5 | REFSENS |
| CA_n3A-n41A | 1 | n3 | 5 | REFSENS2Rx +8.2 |
|  |  | n41 | 10 | REFSENS |
|  | 2 | n3 | 5 | REFSENS2Rx +0.6 |
|  |  | n41 | 100 | REFSENS |
| CA_n3A-n77A | 1 | n3 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n3 | 10 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n3 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n3 | 5 | REFSENS2Rx +26.0 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n3 | 5 | REFSENS2Rx +8.0 |
|  |  | n77 | 10 | REFSENS |
| CA_n3A-n78A | 1 | n3 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n3 | 10 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n3 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n3 | 5 | REFSENS2Rx +26 |
|  |  | n78 | 10 | REFSENS |
|  | 5 | n3 | 5 | REFSENS2Rx +8 |
|  |  | n78 | 10 | REFSENS |
| CA_n5A-n66A | 1 | n5 | 5 | REFSENS2Rx +30 |
|  |  | n66 | 5 | REFSENS |
| CA_n5A-n77A | 1 | n5 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +10.5 |
|  | 2 | n5 | 5 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +1.4 |
|  | 3 | n5 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +10.4 |
|  | 4 | n5 | 5 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +0.7 |
|  | 5 | n5 | 20 | REFSENS2Rx +2.7 |
|  |  | n77 | 20 | REFSENS |
|  | 6 | n5 | 5 | REFSENS2Rx +5.7 |
|  |  | n77 | 10 | REFSENS |
|  | 7 | n5 | 5 | REFSENS2Rx +8.3 |
|  |  | n77 | 10 | REFSENS |
|  | 8 | n5 | 5 | REFSENS2Rx +5.5 |
|  |  | n77 | 10 | REFSENS |
| CA_n5A-n78A | 1 | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +10.5 |
|  | 2 | n5 | 5 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +1.4 |
|  | 3 | n5 | 5 | REFSENS2Rx +8.3 |
|  |  | n78 | 10 | REFSENS |
| CA_n7A-n78A | 1 | n7 | 5 | REFSENS2Rx +4.5 |
|  |  | n78 | 100 | REFSENS |
| CA_n8A-n78A | 1 | n8 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +10.8 |
|  | 2 | n8 | 5 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +1.4 |
|  | 3 | n8 | 5 | REFSENS2Rx +8.3 |
|  |  | n78 | 10 | REFSENS |
| CA_n14A-n77A | 1 | n14 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +10.4 |
|  | 2 | n14 | 5 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +0.7 |
|  | 3 | n14 | 5 | REFSENS2Rx + 5.5 |
|  |  | n77 | 10 | REFSENS |
|  | 4 | n14 | 5 | REFSENS2Rx + 31 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n14 | 10 | REFSENS2Rx + 28 |
|  |  | n77 | 10 | REFSENS |
| CA_n20A-n78A | 1 | n20 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +10.8 |
|  | 2 | n20 | 5 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +1.4 |
|  | 3 | n20 | 5 | REFSENS2Rx + 11 |
|  |  | n78 | 10 | REFSENS |
| CA_n25A-n66A | 1 | n66 | 5 | REFSENS |
|  |  | n25 | 5 | REFSENS2Rx +20 |
|  | 2 | n66 | 5 | REFSENS2Rx +23 |
|  |  | n25 | 5 | REFSENS |
|  | 3 | n66 | 5 | REFSENS2Rx +4 |
|  |  | n25 | 5 | REFSENS |
| CA_n25A-n77A | 1 | n25 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n25 | 10 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n25 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n25 | 5 | REFSENS2Rx +6.7 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n25 | 40 | REFSENS2Rx +1.1 |
|  |  | n77 | 20 | REFSENS |
|  | 6 | n25 | 5 | -96.5+26+TT |
|  |  | n77 | 10 | REFSENS |
|  | 7 | n25 | 5 | -96.5+8+TT |
|  |  | n77 | 10 | REFSENS |
|  | 8 | n25 | 5 | -96.5+5+TT |
|  |  | n77 | 10 | REFSENS |
| CA_n25A-n78A | 1 | n25 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n25 | 10 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n25 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n25 | 5 | -96.5+26+TT |
|  |  | n78 | 10 | REFSENS |
| CA_n26A-n66A | 1 | n26 | 5 | REFSENS2Rx + 30 |
|  |  | n66 | 5 | REFSENS |
| CA_n26A-n70A | 1 | n26 | 5 | REFSENS2Rx + 30 |
|  |  | n70 | 5 | REFSENS |
| CA_n28A-n40A | 1 | n28 | 5 | REFSENS2Rx +37.8 |
|  |  | n40 | 5 | REFSENS |
|  | 2 | n28 | 20 | REFSENS2Rx +30.3 |
|  |  | n40 | 20 | REFSENS |
| CA_n28A-n77A | 1 | n28 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +10.4 |
|  | 2 | n28 | 5 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +0.7 |
|  | 3 | n28 | 5 | REFSENS2Rx +0.7 |
|  |  | n77 | 10 | REFSENS |
| CA_n28A-n78A | 1 | n28 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +10.4 |
|  | 2 | n28 | 5 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +0.7 |
|  | 3 | n28 | 5 | REFSENS2Rx + 31 |
|  |  | n78 | 10 | REFSENS |
| CA_n29A-n71A | 1 | n29 | 5 | REFSENS2Rx + 17.5 |
|  |  | n71 | 20 | REFSENS |
| CA_n30A-n66A | 1 | n30 | 10 | REFSENS |
|  |  | n66 | 15 | REFSENS2Rx + 8.3 |
| CA_n30A-n77A | 1 | n30 | 5 | REFSENS2Rx + 10.4 |
|  |  | n77 | 10 | REFSENS |
|  | 2 | n30 | 10 | REFSENS2Rx + 8 |
|  |  | n77 | 10 | REFSENS |
|  | 3 | n30 | 5 | REFSENS2Rx + 8 |
|  |  | n77 | 10 | REFSENS |
| CA_n39A-n41A | 1 | n39 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS2Rx +8.1 |
|  | 2 | n39 | 5 | REFSENS |
|  |  | n41 | 100 | REFSENS2Rx +3.3 |
|  | 3 | n39 | 5 | REFSENS2Rx +4.3 |
|  |  | n41 | 10 | REFSENS |
|  | 4 | n39 | 40 | REFSENS2Rx + 0.8 |
|  |  | n41 | 10 | REFSENS |
|  | 5 | n39 | 40 | REFSENS |
|  |  | n41 | 10 | REFSENS2Rx +3.3 |
|  | 6 | n39 | 5 | REFSENS2Rx +1.6 |
|  |  | n41 | 100 | REFSENS |
| CA_n40A-n77A | 1 | n40 | 10 | REFSENS2Rx +10.4 |
|  |  | n77 | 20 | REFSENS |
|  | 2 | n40 | 100 | REFSENS2Rx +0.9 |
|  |  | n77 | 20 | REFSENS |
|  | 3 | n40 | 10 | REFSENS2Rx +4.5 |
|  |  | n77 | 100 | REFSENS |
|  | 4 | n40 | 100 | REFSENS2Rx +4.5 |
|  |  | n77 | 100 | REFSENS |
| CA_n41A-n66A | 1 | n41 | 100 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +10.5 |
| CA_n41A-n71A | 1 | n41 | 10 | REFSENS2Rx + 10.8 |
|  |  | n71 | 5 | REFSENS |
|  | 2 | n41 | 100 | REFSENS2Rx + 1.4 |
|  |  | n71 | 5 | REFSENS |
| CA_n41A-n77A | 1 | n41 | 10 | REFSENS2Rx +10.4 |
|  |  | n77 | 20 | REFSENS |
|  | 2 | n41 | 100 | REFSENS2Rx +6.3 |
|  |  | n77 | 20 | REFSENS |
|  | 3 | n41 | 60 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +2.7 |
|  | 4 | n41 | 100 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +2.7 |
|  | 5 | n41 | 100 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +8.3 |
|  | 6 | n41 | 10 | REFSENS2Rx +4.5 |
|  |  | n77 | 100 | REFSENS |
|  | 7 | n41 | 100 | REFSENS2Rx +4.5 |
|  |  | n77 | 100 | REFSENS |
| CA_n48A-n66A | 1 | n48 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx + 5.0 |
|  | 2 | n48 | 5 | REFSENS2Rx +27.1 |
|  |  | n66 | 5 | REFSENS |
|  | 3 | n48 | 100 | REFSENS2Rx + 13.8 |
|  |  | n66 | 40 | REFSENS |
|  | 4 | n48 | 10 | REFSENS2Rx + 1.9 |
|  |  | n66 | 5 | REFSENS |
| CA_n48A-n70A | 1 | n48 | 10 | REFSENS |
|  |  | n70 | 15 MHz UL /25 MHz DL | REFSENS2Rx +26 |
| CA_n66A-n71A | 1 | n66 | 5 | REFSENS2Rx +5 |
|  |  | n71 | 5 | REFSENS |
| CA_n66A-n77A | 1 | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n66 | 20 | REFSENS |
|  |  | n77 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +1.1 |
|  | 4 | n66 | 5 | REFSENS2Rx +31 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n66 | 5 | REFSENS2Rx +5 |
|  |  | n77 | 10 | REFSENS |
| CA_n66A-n78A | 1 | n66 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +23.9 |
|  | 2 | n66 | 20 | REFSENS |
|  |  | n78 | 100 | REFSENS2Rx +13.8 |
|  | 3 | n66 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +1.1 |
| CA_n70A-n71A | 1 | n70 | 25 | REFSENS2Rx +4.1 |
|  |  | n71 | 5 | REFSENS |
|  | 2 | n70 | 5 | REFSENS2Rx +9.9 |
|  |  | n71 | 5 | REFSENS |
|  | 3 | n70 | 5 | REFSENS2Rx +5 |
|  |  | n71 | 5 | REFSENS |
| CA_n71A-n77A | 1 | n71 | 5 | REFSENS2Rx +5.5 |
|  |  | n77 | 10 | REFSENS |
| CA_n71A-n78A | 1 | n71 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +10.4 |
|  | 2 | n71 | 5 | REFSENS2Rx +5.5 |
|  |  | n78 | 10 | REFSENS |
| CA_n78A-n79A | 1 | n78 | 100 | REFSENS |
|  |  | n79 | 40 | REFSENS2Rx +2 |
|  | 2 | n78 | 10 | REFSENS2Rx +2.6 |
|  |  | n79 | 100 | REFSENS |
|  | 3 | n78 | 100 | REFSENS2Rx +2.6 |
|  |  | n79 | 100 | REFSENS |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: The symbol “REFSENS2Rx” in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b. The requirements in this column are applicable for 2Rx, 4Rx or 8Rx antenna ports NR band in the CA configuration. Note 10: The symbol "REFSENS" in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b for 4 antenna port NR band, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 antenna port NR band. |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 8, Note 9) |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | 1 | n1 | 5 | REFSENS2Rx +17.8 |
|  |  | n78 | 10 | REFSENS |
| CA_n2A-n77A | 4 | n2 | 5 | REFSENS2Rx +9.1 |
|  |  | n775 | 10 | REFSENS |
|  | 5 | n2 | 5 | REFSENS2Rx +6.7 |
|  |  | n775 | 20 | REFSENS |
|  | 6 | n2 | 5 | REFSENS2Rx +32.1 |
|  |  | n775 | 10 | REFSENS |
|  | 7 | n2 | 5 | REFSENS2Rx +19.1 |
|  |  | n775 | 10 | REFSENS |
|  | 9 | n2 | 5 | REFSENS2Rx +1.0 |
|  |  | n775 | 100 | REFSENS |
| CA_n3A-n41A | 1 | n3 | 5 | REFSENS2Rx +18.4 |
|  |  | n41 | 10 | REFSENS |
|  | 2 | n3 | 5 | REFSENS2Rx +2.3 |
|  |  | n41 | 100 | REFSENS |
| CA_n3A-n77A | 4 | n3 | 5 | REFSENS2Rx +31.9 |
|  |  | n775 | 10 | REFSENS |
|  | 5 | n3 | 5 | REFSENS2Rx +18.5 |
|  |  | n775 | 10 | REFSENS |
|  | 6 | n3 | 5 | REFSENS2Rx +8.1 |
|  |  | n775 | 10 | REFSENS |
|  | 7 | n3 | 40 | REFSENS2Rx +1.0 |
|  |  | n775 | 40 | REFSENS |
| CA_n3A-n78A | 6 | n3 | 5 | REFSENS2Rx + 8.1 |
|  |  | n78 | 5 | REFSENS |
|  | 7 | n3 | 40 | REFSENS2Rx + 1.0 |
|  |  | n78 | 40 | REFSENS |
| CA_n5A-n77A | 5 | n5 | 20 | REFSENS2Rx +4.3 |
|  |  | n77 | 20 | REFSENS |
|  | 6 | n5 | 10 | REFSENS2Rx +8.1 |
|  |  | n77 | 10 | REFSENS |
|  | 7 | n5 | 5 | REFSENS2Rx +18.6 |
|  |  | n77 | 10 | REFSENS |
| CA_n14A_n77A | 4 | n14 | 5 | REFSENS2Rx + 34 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n14 | 10 | REFSENS2Rx + 31 |
|  |  | n77 | 10 | REFSENS |
| CA_n30A-n77A | 1 | n30 | 5 | REFSENS2Rx + 13.2 |
|  |  | n77 | 10 | REFSENS |
|  | 2 | n30 | 10 | REFSENS2Rx + 10.6 |
|  |  | n77 | 10 | REFSENS |
|  | 3 | n30 | 5 | REFSENS2Rx + 17.6 |
|  |  | n77 | 10 | REFSENS |
| CA_n41A-n77A | 1 | n41 | 10 | REFSENS2Rx +13.2 |
|  |  | n775 | 20 | REFSENS |
|  | 2 | n41 | 100 | REFSENS2Rx +8.8 |
|  |  | n775 | 20 | REFSENS |
|  | 5 | n41 | 100 | REFSENS |
|  |  | n775 | 10 | REFSENS2Rx +10.5 |
|  | 6 | n41 | 10 | REFSENS2Rx +6.5 |
|  |  | n775 | 100 | REFSENS |
|  | 7 | n41 | 100 | REFSENS2Rx +6.5 |
|  |  | n775 | 100 | REFSENS |
|  | 8 | n41 | 10 | REFSENS2Rx +32 |
|  |  | n775 | 10 | REFSENS |
|  | 9 | n41 | 10 | REFSENS2Rx +32 |
|  |  | n775 | 10 | REFSENS |
| CA_n41A-n79A | 1 | n41 | 100 | REFSENS |
|  |  | n79 | 10 | REFSENS2Rx +3.1 |
|  | 2 | n41 | 10 | REFSENS2Rx +3.5 |
|  |  | n79 | 100 | REFSENS |
| CA_n66A-n77A | 4 | n66 | 5 | REFSENS2Rx +34.33 |
|  |  | n77 | 10 | REFSENS |
|  | 5 | n66 | 5 | REFSENS2Rx +11.27 |
|  |  | n77 | 10 | REFSENS |
| CA_n78A-n79A | 1 | n78 | 100 | REFSENS |
|  |  | n79 | 40 | REFSENS2Rx +5 |
|  | 2 | n78 | 10 | REFSENS2Rx +5.6 |
|  |  | n79 | 100 | REFSENS |
|  | 3 | n78 | 100 | REFSENS2Rx +5.6 |
|  |  | n79 | 100 | REFSENS |
|  | 4 | n78 | 100 | REFSENS |
|  |  | n79 | 100 | REFSENS2Rx +5 |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: The symbol “REFSENS2Rx” in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b. The requirements in this column are applicable for 2Rx, 4Rx or 8Rx antenna ports NR band in the CA configuration. Note 9: The symbol "REFSENS" in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b for 4 antenna port NR band, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 antenna port NR band. |  |  |  |  |

Table 7.3A.1_1.5-2: Reference sensitivity requirement for intraband non-contiguous CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n71(2A) | 1 | n71 | 15 | -97.2 +TT for PCC -97.2 + 4.0 +TT for SCC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n71 | 15 |  | -94.0 +22.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n71 | 15 |  | -94.0 +5.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <End of Changes>