3GPP TSG-RAN5 Meeting #105	R5-247130r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0617 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS NR5GC testcase 6.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For verifying the TP1, the UE is made to update its capability and disable its SMS over IMS capability, which results in UE initiating a re-REGISTER. After this is done, it makes sense to make the UE enable this capability again. This would help the UE return to the capabilities it was in Preamble, and will not impact regressions. If this is not done, then other testcases that are run after this testcase would fail because the UE would not include the "+g.3gpp.smsip" feature tag in the REGISTER message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Note in Step 1, which indicates that an AT/MMI command to be used to make the UE turn off SMS over IMS capability New steps [Steps 6-8] have been added to allow the UE to revert to its normal capabilities and activate its SMS over IMS capability. Introduced Table 6.6.3.3-5 for Step 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose causes failures in regression testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-247130r1: Implemented the changes from R5-246638 for testcase 6.6 to resolve the CR overlap. Basically Made editorial updates to Table 6.6.3.2-1 and Table 6.6.3.4-4. Also updated the nonce-count in Table 6.6.3.3-5 to 3 |  |  |  |  |  |  |  |  |

## 6.6	Re-Registration after capability update / 5GS

6.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to update its capabilities }

then { UE re-registers }

}

6.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.1.4.1]:

Unless either the user or the application within the UE has determined that a continued registration is not required the UE shall reregister an already registered public user identity either 600 seconds before the expiration time if the previous registration was for greater than 1200 seconds, or when half of the time has expired if the previous registration was for 1200 seconds or less, or when the UE intends to update its capabilities according to RFC 3840 [62] or when the UE needs to modify the ICSI values that the UE intends to use in a g.3gpp.icsi-ref media feature tag or IARI values that the UE intends to use in the g.3gpp.iari-ref media feature tag.

6.6.3	Test description

6.6.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE.

-	SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [16] clause 6.1 and RFC 3310 [15].

- 	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	1 NR Cell

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is registered to IMS services, by executing the generic test procedure in Annex A.2 up to the last step.

-	UE is able to be made change its capabilities, manifested through a specific instance which is setting the AT Command +CASIMS (Availability for SMS using IMS, defined in 3GPP TS 27.007 [22] 8.72) to 0.

Preamble:

-	 UE complete the registration procedure and IMS PDU session establishment procedure according to Steps 2-19a1 of TS 38.508-1 table 4.5.2.2-2.

6.6.3.2	Test procedure sequence

Table 6.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to turn off its SMS over IMS capability. (Note) | - | - | - | - |
| 2 | Check: does the UE initiate a re-registration procedure, and indicating the changed capabilities in the REGISTER message? | --> | REGISTER | 1 | P |
| 3 | Void |  |  |  |  |
| 4 | Void |  |  |  |  |
| 5 | SS responds with 200 OK for REGISTER | <-- | 200 OK | - | - |
| 6 | UE is made to turn on its SMS over IMS capability (Note) | - | - | - | - |
| 7 | UE initiates re-REGISTER | --> | REGISTER | - | - |
| 8 | SS responds with 200 OK for REGISTER | <-- | 200 OK | - | - |
| NOTE: This is achieved by MMI or AT command. |  |  |  |  |  |

6.6.3.3	Specific message contents

Table 6.6.3.3-1: REGISTER (step 2, table 6.6.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.1.1, Condition A2, A17 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| feature-param |  | does not contain "+g.3gpp.smsip" |  |  |
| Authorization |  |  |  |  |
| nonce-count |  | 2 |  |  |

Table 6.6.3.3-2: Void

Table 6.6.3.3-3: Void

Table 6.6.3.3-4: 200 OK for REGISTER (step 5, 8 table 6.6.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.1.3

Table 6.6.3.3-5: REGISTER (step 7, table 6.6.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.1.1, Condition A2, A17 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| feature-param |  | contains "+g.3gpp.smsip" |  |  |
| Authorization |  |  |  |  |
| nonce-count |  | 3 |  |  |