3GPP TSG-RAN5 Meeting #104			R5-247145

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Agenda item:	5.3.28.9

Source: 	Qualcomm Inc

Title: 	Discussion on UE beam correspondence for initial access

Document for:	Discussion and Endorsement

## 1.	Introduction

In response to the RAN4 LS [1] to RAN5 for the FR2 UE beam correspondence for initial access and RRC_inactive requirements, there has been several discussions in previous RAN5 meetings. The latest WF is captured in [2].

In [4] an alternate method without requiring beam lock test function was proposed. Action point AP#104.24 was created to include evaluating the testability aspects for testing beam correspondence requirement without beamlock function in addition to with beamlock

| AP#104.24 | RF | Check the testability aspects [with beam lock / without beam lock] for testing beam correspondence requirement in IDLE and INACTIVE mode. If the Beam lock function is confirmed not feasible the alternative method is considered. | Qualcomm, Huawei, Apple, Nokia | R5-244538, R5-246054, R5-245019, R5-244945 | RAN5#106 | Open |
| --- | --- | --- | --- | --- | --- | --- |

This paper further investigates the testability for beam correspondence requirement without beamlock function.

## 2.	Discussion

### 2.1 Test configuration and flow without the use of Beamlock

As mentioned in [4], reusing the RRM SA FR2 contention based random access test case configuration with the below exceptions.

Enabling multiple PRACH transmissions

Modify the preambleTransMax IE value to a large value (n200) to allow multiple PRACH transmissions.

For each received PRACH transmission, TE shall send a RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

UE computed power reaches Pcmax before PRACH EIRP measurement

Modify the preambleReceivedTargetPower IE to a large value (-60) such that UE computed Tx power reaches Pcmax.

Accumulated PRACH power measurement period is atleast 1ms

prach-ConfigurationIndex value 190 corresponds to PRACH format C2. Each RACH is 6 symbol duration with a CP+Seq duration of 41.67 us. This means for 1ms accumulated PRACH measurement period, 24 PRACH is needed.

An even longer applicable PRACH format B4 can be configured for the test to reduce the number of PRACH needed to fulfil the 1ms PRACH measurement period. In this case only 10 PRACH is needed to fulfil 1ms accumulated PRACH.

# of SSB and SSB power level

Configuring a single SSB for this test will ensure UE has only one SSB to measure and send its PRACH towards the direction of the SSB.

The SA FR2 PRACH beam correspondence test flow without beamlock is captured in the below flow diagram

Ensure UE is in RRC_Idle as per 38.508-1 clause 4.5.2

(attach procedure followed by RRC Release)

Configure the test parameters as per RRM TC 7.3.2.2.1 with exceptions

Rotate the UE to a starting coordinate for spherical scan

Place UE in a starting coordinate based on Annex K.1.2 or Annex H.1.2

UE starts sending PRACH

TE sends RAR containing Random access preamble identifier that do not match the transmitted random access preamble

TE waits for a time = T304 timer and collects all the PRACH sent by the UE

1. TE picks a set of X (X = number of PRACH needed to collect total accumulated PRACH measurement period of 1ms) consecutive PRACH among the total PRACH received

2. Calculate the average PRACH power among the best set of X consecutive PRACH. This is the EIRP PRACH power for this coordinate

Continue with above steps for each UE coordinate to complete the spherical scan

Exceptions

- Only 1 SSB configured in the test

- Modify preambleTransMax IE under SIB1 = n200

- Modify preambleReceivedTargetPower IE = -60

- Modify T304 = ms2000

- PRACH configuration index 190 Format C2 (default) or Format B4

Figure 2.0: SA FR2 PRACH Beam correspondence test flow without the use of beamlock

Proposal 1: Configure the FR2 beam correspondence for initial access as per the test flow mentioned in figure 2.0.

### 2.2	Thoughts on beam correspondence procedure with UE beamlock

One of the concerns raised in RAN5-104 by the proponents of UE beamlock based procedure is that UE can switch beams in the withoutbeamlock case. Counter argument would be that in the case of UE beamlock, the UE might end up in a sub optimal beam after Beamlock in which case is more detrimental to the UE.

But as mentioned in [4], for the withoutBeamlock procedure, a carefully chosen test parameter will ensure UE settles on an optimal beam before sending its PRACH transmission.

Observation 1: In the case of UE beamlock based procedure, a UE can end up in a sub optimal beam after locking its beam which is bad for the UE.

### 2.3	Coexistence of withBeamlock and withoutBeamlock procedure

RAN4 LS has suggested (not mandated which is why beamlock is nowhere specified in the 38.101-2 spec) to use UE Beamlock function for verifying the Beam correspondence for initial access. But as demonstrated in [4] and this paper, it is very well possible to verify the RAN4 requirements without using UE beamlock.

Given there are proponents for both options demonstrating the feasibility, in the interest of making progress for this WI, one approach RAN5 can take is to define the test procedure with both solutions (with and without UE beamlock).

Without UE beamlock procedure requires no additional core UE requirement in TS 38.509.

Although with UE Beamlock procedure requires core UE requirement in the form of defining UE beamlock function in TS 38.509, our suggestion is to keep the UE support for beamlock as an optional capability.

Proposal 2a: Define test procedure with both with and without UE Beamlock

Proposal 2b: For the withBeamlock procedure, define UE beam lock function in TS 38.509 as an optional capability.

## 3.	Conclusion

Proposal 1: Configure the FR2 beam correspondence for initial access as per the test flow mentioned in figure 2.0.

Observation 1: In the case of UE beamlock based procedure, a UE can end up in a sub optimal beam after locking its beam which is bad for the UE.

Proposal 2a: Define test procedure with both with and without UE Beamlock

Proposal 2b: For the withBeamlock procedure, define the UE beam lock requirements in TS 38.509 as optional capability.

## 4.		References

[1]		R4-2317764, LS on testability issues of beam correspondence requirement for initial access, 	Huawei

[2]		R5-243159, Discussion on FR2 beam correspondence for initial access, Huawei

[3]	TS 38.101-2 v18.7.0

[4]	R5-245297, UE beam correspondence for initial access, Qualcomm