3GPP TSG-RAN5 Meeting #105	R5-247523

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5370 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ACB per PLMN test case 8.1.2.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As outlined in discussion paper R5-242949, it is necessary to introduce test cases for the ac-BarringPerPLMN-List parameters to meet Japanese requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.2.9a – " RRC Connection Establishment / 0% access probability for MO data, non-zero percent access probability for MO signalling / AC-Barring per PLMN". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not guaranteed up on reception of IEs related to Access Control per PLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.9a (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1473 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-247523 is the final version of R5-246613 with below changes: 1. Updated step number in specific message content Table 8.1.2.9a.3.3-2. 2. ASN.1 corrections in specific message content tables. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.2.9a	RRC Connection Establishment / 0% access probability for MO data, non-zero percent access probability for MO signalling / AC-Barring per PLMN

8.1.2.9a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE reselects a new cell which belongs to different TA and has received SystemInformationBlockType2 message including ac-BarringForMO-Signalling-r12 indicating non zero percent access probability for MO signalling }

then { UE does not transmit any RRCConnectionRequest message when access to the cell is considered as barred and UE transmits an RRCConnectionRequest message when access to the cell is considered as not barred }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received SystemInformationBlockType2 message including ac-BarringforMO-Data-r12 indicating 0% access probability for MO data }

ensure that {
  when { UE has user data pending }

then { UE does not transmit any RRCConnectionRequest message }

}

8.1.2.9a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.3.3.2 and 5.3.3.11. Unless otherwise stated these are Rel-12 requirements.

[TS 36.331, clause 5.3.3.2]

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE.

Upon initiation of the procedure, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

...

1>	if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

...

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

...

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

- initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

- NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

[TS 36.331, clause 5.3.3.11]

1>	if timer T302 or "Tbarring" is running:

2>	consider access to the cell as barred;

1>	else if SystemInformationBlockType2 includes "AC barring parameter":

2>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

2>	for at least one of these valid Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in "AC barring parameter" is set to zero:

3>	consider access to the cell as not barred;

2>	else:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by ac-BarringFactor included in "AC barring parameter":

4>	consider access to the cell as not barred;

3>	else:

4>	consider access to the cell as barred;

1>	else:

2>	consider access to the cell as not barred;

1>	if access to the cell is barred and both timers T302 and "Tbarring" are not running:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer "Tbarring" with the timer value calculated as follows, using the ac-BarringTime included in "AC barring parameter":

"Tbarring" = (0.7+ 0.6 * rand) * ac-BarringTime.

8.1.2.9a.3	Test description

8.1.2.9a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and 11, as specified in TS 36.508 [18] clause 4.4.1.2 are configured according to Table 4.4.2-2 in [18] except for multiple broadcasted PLMN identities as shown in Table 8.1.2.9a.3.1-1: PLMN identifiers.

Table 8.1.2.9a.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1,11 | PLMN1 | 001 | 01 |
|  | PLMN2 | 001 | 02 |

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 according to [18].

8.1.2.9a.3.2	Test procedure sequence

Table 8.1.2.9a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.2.9a.3.2-2.

Table 8.1.2.9a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are assigned to satisfy RCell 1 < RCell 11. |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 11. |

Table 8.1.2.9a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 3 | The SS notifies the UE of change of System Information. SS adjusts SIB2 of Cell 1 as per table 8.1.2.9a.3.3-2 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Wait for 2.1* modification period to allow the new system information to take effect. (Note 1). | - | - | - | - |
| 5 | The SS initializes internal counters: K to 0 and L to 0. | - | - | - | - |
| - | EXCEPTION: Steps 6 to 7b15 shall be repeated maximum 50 times unless K > 0 and L >0. (Note 2) | - | - | - | - |
| 6 | The SS changes the power level setting according to the row “T1” in table 8.1.2.9a.3.2-1. | - | - | - | - |
| - | EXCEPTION: Steps 7a1 to 7b15 describe behaviours which vary depending on whether the UE transmits an RRCConnectionRequest message or not. | - | - | - | - |
| 7a1 | IF the UE does not transmit any RRCConnectionRequest message within 34 s when K=0 and L=0 and within 8 s for other repeat times, i.e. K ≠0 or L≠0 (Note 3) THEN the SS increments the counter K by 1. | - | - | - | - |
| 7a2 | The SS changes the power level setting according to the row “T2” in table 8.1.2.9a.3.2-1. | - | - | - | - |
| 7a3 | The SS waits for 20s for the UE to perform cell reselection procedure. (Note 4) | - | - | - | - |
| 7b1 | ELSE IF the UE transmits an RRCConnectionRequest message on Cell 11 THEN the SS increments the counter L by 1. | --> | RRCConnectionRequest | - | - |
| 7b2-7b6 | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 7b7 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 7b8 | The SS changes the power level setting according to the row “T2” in table 8.1.2.9a.3.2-1. | - | - | - | - |
| 7b9 | The UE transmits an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 7b10-7b14 | Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 1. Note: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 7b15 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 8 | Check: Is the counter K > 0 and L > 0? | - | - | 1 | P |
| 9 | Generic test procedure in TS 36.508 [18] subclause 4.5.3.3 is performed on Cell 1. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 10 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. | - | - | - | - |
| 11 | Wait for 1 s after the IP packet has been transmitted in step 8. (Note 5) | - | - | - | - |
| 12 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionRequest message on Cell 1 within 30s? | --> | RRCConnectionRequest | 2 | F |
| - | EXCEPTION: Step 23 and Step 24a1 can happen in any order. | - | - | - | - |
| 14-23 | Check: Does the test result of steps 1 to 10 generic test procedure in TS 36.508 [18] subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1,2 | - |
| - | EXCEPTION: Step 24a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 24a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 8 on the DRB associated with the default EPS bearer context on Cell 1 within 10 s. | - | - | - | - |
| Note 1: The wait time of 2.1* modification period in step 4 is to allow for the network to paging the system information change during the next modification period and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. Note 2: There is an extremely low probability that the UE originating calls are barred, or not barred, 50 times consecutively. (The probability is 1.7810-15). Therefore, 50 times is enough number of trials for this test case. Note 3: The UE starts T305. Note 4: The UE stops T305 upon cell reselection. Note 5: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 8 to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 10. |  |  |  |  |  |

8.1.2.9a.3.3	Specific message contents

Table 8.1.2.9a.3.3-1: SystemInformationBlockType1 for Cell 1 and 11 (preamble and all steps, Table 8.1.2.9a.3.2-2)

| Derivation path: 36.508[18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityList SEQUENCE (SIZE (1.. maxPLMN-r11)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | 00101 | PLMN1 |  |
| cellReservedForOperatorUse | notReserved |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | 00102 | PLMN2 |  |
| cellReservedForOperatorUse | reserved |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9a.3.3-2: SystemInformationBlockType2 for Cell 1 (step 3 and all subsequent steps, Table 8.1.2.9a.3.2-2)

| Derivation Path: 36.508[18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| ac-BarringForMO-Data-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s512 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9a.3.3-3: RRCConnectionSetupComplete (preamble, step 7b3, step 7b11 and step 17 Table 8.1.2.9a.3.2-2)

| Derivation Path: 36.508[18], Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9a.3.3-4: SystemInformationBlockType2 for Cell 11 (preamble and all steps, Table 8.1.2.9a.3.2-2)

| Derivation Path: 36.508[18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| ac-BarringForMO-Signalling-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p50 |  |  |
| ac-BarringTime | s64 |  |  |
| ac-BarringForSpecialAC | '00000'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.9a.3.3-5: CLOSE UE TEST LOOP (preamble, Table 8.1.2.9a.3.2-2)

| Derivation Path: 36.508[18], Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

Table 8.1.2.9a.3.3-6: RRCConnectionRequest (step 7b1 and step 7b9, Table 8.1.2.9a.3.2-2)

| Derivation Path: 36.508[18], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | mo-Signalling |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>