3GPP TSG-RAN5 Meeting #105	R5-247543

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4787 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Idle mode test case 6.1.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wait time for FR2 is missing at step 2. The wait time calculation is based on TS 38.133, Table 4.2.2.4-1 plus the t-ReselectionNR timer which comes out to be 137 seconds. The notes in cell power level tables for conducted and OTA testing, which are Table 6.1.2.5.3.2-1 and Table 6.1.2.5.3.2-2 respectively, were not consistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the wait time for FR2. Aligned the notes for conducted and OTA cell power tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-247244->R5-247543 |  |  |  |  |  |  |  |  |

#### 6.1.2.5	Cell reselection for interband operation using Pcompensation / Between FDD and TDD

6.1.2.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state on a FDD band }

ensure that {

when { UE detects the cell selection using Pcompensation & reselection criteria is met for the cell on a TDD band }

then { UE reselects the new cell }

}

6.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.304: clause 5.2.3.2 and 5.2.4.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.2.3.2]

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm). If the UE supports SUL frequency for this cell, Qrxlevmin is obtained from RxLevMinSUL, if present, in SIB1, SIB2 and SIB4, additionally, if QrxlevminoffsetcellSUL is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell; else Qrxlevmin is obtained from q-RxLevMin in SIB1, SIB2 and SIB4, additionally, if Qrxlevminoffsetcell is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell. |
| Qqualmin | Minimum required quality level in the cell (dB). Additionally, if Qqualminoffsetcell is signalled for the concerned cell, this cell specific offset is added to achieve the required minimum quality level in the concerned cell. |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9] |
| Pcompensation | If the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| PEMAX1, PEMAX2 | Maximum TX power level of a UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 38.101 [15]. If UE supports SUL frequency for this cell, PEMAX1 and PEMAX2 are obtained from the p-Max for SUL in SIB1 and NR-NS-PmaxList for SUL respectively in SIB1, SIB2 and SIB4 as specified in TS 38.331 [3], else PEMAX1 and PEMAX2 are obtained from the p-Max and NR-NS-PmaxList respectively in SIB1, SIB2 and SIB4 for regular UL as specified in TS 38.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101 [15] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN (TS 23.122 [9]). During this periodic search for higher priority PLMN, the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.[TS 38.304, clause 5.2.4.5]

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in subclause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

6.1.2.5.3	Test description

6.1.2.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is set to NR FDD mode and NR Cell 10 is set to NR TDD mode.

-	System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.5.3.2	Test procedure sequence

Table 6.1.2.5.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.1.2.5.3.2-3.

Table 6.1.2.5.3.2-1: Time instances of cell power level and parameter changes for conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -80 | NR Cell 1 become lower than Threshserving, low and NR Cell 10 become stronger than ThreshNR Cell 10, low. |
| Note 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.1-3. Note 2: The downlink signal level uncertainty is specified in TS 38.508-1 [4], clause 6.2.2.1. |  |  |  |  |  |

Table 6.1.2.5.3.2-2: Time instances of cell power level and parameter changes for OTA test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | NR Cell 1 become lower than Threshserving, low and NR Cell 10 become stronger than ThreshNR Cell 10, low. |
| Note 1: Power level “Off” is defined in TS 38.508-1[4], Table 6.2.2.2.2-2. Note 2: The downlink signal level uncertainity is specified in TS 38.508-1 [4], clause 6.2.2.2. |  |  |  |  |  |

Table 6.1.2.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 10 SS/PBCH EPRE level according to the row "T1" in table 6.1.2.5.3.2-1/2. | - | - | - | - |
| 2 | Wait for 41[FR1]/137[FR2] seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 10? | - | - | 1 | - |
| Note: The wait time at step 2 is Kcarrier*Tdetect,NR_Inter (as per TS 38.133 [30], clause 4.2.2.4) plus the time to read the system information TSI-NR and t-ReselectionNR |  |  |  |  |  |

6.1.2.5.3.3	Specific message contents

Table 6.1.2.5.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 10 | This condition applies to system information transmitted on NR Cell 10. |

Table 6.1.2.5.3.3-2: SIB1 of NR Cell 1 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE{ |  |  |  |
| uplinkConfigCommon SEQUENCE{ |  |  |  |
| FrequencyInfoUL SEQUENCE { |  |  |  |
| p-Max | 33dBm |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5.3.3-3: SIB2 of NR Cell 1 and NR Cell10 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE{ |  |  |  |
| threshServingLowP | 9 | 18 dB | NR Cell 1 |
| cellReselectionPriority | 5 |  | NR Cell 1 |
|  | 1 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5.3.3-4: SIB4 of NR Cell 1 and NR Cell 10 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 10 |  | NR Cell 1 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 10 |
| p-Max | 33dBm |  | NR Cell 1 |
| t-ReselectionNR | 7 | seconds |  |
| threshX-HighP | 14 | 28 dB |  |
| threshX-LowP | 6 | 12 dB |  |
| cellReselectionPriority | 1 |  | NR Cell 1 |
|  | 5 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |