3GPP TSG-RAN5 Meeting #105	R5-247565

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4738 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SL SIG test case 12.1.8.2.3 - PC5 only TBS selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add NR SL SIG test case 12.1.8.2.3 - PC5-only operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SL SIG test case 12.1.8.2.3 - PC5-only operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.8.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0528 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-246744r2 |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.1.8.2.3	PC5-only operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection

12.1.8.2.3.1	Test Purpose (TP)

(1)

with { UE is configured to perform sidelink reception }

ensure that {

when { UE receives 1st stage SCI indicating a SL-SCH TB corresponding to Frequency resource assignment, Time resource assignment, Modulation and coding scheme and PSFCH overhead indication }

then { UE decodes the received SL-SCH TB transport block of size correctly and forwards it to higher layers }

}

12.1.8.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, subclause 8.3.1.1, TS 38.214, subclause 8.1.2.1, 8.1.2.2, 8.1.3.1 and 8.1.3.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.212, subclause 8.3.1.1]

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

…

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

…

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

…

-	PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

…

[TS 38.214, subclause 8.1.2.1]

The UE shall transmit the PSSCH in the same slot as the associated PSCCH.

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are not configured for sidelink. A symbol is configured for sidelink, according to higher layer parameters sl-StartSymbol and sl-LengthSymbols, where sl-StartSymbol is the symbol index of the first symbol of sl-LengthSymbols consecutive symbols configured for sidelink.

-	Within the slot, PSSCH resource allocation starts at symbol sl-StartSymbol+1.

-	The UE shall not transmit PSSCH in symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

-	The UE shall not transmit PSSCH in the last symbol configured for sidelink.

-	The UE shall not transmit PSSCH in the symbol immediately preceding the symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

…

[TS 38.214, subclause 8.1.2.2]

The resource allocation unit in the frequency domain is the sub-channel.

The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI.

The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted.

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

[TS 38.214, subclause 8.1.3.1]

IMCS is given by the 'Modulation and coding scheme' field in SCI format 1-A.

The MCS table is determined as follows: Table 5.1.3.1-1 is used if no additional MCS table is configured by higher layer parameter sl-Additional-MCS-Table;

…

The UE shall use IMCS and the MCS table determined according to the previous step to determine the modulation order (Qm) and Target code rate (R) used in the physical sidelink shared channel.

[TS 38.214, subclause 8.1.3.2]

For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot.

-	A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where

-	NscRB=12 is the number of subcarriers in a physical resource block,

-	NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers,

-	NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	NohPRB is the overhead given by higher layer parameter sl-X-Overhead,

-	NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList.

Table 8.1.3.2-1: NREDMRS according to higher layer parameter sl-PSSCH-DMRS-TimePatternList

| sl-PSSCH-DMRS-TimePatternList | NREDMRS |
| --- | --- |
| {2} | 12 |
| {3} | 18 |
| {4} | 24 |
| {2,3} | 15 |
| {2,4} | 18 |
| {3,4} | 21 |
| {2,3,4} | 18 |

-	A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

-	nPRB is the total number of allocated PRBs for the PSSCH,

-	NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DM-RS.

-	NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0.

The UE determines TBS according to Steps 2), 3), and 4) in clause 5.1.3.2.

A UE is not expected to receive an SCI indicating 28≤IMCS≤31 if Table 5.1.3.1-2 is used, or 29≤IMCS≤31 otherwise.

12.1.8.2.3.3	Test description

12.1.8.2.3.3.1	Pre-test conditions

NR SL carrier bandwidth and BWP bandwidth are set to maximum for the NR band under test as specified in TS 38.101-1 [16] Table 5.3E.1-1 (to enable testing of NPRB up to maximum value) in pre-configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1.8.1 using the common highest mandatory channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.7

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.8.2.3.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.1.8.2.3.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.8.2.3.3.2	Test procedure sequence

Table 12.1.8.2.3.3.2-1: Maximum TBS for different UE categories for sidelink UE

| UE Category | Maximum number of bits of a SL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.5 require UE to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 12.1.8.2.3.3.2-2: Number of sidelink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 152 ≤ TBS ≤ 12152 (note 2) | 1 | 8*FLOOR((TBS -152) / 8) |
| 12153 ≤ TBS ≤ 24224 | 2 | 8*FLOOR((TBS -224) / 16) |
| 24225 ≤ TBS ≤ 36296 | 3 | 8*FLOOR((TBS -296) / 24) |
| 36297 ≤ TBS ≤ 48368 | 4 | 8*FLOOR((TBS -368) / 32) |
| 48369 ≤ TBS ≤ 60440 | 5 | 8*FLOOR((TBS -440) / 40) |
| 60441 ≤ TBS ≤ 72512 | 6 | 8*FLOOR((TBS -512) / 48) |
| 72513 ≤ TBS ≤ 84584 | 7 | 8*FLOOR((TBS -584) / 56) |
| 84585 ≤ TBS ≤ 96656 | 8 | 8*FLOOR((TBS -656) / 64) |
| 96657 ≤ TBS ≤ 108728 | 9 | 8*FLOOR((TBS -728) / 72) |
| 108729 ≤ TBS ≤ 120800 | 10 | 8*FLOOR((TBS -800) / 80) |
| 120801 ≤ TBS ≤ 132872 | 11 | 8*FLOOR((TBS -872) / 88) |
| 132873 ≤ TBS ≤ 144944 | 12 | 8*FLOOR((TBS -944) / 96) |
| 144945 ≤ TBS ≤ 145416 | 13 | 8*FLOOR((TBS -1016) / 104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = 8 * FLOOR((TBS - SL-SCH subheader size - RLC Status PDU size - subheader for RLC Status PDU size) / 8N) - (PDCP header size + AMD PDU header size + subheader for MAC subPDU size) (in bits) where PDCP header size is 32 bits (12-bit PDCP SN case); AMD PDU header size is 16 bits (12 bit RLC SN case); SL-SCH subheader size is 32 bits. subheader for MAC subPDU or RLC status PDU = 24 bits (consider the worst L = 16 bits case). RLC Status PDU size = 24 bits (only 1 ACK_SN case). This gives: PDCP SDU size = 8 * FLOOR((TBS - 80) / 8N) - 72 Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 152 bits. |  |  |

Table 12.1.8.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 7 are repeated for allowed values of Nsubchannel from 1 to NRBSL BWPnsubch in BWP and IMCS from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of Nsubchannel and IMCS. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 7 are performed if TBS is less than or equal to UE capability "Maximum number of bits of a SL-SCH transport block received within a TTI" as specified in Table 12.1.8.2.3.3.2-1 and larger than or equal to 152 bits as specified in Table 12.1.8.2.3.3.2-2. | - | - | - | - |
| 2 | The SS triggers UE to close UE test loop mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 3 | The SS creates one or more PDCP SDUs each containing a V2X packet, depending on TBS, in accordance with Table 12.1.8.2.3.3.2-2 | - | - | - | - |
| 4 | The SS transmits the PDCP SDUs concatenated into a MAC PDU in logical slot without presence of PSFCH, and sets the fields in associated SCI format 1-A as follows: - Frequency resource assignment: Nsubchannel - Modulation and coding scheme: IMCS - PSFCH overhead indication: “0” (doesn’t consider PSFCH when determining TBS) | <-- | MAC PDU (N PDCP SDUs each containing a V2X packet) SCI: (SCI Format 1-A) |  |  |
| 5 | the SS requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 6 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to N, where N is the PDCP SDUs sent in step 4? NOTE: The UE reporting the counter of successful reception of SDAP SDU/PSSCH/PSCCH packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 1 | P |
| 7 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| - | EXCEPTION: Steps 8 to 14 are repeated for allowed values of Nsubchannel from 1 to NRBSL BWPnsubch in BWP and IMCS from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 8 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of Nsubchannel and IMCS. | - | - | - | - |
| - | EXCEPTION: Steps 9 to 14 are performed if TBS is less than or equal to UE capability "Maximum number of bits of a SL-SCH transport block received within a TTI" as specified in Table 12.1.8.2.3.3.2-1 and larger than or equal to 152 bits as specified in Table 12.1.8.2.3.3.2-2. | - | - | - | - |
| 9 | The SS triggers UE to close UE test loop mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 10 | The SS creates one or more PDCP SDUs each containing a V2X packet, depending on TBS, in accordance with Table 12.1.8.2.3.3.2-2 | - | - | - | - |
| 11 | The SS transmits the PDCP SDUs concatenated into a MAC PDU in logical slot with presence of PSFCH, and sets the fields in associated SCI format 1-A as follows: - Frequency resource assignment: Nsubchannel - Modulation and coding scheme: IMCS - PSFCH overhead indication: “1” (doesn’t consider PSFCH when determining TBS) | <-- | MAC PDU (N PDCP SDUs each containing a V2X packet) SCI: (SCI Format 1-A) |  |  |
| 12 | the SS requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 13 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to N, where N is the PDCP SDUs sent in step 4? NOTE: The UE reporting the counter of successful reception of SDAP SDU/PSSCH/PSCCH packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 1 | P |
| 14 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |

12.1.8.2.3.3.3	Specific message contents

Table 12.1.8.2.3.3.3-1: +CCUTLE (Table 12.1.8.2.3.3.2-4, Step 2 and 9)

Derivation Path: TS 38.508-1 [4] Table 4.7B-1 with condition Close and Receive

Table 12.1.8.2.3.3.3-2: +CCUTLE (Table 12.1.8.2.3.3.2-4, Step 7 and 14)

Derivation Path: TS 38.508-1 [4] Table 4.7B-1 with condition Open

Table 12.1.8.2.3.3.3-3: +CUSPCREQ (Table 12.1.8.2.3.3.2-4, Step 5 and 12)

| Derivation Path: 36.508 [7] Table 4.7I-4 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value/remark | Comment | Condition |
| +CUSPCREQ = |  |  |  |  |
| < type1> |  | 2 | NR PSCCH transport blocks |  |
| < format > |  | 1 |  |  |
| <length1> |  | Not Checked |  |  |
| <counter1> |  | Not Checked |  |  |
| < type2> |  | 2 | NR STCH SDAP SDU packets |  |
| < format > |  | 1 |  |  |
| <length2> |  | Not Checked |  |  |
| <counter2> |  | N | Value of N is given in Table 12.1.8.2.3.3.2-2 |  |
| < type3> |  | 2 | NR PSSCH transport blocks |  |
| < format > |  | 1 |  |  |
| <length3> |  | Not Checked |  |  |
| <counter3> |  | Not Checked |  |  |

<End of Change 1>