3GPP TSG-RAN5 Meeting #105	R5-247570

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4743 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SL SIG test case 12.2.10.2.1 - Concurrent ACK-NACK HARQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add NR SL SIG test case 12.2.10.2.1 - Inter-concurrent operation / MAC / SL-SCH data reception / ACK-NACK HARQ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SL SIG test case 12.2.10.2.1 - Inter-concurrent operation / MAC / SL-SCH data reception / ACK-NACK HARQ is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.10.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0528 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-246749r1 |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2.10.2	Inter-concurrent operation / MAC / SL-SCH data reception

12.2.10.2.1	Inter-concurrent operation / MAC / SL-SCH data reception / ACK-NACK HARQ

12.2.10.2.1.1	Test Purpose (TP)

(1)

with { UE is configured to perform sidelink reception }

ensure that {

when { a SL-SCH TB is received from PHY layer and HARQ feedback is disabled by its associated SCI. UE successfully decodes the SL-SCH MAC PDU }

then { UE doesn't sent PSFCH }

}

(2)

with { UE is configured to perform sidelink reception }

ensure that {

when { a SL-SCH TB is received from PHY layer and ACK-NACK HARQ feedback is indicated by its associated SCI. UE successfully decodes the SL-SCH MAC PDU }

then { UE sends an ACK feedback on PSFCH }

}

(3)

with { UE is configured to perform sidelink reception }

ensure that {

when { a SL-SCH TB is received from PHY layer and ACK-NACK HARQ feedback is enabled by its associated SCI. UE fails to decode the SL-SCH MAC PDU }

then { UE sends a NACK feedback on PSFCH }

}

(4)

with { UE is configured to perform sidelink reception }

ensure that {

when { a MAC PDU associated to unicast is received and its associated destination L2 ID doesn't match with any source L2 ID of the UE or its associated source L2 ID doesn't match with any destination L2 ID of the UE }

then { UE discards this MAC PDU }

}

12.2.10.2.1.2	Conformance requirements

Same as the conformance requirements given in subclause 12.1.8.2.1.2.

12.2.10.2.1.3	Test description

12.2.10.2.1.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.2.10.2.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.2.10.2.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 |  |

Preamble:

-	The UE is in state 1N-B as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.10.2.1.3.2	Test procedure sequence

Same as the test procedure sequence given in subclause 12.1.8.2.1.3.2.

12.2.10.2.1.3.3	Specific message contents

Same as the specific message contents given in subclause 12.1.8.2.1.3.3.

<End of Change 1>