3GPP TSG-RAN WG5 Meeting # 105	R5-247709

# Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0751 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of band n54, n98 and n109 for physical layer baseline implementation capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing baseline implementation capabilities for band n54, n98 and n109 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding implementation capabilities for band n54, n98 and n109 into related tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF baseline implementation capabilities will not be defined for band n54, n98 and n109 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1, A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-247250 |  |  |  |  |  |  |  |  |

### A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" for bands are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

NOTE:	See Annex B for status of completed NR bands and power classes in this version of 3GPP UE conformance test specifications.

Table A.4.3.1-1: NR FDD FR1 RF Baseline Implementation Capabilities

| Item | NR FDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand1_Supp | NR FDD FR1 Band n1 |
| 2 | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand2_Supp | NR FDD FR1 Band n2 |
| 3 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand3_Supp | NR FDD FR1 Band n3 |
| 4 | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand5_Supp | NR FDD FR1 Band n5 |
| 5 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand7_Supp | NR FDD FR1 Band n7 |
| 6 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand8_Supp | NR FDD FR1 Band n8 |
| 6a to 6c | Reserved |  |  |  |  |
| 6d | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand12_Supp | NR FDD FR1 Band n12 |
| 6e | NR Frequency band: 777-787 MHz (UL), 746-756 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand13_Supp | NR FDD FR1 Band n13 |
| 6f | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand14_Supp | NR FDD FR1 Band n14 |
| 6g to 6i | Reserved |  |  |  |  |
| 6j | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand18_Supp | NR FDD FR1 Band n18 |
| 6k | Reserved |  |  |  |  |
| 7 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand20_Supp | NR FDD FR1 Band n20 |
| 7a to 7c | Reserved |  |  |  |  |
| 7d | NR Frequency band: 1626.5-1660.5 MHz (UL), 1525-1559 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand24_Supp | NR FDD FR1 Band n24 |
| 7e | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand25_Supp | NR FDD FR1 Band n25 |
| 7f | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand26_Supp | NR FDD FR1 Band n26 |
| 7g | Reserved |  |  |  |  |
| 8 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand28_Supp | NR FDD FR1 Band n28 |
| 8a | Reserved |  |  |  |  |
| 8b | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand30_Supp | NR FDD FR1 Band n30 |
| 8c | NR Frequency band: 452.5-457.5 MHz (UL), 462.5-467.5 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand31_Supp | NR FDD FR1 Band n31 |
| 8d | Reserved |  |  |  |  |
| 8e | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand65_Supp | NR FDD FR1 Band n65 |
| 9 | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand66_Supp | NR FDD FR1 Band n66 |
| 9a to 9c | Reserved |  |  |  |  |
| 10 | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand70_Supp | NR FDD FR1 Band n70 |
| 11 | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand71_Supp | NR FDD FR1 Band n71 |
| 12 | NR Frequency band: 451-456 MHz (UL), 461-466 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand72_Supp | NR FDD FR1 Band n72 |
| 13 | Reserved |  |  |  |  |
| 14 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand74_Supp | NR FDD FR1 Band n74 |
| 14a | NR Frequency band: 698-716 MHz (UL), 728-746 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand85_Supp | NR FDD FR1 Band n85 |
| 15 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand91_Supp | NR FDD FR1 Band n91 |
| 16 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand92_Supp | NR FDD FR1 Band n92 |
| 17 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand93_Supp | NR FDD FR1 Band n93 |
| 18 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand94_Supp | NR FDD FR1 Band n94 |
| 19 | NR Frequency band: 874.4-880 MHz (UL), 919.4-925 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand100_Supp | NR FDD FR1 Band n100 |
| 20 to 24 | Reserved |  |  |  |  |
| 25 | NR Frequency band: 896 MHz-901 MHz (UL), 935 MHz-940 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand106_Supp | NR FDD FR1 Band n106 |
| 26 | NR Frequency band: 703 MHz-733 MHz (UL), 1432 MHz-1517 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand109_Supp | NR FDD FR1 Band n109 |

Table A.4.3.1-2: NR TDD FR1 RF Baseline Implementation Capabilities

| Item | NR TDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2010-2025 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand34_Supp | NR TDD FR1 Band n34 |
| 0a to 0c | Reserved |  |  |  |  |
| 1 | NR Frequency band: 2570-2620 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand38_Supp | NR TDD FR1 Band n38 |
| 1a | NR Frequency band: 1880-1920 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand39_Supp | NR TDD FR1 Band n39 |
| 1b | NR Frequency band: 2300-2400 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand40_Supp | NR TDD FR1 Band n40 |
| 2 | NR Frequency band: 2496-2690 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand41_Supp | NR TDD FR1 Band n41 |
| 2a to 2d | Reserved |  |  |  |  |
| 2e | NR Frequency band: 5150-5925 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand46_Supp | NR TDD FR1 Band n46 |
| 2f | Reserved |  |  |  |  |
| 2g | NR Frequency band: 3550-3700 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand48_Supp | NR TDD FR1 Band n48 |
| 2h | Reserved |  |  |  |  |
| 2i | NR Frequency band: 1432-1517 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand50_Supp | NR TDD FR1 Band n50 |
| 2j | NR Frequency band: 1427-1432 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand51_Supp | NR TDD FR1 Band n51 |
| 2k | Reserved |  |  |  |  |
| 2l | NR Frequency band: 2483.5-2495 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand53_Supp | NR TDD FR1 Band n53 |
| 2m | NR Frequency band: 1670-1675 MHz (UL / DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand54_Supp | NR TDD FR1 Band n54 |
| 3 | NR Frequency band: 3300–4200 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand77_Supp | NR TDD FR1 Band n77 |
| 4 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand78_Supp | NR TDD FR1 Band n78 |
| 5 | NR Frequency band: 4400–5000 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand79_Supp | NR TDD FR1 Band n79 |
| 6 | Void |  |  |  |  |
| 7-11 | Reserved |  |  |  |  |
| 12 | NR Frequency band: 5925–7125 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand96_Supp | NR TDD FR1 Band n96 |
| 13 | NR Frequency band: 1900–1910 MHz (UL / DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand101_Supp | NR TDD FR1 Band n101 |
| 14 | NR Frequency band: 5925 MHz – 6425 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand102_Supp | NR TDD FR1 Band n102 |

Table A.4.3.1-3: NR TDD FR2 RF Baseline Implementation Capabilities

| Item | NR TDD FR2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand257_Supp | NR TDD FR2 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand258_Supp | NR TDD FR2 Band n258 |
| 2a | NR Frequency band: 39500-43500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-16 | pc_nrBand259_Supp | NR TDD FR2 Band n259 |
| 3 | NR Frequency band: 37000–40000 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand260_Supp | NR TDD FR2 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand261_Supp | NR TDD FR2 Band n261 |

Table A.4.3.1-4: NR FR1 PC2 RF Baseline Implementation Capabilities

| Item | NR FR1 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2300-2400 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand40_PC2_Supp | NR FR1 PC2 Band n40 |
| 1 | NR Frequency band: 2496-2690 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand41_PC2_Supp | NR FR1 PC2 Band n41 |
| 2 | NR Frequency band: 3300-4200 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand77_PC2_Supp | NR FR1 PC2 Band n77 |
| 3 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand78_PC2_Supp | NR FR1 PC2 Band n78 |
| 4 | NR Frequency band: 4400–5000 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand79_PC2_Supp | NR FR1 PC2 Band n79 |
| 5 | NR Frequency band: 2010–2025 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand34_PC2_Supp | NR FR1 PC2 Band n34 |
| 6 | NR Frequency band: 1880–1920 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand39_PC2_Supp | NR FR1 PC2 Band n39 |
| 7 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 6.2.1 | Rel-17 | pc_nrBand1_PC2_Supp | NR FR1 PC2 Band n1 |
| 8 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 6.2.1 | Rel-17 | pc_nrBand3_PC2_Supp | NR FR1 PC2 Band n3 |
| 9 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand8_PC2_Supp | NR FR1 PC2 Band n8 |
| 10 | NR Frequency band: 1880–1920 MHz (UL) | 38.101-1, 6.2.1 | Rel-17 | pc_nrBand98_PC2_Supp | NR FR1 PC2 Band n98 |

Table A.4.3.1-4a: NR FR2 PC2 RF Baseline Implementation Capabilities

| Item | NR FR2 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC2_Supp | NR FR2 PC2 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC2_Supp | NR FR2 PC2 Band n258 |
| 3 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC2_Supp | NR FR2 PC2 Band n261 |

Table A.4.3.1-4b: NR FR1 PC1 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand14_PC1_Supp | NR FR1 PC1 Band n14 |
| 2 | NR Frequency band: 874.4-880 MHz (UL), 919.4-925 MHz (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand100_PC1_Supp | NR FR1 PC1 Band n100 |
| 3 | NR Frequency band: 1900–1910 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand101_PC1_Supp | NR FR1 PC1 Band n101 |
| 4 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand7_PC1_Supp | NR FR1 PC1 Band n7 |
| 5 | FFS |  |  |  |  |
| 6 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand78_PC1_Supp | NR FR1 PC1 Band n78 |
| 7 | NR Frequency band: 2496–2690 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand41_PC1_Supp | NR FR1 PC1 Band n41 |
| 8 | NR Frequency band: 1850–1915 MHz (UL), 1930–1995 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand25_PC1_Supp | NR FR1 PC1 Band n25 |
| 9 | NR Frequency band: 2300–2400 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand40_PC1_Supp | NR FR1 PC1 Band n40 |
| 10 | NR Frequency band: 1710–1780 MHz (UL), 2110–2200 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand66_PC1_Supp | NR FR1 PC1 Band n66 |
| 11 | NR Frequency band: 663–698 MHz (UL), 617–652 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand71_PC1_Supp | NR FR1 PC1 Band n71 |
| 12 | NR Frequency band: 3300–4200 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand77_PC1_Supp | NR FR1 PC1 Band n77 |
| 13 | NR Frequency band: 698–716 MHz (UL), 728–746 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand85_PC1_Supp | NR FR1 PC1 Band n85 |

Table A.4.3.1-4c: NR FR2 PC1 RF Baseline Implementation Capabilities

| Item | NR FR2 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC1_Supp | NR FR2 PC1 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC1_Supp | NR FR2 PC1 Band n258 |
| 3 | NR Frequency band: 37000-40000 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC1_Supp | NR FR2 PC1 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC1_Supp | NR FR2 PC1 Band n261 |

Table A.4.3.1-4d: NR FR2 PC4 RF Baseline Implementation Capabilities

| Item | NR FR2 PC4 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC4_Supp | NR FR2 PC4 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC4_Supp | NR FR2 PC4 Band n258 |
| 3 | NR Frequency band: 37000-40000 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC4_Supp | NR FR2 PC4 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC4_Supp | NR FR2 PC4 Band n261 |

Table A.4.3.1-4e: NR FR1 PC1.5 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1.5 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 3300-4200 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand77_PC1.5_Supp | NR FR1 PC1.5 Band n77 |
| 2 | NR Frequency band: 3300–3800 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand78_PC1.5_Supp | NR FR1 PC1.5 Band n78 |
| 3 | NR Frequency band: 4400-5000 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand79_PC1.5_Supp | NR FR1 PC1.5 Band n79 |
| 4 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand41_PC1.5_Supp | NR FR1 PC1.5 Band n41 |
| 5 | NR Frequency band: 2010-2025 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand34_PC1.5_Supp | NR FR1 PC1.5 Band n34 |
| 6 | NR Frequency band: 2300-2400 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand40_PC1.5_Supp | NR FR1 PC1.5 Band n40 |
| 7 | NR Frequency band: 1880-1920 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand39_PC1.5_Supp | NR FR1 PC1.5 Band n39 |

Table A.4.3.1-4f: NR FR1 maxNumberSRS-Ports-PerResource RF Baseline Implementation Capabilities (Rel-15)

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Parameter Type | Supported Value | Supported UE capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxNumberSRS-Ports-PerResource | 38.306, 4.2.7.7 | Rel-15 | pc_nrBand41_maxNumberSRS-Ports-PerResource_r15 | enumerated | n1, n2, n4 |  |

Table A.4.3.1-4g: NR FR1 maxUplinkDutyCycle-PC2-FR1 RF Baseline Implementation Capabilities

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic Parameter Name | Parameter Type | Supported Value | Supported UE capability (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxUplinkDutyCycle-PC2-FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_nrBand41_maxUplinkDutyCycle_PC2_FR1 | enumerated | n60, n70, n80, n90, n100 |  |
| 2 | n79 | NR_n79 maxUplinkDutyCycle-PC2-FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_nrBand79_maxUplinkDutyCycle_PC2_FR1 | enumerated | n60, n70, n80, n90, n100 |  |
| NOTE 1: The UE supplier shall indicate the supported maxUplinkDutyCycle-PC2-FR1 as per RF-Parameters in TS 38.331 Section 6.3.3 UE capability information elements and choose the supported value. |  |  |  |  |  |  |  |  |

Table A.4.3.1-4h: NR FR1 maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 RF Baseline Implementation Capabilities

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic Parameter Name | Parameter Type | Supported Value | Supported UE capability (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 | 38.306, 4.2.7.2 | Rel-16 | pc_nrBand41_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 | enumerated | n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100 |  |
| 2 | n79 | NR_n79 maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 | 38.306, 4.2.7.2 | Rel-16 | pc_nrBand79_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 | enumerated | n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100 |  |
| NOTE 1: The UE supplier shall indicate the supported maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 as per RF-Parameters in TS 38.331 Section 6.3.3 UE capability information elements and choose the supported value. |  |  |  |  |  |  |  |  |

Table A.4.3.1-4i: NR FR2 PC6 RF Baseline Implementation Capabilities

| Item | NR FR2 PC4 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand257_PC6_Supp | NR FR2 PC6 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand258_PC6_Supp | NR FR2 PC6 Band n258 |
| 3 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand261_PC6_Supp | NR FR2 PC6 Band n261 |

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item | NR SUL FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1710-1785 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand80_Supp | NR SUL FR1 Band n80 |
| 2 | NR Frequency band: 880-915 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand81_Supp | NR SUL FR1 Band n81 |
| 3 | NR Frequency band: 832-862 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand82_Supp | NR SUL FR1 Band n82 |
| 4 | NR Frequency band: 703-748 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand83_Supp | NR SUL FR1 Band n83 |
| 5 | NR Frequency band: 1920-1980 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand84_Supp | NR SUL FR1 Band n84 |
| 6 | NR Frequency band: 1710-1780 MHz (UL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand86_Supp | NR SUL FR1 Band n86 |
| 6a to 6b | Reserved |  |  |  |  |
| 6c | Reserved |  |  |  |  |
| 7 | NR Frequency band: 2010-2025 MHz (UL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand95_Supp | NR SUL FR1 Band n95 |
| 8 | NR Frequency band: 2300 MHz – 2400 MHz (UL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand97_Supp | NR SUL FR1 Band n97 |
| 8a | NR Frequency band: 1880 MHz – 1920 MHz (UL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand98_Supp | NR SUL FR1 Band n98 |
| 9 | NR Frequency band: 1626.5-1660.5 MHz (UL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand99_Supp | NR SUL FR1 Band n99 |

Table A.4.3.1-6: NR SDL FR1 RF Baseline Implementation Capabilities

| Item | NR SDL FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 717-728 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand29_Supp | NR SDL FR1 Band n29 |
| 1 | NR Frequency band: 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand75_Supp | NR SDL FR1 Band n75 |
| 2 | NR Frequency band: 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand76_Supp | NR SDL FR1 Band n76 |

Table A.4.3.1-7: UE Power Class implementation Capabilities (for one or more of the supported UE Power Class Implemented Capabilities in Table A.4.3.1-4, Table A.4.3.1-4a, Table A.4.3.1-4b, Table A.4.3.1-4c, Table A.4.3.1-4d and Table A.4.3.1-4e)

| Item | UE Power Class implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE Power Class 1 in FR1 | 38.101-1, 6.2.1 | Rel-16 | pc_FR1_PC1 | Applicable to the bands in Table A.4.3.1-4b |
| 1a | UE Power Class 1 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC1 | Applicable to the bands in Table A.4.3.1-4c |
| 2 | UE Power Class 2 in FR1 | 38.101-1, 6.2.1 | Rel-15 | pc_FR1_PC2 | Applicable to the bands in Table A.4.3.1-4 |
| 2a | UE Power Class 2 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC2 | Applicable to the bands in Table A.4.3.1-4a |
| 3 | UE Power Class 3 in FR1 | 38.101-1, 6.2.1 | Rel-15 | pc_FR1_PC3 | All applicable FR1 NR bands |
| 3a | UE Power Class 3 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC3 | All applicable FR2 NR bands |
| 4 | UE Power Class 4 in FR2 | 38.101-2, 6.2.1 | Rel-15 | pc_FR2_PC4 | Applicable to the bands in Table A.4.3.1-4d |
| 5 | UE Power Class 1.5 in FR1 | 38.101-1, 6.2.1 | Rel-15 | pc_FR1_PC1.5 | Applicable to the bands in Table A.4.3.1-4e |
| 6 | UE Power Class 6 in FR2 | 38.101-2, 6.2.1 | Rel-17 | pc_FR2_PC6 | Applicable to the bands in Table A.4.3.1-4i |

Table A.4.3.1-7a: NR FR1 Rx implementation Capabilities

| Item | UE 2Rx/4Rx/8Rx implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE 2Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_2Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4c |
| 2 | UE FDD 4Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_FDD_4Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4a |
| 3 | UE TDD 4Rx in FR1 | 38.101-1, 7.3 | Rel-15 | pc_FR1_TDD_4Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4b |
| 4 | UE only supports 1Rx in FR1 | 38.101-1, 7.3I | Rel-17 | pc_FR1_1Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated in Table A.4.3.9-4e |
| 5 | UE supporting 8Rx in FR1 | 38.101-1, 7.3 | Rel-17 | pc_FR1_8Rx | If the capability is supported then the Band(s) for which it is supported shall be indicated by the UE |

Table A.4.3.1-8: Void

Table A.4.3.1-9: NR Sidelink FR1 RF Baseline Implementation Capabilities

| Item | NR Sidelink FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 2570-2620 MHz (Transmission), 2570-2620 MHz (Reception) | 38.101-1, 5.2E | Rel-16 | pc_nrBand38_NRSL_Supp | NR Sidelink FR1 Band n38 |
| 2 | NR Frequency band: 5855-5925 MHz (Transmission), 5855-5925 MHz (Reception) | 38.101-1, 5.2E | Rel-16 | pc_nrBand47_NRSL_Supp | NR Sidelink FR1 Band n47 |

Table A.4.3.1-10: NR FR2 PC7 RF Baseline Implementation Capabilities

| Item | NR FR2 PC7 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand257_PC7_Supp | NR FR2 PC7 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand258_PC7_Supp | NR FR2 PC7 Band n258 |
| 3 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-17 | pc_nrBand261_PC7_Supp | NR FR2 PC7 Band n261 |

Table A.4.3.1-11: FR1-NTN RF Baseline Implementation Capabilities (NTN satellite bands in FR1-NTN)

| Item | FR1-NTN RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | FR1-NTN Frequency band: 1980–2010 MHz (Transmission), 2170–2200 MHz (Reception) | 38.101-5, 5.2.2 | Rel-17 | pc_nrBand256_FR1_NTN | FR1-NTN Band n256 |
| 2 | FR1-NTN Frequency band: 1626.5–1660.5 MHz (Transmission), 1525–1559 MHz (Reception) | 38.101-5, 5.2.2 | Rel-17 | pc_nrBand255_FR1_NTN | FR1-NTN Band n255 |

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |
| 3 | MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver | 38.101-4, 5 | Rel-15 | pc_nr_mmse_irc_receiver | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference. |
| 4 | Baseline SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_baseline_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 |
| 5 | Simplified SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_simplified_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1 |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for Rel-15 FR2 power class 3 UE

| Item | Supported FR2 bands set | Ref. | Release | peak EIRP relaxation factor per band, MBp (dB) (Note 1) n257 | peak EIRP relaxation factor per band, MBp (dB) (Note 1) n258 | peak EIRP relaxation factor per band, MBp (dB) (Note 1) n260 | peak EIRP relaxation factor per band, MBp (dB) (Note 1) n261 | Maximum sum of MBp, ∑MBP (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.3 |  |
| 2 | n257, n260 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A |  | N/A | 1.0 |  |
| 3 | n258, n260 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  |  | N/A | 1.0 |  |
| 4 | n258, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  | N/A |  | 1.0 |  |
| 5 | n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A | N/A |  |  | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 | 38.101-2, 6.2.1.3 | Rel-15 |  |  |  | N/A | 1.7 |  |
| 7 | n257, n258, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A |  | 1.7 |  |
| 8 | n257, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A |  |  | 0.5 |  |
| 9 | n258, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  |  |  | 1.5 |  |
| 10 | n257, n258, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  |  |  |  | 1.7 |  |
| 11 | n257, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A | N/A |  | 0.0 | No relaxation factor allowed |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for Rel-15 FR2 power class 3 UE

| Item | Supported FR2 bands set | Ref. | Release | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) n257 | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) n258 | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) n260 | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) n261 | Maximum sum of MBs, ∑MBs (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.25 |  |
| 2 | n257, n260 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 3 | n258, n260 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 4 | n258, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  | N/A |  | 1.25 |  |
| 5 | n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A | N/A |  |  | 0.75 | No relaxation allowed for n260 |
| 6 | n257, n258, n260 | 38.101-2, 6.2.1.3 | Rel-15 |  |  |  | N/A | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 7 | n257, n258, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A |  | 1.75 |  |
| 8 | n257, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 9 | n258, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 | N/A |  |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 10 | n257, n258, n260, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  |  |  |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 11 | n257, n261 | 38.101-2, 6.2.1.3 | Rel-15 |  | N/A | N/A |  | 0.0 | No relaxation factor allowed |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 | Rel-15 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 | Rel-15 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 5 | FDD Band n5 | 38.101-1, 7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 7 | FDD Band n7 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 8 | FDD Band n8 | 38.101-1.7.3.2 | Rel-17 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 20 | FDD Band n20 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 25 | FDD Band n25 | 38.101-1, 7.3.2 | Rel-18 |  |
| 26 | FDD Band n26 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 28 | FDD Band n28 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 30 | FDD Band n30 | 38.101-1, 7.3.2 | Rel-16 |  |
| ... |  |  |  |  |
| 66 | FDD Band n66 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 70 | FDD Band n70 | 38.101-1, 7.3.2 | Rel-15 |  |
| 71 | FDD Band n71 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 34 | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 38 | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 40 | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |
| 48 | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |
| 77 | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3.2 |  |
| 5 | FDD Band n7 | 38.101-1, 7.3.2 | NOTE 2 |
| 6 | FDD Band n8 | 38.101-1, 7.3.2 |  |
| 7 | FDD Band n12 | 38.101-1, 7.3.2 |  |
| 7b | FDD Band n14 | 38.101-1, 7.3.2 |  |
| 8 | FDD Band n20 | 38.101-1, 7.3.2 |  |
| 8d | FDD Band n24 | 38.101-1, 7.3.2 |  |
| 9 | FDD Band n25 | 38.101-1, 7.3.2 |  |
| 9a | FDD Band n26 | 38.101-1, 7.3.2 |  |
| 10 | FDD Band n28 | 38.101-1, 7.3.2 |  |
| 10a | SDL Band n29 | 38.101-1, 7.3.2 |  |
| 10b | FDD Band n30 | 38.101-1, 7.3.2 |  |
| 10c | FDD Band n31 | 38.101-1, 7.3.2 |  |
| 11 | TDD Band n34 | 38.101-1, 7.3.2 |  |
| 12 | TDD Band n38 | 38.101-1, 7.3.2 | NOTE 2 |
| 13 | TDD Band n39 | 38.101-1, 7.3.2 |  |
| 14 | TDD Band n40 | 38.101-1, 7.3.2 |  |
| 15 | TDD Band n41 | 38.101-1, 7.3.2 | NOTE 2 |
| 16 | TDD Band n48 | 38.101-1, 7.3.2 |  |
| 17 | TDD Band n50 | 38.101-1, 7.3.2 |  |
| 18 | TDD Band n51 | 38.101-1, 7.3.2 |  |
| 18a | Reserved |  |  |
| 18b | TDD Band n53 | 38.101-1, 7.3.2 |  |
| 18c | TDD Band n54 | 38.101-1, 7.3.2 |  |
| 19 | FDD Band n65 | 38.101-1, 7.3.2 |  |
| 20 | FDD Band n66 | 38.101-1, 7.3.2 |  |
| 21 | FDD Band n70 | 38.101-1, 7.3.2 |  |
| 22 | FDD Band n71 | 38.101-1, 7.3.2 |  |
| 22a | FDD Band n72 | 38.101-1, 7.3.2 |  |
| 23 | FDD Band n74 | 38.101-1, 7.3.2 |  |
| 24 | TDD Band n77 | 38.101-1, 7.3.2 | NOTE 2 |
| 25 | TDD Band n78 | 38.101-1, 7.3.2 | NOTE 2 |
| 26 | TDD Band n79 | 38.101-1, 7.3.2 | NOTE 2 |
| 26a | FDD Band n85 | 38.101-1, 7.3.2 |  |
| 27 | FDD Band n91 | 38.101-1, 7.3.2 |  |
| 28 | FDD Band n92 | 38.101-1, 7.3.2 |  |
| 29 | FDD Band n93 | 38.101-1, 7.3.2 |  |
| 30 | FDD Band n94 | 38.101-1, 7.3.2 |  |
| 31 | FDD Band n100 | 38.101-1, 7.3.2 |  |
| 32 | TDD Band n101 | 38.101-1, 7.3.2 |  |
| 33 | FDD Band n106 | 38.101-1, 7.3.2 |  |
| 34 | FDD Band n109 | 38.101-1, 7.3.2 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |

Table A.4.3.9-4d: Enhanced transient capabilities

| Item | Band | Ref | Release | enhanced transient capability per band 2us | enhanced transient capability per band 4us | enhanced transient capability per band 7us | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 2 | FDD Band n2 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 3 | FDD Band n3 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 4 | FDD Band n5 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 5 | FDD Band n7 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 6 | FDD Band n8 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 7 | FDD Band n12 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 8 | FDD Band n14 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 9 | FDD Band n20 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 10 | FDD Band n24 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 11 | FDD Band n25 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 12 | FDD Band n26 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 13 | FDD Band n28 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 14 | FDD Band n30 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 15 | TDD Band n34 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 16 | TDD Band n38 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 17 | TDD Band n39 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 18 | TDD Band n40 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 19 | TDD Band n41 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 20 | TDD Band n46 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 21 | TDD Band n48 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 22 | TDD Band n50 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 23 | TDD Band n51 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 24 | TDD Band n53 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 25 | FDD Band n65 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 26 | FDD Band n66 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 27 | FDD Band n70 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 28 | FDD Band n71 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 29 | FDD Band n74 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 30 | TDD Band n77 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 31 | TDD Band n78 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 32 | TDD Band n79 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 33 | TDD Band n96 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported with enhanced transient capability of 2us, 4us or 7us if UE has indicated support of the capability defined in Table A.4.3.2-1/79. NOTE 2: Indicate transient capability for each band by ticking the cell corresponding to the smallest enhanced transient capability that the UE supports for that band. |  |  |  |  |  |  |  |

Table A.4.3.9-4e: 1 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3I.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3I.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3I.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3I.2 |  |
| 5 | FDD Band n8 | 38.101-1, 7.3I.2 |  |
| 6 | FDD Band n12 | 38.101-1, 7.3I.2 |  |
| 7 | FDD Band n13 | 38.101-1, 7.3I.2 |  |
| 8 | FDD Band n14 | 38.101-1, 7.3I.2 |  |
| 9 | FDD Band n18 | 38.101-1, 7.3I.2 |  |
| 10 | FDD Band n20 | 38.101-1, 7.3I.2 |  |
| 11 | FDD Band n24 | 38.101-1, 7.3I.2 |  |
| 12 | FDD Band n25 | 38.101-1, 7.3I.2 |  |
| 13 | FDD Band n26 | 38.101-1, 7.3I.2 |  |
| 14 | FDD Band n28 | 38.101-1, 7.3I.2 |  |
| 15 | FDD Band n30 | 38.101-1, 7.3I.2 |  |
| 16 | TDD Band n34 | 38.101-1, 7.3I.2 |  |
| 17 | TDD Band n39 | 38.101-1, 7.3I.2 |  |
| 18 | TDD Band n40 | 38.101-1, 7.3I.2 |  |
| 19 | TDD Band n50 | 38.101-1, 7.3I.2 |  |
| 20 | TDD Band n51 | 38.101-1, 7.3I.2 |  |
| 21 | TDD Band n53 | 38.101-1, 7.3I.2 |  |
| 21a | TDD Band n54 | 38.101-1, 7.3I.2 |  |
| 22 | FDD Band n65 | 38.101-1, 7.3I.2 |  |
| 23 | FDD Band n66 | 38.101-1, 7.3I.2 |  |
| 24 | FDD Band n70 | 38.101-1, 7.3I.2 |  |
| 25 | FDD Band n71 | 38.101-1, 7.3I.2 |  |
| 26 | FDD Band n74 | 38.101-1, 7.3I.2 |  |
| 27 | FDD Band n85 | 38.101-1, 7.3I.2 |  |
| 28 | FDD Band n91 | 38.101-1, 7.3I.2 |  |
| 29 | FDD Band n92 | 38.101-1, 7.3I.2 |  |
| 30 | FDD Band n93 | 38.101-1, 7.3I.2 |  |
| 31 | FDD Band n94 | 38.101-1, 7.3I.2 |  |
| 32 | FDD Band n100 | 38.101-1, 7.3I.2 | NOTE 2 |
| 33 | TDD Band n101 | 38.101-1, 7.3I.2 |  |
| 34 | Void |  |  |
| 35 | TDD Band n41 | 38.101-1, 7.3I.2 |  |
| 36 | TDD Band n77 | 38.101-1, 7.3I.2 |  |
| 37 | TDD Band n78 | 38.101-1, 7.3I.2 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/4. NOTE 2: HD-FDD is not supported. |  |  |  |

Table A.4.3.9-5: Beam Peak Search Vendor Declarations with respect to test frequency range for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1. |
| 3 | n260 | n260 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 5 | n261 | n261 single CC beam peak is leveraged from n257 single CC mid channel to n261 low, mid and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 2 |
| NOTE 1: The beam peak searches shall be performed for every test frequency range by default unless the device manufacturer explicitly declares that the beam peak at the mid test frequency range is applicable for the remaining (low, high) test frequency ranges. NOTE 2: Beam peak search results can be re-used from bands that completely contain the target bands if explicitly declared by the manufacturer. |  |  |  |  |  |

Table A.4.3.9-6: Beam Peak Search Vendor Declarations with respect to test frequency range for different CA BW classes

| Item | Bands | NR CA bandwidth class | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258, n260, n261 | A, B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | The beam peak is leveraged from a reference (frequency band, CBW) or (frequency band combination, CA BW class) to a group of other intra-band contiguous combinations and CA BW classes | 38.521-2, K.1.1 & K.1.2 | Rel-15 | A beam peak search shall be performed for every intra-band contiguous combination and CA BW class by default unless the device manufacturer explicitly declares that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes. |

Table A.4.3.9-7: Beam Peak Search Vendor Declarations with respect to modulation for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulations. |  |  |  |  |  |

Table A.4.3.9-8: Beam Peak Search Vendor Declarations with respect to waveform for single CC

| Item | Band | Intent | Reference Waveform | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform. |  |  |  |  |  |  |

Table A.4.3.9-9: Reference Point Vendor Declaration for grey-box test approach

| Item | Band | Positioning Reference Point: Offset (x/y/z) from geometric centre of DUT [cm] | Minimum QZ required to contain all active antennas within the quiet zone | Ref. | Release |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 2 | n258 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 3 | n260 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 4 | n261 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| NOTE: The available QZ sizes are defined in TS 38.508-1, Clause B.2.2.2, i.e., 20cm, 30cm, 40cm, and 55cm |  |  |  |  |  |

Table A.4.3.9-10: Vendor Declarations with respect to PC3 antenna configuration

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1a | n257 | n257 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 2a | n258 | n258 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 3a | n259 | n259 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-16 | NOTE 1 |
| 4a | n260 | n260 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 5a | n261 | n261 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 1b | n257 | n257 PC3 measurement grids can be relaxed based on 6x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 2 |
| 2b | n258 | n258 PC3 measurement grids can be relaxed based on 6x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 2 |
| 3b | n259 | n259 PC3 measurement grids can be relaxed based on 6x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-16 | NOTE 2 |
| 4b | n260 | n260 PC3 measurement grids can be relaxed based on 6x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 2 |
| 5b | n261 | n261 PC3 measurement grids can be relaxed based on 6x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 2 |
| NOTE 1: The fine PC3 measurement grids based on the 8x2 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with M ≤ 4 and N ≤ 2 for each band. NOTE 2: The fine PC3 measurement grids based on the 8x2 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with 4<M≤6 and N≤2 for each band. |  |  |  |  |  |

Table A.4.3.9-10a: Vendor Declarations with respect to PC5 antenna configuration

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 PC5 measurement grids can be relaxed based on 6x6 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 PC5 measurement grids can be relaxed based on 6x6 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| NOTE 1: The fine PC5 measurement grids based on the 12x12 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with M ≤ 6 and N ≤ 6 for each band. |  |  |  |  |  |

Table A.4.3.9-11: Antenna Aperture Vendor Declaration

| Item | Band | Antenna Aperture Declaration | Ref. | Release |
| --- | --- | --- | --- | --- |
| 1 | n257 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 2 | n258 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 3 | n260 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 4 | n261 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |

Table A.4.3.9-12: NR FR1 UL MIMO Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz, 2110-2170 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 1 |
| 2 | NR Frequency band: 1850-1910 MHz, 1930-1990 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 2 |
| 3 | NR Frequency band: 1710-1785 MHz, 1805-1880 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 3 |
| ... |  |  |  |
| 5 | NR Frequency band: 824-849 MHz , 869-894 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 5 |
| ... |  |  |  |
| 7 | NR Frequency band: 832-862 MHz, 791-821 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 7 |
| 8 | NR Frequency band: 880-915 MHz, 925-960 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 8 |
| ... |  |  |  |
| 24 | NR Frequency band: 1626.5-1660.5 MHz, 1525-1559 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 24 |
| 25 | NR Frequency band: 1850-1915 MHz, 1930-1995 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 25 |
| ... |  |  |  |
| 301 | NR Frequency band: 2305-2315 MHz, 2350-2360 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 30 |
| ... |  |  |  |
| 34 | NR Frequency band: 2010-2025 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 34 |
| ... |  |  |  |
| 38 | NR Frequency band: 2570-2620 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 38 |
| 39 | NR Frequency band: 1880-1920 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 39 |
| 40 | NR Frequency band: 2300-2400 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 40 |
| 41 | NR Frequency band: 2496-2690 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 41 |
| ... |  |  |  |
| 46 | NR Frequency band: 5150-5925 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 46 |
| ... |  |  |  |
| 48 | NR Frequency band: 3550-3700 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 48 |
| ... |  |  |  |
| 66 | NR Frequency band: 1710-1780 MHz, 2110-2200 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 66 |
| ... |  |  |  |
| 70 | NR Frequency band: 1695-1710 MHz, 1995-2020 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 70 |
| 712 | NR Frequency band: 663-698 MHz, 617-652 MHz | 38.101-1, 5.2D | NR FDD FR1 Band 71 |
| ... |  |  |  |
| 77 | NR Frequency band: 3300–4200 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 77 |
| 78 | NR Frequency band: 3300–3800 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 78 |
| 79 | NR Frequency band: 4400–5000 MHz | 38.101-1, 5.2D | NR TDD FR1 Band 79 |
| 80 | NR Frequency band: 1710–1785 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 80 |
| 84 | NR Frequency band: 1920–1980 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 84 |
| 95 | NR Frequency band: 2010–2025 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 95 |
| 97 | NR Frequency band: 2300–2400 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 97 |
| 98 | NR Frequency band: 1880–1920 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 98 |
| 99 | NR Frequency band: 1626.5–1660.5 MHz | 38.101-1, 5.2D | NR SUL FR1 Band 99 |
| NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor. |  |  |  |

Table A.4.3.9-13: NR FR2 UL MIMO Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 257 | NR Frequency band: 26500-29500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 257 |
| 258 | NR Frequency band: 24250-27500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 258 |
| 259 | NR Frequency band: 39500-43500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 259 |
| 260 | NR Frequency band: 37000–40000 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 260 |
| 261 | NR Frequency band: 27500–28350 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 261 |

Table A.4.3.9-14: NR FR1 TxD Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz, 2110-2170 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n1 |
| 2 | NR Frequency band: 1850-1910 MHz, 1930-1990 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n2 |
| 3 | NR Frequency band: 1710-1785 MHz, 1805-1880 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n3 |
| ... |  |  |  |
| 5 | NR Frequency band: 824-849 MHz , 869-894 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n5 |
| ... |  |  |  |
| 7 | NR Frequency band: 832-862 MHz, 791-821 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n7 |
| 8 | NR Frequency band: 880-915 MHz, 925-960 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n8 |
| … |  |  |  |
| 12 | NR Frequency band: 699-716 MHz, 729-746 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n12 |
| 13 | NR Frequency band: 777-787 MHz (UL), 746-756 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n13 |
| 14 | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n14 |
| … |  |  |  |
| 18 | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n18 |
| … |  |  |  |
| 20 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n20 |
| ... |  |  |  |
| 24 | NR Frequency band: 1626.5-1660.5 MHz, 1525-1559 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n24 |
| 25 | NR Frequency band: 1850-1915 MHz, 1930-1995 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n25 |
| 26 | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n26 |
| … |  |  |  |
| 28 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n28 |
| … |  |  |  |
| 301 | NR Frequency band: 2305-2315 MHz, 2350-2360 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band n30 |
| ... |  |  |  |
| 34 | NR Frequency band: 2010-2025 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n34 |
| ... |  |  |  |
| 38 | NR Frequency band: 2570-2620 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n38 |
| 39 | NR Frequency band: 1880-1920 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n39 |
| 40 | NR Frequency band: 2300-2400 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n40 |
| 41 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n41 |
| ... |  |  |  |
| 48 | NR Frequency band: 3550-3700 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n48 |
| … |  |  |  |
| 50 | NR Frequency band: 1432-1517 MHz (UL / DL) | 38.101-1, 6.2G.1 | NR TDD FR1 Band n50 |
| 51 | NR Frequency band: 1427-1432 MHz (UL / DL) | 38.101-1, 6.2G.1 | NR TDD FR1 Band n51 |
| … |  |  |  |
| 53 | NR Frequency band: 2483.5-2495 MHz (UL / DL) | 38.101-1, 6.2G.1 | NR TDD FR1 Band n53 |
| 54 | NR Frequency band: 1670-1675 MHz (UL / DL) | 38.101-1, 6.2G.1 | NR TDD FR1 Band n54 |
| ... |  |  |  |
| 65 | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n65 |
| 66 | NR Frequency band: 1710-1780, 2110-2200 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band 66 |
| ... |  |  |  |
| 70 | NR Frequency band: 1695-1710, 1995-2020 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band 70 |
| 71 | NR Frequency band: 663-698 MHz, 617-652 MHz | 38.101-1, 6.2G.1 | NR FDD FR1 Band 71 |
| … |  |  |  |
| 74 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n74 |
| ... |  |  |  |
| 77 | NR Frequency band: 3300–4200 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band 77 |
| 78 | NR Frequency band: 3300–3800 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band 78 |
| 79 | NR Frequency band: 4400–5000 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band 79 |
| … |  |  |  |
| 91 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n91 |
| 92 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n92 |
| 93 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n93 |
| 94 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n94 |
| 100 | NR Frequency band: 874.4-880 MHz (UL), 919.4-925 MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n100 |
| 101 | NR Frequency band: 1900–1910 MHz (UL / DL) | 38.101-1, 6.2G.1 | NR TDD FR1 Band n101 |
| 102 | NR Frequency band: 5925 MHz – 6425 MHz | 38.101-1, 6.2G.1 | NR TDD FR1 Band n102 |
| … |  |  |  |
| 109 | NR Frequency band: 703 MHz – 733 MHz (UL), 1432 MHz – 1517MHz (DL) | 38.101-1, 6.2G.1 | NR FDD FR1 Band n109 |
| NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. |  |  |  |