3GPP TSG-RAN WG5 Meeting #105	R5-247767

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3056 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update ASE TC for n41 CBW 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-9-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ASE TC needs to be updated for adding n41 CBW 5MHz according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ASE TC has been updated for adding n41 CBW 5MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Align with TS 38.101-1 i70 version |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.5.3.3	Additional spurious emissions

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined.

.

Test requirement for NS_38, NS_39, NS_40 are incomplete.

The requirements of this test case for NS_44, NS_46, NS_47, NS_48, and NS_49 apply to all types of NR UE release 16 forward, and release 15 if the corresponding channel bandwidths are supported.

Clarification from RAN4 is needed for NOTE 1 in Table 6.5.3.3.3.10-1 of 38.101-1 as a UE's output power may not be able to reach 15dBm.

6.5.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5.3.3.2	Test applicability

This test case applies for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_47, NS_48, NS_49, NS_50, NS_56 and NS_62 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.5.3.3.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.3.3.1	Minimum conformance requirements (network signalling value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.1-1.  This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.1-1: Additional requirements for “NS_04”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.3.1.

<< UNCHANGED PARTS SKIPPED >>

6.5.3.3.5.1	Test requirement (network signalling value “NS_04”)

When "NS_04" is indicated in the cell,

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-2 for power class 2 UE, and Table 6.2.3.5-3 for power class 3 UE.

The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5.1-1: Additional requirements for "NS_04"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 (MHz) |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

<< END OF CHANGES >>