3GPP TSG-RAN5 Meeting #105	R5-247796

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0052 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 253 into test case 7.3A Reference sensitivity power level for UE category M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_extLband-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Reference sensitivity power level requirement for category M1 band 253 is missing and needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Reference sensitivity power level requirement for category M1 band 253 have been added into clause 7.3A.1 Updated minimum requirements into clause 7.3A.3 Typo corrected in Table 7.3A.3-3. Updated test requirement into clause 7.3A.5 Added MU and TT for the TC in Annex F |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 253 Reference sensitivity power level requirement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A, F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-246298->R5-247796 |  |  |  |  |  |  |  |  |

<Start of Change1 >

7.3A	Reference sensitivity power level for UE category M1

7.3A.1	Test purpose

To verify the category M1 UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area of an e-NodeB.

7.3A.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category M1 that support satellite access operation.

7.3A.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A.3-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A.3-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3A.3-1/Table 7.3A.3-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A.3-3.

NOTE:	Table 7.3A.3-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G] (informative).

Table 7.3A.3-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 253 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.3A.1.

<End of Change1 >

<Skipped Unchanged >

<Start of Change2 >

7.3A.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 with parameters specified in Table 7.3A.5-1 and Table 7.3A.5-2.

Table 7.3A.5-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.0 | FDD |
| 254 | -101.5 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A.5-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.8 | HD-FDD |
| 254 | -102.4 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

<End of Change2 >

<Skipped Unchanged >

<Start of Change3 >

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3A Reference sensitivity power level for UE category M1 | Same as clause 7.3EA in TS 36.521-1 [14] | Same as clause 7.3EA in TS 36.521-1 [14] |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | Same as clause 7.3F.1 in TS 36.521-1 [14] |  |
| 7.4A Maximum input level for category M1 | Same as clause 7.4EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 7.4B Maximum input level for category NB1 and NB2 | Same as clause 7.4F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 7.5A Adjacent Channel Selectivity for category M1 | Same as clause 7.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 | Same as clause 7.5F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |  |
| 7.6A.2 In-band blocking for category M1 | Same as clause 7.6.1EA in TS 36.521-1 [14]. | Same as clause 7.6.1EA in TS 36.521-1 [14]. |
| 7.6A.3 Out-of-band blocking for category M1 | Same as clause 7.6.2EA in TS 36.521-1 [14]. | Same as clause 7.6.2EA in TS 36.521-1 [14]. |
| 7.6A.4 Narrow band blocking for category M1 | Same as clause 7.6.3EA in TS 36.521-1 [14]. | Same as clause 7.6.3EA in TS 36.521-1 [14]. |
| 7.6B.2 In-band blocking for category NB1 and NB2 | Same as clause 7.6.1F in TS 36.521-1 [14]. | Same as clause 7.6.1F in TS 36.521-1 [14]. |
| 7.6B.3 Out-of-band blocking for category NB1 and NB2 | Same as clause 7.6.2F in TS 36.521-1 [14]. | Same as clause 7.6.2F in TS 36.521-1 [14]. |
| 7.7A Spurious response for category M1 | Same as clause 7.6A.3. | Same as clause 7.6A.3. |
| 7.7B Spurious response for category NB1 and NB2 | Same as clause 7.6B.3. | Same as clause 7.6B.3. |
| 7.8A Intermodulation characteristics for category M1 | Same as clause 7.8.1EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.8.1EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 7.8B Intermodulation characteristics for category NB1 and NB2 | Same as clause 7.8.1F in TS 36.521-1 [14]. | Same as clause 7.8.1F in TS 36.521-1 [14]. |
| 7.9A Spurious emissions for category M1 | Same as clause 7.9EA in TS 36.521-1 [14] except for the spurious frequency range to be limited to up to 12.75GHz. |  |
| 7.9B Spurious emissions for category NB1 and NB2 | Same as clause 7.9F in TS 36.521-1 [14]. |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

<End of Change3 >

<Skipped Unchanged >

<Start of Change4 >

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 7.3A Reference sensitivity power level for UE category M1 | Same as clause 7.3EA in TS 36.521-1 [14] | Same as clause 7.3EA in TS 36.521-1 [14] | Same as clause 7.3EA in TS 36.521-1 [14] |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | Same as clause 7.3F.1 in TS 36.521-1 [14] | Same as clause 7.3F.1 in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.3F.1 in TS 36.521-1 [14] |
| 7.4A Maximum input level for category M1 | Same as clause 7.4EA in TS 36.521-1 [14] | Same as clause 7.4EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.4EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 7.4B Maximum input level for category NB1 and NB2 | Same as clause 7.4F in TS 36.521-1 [14] | Same as clause 7.4F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.4F in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. |
| 7.5A Adjacent Channel Selectivity for category M1 | Same as clause 7.5EA in TS 36.521-1 [14] for FDD band with “Case 1, channel bandwidth=1.4MHz” and “Case 2, channel bandwidth=1.4MHz”. | Same as clause7.5EA in TS 36.521-1 [14] for FDD band with “f ≤ 3.0GHz”. | Same as clause7.5EA in TS 36.521-1 [14] |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 | Same as clause 7.5F in TS 36.521-1 [14] | Same as clause 7.5F in TS 36.521-1 [14] | Same as clause 7.5F in TS 36.521-1 [14] |
| 7.6A.2 In-band blocking for category M1 | Same as clause 7.6.1EA in TS 36.521-1 [14]. | Same as clause 7.6.1EA in TS 36.521-1 [14]. | Same as clause 7.6.1EA in TS 36.521-1 [14]. |
| 7.6A.3 Out-of-band blocking for category M1 | Same as clause 7.6.2EA in TS 36.521-1 [14]. | Same as clause 7.6.2EA in TS 36.521-1 [14]. | Same as clause 7.6.2EA in TS 36.521-1 [14]. |
| 7.6A.4 Narrow band blocking for category M1 | Same as clause 7.6.3EA in TS 36.521-1 [14]. | Same as clause 7.6.3EA in TS 36.521-1 [14]. | Same as clause 7.6.3EA in TS 36.521-1 [14]. |
| 7.6B.2 In-band blocking for category NB1 and NB2 | Same as clause 7.6.1F in TS 36.521-1 [14]. | Same as clause 7.6.1F in TS 36.521-1 [14]. | Same as clause 7.6.1F in TS 36.521-1 [14]. |
| 7.6B.3 Out-of-band blocking for category NB1 and NB2 | Same as clause 7.6.2F in TS 36.521-1 [14]. | Same as clause 7.6.2F in TS 36.521-1 [14]. | Same as clause 7.6.2F in TS 36.521-1 [14]. |
| 7.7A Spurious response for category M1 | Same as clause 7.7EA in TS 36.521-1 [14]. | Same as clause 7.7EA in TS 36.521-1 [14]. | Same as clause 7.7EA in TS 36.521-1 [14]. |
| 7.7B Spurious response for category NB1 and NB2 | Same as clause 7.7F in TS 36.521-1 [14]. | Same as clause 7.7F in TS 36.521-1 [14]. | Same as clause 7.7F in TS 36.521-1 [14]. |
| 7.8A Intermodulation characteristics for category M1 | Same as clause 7.8.1EA in TS 36.521-1 [14]. | Same as clause 7.8.1EA in TS 36.521-1 [14]. | Same as clause 7.8.1EA in TS 36.521-1 [14]. |
| 7.8B Intermodulation characteristics for category NB1 and NB2 | Same as clause 7.8.1F in TS 36.521-1 [14]. | Same as clause 7.8.1F in TS 36.521-1 [14]. | Same as clause 7.8.1F in TS 36.521-1 [14]. |
| 7.9A Spurious emissions for category M1 | Same as clause 7.9EA in TS 36.521-1 [14] except for the spurious frequency range to be limited to up to 12.75GHz. | Same as clause 7.9EA in TS 36.521-1 [14]. | Same as clause 7.9EA in TS 36.521-1 [14]. |
| 7.9B Spurious emissions for category NB1 and NB2 | Same as clause 7.9F in TS 36.521-1 [14]. | Same as clause 7.9F in TS 36.521-1 [14]. | Same as clause 7.9F in TS 36.521-1 [14]. |

<End of Change4 >