3GPP TSG-RAN5 Meeting #105	R5-247966

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | 0019 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Browsing Mode FR1 TRP and TRS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_TRP_TRS_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements for Browsing mode FR1 TRP and TRS tests have been introduced in the core specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the minimum conformance requirements for Browsing Mode TRP and TRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing minimum conformance requirements for Browsing Mode TRP TRS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1, 7.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 updates the NOTE in minimum conformance requirements section. And also updates the TT for n1 and n28 as MU/TT for browsing mode has already been completed for this frequency range.Along with a correction within tables with missing entries for n1. R5-247496r1 -> R5-247966 |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

##### 6.2.1.1.1	Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom

6.2.1.1.1.1	Test Purpose

To verify that the total radiated power (TRP) of a 5G NR FR1 UE, in browsing mode with Hand phantom only, does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.1.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE Release 17 and forward that do not support Transmit Diversity.

6.2.1.1.1.3	Minimum conformance requirements

The average measured Total Radiated Power (TRP) of low, mid and high channel for handheld UE shall be higher than the average TRP requirement specified in subclauses 6.2.1.1 and 6.2.1.2. The averaging shall be done in linear scale for the TRP results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRP results of both right and left hand phantom measurements.

TRPaverage=10log10Pleft_low/10+10Pleft_mid/10+10Pleft_high/10+10Pright_low/10+10Pright_mid/10+10Pright_high/106

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of the present document.

6.2.1.1.1.3.1	Hand phantom browsing mode

Hand phantom browsing mode positions are defined in Clause B.3.1.

6.2.1.1.1.3.2	Minimum conformance requirements for NR FR1 in hand phantom browsing position

Handheld UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode are defined in Tables 6.2.1.1.1.3.2-1 and 6.2.1.1.1.3.2-2.

Table 6.2.1.1.1.3.2-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  | 11.6 |
| n28 | 20 | HL and HR |  | 101 |
| n41 | 100 | HL and HR |  | 9.5 |
| n78 | 100 | HL and HR |  | 10 |
| n79 | 100 | HL and HR |  |  |
| NOTE 1: 0.5dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. |  |  |  |  |

Table 6.2.1.1.1.3.2-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n28 | 20 | HL and HR |  |  |
| n41 | 100 | HL and HR |  | 12.5 |
| n78 | 100 | HL and HR |  | 13 |
| n79 | 100 | HL and HR |  |  |

The normative reference for this requirement is TS 38.161 [1] clause 6.2.1.1.1.

6.2.1.1.1.4	Test description

6.2.1.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 5.3-1. The details of the uplink reference measurement channels (RMCs) mainly comprising the uplink modulation and uplink RB allocation are also specified in Table 5.3-1 . Configurations of PDSCH and PDCCH before measurement are specified in Annex A.0

The following steps are recommended as per TR 38.834 [10]:

-	P-MPRc shall be 0 dB.

-	FR1 TRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions being OFF should be based on manufacturer declaration; if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

-	The NR SS should send continuous uplink power control “up” commands to the DUT to ensure the DUT’s transmitter is at maximum output power during the SA TRP and TRS test.

-	It is recommended to disable Transmit Antenna Switching (TAS) as per the procedure defined in Annex A.3.6

1.	For DUT containing multiple Tx antennas, it shall be ensured that the Tx Antenna Switching (TAS) function should be OFF, and the TRP should be measured for each Tx antenna individually.

NOTE: The primary antenna should be selected based on manufacturer declaration. In the absence of manufacturer declaration, the antenna with better TRP is identified as the primary antenna, and the corresponding TRP result will be used to determine the pass/fail compliance.

2.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [2] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

3.	The parameter settings for the cell are set up according to TS 38.508-1 [2] subclause 4.4.3.

4.	Downlink signals are initially set up according to Annex C in TS 38.521-1 [11and uplink signals according to Annex G in TS. 38.521-1 [11].

5.	The UL and DL parameters are set according to Table 5.3-1.

6.	Propagation conditions are set to Static.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [2] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.1.1.4.2	Test procedure

For TRP measurement, the evaluations shall be performed at maximum transmit power. The applicability rules for testing of UE power class in clause 4.4 shall be followed. The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the UE positioning guidelines defined in Annex B.3.1.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 5.3-1. For any additional RMC parameters beyond Table 5.3-2, refer to Table 6.2.1.4.1-1 of TS 38.521-1 [11].. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Ensure the DUT transmits with its maximum power by continuously sending uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	The TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

5.	The measurement is performed with a constant sampling step in both theta () and phi () axes using any of the measurement grids and quadrature options outlined and applicable to TRP in Table A.4.2.12-1. Any of the measurement grids in Table A.4.2.12-1 could be used for testing. Measurements at theta = 0 and 180 degrees only require one measurement each. For test systems that cannot measure 180º EIRP, an extrapolation approach shall be adopted when generating the 3D antenna pattern and calculating TRP.

6.	All the measured power values will be integrated to TRP, as defined in Annex A.3.5.1 with the procedure further detailed in A.3.3.2.

6.2.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.2.1.1.1.5	Test requirement

The TRP across low, mid and high channels tested with hand phantom browsing mode position shall be higher than test performance requirements shown in Table 6.2.1.1.1.5-1 for Power Class 3 and 6.2.1.1.1.5-2 for Power Class 2.

Table 6.2.1.1.1.5-1: Handheld PC3 UE TRP test requirements for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR | FFS | 11.6-TT |
| n28 | 20 | HL and HR | FFS | 101-TT |
| n41 | 100 | HL and HR | FFS | 9.5-TT |
| n78 | 100 | HL and HR | FFS | 10-TT |
| n79 | 100 | HL and HR | FFS | FFS |
| NOTE 1: 0.5dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. |  |  |  |  |

Table 6.2.1.1.1.5-2: Handheld PC2 UE TRP test requirements for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n28 | 20 | HL and HR | FFS | FFS |
| n41 | 100 | HL and HR | FFS | 12.5-TT |
| n78 | 100 | HL and HR | FFS | 13-TT |
| n79 | 100 | HL and HR | FFS | FFS |

Table 6.2.1.1.1.5-3: Test Tolerance (NR FR1 TRP)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n1 | 1.02 |
| n28 | 1.02 |
| n41 | 1.02 |
| n78 | 1.07 |

<Unchanged sections skipped>

##### 7.2.1.1.1	Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom

7.2.1.1.1.1	Test purpose

The purpose of this test is to ensure that TRSaverage of the UE, in browsing mode with hand phantom only, is above specified limit. The lack of the reception sensitivity decreases the coverage area at the far side from Node B.

7.2.1.1.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UEs Release 17 and forward.

7.2.1.1.1.3	Minimum conformance requirements

The average measured total radiated sensitivity (TRS) of low, mid and high channel for handheld UE shall be lower than the average TRS requirement specified in subclauses 7.2.1.1 and 7.2.1.2. The averaging shall be done in linear scale for the TRS results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRS results of both right and left hand phantom measurements.

TRSaverage=10log6/(110Pleft_low10+110Pleft_mid10+110Pleft_high10+110Pright_low10+110Pright_mid10+110Pright_high10)

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of the present document.

7.2.1.1.1.3.1	Hand phantom browsing mode

Hand phantom browsing mode positions are defined in Clause B.3.1.

7.2.1.1.1.3.2	Minimum conformance requirements for NR FR1 in hand phantom browsing position

Handheld UE TRS minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode are defined in Tables 7.2.1.1.1.3.2-1.

Table 7.2.1.1.1.3.2-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  | -88.5 |
| n28 | 20 | HL and HR |  | -83.5 |
| n41 | 100 | HL and HR |  | -80 |
| n78 | 100 | HL and HR |  | -81.2 |
| n79 | 100 | HL and HR |  |  |

The normative reference for this requirement is TS 38.161 [1] clause 7.2.1.1.1.

7.2.1.1.1.4	Test description

7.2.1.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations that the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2.1-1. All these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 5.3-2. The details of the uplink and downlink test parameters mainly comprising the uplink modulation and uplink RB allocation are also specified in Table 5.3-2. Configurations of PDSCH and PDCCH before measurement are specified in Annex A.0

The following steps are recommended as per TR 38.834 [10]:

-	P-MPRc shall be 0 dB.

-	FR1 TRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions being OFF should be based on manufacturer declaration; if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

-	The NR SS should send continuous uplink power control “up” commands to the DUT to ensure the DUT’s transmitter is at maximum output power during the SA TRS test.

-	For TRS measurement, no specific setting is needed for Rx antennas. By default, the maximum number of Rx antennas supported at each band should be enabled during the TRS test.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [2] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [2] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C in TS 38.521-1 [11] and uplink signals according to Annex G in TS 38.521-1 [11].

4.	The UL and DL parameters are set according to Table 5.3-2.

5.	Propagation conditions are set to Static.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [2] clause 4.5. Message contents are defined in clause 7.2.1.4.3.

7.2.1.1.1.4.2	Test procedure

For TRS measurement, the evaluations shall be performed at maximum transmit power. The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the UE positioning guidelines defined in Annex B.3.1.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.3-2. For any additional RMC parameters beyond Table 5.3-2 refer to Table 7.3.2.4.1-1 of TS 38.521-1 [11]. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL parameters defined in Table 5.3-2 and any additional parameters from Table 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

4.	Ensure the DUT transmits with its maximum power by continuously sending uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

5.	The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. Determine each EIS by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

6.	The measurement is performed with a constant sampling step of in both theta () and phi () axes using any of the measurement grids and quadratures' options outlined and applicable to TRS in Table A.4.2.12-1. Any of the measurement grids in Table A.4.2.12-1 could be used for testing. Measurements at theta = 0 and 180 degrees only require one measurement each. For test systems that cannot measure 180º EIS, an extrapolation approach shall be adopted when generating the 3D antenna pattern and calculating TRS.

7.	All of the measured power values at each position or measurement angle will be integrated to TRS, as defined in Annex A.3.5.2. with the procedure further detailed in Annex A.3.3.3

8.	The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If the alternate EIS search procedure is applied as described in A.3.3.3.1 to assess the EIS at each grid point, no correction of TRS shall be applied.

7.2.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 7.2.1.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

7.2.1.1.1.5	Test requirement

The TRS across low, mid and high channels tested with hand phantom browsing mode position shall be lower than test performance requirements shown in Table 7.2.1.1.1.5-1.

Table 7.2.1.1.1.5-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR | FFS | -88.5+TT |
| n28 | 20 | HL and HR | FFS | -83.5+TT |
| n41 | 100 | HL and HR | FFS | -80+TT |
| n78 | 100 | HL and HR | FFS | -81.2+TT |
| n79 | 100 | HL and HR | FFS | FFS |

Table 7.2.1.1.1.5-2: Test Tolerance (NR FR1 TRS)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n1 | 1.28 |
| n28 | 1.28 |
| n41 | 1.28 |
| n78 | 1.32 |

<End of changes>