| 3GPP TR 23.946 V1.0.0 (2024-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Guidelines for CAPIF Usage; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview of CAPIF	7

4.1	Introduction	7

4.2	Functional Architecture	7

4.3	Functional Entities	9

4.4	RNAA and CAPIF	9

4.5	Relationship between RNAA and OAuth 2.0	10

4.6	Overview CAPIF operations	11

4.6.1	General	11

4.6.2	Usage of CAPIF by API invoker	11

4.6.3	Usage of CAPIF by API provider	12

5	Role of Stakeholders for API Exposure	15

5.1	Stakeholders in CAPIF	15

5.2	Basic roles	15

5.2.1	CAPIF provider	15

5.2.2	Service API provider	15

5.2.3	API invoker	16

5.2.4	Resource owner	16

5.3	Mapping of stakeholders to CAPIF roles	16

5.4	Usage of CAPIF in different phases of an application business lifecycle	17

6	Exemplary Use Cases and Adoption	18

6.1	The deployment of the API Invoker as AF on the UE for RNAA	18

6.2	The deployment of the API Invoker as AF on the network for RNAA	20

6.3	GSMA Operator Platform deployment via CAPIF	22

6.4	CAPIF Vendor Extensibility	23

6.5	ETSI MEC deployment based on CAPIF	23

6.6	MEC Platform as an API Provider	24

6.6.1	Introduction	24

6.6.2	Registering the MEC Platform as an API Provider in CAPIF	24

6.6.3	Publishing an API for the MEC Platform	25

6.7	Open-source implementation of CAPIF	27

7	Summary	28

Annex A: Examples from OpenCAPIF	29

Annex B: CAPIF Vendor Extensibility	29

B.1	Include Vendor Specific information	29

B.2	Query for Vendor Specific information	30

B.3	ETSI MEC extensibility to CAPIF	30

Annex C: Open-source implementation of CAPIF	33

C.1	NEF Publishes an API	33

C.2	API consumption from an API Invoker	45

Annex D: CAPIF Test cases	56

D.1	General	56

D.2	Test Plan for CAPIF API Invoker Management	56

D.3	Test Plan for CAPIF API Provider Management	61

D.4	Test Plan for CAPIF API Publish Service	65

D.5	Test Plan for CAPIF Discover Service API	75

D.6	Test Plan for CAPIF API Security Service	79

D.7	 Test Plan for CAPIF API Access Control Policy Service	96

D.8	Test Plan for CAPIF API Logging Service	107

D.9	Test Plan for CAPIF API Events Service	110

D.10	Test Plan for CAPIF API Auditing Service	121

Annex E: Change history	125

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This TR provides guidelines for CAPIF usage for the benefit of the Application developer and API provider communities.

# 1	Scope

This TR provides guidelines for CAPIF usage for the benefit of the Application developer and API provider communities. This document also describes the usage and deployment options in CAPIF.

The work takes into consideration the work done for CAPIF in 3GPP TS 23.222 [2] and 3GPP TS 29.222 [3] and 3GPP TS 33.122 [4], and other related work outside 3GPP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 (V3.3.1), "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations".

[7]	EVOLVED-5G ICT-41 project, "EVOLVED-5G", https://evolved-5g.eu.

[8]	EVOLVED-5G NEF emulator (V2.2.3), "EVOLVED-5G NEF emulator", https://github.com/EVOLVED-5G/NEF_emulator.

[9]	ETSI Software Development Group OpenCAPIF, "OpenCAPIF", https://ocf.etsi.org.

[10]	ETSI Labs OCF capif, "capif", https://labs.etsi.org/rep/ocf/capif.

[11]		GSMA PRD OPG.02 - "Operator Platform Telco Edge Requirements Version 7.0", https://www.gsma.com/solutions-and-impact/technologies/networks/wp-content/uploads/2024/09/OPG.02-v7.0-Operator-Platform-Requirements-and-Architecture.pdf.

[12]	OpenCAPIF ETSI SDG OCF Documentation – Postman, "ETSI SDG OCF Documentation - Postman", https://ocf.etsi.org/documentation/develop/testing/postman/.

[13]	OpenCAPIF ETSI SDG OCF Documentation - Common Operations, "ETSI SDG OCF Documentation - Common Operations", https://ocf.etsi.org/documentation/develop/testing/testplan/common_operations/.

[14]		3GPP TS 29.122: "T8 reference point for Northbound APIs".

[15]	IETF RFC 6749 (October 2012): "The OAuth 2.0 Authorization Framework".

[16]		3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the terms given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the abbreviations given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

# 4	Overview of CAPIF

## 4.1	Introduction

3GPP specifies multiple northbound API-related specifications. To avoid duplication and inconsistency of approaches between different API specifications and to specify common services (e.g. authorization), 3GPP developed a common API framework (CAPIF) that includes aspects applicable to any northbound service APIs. The key CAPIF specifications are:

-	3GPP TS 23.222 [2] defines the CAPIF architecture and procedures;

-	3GPP TS 29.222 [3] defines the API messages and protocol for CAPIF APIs; and

-	3GPP TS 33.122 [4] defines CAPIF security procedures.

## 4.2	Functional Architecture

The CAPIF functional architecture is specified in 3GPP TS 23.222 [2] and consists of CAPIF core function, API Provider Domain Functions and API invoker. CAPIF architecture is based on the well-known Service Oriented Architecture (SOA) design paradigm, where a Service producer (i.e. API provider) is able to publish (1) the offered service APIs which can be discovered (2) by the Service consumers (e.g. API invokers) and further can invoke (3) the discovered service APIs as per the authorization.

Figure 4.2-1: CAPIF based on SOA

Figure 4.2-1 provides an illustration of CAPIF based on SOA with the following relationship:

1.	The functionalities related to Service Access Controller are supported by CAPIF core function.

2.	The Service producer is the API Provider Domain Functions.

3.	The Service consumer is the API Invoker.

Most CAPIF APIs are provided by a CAPIF core function entity and can be consumed by applications (API invokers) and application providers (CAPIF defines an API provider as three functions: API exposure, API publishing, and API management). CAPIF core function APIs enable onboarding, publishing, discovery, authentication, registration, authorization, logging, charging, monitoring, configuration, topology hiding, and other procedures. One CAPIF API (AEF security) is provided by the API exposure function.

Figure 4.2-2: Functional model for the CAPIF to support 3rd party API providers

(adapted from 3GPP TS 23.222 [2] Figure 6.2.1-1: Functional model for the CAPIF to support 3rd party API providers).

The CAPIF architecture includes multiple deployment models e.g. centralized vs. distributed, single vs. multiple API providers (see 3GPP TS23.222 [2] clause 7).

## 4.3	Functional Entities

API invoker: Typically provided by a 3rd party application provider who has service agreement with the PLMN operator.

The API invoker supports several CAPIF capabilities such as supporting authentication, obtaining authorization, and discovering service APIs and invoking service APIs.

CAPIF core function (CCF): Supports capabilities used by other functional entities, for example the following:

-	For the API invoker: authenticating the API invoker, providing authorization information, and service API discovery.

-	For the API exposing function: (AEF) providing the service API access policy, providing API routing information, and charging of service API invocations.

-	For the API publishing function: publishing and storing the service APIs information.

-	For the API management function: providing the service API invocation log for auditing, storing configurations of the API provider policies, and updating registration information of API provider domain functions.

A CAPIF core function can also interact with another CAPIF core function for API publishing and discovery.

API exposing function (AEF): The API exposing function (AEF) is the provider of the Service APIs and is also the service communication entry point of the Service API to API invokers.

API publishing function (APF): The API publishing function (APF) enables the API provider to publish the Service APIs information to enable the discovery of Service APIs by the API invoker.

API management function (AMF): The API management function (AMF) enables the API provider to manage service APIs such as querying the Service API invocation log for auditing, monitoring the events, and configuring the API provider policies.

Authorization function: The authorization function enables receiving authorization from the resourse owner function and providing the API invoker with the authorization information which is needed to access the resource owner’s resources. This function is used in RNAA scenarios.

Resource owner function (ROF): The resource owner function enables the authorization for resource access and managing and revoking authorization for resource access. This function is used in RNAA scenarios.

## 4.4	RNAA and CAPIF

RNAA (Resource owner-aware Northbound API Access) is an OAuth 2.0-based authorization mechanism for CAPIF and enables API invokers to have authorized access to resources of a resource owner provided by service APIs offered by the API exposing function.

The relationship between the RNAA and CAPIF is described in this clause. The CAPIF architecture given in clause 6.2.0 of 3GPP TS 23.222 [2] can use OAuth 2.0 token's mechanism to authorize API invokers. As per OAuth 2.0, API invoker performs the function of the client, the CAPIF core function performs the function of the authorization server, and the API exposing function performs the exposure of the protected resources. The API invoker is authorized with an authorization grant type of client credentials described in clause 6.5 of 3GPP TS 33.122 [4].

The RNAA architecture given in clause 6.2.3 of 3GPP TS 23.222 [2] supports an authorization grant type of authorization code grant. The resource owner function in the RNAA architecture has the role of the resource owner, authorizing the API invoker to invoke the API exposing function.

Figure 4.4-1 shows the architectural model for the RNAA which allows the resource owner to provide authorization to the API invocation. The authorization function and the resource owner function are used in RNAA scenarios.

Figure 4.4-1: High level functional architecture for CAPIF supporting RNAA

## 4.5	Relationship between RNAA and OAuth 2.0

This clause shows the relationship between RNAA and OAuth 2.0. The Table 4.5-1 shows the mapping of OAuth 2.0 and RNAA. The details of OAuth 2.0 are specified in IETF RFC 6749 [15] and the details of RNAA are in 3GPP TS 23.222 [2].

Table 4.5-1: RNAA relationship with OAuth 2.0

| Role | OAuth 2.0 | RNAA |
| --- | --- | --- |
| The entity capable of granting access to a protected resource. | Resource Owner | Resource owner via Resource owner function |
| The server hosting the protected resources, capable of accepting and responding to protected resource requests using access tokens. | Resource Server | API exposing function |
| The application making protected resource requests on behalf of the resource owner and with its authorization. | Client | API invoker |
| The server issuing access tokens to the client after successfully authenticating the resource owner and obtaining authorization. | Authorization Server | Authorization function |
| The software agent responsible for retrieving and facilitating end-user interaction for authorization. | User Agent | Resource owner function hosting environment |

## 4.6	Overview CAPIF operations

### 4.6.1	General

This clause provides the overview of CAPIF operations from the perspective of an Application (API invoker) and API provider. The detail of the CAPIF overall operations is shown in Annex A of 3GPP TS 23.222 [2].

### 4.6.2	Usage of CAPIF by API invoker

Figure 4.6.2-1 describes the usage of CAPIF by API invoker for APIs which does not involve resource owner authorization.

Figure 4.6.2-1: Usage of CAPIF by API invoker for APIs which does not involve resource owner

1.	API invoker onboards to the CAPIF core function, as specified in clause 8.1 of 3GPP TS 23.222 [2].

2.	The API invoker authenticates with the CAPIF core function, as specified in clause 8.10 of 3GPP TS 23.222 [2].

3.	The API invoker discovers the service API, as specified in clause 8.7 of 3GPP TS 23.222 [2].

4.	To access the service API, the API invoker obtains authorization with the CAPIF core function, as specified in clause 8.11 of 3GPP TS 23.222 [2].

5.	The API invoker performs service API invocation, as specified in clause 8.16 of 3GPP TS 23.222 [2].

Figure 4.6.2-2 describes the usage of CAPIF by API invoker for APIs which involves resource owner authorization.

Figure 4.6.2-2: Usage of CAPIF by API invoker for APIs which involve resource owner

1.	API invoker onboards to the CAPIF core function, as specified in clause 8.1 of 3GPP TS 23.222 [2].

2.	The API invoker authenticates with the CAPIF core function, as specified in clause 8.10 of 3GPP TS 23.222 [2].

3.	The API invoker discovers the service API, as specified in clause 8.7 of 3GPP TS 23.222 [2].

4.	To access the service API, the API invoker obtains authorization with the CAPIF core function, and then requests service API invocation as specified in clause 8.31 of 3GPP TS 23.222 [2].

5.	The API invoker performs service API invocation, as specified in clause 8.16 of 3GPP TS 23.222 [2].

### 4.6.3	Usage of CAPIF by API provider

Figure 4.6.3-1 describes the usage of CAPIF by API provider for APIs which does not involve resource owner authorization.

Figure 4.6.3-1: Usage of CAPIF by API provider for APIs which does not involve resource owner

1.	The API publishing function sends service API publish request to the CAPIF core function, as specified in clause 8.3 of 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in clause 8.14 of 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in clause 8.15 of 3GPP TS 23.222 [2].

4.	The API provider authorizes the API invoker to access, as specified in clause 8.16 of 3GPP TS 23.222 [2].

5.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in clause 8.16 and 8.17 of 3GPP TS 23.222 [2].

6.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in clause 8.19 of 3GPP TS 23.222 [2].

7.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in clause 8.20 of 3GPP TS 23.222 [2].

8.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in clause 8.21 of 3GPP TS 23.222 [2].

9.	The API provider utilizes the API management function for auditing, as specified in clause 8.22 of 3GPP TS 23.222 [2].

Figure 4.6.3-2 describes the usage of CAPIF by API provider for APIs which involves resource owner authorization.

Figure 4.6.3-2: Usage of CAPIF by API provider for APIs which involve resource owner

1.	The API publishing function sends service API publish request to the CAPIF core function, as specified in clause 8.3 of 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in clause 8.14 of 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in clause 8.15 of 3GPP TS 23.222 [2].

4.	API authorization is based on the authorization information obtained from the resource owner function, as specified in clause 8.31 of 3GPP TS 23.222 [2].

5.	The API provider authorizes the API invoker to access, as specified in clause 8.16 of 3GPP TS 23.222 [2].

6.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in clause 8.16 and 8.17 of 3GPP TS 23.222 [2].

7.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in clause 8.19 of 3GPP TS 23.222 [2].

8.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in clause 8.20 of 3GPP TS 23.222 [2].

9.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in clause 8.21 of 3GPP TS 23.222 [2].

10.	The API provider utilizes the API management function for auditing, as specified in clause 8.22 of 3GPP TS 23.222 [2].

# 5	Role of Stakeholders for API Exposure

## 5.1	Stakeholders in CAPIF

Several stakeholders are considered to interact within the API exposure ecosystem, for instance: MNO, NPN owners, channel aggregators, application service developers, application service providers (ASP), authorization service providers, identity service providers, resource owners, API invokers.

The functional model description for CAPIF is given in clause 6.2.0 of 3GPP TS 23.222 [2]. Different business relationships are considered for CAPIF as follows:

-	Basic CAPIF business relationships are given in clause 5.1 of 3GPP TS 23.222 [2].

-	CAPIF business relationships for RNAA are provided in clause 5.2 of 3GPP TS 23.222 [2].

-	In case of interconnection among CAPIF providers, CAPIF business relationships are given in clause 4.12 of 3GPP TS 23.222 [2].

Some exemplary business relationships are reproduced in Figure 5.1-1.

Figure 5.1-1: Basic business relationships in CAPIF

CAPIF has three main stakeholder roles (CAPIF provider, API provider, and API invoker) and RNAA has resource owner role.

## 5.2	Basic roles

### 5.2.1	CAPIF provider

The CAPIF provider contains an instance of CAPIF core function, and API provider, and API invokers and it could be a PLMN, SNPN or 3rd party. The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), in which case the business relationship between the two is internal to a single organization. The CAPIF provider and the API provider can be part of different organizations, in which case the business relationship between the two must exist in clause 5.1 of 3GPP TS 23.222 [2].

### 5.2.2	Service API provider

The service API provider hosts one or more service APIs and has a service API arrangement with CAPIF provider to offer the service APIs to the API invoker.

Requirements for enabling API invoker is given in clause 4.1.2 of 3GPP TS 23.222 [2].

### 5.2.3	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator and consumes the CAPIF APIs and service APIs. The API invoker may reside within the same trust domain as the PLMN operator network. There are various API invokers like application management client (used by application developers, application service provider), hosted applications (on cloud, edge or UE), and channel aggregator (who aggregates the CAPIF APIs and/or the service APIs).

Requirements for supporting 3rd party API providers is given in clause 4.1.3 of 3GPP TS 23.222 [2].

### 5.2.4	Resource owner

The resource owner is a UE user or an MNO subscriber capable of granting access to a protected resource related to the resource exposed by the API provider. In the current release, it is a user of the UE hosting the API invoker that can authorize the API access in clause 6.3.8 of 3GPP TS 23.222 [2].

Requirements for supporting RNAA is given in clause 4.17.2 of 3GPP TS 23.222 [2].

## 5.3	Mapping of stakeholders to CAPIF roles

The figure 5.3-1 provides the mapping of the stakeholders to CAPIF roles and illustrates the usage of CAPIF capabilities.

The following examples of API invoker roles are illustrated:

1.	Application management client: The application developers utilize the CAPIF APIs using an Application management client as an API invoker to obtain the service APIs information to implement the application program. Such application programs are hosted on cloud, edge or on a UE.

2.	Hosted applications: The Hosted applications (which are programmed to utilize the service APIs) as an API invoker invoke the CAPIF APIs and service APIs as per the application business logic.

3.	Channel Aggregator Platform: The Channel Aggregator aggregates the APIs from one or more southbound CAPIF providers with the intention to re-expose such APIs or expose value-added APIs developed using the APIs from the southbound CAPIF providers to the northbound side Application management client or Hosted applications. The Channel Aggregator Platform as API invoker invokes the CAPIF APIs and service APIs as per its business logic.

4.	BSS/OSS System: The BSS/OSS system of MNO enables the business relationship with the Application Service Providers (the consumers of the service APIs exposed by the MNO's exposure platform). The BSS/OSS system as an API invoker invoke the CAPIF APIs as per its business logic.

Figure 5.3-1: Mapping of the stakeholders to CAPIF roles

## 5.4	Usage of CAPIF in different phases of an application business lifecycle

This clause shows an example of CAPIF usage in application service system. In this example, there are following stakeholders/roles: application service provider, application developer, and CAPIF provider and service API provider as defined in 3GPP TS 23.222 [2].

Figure 5.4-1 shows the whole service flow of CAPIF usage by the 3rd party during the application business lifecycle. The application business lifecycle includes 3 phases:

Phase 1.	Application development preparation phase: Before the application developer develops the application, the developer determines the candidate service APIs to be used for the application. The developer selects the CAPIF provider and the service API provider. Then the developer searches the interested service API(s) information, e.g. available service API, service API name, and so on via the utilizes the application management client (act as API invoker), which invokes the CAPIF_Discover_Service_API to obtain that service API information from the CCF. Then, the developer creates an account and gets authorization to access CAPIF core function. This procedure follows onboarding of API invoker as specified in clause 8.1 in 3GPP TS 23.222 [2]. The developer may find the service API provider at any time later.

Phase 2.	Application development and testing phase: The application developer obtains the detailed service API information including API usage instructions which enables the application developer to program the service API into the application. In addition, the developer may also obtain the security credential of the service API before programming it (option #1). Later, the application developer can test the application which invokes the corresponding CAPIF API and service API (e.g. in a sandbox mode).

Phase 3.	Application deployment and execution phase: After the development, the application is commercial released. The host application (acting as API invoker) obtains the authorization from CAPIF core function to invoke the service API as specified in clause 8.11 of 3GPP TS 23.222 [2]. The host application may obtain the security credential of the service API invocation (Option #2). Upon receiving the access token from CAPIF core function, the application initiates the service API invocation, as specified in clause 8.16 in 3GPP TS 23.222 [2]. In RNAA scenario, the API invoker obtains authorization with the CAPIF core function as specified in clause 8.31 of 3GPP TS 23.222 [2], and then requests service API invocation.

NOTE:	There are two options about obtaining the security credential of the service API invocation. Option #1 is fetching it during phase 1 or phase 2 and program it into the host application. In this option, the security can not be dynamically refreshed at any time. Option #2 is fetching it during the host application is running. In this option, the security can be dynamically refreshed at any time.

Figure 5.4-1: Usage of CAPIF in different phases of an application business lifecycle

# 6	Exemplary Use Cases and Adoption

## 6.1	The deployment of the API Invoker as AF on the UE for RNAA

Several RNAA deployments are described in clause 7.5 of 3GPP TS 23.222 [2], and the following is an example of a RNAA deployment in which API invoker is deployed as AF on the UE.

In this deployment scenario, UE, which functions as both the API Invoker (AF) and the resource owner function, requests resource owner consent from the resource owner function through CAPIF core function to obtain the resource (e.g., QoS).

Figure 6.1-1 illustrates integrated fundamental deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.1-1: The deployment of the API Invoker as AF on the UE for RNAA

Figure 6.1-2 illustrates the procedure for the deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.1-2: The deployment of the API Invoker as AF on the UE for RNAA

The following procedure is for the deployment of the API Invoker as AF on the UE for RNAA:

1.	The API invoker triggers onboard API invoker request towards the CAPIF core function. The CAPIF core function then verifies the request and responds to the API invoker with the result.

2.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API. The CAPIF core function validates the authentication of the API invoker and checks whether the API invoker is permitted to access the requested service API. Then, the CAPIF core function responds to the API invoker with the authorization information required to access the service APIs.

3.	 The API invoker sends a service API discover request to the CAPIF core function and the CAPIF core function verifies the identity of the API invoker. Then the CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

4.	The AEF sends an obtain access control policy request to the CAPIF core function for obtaining the policy to perform the access control on service API invocations. The CAPIF core function checks whether the AEF is authorized to receive the access control policy corresponding to the service APIs requested. The AEF is provided the access control policy for the service API via an obtain access control policy response.

5.	The API invoker requests the resource owner's authorization information to invoke the service API, exposed by the API exposing function. The authorization function facilitates this request by involving the resource owner. Then, the API invoker sends a service API invocation request to the API exposing function, which in turn sends a response back to the API invoker.

6.	The API invoker (UE) sends service API invocation request to the and the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information. The API invoker receives the service API invocation response as a result of the service API invocation.

## 6.2	The deployment of the API Invoker as AF on the network for RNAA

In this deployment scenario, the UE, which functions as both the application and the resource owner function, initiates the application's requests. The API Invoker then requests resource owner consent from the resource owner function through CAPIF core function to obtain the resource (e.g., QoS).

Figure 6.6-1 illustrates integrated fundamental deployment of the API Invoker as AF on the network for RNAA.

Figure 6.6-1: The deployment of the API Invoker as AF on the network for RNAA

Figure 6.6-2 illustrates the procedure for the deployment of the API Invoker as AF on the network for RNAA.

Figure 6.2-2: The procedure for the deployment of the API Invoker as AF on the network for RNAA

The following procedure is for API Invoker deployment on the network as AF for RNAA:

1.	The API invoker triggers onboard API invoker request towards the CAPIF core function. The CAPIF core function then verifies the request and responds to the API invoker with the result.

2.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API. The CAPIF core function validates the authentication of the API invoker and checks whether the API invoker is permitted to access the requested service API. Then, the CAPIF core function responds to the API invoker with the authorization information required to access the service APIs.

3.	The API invoker sends a service API discover request to the CAPIF core function and the CAPIF core function verifies the identity of the API invoker. Then the CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

4.	The AEF sends an obtain access control policy request to the CAPIF core function for obtaining the policy to perform the access control on service API invocations. The CAPIF core function checks whether the AEF is authorized to receive the access control policy corresponding to the service APIs requested. The AEF is provided the access control policy for the service API via an obtain access control policy response.

5.	The API exposing function obtains the resource owner consent from the resource owner. The API invoker requests the resource owner's authorization information to invoke the service API, exposed by the API exposing function. The authorization function facilitates this request by involving the resource owner. Then, the API invoker sends a service API invocation request to the API exposing function, which in turn sends a response back to the API invoker.

6.	From the application’s request on UE, the API invoker sends service API invocation request to the AEF and the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information. The API invoker receives the service API invocation response as a result of the service API invocation.

## 6.3	GSMA Operator Platform deployment via CAPIF

The GSMA Operator Platform defines a common platform exposing operator services / capabilities to customers / application providers in the 5G-era. The requirement that an Operator Platform’s SBI-NR can work with CAPIF when available is specified in clause 5.1.4.2.2 of GSMA PRD OPG.02 [11] and application session continuity support for handovers between 4G and 5G based on combined SCEF+NEF via CAPIF is described in clause 5.2.2.6.7 of GSMA PRD OPG.02 [11].

Figure 6.3-1 illustrates integrated fundamental GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF. In this deployment model, the GSMA Operator Platform functions as the API invoker.

NOTE:	An Operator Platform's SBI-NR can work without CAPIF. If CAPIF is not supported, the SBI-NR API will provide an alternate means of providing these functions.

Figure 6.3-1: Integrated GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF

## 6.4	CAPIF Vendor Extensibility

Vendor extensibility is a CAPIF feature that enables third-party API frameworks to re-use CAPIF functionality, adapted to their requirements, by extending either CAPIF APIs or CAPIF data models. A detailed description of vendor extensibility mechanisms is provided on 3GPP TS 29.122 [14] and 3GPP TS 29.500 [16]. The CAPIF APIs supporting this feature are the following:

-	CAPIF_Publish_Service_API, including “VendorExt” option in the supported feature list (see Table 8.1.6-1 of 3GPP TS 29.222 [3], which lists 3 features), indicating the support for CAPIF vendor specific extensions on the ServiceAPIDescription data model.

-	CAPIF_Discover_Service_API, including “VendSpecQueryParams” option in the supported feature list (see Table 8.2.6-1 of 3GPP TS 29.222 [3], which lists 9 features), indicating the support of vendor specific query parameters in API discovery requests.

Annex B provides more details of CAPIF vendor extensibility and illustrates ETSI MEC leveraging the CAPIF vendor extensibility feature.

## 6.5	ETSI MEC deployment based on CAPIF

The ETSI MEC platform is the collection of essential functionalities required to run MEC applications on a particular Virtualisation infrastructure and enable them to provide and consume MEC services. In clause 9 of ETSI GS MEC 011 V3.3.1 [5], a description is provided on how the MEC service management APIs can be realized by CAPIF APIs. The clause describes how MEC deployments can reuse CAPIF functionality to harmonize between MEC and CAPIF, e.g., to integrate MEC applications into a joint 3GPP-MEC deployment. To enable such harmonization, a variant of the MEC service management functionality is defined that leverages the CAPIF Publish Service API, CAPIF Discover Service API and CAPIF Events API. The variant is referred to as the "MEC profile of CAPIF".

This deployment option for ETSI MEC based on CAPIF is also depicted at a high-level in clause 7.2 of 3GPP TR 23.958 [6].

## 6.6	MEC Platform as an API Provider

### 6.6.1	Introduction

This clause outlines the registration process for the MEC platform as an API provider within the CAPIF framework, describing the necessary steps and requirements for API publication and management.

### 6.6.2	Registering the MEC Platform as an API Provider in CAPIF

To register the MEC platform as an API provider in CAPIF, an access token (access_token) is required to authorize the provider’s onboarding. Once the required information is available, the provider submits an HTTP POST request to the following URI:

POST {apiRoot}/api-provider-management/v1/registrations

Request Headers:

•	Authorization: Bearer {access_token}

•	Content-Type: application/json

In the request body, include the regSec field (holding the token) and provide a Certificate Signing Request (CSR) along with relevant details for each provider function: Application Management Function (AMF), API Publishing Function (APF), and API Exposing Function (AEF). Additionally, a domain name and other provider-related information can be specified.

Example Request Body:

HTTP POST https://capif.mec.example.com/api-provider-management/v1/registrations

Request headers:

Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9

Content-Type: application/json

Request body:

{

"regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9",

"apiProvFuncs": [

{

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR Body\n-----END CERTIFICATE REQUEST-----\n"

},

"apiProvFuncRole": "AEF",

"apiProvFuncInfo": "AEF for MEC Platform"

},

{

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR Body\n-----END CERTIFICATE REQUEST-----\n"

},

"apiProvFuncRole": "APF",

"apiProvFuncInfo": "APF for MEC Platform"

},

{

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR Body\n-----END CERTIFICATE REQUEST-----\n"

},

"apiProvFuncRole": "AMF",

"apiProvFuncInfo": "AMF for MEC Platform"

}

],

"apiProvDomInfo": "MEC Platform",

"apiProvName": "OCF_MEC"

}

Upon successful registration, CAPIF returns the location of the newly created resource in the Location field of the header. The response body includes signed certificates and IDs for each provider function, as well as the domain ID.

Response headers:

Location: https://capif.mec.example.com/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb

Response body:

{

"apiProvDomId": "3a4df5f6e30a5ee34de346004ee6bb",

"regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9",

"apiProvFuncs": [

{

"apiProvFuncId": "AEFb1a522967511fd6d6def0cbb3b5f05",

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR\n-----END CERTIFICATE REQUEST-----\n",

"apiProvCert": "-----BEGIN CERTIFICATE -----\nAEF CERTIFICATE\n-----END CERTIFICATE-----\n"

},

"apiProvFuncRole": "AEF",

"apiProvFuncInfo": "AEF for MEC Platform "

},

{

"apiProvFuncId": "APF54f49ee44f67c81fc7d7f321195531",

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR\n-----END CERTIFICATE REQUEST-----\n",

"apiProvCert": "-----BEGIN CERTIFICATE -----\nAPF CERTIFICATE\n-----END CERTIFICATE-----\n"

},

"apiProvFuncRole": "APF",

"apiProvFuncInfo": "APF for MEC Platform"

},

{

"apiProvFuncId": "AMFOdbe115f6759f0fec943321dc2184d",

"regInfo": {

"apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nCSR\n-----END CERTIFICATE REQUEST-----\n",

"apiProvCert": "-----BEGIN CERTIFICATE -----\nAMF CERTIFICATE\n-----END CERTIFICATE-----\n"

},

"apiProvFuncRole": "AMF",

"apiProvFuncInfo": "AMF for MEC Platform"

}

],

"apiProvDomInfo": "MEC Platform",

"apiProvName": "OCF_MEC"

}

### 6.6.3	Publishing an API for the MEC Platform

To publish an API, the MEC platform uses mutual TLS authentication with the APF certificate to make an HTTP POST request to the following URI:

POST {apiRoot}/published-apis/v1/{apfId}/service-apis

The request body should specify details of the API to be published, including name, status, version, resources, and protocol details.

Example Request Body:

HTTP POST https://capif.mec.example.com/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis

Request body:

{

"apiName": "Application Instance Registration",

"apiStatus": {

"aefIds": ["AEFb1a522967511fd6d6def0cbb3b5f05"]

},

"aefProfiles": [

{

"aefId": "AEFb1a522967511fd6d6def0cbb3b5f05",

"versions": [

{

"apiVersion": "v1",

"expiry": "2100-11-30T10:32:02.004Z",

"resources": [

{

"resourceName": "MEC RESOURCE NAME",

"commType": "REQUEST_RESPONSE",

"uri": "/mec_app_support/v2/registrations",

"custOpName": "http_post",

"operations": [

"POST"

],

"description": "App Registration Resource"

},]

}],

"protocol": "HTTP_1_1",

"dataFormat": "JSON",

"securityMethods": [

"OAUTH",

],

"interfaceDescriptions": [

{

"ipv4Addr":”X.X.X.X",

"port": NNNN,

"securityMethods": [

"OAUTH"

]

}

]

}

],

"description": "MEC App Registration API",

"supportedFeatures": "1ff",

"shareableInfo": {

"isShareable": true,

"capifProvDoms": [

"MEC Platform"

]

},

"serviceAPICategory": "MEC Service Management",

"apiProvName": "OCF_MEC"

}

Upon successful API publication, CAPIF returns the location of the created resource in the Location field of the header and provides the service API ID of the published API in the response body.

This contribution provides a structured approach for onboarding and publishing APIs for the MEC platform within CAPIF, ensuring alignment with CAPIF’s standard onboarding and security procedures.

## 6.7	Open-source implementation of CAPIF

As stated in Annex B.2.2.3 of 3GPP TS 23.222 [2], the NEF can implement the functionalities of the API provider domain functions, as reproduced in Figure 6.7-1.

Figure 6.7-1: NEF implements the service specific aspect compliant with the CAPIF architecture

Evolved-5G [7] (ICT-41-2020 project from call H2020-ICT-2018-2020) developed an open-source project of the CAPIF Release 17 under the Apache-2.0 license. EVOLVED-5G implementation of CAPIF works with an open-source implementation of a NEF emulator available in [8]. The NEF emulator used for Evolved-5G implementation supports two services, namely Nnef_EventExposure and Nnef_AFsessionWithQoS [8].

Evolved-5G implementation has been used as the seed code for OpenCAPIF SDG [9] created by ETSI in January 2024.

OpenCAPIF has adopted the following APIs from CAPIF:

-	CAPIF API Invoker Management API

-	CAPIF API Provider Management API

-	CAPIF Publish Service API

-	CAPIF Discover Service API

-	CAPIF Security API, including obtaining the OAuth 2.0 authorization information with JWT access tokens

-	CAPIF Access Control Policy API

-	CAPIF Logging API Invocation API

-	CAPIF Events API

-	CAPIF Auditing API

The implementation also includes a Testing Suite of the above services.

Every CAPIF API is implemented in a separate container for future scalability adaptation. All APIs are exposed and support mutual TLS authentication as specified in 3GPP TS 33.122 [4].

The Source Code of OpenCAPIF and the instructions to deploy it are included in the ETSI GitLab repository [10].

Annex C provides more details of Open-source implementation of CAPIF for NEF publishing API and API consumption from an API Invoker.

# 7	Summary

This guide presents a number of use cases and adoption strategies for CAPIF. Several stakeholders (e.g., developers, aggregators, CSPs, etc.) can compare them with their own use cases to understand how to apply/map CAPIF capabilities to their own needs.

Annex A:
Examples from OpenCAPIF

All the requests shown in this document for OpenCAPIF can be tested using the OpenCAPIF Postman collection [12]. The collection includes a guide on its use and how to correctly test each of the requests. The following table shows the mapping between each of the requests presented in this document to the corresponding request in the OpenCAPIF Postman collection.

| Table A-1: The OpenCAPIF Postman collection |  |  |
| --- | --- | --- |
| Request | Section | OCF Postman Example |
| CAPIF_API_Invoker_Management_API | 6.8.1 | https://ocf.etsi.org/documentation/develop/testing/postman/#07-onboard_invoker |
| CAPIF_API_Provider_Management_API | 6.4.1 | https://ocf.etsi.org/documentation/develop/testing/postman/#04-onboard_provider |
| CAPIF_Publish_Service_API | 6.4.3 | https://ocf.etsi.org/documentation/develop/testing/postman/#05-publish_api |
| CAPIF_Discover_Service_API | 6.8.3 | https://ocf.etsi.org/documentation/develop/testing/postman/#08-discover |
| CAPIF_Security_API create context | 6.8.5 | https://ocf.etsi.org/documentation/develop/testing/postman/#09-security_context |
| CAPIF_Security_API authorization info | 6.8.7 | https://ocf.etsi.org/documentation/develop/testing/postman/#10-get_token |
| API Invocation | 6.8.9 | https://ocf.etsi.org/documentation/develop/testing/postman/#11-call_service |

Annex B:
CAPIF Vendor Extensibility

# B.1	Include Vendor Specific information

An API provider can include a vendor specific attribute in the ServiceAPIDescription model by enabling “VendorExt” supported feature option and adding the field in the following manner (see clause 5.2.13.2 of 3GPP TS 29.122 [14]):

"vendorSpecific-<vendor-identification>": {

...

}

where “vendor-identification” represents a unique name of the vendor publishing the API, using one of the following structures:

- IANA-assigned enterprise code (e.g., vendorSpecific-010415 for “3GPP”)

- Domain name (e.g., vendorSpecific-3gpp.org)

- URN (e.g., vendorSpecific-urn:3gpp:example)

# B.2	Query for Vendor Specific information

An API invoker can discover API services including vendor specific attributes by adding the following query parameters (see clause 5.2.13.3 of 3GPP TS 29.122 [14]):

- supported-features, enabling “VendSpecQueryParams” option

- vend-spec-<query-parameter-name> = {

"target": "...",
"value": "..."

}

where “query-parameter-name” represents the actual name of the query parameter, “target” is a JSON pointer towards the targeted attribute in the targeted resource representation and “value” is the actual value of the query parameter to be used for filtering.

# B.3	ETSI MEC extensibility to CAPIF

According to ETSI GS MEC 011 V3.3.1 [5] specification, MEC framework, leveraging CAPIF extensibility feature, can engage CAPIF and re-use its functionalities. A concrete example is presented in Table B.3-1 and Table B.3-4, describing the request that a MEC platform has to send towards CAPIF to publish an API, as well as a request of an invoker searching an API with MEC specific attributes.

ETSI MEC supports two extensibility attributes on CAPIF:

- “vendorSpecific-urn:etsi:mec:capifext:service-info” in ServiceAPIDescription and ServiceAPIDescriptionPatch data types, and

- “vendorSpecific-urn:etsi:mec:capifext:transport-info” in AefProfile data type

Both attributes are included on the following API Publish request.

Table B.3-1: Publish MEC API with vendor specific attributes in CAPIF (schema)

HTTP POST {apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ], "vendorSpecific-urn:etsi:mec:capifext:transport-info": { "name": "string", "description": "string", "type": "string", "protocol": "string", "version": "string", "security": { "grantTypes": "string", "tokenEndpoint": "string" } } } ], "vendorSpecific-urn:etsi:mec:capifext:service-info": { "serializer": "string", "state": "string", "scopeOfLocality": "string", "consumedLocalOnly": "boolean", "isLocal": "boolean", "category": { "href": "string", "id": "string", "name": "string", "version": "string" } }, "description": "string", "supportedFeatures": "string", "apiProvName": "string" }

When APIs are published including Vendor Specific information, API Invokers can filter the Discovery of APIs including query parameters to filter by vendor specific information. Here we show some examples of CAPIF Discover API including vendor specific filtering.

The schema for discovering APIs using vendor specific information is shown in Table B.3-2 and Table B.3-3:

Table B.3-2: Discover APIs with vendor specific query parameters (schema for transport-info)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs? api-invoker-id="string"& supported-features="string"& vend-spec-type={"target": "...", "value": "string"}

Table B.3-3: Discover APIs with vendor specific query parameters (schema for service-info)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs? api-invoker-id="string"& supported-features="string"& vend-spec-state={"target": "...", "value": "string"}

Based in these schemas, the following examples show how to use CAPIF Extensibility. Table B.3-4 shows an API Publication that includes both transport-info and service-info as part of the API description published.

Table B.3-4: Publish MEC API with vendor specific attributes in CAPIF (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "api_demo_v2", "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v2", "expiry": "2021-11-30T10:32:02.004Z", "resources": [ { "resourceName": "endpoint_1", "commType": "REQUEST_RESPONSE", "uri": "/endpoint", "custOpName": "string", "operations": [ "GET" ], "description": "Endpoint to receive a welcome message" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "GET" ], "description": "string" } ] } ], "interfaceDescriptions": [ { "ipv4Addr": "localhost", "port": 8089, "securityMethods": ["Oauth"] } ], "vendorSpecific-urn:etsi:mec:capifext:transport-info": { "name": "trasport1", "description": "Transport Info 1", "type": "REST_HTTP", "protocol": "HTTP", "version": "2", "security": { "grantTypes": "OAUTH2_CLIENT_CREDENTIALS", "tokenEndpoint": "https://token-endpoint.example.com/" } } } ], "vendorSpecific-urn:etsi:mec:capifext:service-info": { "serializer": "JSON", "state": "ACTIVE", "scopeOfLocality": "MEC_SYSTEM", "consumedLocalOnly": "True", "isLocal": "True", "category": { "href": "https://location.example.com", "id": "Location", "name": "Location", "version": "1.0" } }, "description": "Hello api services", "supportedFeatures": "4", "apiProvName": "OCF_MEC" }

The next example in Table B.3-5 shows how to query CAPIF using transport-info to query for APIs which vend-spec-type is “REST_HTTP”.

Table B.3-5: Discover APIs with vendor specific query parameters (OCF example for transport-info)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs? api-invoker-id=INVd4d70e16c52bace2b8f40ed6f6badS& supported-features=4& vend-spec-type={"target": "/vendorSpecific-urn:etsi:mec:capifext:transport-info", "value": "REST_HTTP"}

Similarly, the next example in Table B.3-6 shows how to query CAPIF for APIs that contains service-info which vend-spec-state is “ACTIVE”.

Table B.3-6: Discover APIs with vendor specific query parameters (OCF example for service-info)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs? api-invoker-id=INVd4d70e16c52bace2b8f40ed6f6badS& supported-features=4& vend-spec-state={"target": "/vendorSpecific-urn:etsi:mec:capifext:service-info", "value": "ACTIVE"}

Annex C:
Open-source implementation of CAPIF

# C.1	NEF Publishes an API

The 5GS network exposure function (NEF) can use CAPIF capabilities to publish and manage its APIs, including managing charging. One of several possible deployment arrangements is shown in Figure C.1-1.

Figure C.1-1: NEF implements the CAPIF API provider domain functions

Signalling for the NEF to publish its APIs to the CAPIF core function is shown in Figure C.1-2, followed by tables detailing the mandatory attributes of the associated request and response schema and example content for those requests and responses, which is based on publicly available examples provided by the OpenCAPIF [9].

A pre-condition is that the NEF (API management function) has been made aware of the appropriate CAPIF core function endpoint (apiRoot) with which to initiate the registration, as well as having been provided with a token to access the CAPIF core function.

Figure C.1-2: Publish a service API

Details of the signalling flow in Figure C.1-2 are as follows:

NOTE 1:	The JSON schema and example snippets provided in the following clauses are aligned with 3GPP TS 29.222 [3], in which apiVersion is specified as "v1". The example snippets themselves can be tested using the OpenCAPIF Postman collection presented in Annex A and also in the OpenCAPIF Postman documentation [12].

1. CAPIF_API_Provider_Managment_API

The API provider includes:

-	Authorization HTTP header (access token)

-	Security information enabling the CCF to validate the registration (regSec)

-	An array (apiProvFuncs) of at least one API provider domain function profiles

-	Optionally API provider domain information (apiProvDomInfo) and the API provider name (apiProvName)

Table C.1-1: Register API_provider request (schema)

HTTP POST {apiRoot}/api-provider-management/v1/registrations Request headers: Authorization: "string" Request body: { "regSec": "string", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "string", }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

Given the schema provided in Table C.1-1, an example of the API Provider registration process using OpenCAPIF open-source implementation is now described and then provided in Table C.1-1-OCF. The presented traces contain actual values to illustrate the usage of the information elements in the JSON schema.

NOTE 2:	The security related information provided in the examples are truncated for readability, e.g., the value associated with regSec and apiProvPubKey.

Before carrying out onboarding, the provider requires the:

-	OpenCAPIF access token, that will allow the creation of the provider.

-	CA_ROOT certificate, necessary to validate the Certificates in all requests.

-	URL to which to make the request, i.e., <{apiRoot}/api-provider-management/v1/registrations>.

NOTE 3:	In OpenCAPIF, all this information is obtained in a dedicated user registration process [13].

In the presented example, the OpenCAPIF instance is deployed at capif.mobilesandbox.cloud:37211. This instance URL is assumed throughout the presented example traces.

To carry out the onboarding of a provider, the API Provider is required to provide the following fields:

-	regSec: Contains the OpenCAPIF access token obtained during registration process for the creation of the provider. The Token is also provided in the Authorization HTTP Header.

-	apiProvFuncRole: Type of provider function to be created. It can be AMF, APF or AEF.

-	apiProvPubKey: It is the certificate signing request (CSR) of the Provider’s function that OpenCAPIF will use to issue the provider’s valid certificates. Each function needs to include its own CSR to request a valid Certificate.

-	apiProvFuncInfo: Information about the provider function that is being created.

-	apiProvDomInfo: Information about the provider domain that is being created.

The API Provider onboarding request example is shown in Table C.1-1-OCF. The example request is to onboards an API Provider with three functions: AEF, APF and AMF. Therefore, 3 CSRs are provided to obtain 3 Certificates.

Table C.1-1-OCF: Register API provider request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations Request headers: Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9 Content-Type: application/json Request body: { "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" "suppFeat": "3", }

Upon getting this Request, Open CAPIF core function assigns identity IDs for the API provider (apiProvDomId) and each of its functions (apiProvFuncId) and returns them in the response (see next section). Furthermore, it provides certificates (apiProvCert) for each of the API provider functions based on each function’s CSR (apiProvPubKey).

2. 201 Created

The API provider domain has been registered successfully. The URI of the created resource is returned in the "Location" HTTP header, including the assigned registration identifier (registrationId).

Table C.1-2: Register API provider response (schema)

201 Response headers: Location: {apiRoot}/api-provider-management/v1/registrations/{registrationId} Response body: { "apiProvDomId": "string", "regSec": "string", "apiProvFuncs": [ { "apiProvFuncId": "string", "regInfo": { "apiProvPubKey": "string", "apiProvCert": "string" }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

The response received from the successful onboarding request using OpenCAPIF project for the creation of a new API Provider includes the following fields:

-	apiProvDomId: Identifier of the created provider, which is also the ID returned as part of the ‘Location’ header.

-	apiProvCert: Certificate of the provider function created from the CSR sent in the onboarding request, which will be used for subsequent requests from the provider towards OpenCAPIF. Each function is provided with its own Certificate (i.e., one each for the AEF, APF and AMF functions).

-	apiProvFuncId: Identifier of the provider function created.

The example Register API provider response is shown in Table C.1-2-OCF.

Table C.1-2-OCF: Register API provider response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb Response body: { "apiProvDomId": "3a4df5f6e30a5ee34de346004ee6bb", "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "apiProvFuncId": "AEFb1a522967511fd6d6def0cbb3b5f05", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "apiProvFuncId": "APF54f49ee44f67c81fc7d7f321195531", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "apiProvFuncId": "AMFOdbe115f6759f0fec943321dc2184d", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" "suppFeat": "3", }

3. CAPIF_Publish_Service_API

The API publishing function sends information on the offered APIs by providing the API exposing function profiles array (aefProfiles). The published APIs resource created in response to the request represents all published service APIs of the NEF.

NOTE 4:	The example content in Table C.1.3, the attribute apiStatus is omitted, implying the Service API is active at all AEF(s) present in the aefProfiles attribute. Furthermore, it is assumed the exchange is over CAPIF-4, rather than CAPIF-6/6e. As a consequence, the pubApiPath and ccfId attributes are omitted.

Table C.1-3: Publish API request (schema)

HTTP POST {apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" }, { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "protocol": "string", "dataFormat": "string", "securityMethods": [ "string", ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ] } ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string", "apiProvName": "string" }

To publish a service using OpenCAPIF, it is necessary to specify the following fields:

-	apiName: Name of the API to be published, which must be unique in CAPIF.

-	apiStatus: Represents the API status. In addition, if this attribute is omitted, it is understood that the API is available in all AEFs of aefProfiles. It contains the aefIds field which is a list of the AEFs that expose the API. If this list is empty or the attribute is omitted, it is understood that the API is inactive in all aefProfiles.

-	aefProfiles: List of Information about AEFs that expose the API. This information contains the following fields:

  - aefId: AEF identifier of the provider that exposes the service.

  - versions: List of information about API versions. Each version contains the resources supported by the API. Each version contains:

    - apiVersion: usable version of the API.

    - expiry: date on which the API version expires and will no longer be available.

    - resources: A list of resources that contain the API version. Each resource contains the following information:

      - resourceName: Name of the resource for which its information is specified.

      - commType: Type of use of the resource, which can be receiving a response when making a request (REQUEST_RESPONSE) or a subscription to receive notifications (SUBSCRIBE_NOTIFY)

      - uri: URI to make the request to use the described API resource.

      - custOpName: Part of the URI structure used for a custom operation associated with the resource. The attributes custOpName and custOperations are mutually exclusive.

      - custOperations: List of the resource's custom operations associated to this resource. The attributes custOpName and custOperations are mutually exclusive.

      - operations: List of HTTP methods supported by the API resource.

      - description: Descriptive text of the API resource.

    - custOperations: List of custom operations without resource association.

  - protocol: Indicates a protocol and protocol version used by the API.

  - dataFormat: Indicates the data sending format which can be JSON, XML or PROTOBUF3.

  - securityMethods: Indicates the possible security methods of the API.

  - domainName: Domain name to which the API belongs.

  - interfaceDescriptions: List of interfaces to which API requests can be made.

Only domainName or interfaceDescription can be specified.

-	supportedFeatures: Indicates the one or more CAPIF API specific supported features (from the nine features specified in 3GPP TS 29.222 [3]), e.g., support for API status monitoring in the CAPIF layer as part of the SEAL framework, support for RNAA functionality.

-	shareableinfo: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

-	serviceAPICategory: The service API category to which the service API belongs to.

-	apiSuppFeats: Indication provided by the consumer (i.e., related to the AEF) which optional features are supported by the service API.

-	apiProvName: API provider Name that publishes the API.

The example API Publish request to publish “3gpp-as-session-with-qos” API from NEF is shown in Table C.1-3-OCF, for which there are twenty-nine optional service specific features (apiSuppFeats) specified in 3GPP TS 29.122 [14].

Table C.1-3-OCF: Publish API request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis Request body: { "apiName": "3gpp-as-session-with-qos", "apiStatus": { "aefIds": ["AEFb1a522967511fd6d6def0cbb3b5f05" ] }, "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "1ff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiSuppFeats": "1fffffff", "apiProvName": "OCF_NEF" }

4. 201 Created

The service API has been published successfully The URI of the created resource is returned in the "Location" HTTP header, including the assigned service API identifier (serviceApiId) that is also equal to the API identifier (apiId) in the response body.

Table C.1-4: Publish API response (schema)

{ 201 Response headers: Location: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId} Response body: { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ "object" ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string" "apiProvName": "string" }

The response to a successful API Publish request using OpenCAPIF contains the following information:

-	Location: URL with the resource created for the API Published.

-	apiId: The assigned Identifier to the Published API. It is required for the UPDATE or DELETE API procedures once the API has been published.

The information from the API that was sent in the request will be obtained as a response with a new field, the "apiId", which is the identifier of the API that has been created and will enable subsequent modify or delete procedures.

Table C.1-4-OCF: Publish API response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis/f55731ec5f8ce703ac1f69605ad095 Response body: { "apiName": "3gpp-as-session-with-qos", "apiId": "f55731ec5f8ce703ac1f69605ad095", "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "1ff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiSuppFeats": "1fffffff" "apiProvName": "OCF_NEF" }

5. CAPIF administrator creates policy

The procedures for creation of the access control policy at the CAPIF core function by the CAPIF administrator is outside the scope of 3GPP TS 23.222 [2] and can be triggered because of the API management function issuing the "Register API provider" request or prior to that. Either way, sufficient information must be provided to enable the AEF specific access control policy list resource for the service API to be created. Specifically, each policy must include the CCF assigned API invoker identifier (apiInvokerId) and can include allowed total invocations (allowedTotalInvocations), invocations per second (allowedInvocationsPerSecond), invocation time range list (allowedInvocationTimeRangeList). Since an API invoker identifier is mandatory, it is a prerequisite that API invoker has been onboarded prior to invoking the highlighted access control policy procedures.

6. CAPIF_Access_Control_Policy_API

The Access Control List (ACL) service allows the provider to use AEF to obtain the access control list from invokers for an exposing API. The provider is in charge of making the request to the CAPIF Core Function and using the list received to control an invoker's access to one of its APIs.

Table C.1-6: GET access control policy request (schema)

HTTP GET {apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId}?aef-id="string"

The AEF may make a request to OpenCAPIF to obtain all the ACLs of a service that it is exposing, for this it must specify in the URL of the request the identifier of the service (apiId received in the API Publish response) for which it wants the list of ACLs, in addition to the AEF’s own identifier to check that it has access to that list.

Table C.1-6-OCF: ACL Request (OCF example)

HTTP GET https://capif.mobilesandbox.cloud:37211/access-control-policy/v1/accessControlPolicyList/f55731ec5f8ce703ac1f69605ad095?aef-id=AEFb1a522967511fd6d6def0cbb3b5f05

7. 200 OK

The request has been successfully received and the CAPIF Core Function returns the invoker access control information for the requested API. The AEF will use this information to allow or not the use of the API by the invoker.

Table C.1-7: GET access control policy response (schema)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

OpenCAPIF provides the access control policy information for the NEF API Published "3gpp-as-session-with-qos" containing the following information:

-	apiInvokerId: identifier of the invoker to which the access policy belongs.

-	allowedTotalInvocations: Total number of invocations allowed on the service API by the API invoker.

-	allowedInvocationsPerSecond: invocations per second allowed on the service API by the API invoker.

-	allowedInvocationTimeRangeList: the time ranges during which the invocations are allowed on the service API by the API invoker.

Table C.1-7-OCF: GET access control policy response (OCF example)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId”: "INVd4d70e16c52bace2b8f40ed6f6badS", "allowedTotalInvocations": 1200, "allowedInvocationsPerSecond": 2, "allowedInvocationTimeRangeList": [ { "startTime": "2024-08-19T09:00:00.000000+02:00", "stopTime": "2024-08-23T16:00:00.000000+02:00" } ] } ] }

8. Offboarding API Provider

The provider can request its deletion from CAPIF using the offboarding service. By using this service, all information stored in the CAPIF Core Function related to the provider will be deleted, including APIs published by the provider, security contexts of those APIs, ACLs, etc.

Table C.1-8: DELETE API provider request (schema)

HTTP DELETE {apiRoot}/api-provider-management/v1/registrations/{registrationId}

For the process of deleting a Provider in OpenCAPIF, you will need the AMF certificate and the identifier of the provider that you want to delete. This data is obtained in the provider creation process and by simply making the corresponding request we will eliminate the provider.

Table C.1-8-OCF: DELETE API provider request (OCF example)

HTTP DELETE https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb

9. 204 No Content

The response will be a 204 No Content, which indicates that the provider has been successfully deleted along with everything related to it, APIs Published, security contexts associated with its services, ACLs of its services, etc.

# C.2	API consumption from an API Invoker

This section explains how an API Invoker can use OpenCAPIF [9] to discover and consume APIs, based on 3GPP TS 23.222 [2] and 3GPP TS 29.222 [3] for the schema. The API Invoker application needs to obtain the following information from OpenCAPIF during user registration, similarly to the API Provider onboarding:

-	CA_ROOT: Root certificate from the OpenCAPIF Certificate Authority to validate Certificates.

-	Token: Token for the API Invoker onboarding request.

-	Onboarding URL: URL for requesting the API Invoker onboarding.

This section explains:

-	how to onboard an API Invoker in OpenCAPIF, to be able to discover all the APIs published in the CAPIF Core Function.

-	request the creation of a security context, which is necessary to consume an API.

-	request the required OAuth token to consume the API; and finally, how to make the request directly to the published API.

All the requests shown in this document can be tested using the OpenCAPIF Postman collection found in Annex A and in the OpenCAPIF Postman documentation [12].

- CAPIF_API_Invoker_Management (Registration)

Before carrying out onboarding, the invoker will need the CAPIF access token that will allow the creation of the invoker, the CA_ROOT certificate necessary to validate all the requests, and the URL to which to make the request <{apiRoot}/api-invoker-management/v1/onboardedInvokers>.

In OpenCAPIF, all this information is obtained in the user registration process [13].

To onboard an API Invoker in CAPIF, it is necessary to utilize the token received in the Authentication HTTP header. In addition, a Certificate Signing Request (CSR) will have to be created with two keys so that the CAPIF Core Function can issue a valid certificate that will be used by the invoker to be able to make the subsequent requests. In the following example, the {apiList} field has been omitted, as it will discover the APIs that can be used with the discover service.

Table C.2-1: Register API Invoker request (schema)

HTTP POST {apiRoot}/api-invoker-management/v1/onboardedInvokers Request headers: Authorization: "string" Request body: { "onboardingInformation": { "apiInvokerPublicKey": "string" }, "notificationDestination": "string", "apiInvokerInformation": "string", "websockNotifConfig": { "websocketUri": "string", "requestWebsocketURI": "boolean" } }

To perform the onboarding of an API Invoker, the following data will have to be specified:

-	apiInvokerPublicKey: It is the certificate signing request of the invoker that CAPIF will use to issue the invoker’s valid certificate.

-	apiInvokerInformation: Generic information related to the API invoker such as details of the device or the application

-	notificationDestination: URL where the API invoker can receive CAPIF notifications.

Table C.2-1-OCF: Register API Invoker request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/ Request headers: Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9 Content-Type: application/json Request body: { "onboardingInformation": { "apiInvokerPublicKey": "-----BEGIN CERTIFICATE REQUEST----- \nMIICeTCCAWECAQAHISNoGA1ERGAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST----\n" }, "notificationDestination": " http://nef_notification_destination:8080/", "apiInvokerInformation": "NEF Invoker", "websockNotifConfig": { "websocketUri": "websocketUri", "requestWebsocketURI": true } }

The CAPIF Core Function assigns an identity for the API invoker (apiInvokerId) and a certificate (apiInvokerCertificate) based on provided public key(apiInvokerPublicKey).

2. 201 Created

The API Invoker has been registered successfully. The URI of the created resource is returned in the “Location” HTTP header including the assigned registration identifier (apiInvokerId).

Table C.2-2: Register API-Invoker response (schema, then example content)

201 Response headers: Location: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{apiInvokerId} Response body: { "apiInvokerId": "string", "onboardingInformation": { "apiInvokerPublicKey": "string", "apiInvokerCertificate": "string", "onboardingSecret": "string" }, "notificationDestination": "string", "apiInvokerInformation": "string", "websockNotifConfig": { "websocketUri": "string", "requestWebsocketURI": "boolean" } }

The fields that are returned when onboarding an invoker, that are populated by the CCF, are:

-	apiInvokerId: Identifier of the invoker created.

-	apiInvokerCertificate: Certificate of the Invoker created from the CSR sent in the onboarding request, which will be used for the following requests from the Invoker to CAPIF Core Function.

Table C.2-2-OCF: Register API-Invoker response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/INVaa03f9911db37be051b243ce3caad9 Response body: { "apiInvokerId": "INVaa03f9911db37be051b243ce3caad9", "onboardingInformation": { "apiInvokerPublicKey": "-----BEGIN CERTIFICATE REQUEST----- \nMIICeTCCAWECAQAHISNoGA1ERGAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST----\n", "apiInvokerCertificate": "-----BEGIN CERTIFICATE-----\nMIIDgjCCAmqgAwIBAgIUS18nvjZYU5B+ptuAw/ x3gQWYEJVxDha9112mP7lclNg5yd5o1Xwj1Imbht2z33ZJ4jxA=\n-----END CERTIFICATE-----\n" }, "apiInvokerInformation": "NEF Invoker", "websockNotifConfig": { "websocketUri": "websocketUri", "requestWebsocketURI": true } }

3. CAPIF_Discovery_Service_API

After onboarding the API Invoker, the API Discovery function is used by invokers to obtain all the APIs published in CAPIF, providing all the necessary information that an API Invoker needs to be able to make a request to each such API.

Table C.2-3: Discover APIs request (schema, then example content)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId}

The API Invoker must make the request to CAPIF using its identifier {apiInvokerId} in order to verify that the invoker is registered in CAPIF.

Table C.2-3-OCF: Discover APIs request (OCF example)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs?api-invoker-id=INVaa03f9911db37be051b243ce3caad9

4. OK 200

Upon successful request, the API Invoker will receive the 200 OK code and all the information about the APIs published in the CAPIF Core Function containing the AEF profiles exposing the APIs. Using this information, the API Invoker can decide which APIs it wants to use.

Table C.2-4-OCF: Discover APIs response (schema)

200 Response body: { "serviceAPIDescription":[ { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" }, { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "protocol": "string", "dataFormat": "string", "securityMethods": [ "string", ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ] } ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string" "apiProvName": "string" } ] }

The API Invoker will receive a list with the APIs published in CAPIF, including the following information:

-	apiName: Name of the API to be published, which must be unique in CAPIF.

-	apiStatus: Represents the API status, in addition, if this attribute is omitted, it is understood that the API is available in all AEFs of aefProfiles. It contains the aefIds field which is a list of the AEFs that expose the API. If this list is empty or the attribute is omitted, it is understood that the API is inactive in all aefProfiles.

-	aefProfiles: List of Information about AEFs that expose the API. This information contains the following fields:

  - aefId: AEF identifier of the provider that exposes the service.

  - versions: list of information about API versions. Each version contains the resources supported by the API. Each version contains:

    - apiVersion: usable version of the API.

    - expiry: date on which the API version expires and will no longer be available.

    - resources: A list of resources that contain the API version. Each resource contains the following information:

      - resourceName: Name of the resource for which its information is specified.

      - commType: Type of use of the resource, which can be receiving a response when making a request (REQUEST_RESPONSE) or a subscription to receive notifications (SUBSCRIBE_NOTIFY)

      - uri: URI to make the request to use the described API resource.

      - custOpName: Part of the URI structure used for a custom operation associated with the resource. The attributes custOpName and custOperations are mutually exclusive.

      - custOperations: List of the resource's custom operations associated to this resource. The attributes custOpName and custOperations are mutually exclusive.

      - operations: list of HTTP methods supported by the API resource.

      - description: descriptive text of the API resource.

    - custOperations: List of custom operations without resource association.

  - protocol: indicates a protocol and protocol version used by the API.

  - dataFormat: Indicates the data sending format which can be JSON, XML or PROTOBUF3.

  - securityMethods: Indicates the possible security methods of the API.

  - domainName: Domain name to which the API belongs.

  - interfaceDescriptions: List of interfaces to which API requests can be made.

Only domainName or interfaceDescription can be specified.

-	supportedFeatures: Indicates the one or more CAPIF API specific supported features (from the nine features specified in 3GPP TS 29.222 [3]), e.g., support for API status monitoring in the CAPIF layer as part of the SEAL framework, support for RNAA functionality.

-	shareableinfo: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

-	serviceAPICategory: The service API category to which the service API belongs to.

-	apiSuppFeats: Indication provided by the consumer (i.e., related to the AEF) which optional features are supported by the service API.

-	apiProvName: API provider Name that publishes the API.

An example for the Discover APIs response for an NEF that exposes the “3gpp-as-session-with-qos” API is shown in Table C.2-4-OCF, for which there are twenty-nine optional service specific features (apiSuppFeats) specified in 3GPP TS 29.122 [14].

Table C.2-4-OCF: Discover APIs response (OCF example)

200 Response body: { "serviceAPIDescription":[ { "apiName": "3gpp-as-session-with-qos", "apiId": "f55731ec5f8ce703ac1f69605ad095", "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "1ff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiSuppFeats": "1fffffff", "apiProvName": "OCF_NEF" } ] }

5. CAPIF_Security_API

This request is responsible for requesting the creation of a security context to be able to use an API. If this step were not carried out, even if the API invoker had the information on an API it wished to use, the token giving access to the API would be missing, which can only be obtained once the security context has been created.

Table C.2-5: Create Security Context request (schema)

HTTP PUT {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} Request body: { "securityInfo": [ { "prefSecurityMethods": [ "string" ], "aefId": "string", "apiId": "string", "authenticationInfo": "string", "authorizationInfo": "string" } ], "notificationDestination": "string", "supportedFeatures": "string" }

In this request we find the following fields:

-	securityInfo: List of information that represents the details of the interfaces and security methods of the APIs the API invoker wishes to use.

-	prefSecurityMethods: List of preferred security methods for using an API. In the examples, OAuth tokens are used.

-	aefId: Identifier of the API exposing function that expose the API.

-	apiId: Identifier of the API that that the API invoker wishes to use.

-	notificationDestination: Address where the notifications should be delivered to.

The following example asks for OAUTH as the preferred security method. When this method is selected by the CAPIF Core Function, the API invoker must make another request to obtain the necessary security access token to consume the API.

Table C.2-5-OCF: Create Security Context request (OCF example)

HTTP PUT https://capif.mobilesandbox.cloud:37211/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9 Request body: { "securityInfo": [ { "prefSecurityMethods": ["OAUTH"], "selSecurityMethod": "OAUTH", "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "apiId": "f55731ec5f8ce703ac1f69605ad095", "authenticationInfo": "authenticationinfo", "authorizationInfo": "authorizationInfo" } ], "notificationDestination": "http://nef_notification_destination:8080/", "supportedFeatures": "f" }

OpenCAPIF selects one of the preferred methods to make requests to the API based in AEF profile. In this example, only one Security Method is requested by the API Invoker, so OpenCAPIF checks that this Security Method is supported by the API Exposer.

6. 201 Created

The security context has been created successfully and the security context created with the selected security method is returned in the response body.

Table C.2-6: Create Security Context response (schema)

201 Response headers: Location: {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} Response body: { "securityInfo": [ { "prefSecurityMethods": [ "string" ], "selSecurityMethod": "string", "aefId": "string", "apiId": "string", "authenticationInfo": "string", "authorizationInfo": "string" } ], "notificationDestination": "string", "supportedFeatures": "string" }

The {selSecurityMethod} is returned, which specifies the security method that will be used so that the API Invoker can consume the API and the URI of the created resource in the “Location” HTTP header that includes the {apiInvokerId}.

Table C.2-6-OCF: Create Security Context response (OCF example)

201 Response headers: Location: https://capif.mobilesandbox.cloud:3721/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9 Response body: { "securityInfo": [ { "prefSecurityMethods": ["OAUTH"], "selSecurityMethod": "OAUTH", "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "apiId": "f55731ec5f8ce703ac1f69605ad095", "authenticationInfo": "authenticationinfo", "authorizationInfo": "authorizationInfo" } ], "notificationDestination": "http://nef_notification_destination:8080/", "supportedFeatures": "f" }

7. CAPIF_Security_API (Obtain Token)

When the security method selected is OAUTH in the security context created by the API Invoker, the API Invoker must make a request to the CAPIF Core Function to obtain the access token to consume the API.

Table C.2-7: Obtain Access Token request (schema, then example content)

HTTP POST {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}/token Request body: { "grantType": "string", "clientId": "string", "scope": "string" }

The fields necessary to request the access token are:

-	grantType: It is used to specify whether the token request is made by the API Invoker themselves (client_credentials) or for the use of RNAA (authorization_code)

-	clientId: Identifier of the invoker that is requesting the token.

-	scope: a String that contains a list of AEF identifiers and its associated API names for which the access token is authorized for use. It takes the format of: 3gpp#aefId1:apiName1,apiName2;aefId2:apiName1,apiName2

The request to obtain the OAuth token in OpenCAPIF is displayed in the following table:

Table C.2-7-OCF: Obtain Access Token request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/capif-security/v1/trustedInvokers/INVaa03f9911db37be051b243ce3caad9/token Request body: { "grantType": "client_credentials", "clientId": "INVaa03f9911db37be051b243ce3caad9", "scope": "3gpp#AEFb1a522967511fd6d6def0cbb3b5f05:/nef/api/v1/3gpp-as-session-with-qos" }

8. 200 OK

In response, the API Invoker will obtain the token that is needed to consume the API that was specified in the request.

Table C.2-8: Obtain Access Token response (schema)

200 Response body: { "access_token": "string", "token_type": "string", "expires_in": "integer", "scope": "string" }

The fields returned in the response are:

-	access_token: OAuth access token necessary to make the request to the selected API.

-	token_type: Contains the type of token that is returned (i.e. “Bearer”).

-	expires_in: Contains the number of seconds after which the access_token expires.

-	scope: It is the scope requested in the request and that uses the token to enable the use of an API. It takes the following format: 3gpp#aefId1:apiName1,apiName2;aefId2:apiName1,apiName2

Table C.2-8-OCF: Obtain Access Token response (OCF example)

200 Response body: { "access_token": "eyJhbGciOiJSUzI1NiIsInR5cCI6IkpXVCJ9", "token_type": "Bearer", "expires_in": 600, "scope": "3gpp#AEFb1a522967511fd6d6def0cbb3b5f05:3gpp-as-session-with-qos" }

9. CAPIF_API_Invocation

The API invoker’s request to the consuming API does not go through CAPIF, rather it is made directly to the API Exposer. For this request, the data obtained from AEF Profile in the Discover request is used (IP/hostname, port, endpoint, method, etc...), it is also necessary to use the access token obtained from CAPIF that will be in the header of the request.

As an example, the request is to the API that has been published as a NEF provider and the OAuth token that has been obtained as an API Invoker.

Table C.2-9: Invoke API request (schema)

HTTP {operation} {ipv4Addr}:{port}{uri} Request headers: Authorization: "string" Content-Type: "string" Request body: { API BODY }

To make the request to the API, the access token obtained from the CAPIF Core Function must be entered in the Authorization header, from there, everything depends on the description of the published API.

Each description contains either the ipv4Addr or the domainName, both along with the port to which the request can be made. Also, depending on the resource to be called, the appropriate operation and uri specific to that resource. This is in addition to having to pass data and use the format according to the dataFormat field and also specifying this format in the header.

The following example shows how an invoker would use the NEF API that has been published:

Table C.2-9-OCF: Invoke API request (OCF example)

HTTP POST https://127.0.0.1:4443/nef/api/3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions Request headers: Authorization: Bearer eyJhbGciOiJSUzI1NiIsInR5cCI6IkpXVCJ9 Content-Type: "application/json" Request body: { "ipv4Addr": "10.0.0.3", "notificationDestination": "http://localhost:80/api/v1/utils/session-with-qos/callback", "snssai": { "sst": 1, "sd": "000001" }, "dnn": "province1.mnc01.mcc202.gprs", "qosReference": 9, "altQoSReferences": [0], "qosMonInfo": { "reqQosMonParams": ["DOWNLINK"], "repFreqs": ["EVENT_TRIGGERED"], "repThreshDl": 20, "repThreshUl": 20, "repThreshRp": 50, "waitTime": 2, "repPeriod": 3 } }

This would make a request to the NEF API that was used as an example. In this case, the API Invoker would receive the code 201 Created as it is used to create a subscription to the AsSessionWithQoS.

10. CAPIF_API_Invoker_Management (Deregistration)

The invoker can request its deletion from CAPIF using the offboarding service. By using this service, all information stored in the CAPIF Core Function related to the invoker will be deleted, including all the security contexts created for the APIs, ACLs, events subscriptions, etc.

Table C.2-10: DELETE API Invoker request (schema, then example content)

HTTP DELETE {apiRoot}/api-invoker-management/v1/onboardedInvokers/{apiInvokerId}

For the process of deleting an invoker in CAPIF, the certificate and identifier of the invoker to be deleted is required. That information is obtained through the invoker creation process. Then by making the following request, the invoker is eliminated.

Table C.2-10-OCF: DELETE API Invoker request (OCF example)

HTTP DELETE https://capif.mobilesandbox.cloud:37211/api-invoker-management/v1/onboardedInvokers/INVaa03f9911db37be051b243ce3caad9

11. 204 No Content

The successful response will be a 204 No Content, which indicates that the invoker has been successfully deleted along with everything related to it: security context of the APIs; ACLs of the services; events subscriptions etc.

Annex D:
CAPIF Test cases

# D.1	General

This Annex contains CAPIF test cases including the expected responses from CCF. The test cases defined enable testing not only the success responses (200, 201, 204) but also error codes as defined in 3GPP TS 29.222 [3] Annex A (normative) OpenAPI specification. These test cases are aligned with 3GPP TS 29.222 [3].

The annex defines tests for the following CAPIF APIs:

- CAPIF_API_Invoker_Management_API

- CAPIF_API_Provider_Management_API

- CAPIF_Publish_Service_API

- CAPIF_Discover_Service_API

- CAPIF_Events_API

- CAPIF_Security_API

- CAPIF_Access_Control_Policy_API

- CAPIF_Logging_API_Invocation_API

- CAPIF_Auditing_API

# D.2	Test Plan for CAPIF API Invoker Management

This section list test cases for the CAPIF_API_Invoker_Management_API.

### Test Case 1: Onboard Invoker

| Test ID | capif_api_invoker_management-1 |
| --- | --- |
| Description | This test will try to register a new Invoker at CAPIF Core |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was not onboarded previously |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate |
| Information of Test | - Create CSR, public and private key at invoker - Onboard Invoker: - Send POST to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers - “onboardingInformation” -> ”apiInvokerPublicKey”: must contain CSR generated by Invoker - Send in Authorization Header the Bearer access_token obtained previously (Authorization:Bearer ${access_token}) |
| Expected Result | - Onboarding response: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} |

### Test Case 2: Onboard Invoker Already onboarded

| Test ID | capif_api_invoker_management-2 |
| --- | --- |
| Description | This test will check second onboard of same Invoker is not allowed. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was onboarded previously |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Onboard again the Invoker at CCF |
| Information of Test | - Perform Onboard Invoker - Repeat Onboard Invoker: - Send POST to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers - “onboardingInformation” -> ”apiInvokerPublicKey”: must contain CSR generated by Invoker - Send in Authorization Header the Bearer access_token obtained previously (Authorization:Bearer ${access_token}) |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Second Onboard must accomplish: - Status code 403 Forbidden - Error Response Body must accomplish with ProblemDetails data structure with: - status 403 - title with message “Forbidden” - detail with message “Invoker Already registered” - cause with message “Identical invoker public key” |

### Test Case 3: Update Onboarded Invoker

| Test ID | capif_api_invoker_management-3 |
| --- | --- |
| Description | This test will try to update information of previous onboard Invoker at CAPIF Core. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was onboarded previously with {onboardingId} |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Update Onboarding Information at CCF with a minor change on “notificationDestination” |
| Information of Test | - Perform Onboard Invoker - Update information of previously onboarded Invoker: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - “notificationDestination”: "http://host.docker.internal:8086/netapp_new_callback" |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Update Request (PUT) with minor change must contain: - Status code 200 OK - “notificationDestination” on response must contain the new value |

### Test Case 4: Update Not Onboarded Invoker

| Test ID | capif_api_invoker_management-4 |
| --- | --- |
| Description | This test will try to update information of not onboarded Invoker at CAPIF Core. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was not onboarded previously |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Update Onboarding Information at CCF of not onboarded |
| Information of Test | - Perform Onboard Invoker - Update information of not onboarded Invoker: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Update Request (PUT) must contain: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message “Not Found” - detail with message “Please provide an existing Network App ID” - cause with message “Not exist Network App ID” |

### Test Case 5: Offboard Invoker

| Test ID | capif_api_invoker_management-5 |
| --- | --- |
| Description | This test case will check that a Registered Invoker can be deleted. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was onboarded previously |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Offboard Invoker at CCF |
| Information of Test | - Perform Onboard Invoker - Offboard onboarded Invoker: - Send DELETE to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Offboard Request (PUT) must contain: - Status code 204 Not Content |

### Test Case 6: Offboard Not previously Onboarded Invoker

| Test ID | capif_api_invoker_management-6 |
| --- | --- |
| Description | This test case will check that a Non-Registered Network App cannot be deleted. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was not onboarded previously |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Offboard Invoker at CCF |
| Information of Test | - Perform Onboard Invoker - Offboard onboarded Invoker: - Send DELETE to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Offboard Request (DELETE) must contain: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message “Not Found” - detail with message “Please provide an existing Network App ID” - cause with message “Not exist Network App ID” |

### Test Case 7: Update Onboarded Invoker Certificate

| Test ID | capif_api_invoker_management-7 |
| --- | --- |
| Description | This test will try to update public key and get a new signed certificate by CAPIF Core. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Invoker was onboarded previously with {onboardingId} |
| Execution Steps | - Onboard Invoker at CCF - Store signed Certificate - Update Onboarding Information at CCF with a new public key and CSR |
| Information of Test | - Perform Onboard Invoker - Create new CSR, public and private key at invoker - Update information of previously onboarded Invoker: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - Store new invoker certificate. |
| Expected Result | - Response to first Onboard request must accomplish: - Status code 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the certificate signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Update Request (PUT) must contain: - Status code 200 OK - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerCertificate with new certificate on response -> store to use. |

# D.3	Test Plan for CAPIF API Provider Management

This section list test cases for the CAPIF_API_Provider_Management_API.

### Test Case 1: Onboard API Provider

| Test ID | capif_api_provider_management-1 |
| --- | --- |
| Description | This test case will check that Api Provider can be registered at CCF |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Store signed Certificates |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider: - Send POST to https://{CAPIF_HOSTNAME}/api-provider-management/v1/registrations - Each function in list “apiProvFuncs” must contain a CSR generated by Provider in “regInfo”-> ”apiProvPubKey” - Send in Authorization Header the Bearer access_token obtained previously (Authorization:Bearer ${access_token}) |
| Expected Result | - Onboarding response: - Status code 201 Created - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} |

### Test Case 2: Onboard API Provider Already Onboarded

| Test ID | capif_api_provider_management-2 |
| --- | --- |
| Description | This test case will check that an API Provider previously registered cannot be re-registered |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Store signed Certificates - Onboard Again the Provider at CCF |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider: - Send POST to https://{CAPIF_HOSTNAME}/api-provider-management/v1/registrations - Each function in list “apiProvFuncs” must contain a CSR generated by Provider in “regInfo”-> ”apiProvPubKey” - Send in Authorization Header the Bearer access_token obtained previously (Authorization:Bearer ${access_token}) - Onboard Again the Provider: - Send POST to https://{CAPIF_HOSTNAME}/api-provider-management/v1/registrations - Each function in list “apiProvFuncs” must contain a CSR generated by Provider in “regInfo”-> ”apiProvPubKey” - Send in Authorization Header the Bearer access_token obtained previously (Authorization:Bearer ${access_token}) |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Response to Second Onboard must accomplish: - Status code 403 Forbidden - Error Response Body must accomplish with ProblemDetails data structure with: - status 403 - title with message “Forbidden” - detail with message “Provider Already registered” - cause with message “Identical Provider reg sec” |

### Test Case 3: Update Onboarded API Provider

| Test ID | capif_api_provider_management-3 |
| --- | --- |
| Description | This test case will check that a Registered API Provider can be updated |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Update Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider - Update information of previously onboarded Provider: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - “apiProvDomInfo”: "ROBOT_TESTING_MOD" - Use AMF Certificate |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Update Provider must accomplish: - Status code 200 OK - Body returned must accomplish APIProviderEnrolmentDetails data structure, with - “apiProvDomInfo” set to ROBOT_TESTING_MOD |

### Test Case 4: Update API Provider Not Onboarded

| Test ID | capif_api_provider_management-2 |
| --- | --- |
| Description | This test case will check that a Non-Registered API Provider cannot be updated |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Update Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider - Update information of previously onboarded Provider: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - “apiProvDomInfo”: "ROBOT_TESTING_MOD" |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Update Provider must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message “Not Found” - detail with message “Not Exist Provider Enrolment Details” - cause with message “Not found registrations to Send THIS api provider details” |

### Test Case 5: Partially Update Onboarded API Provider

| Test ID | capif_api_provider_management-5 |
| --- | --- |
| Description | This test case will check that a Registered API Provider can be partially updated |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Partial Update Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider - Partial update information of previously onboarded Provider: - Send PATCH to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - “apiProvDomInfo”: "ROBOT_TESTING_MOD" - Use AMF Certificate |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Partial update Provider must accomplish: - Status code 200 OK - Body returned must accomplish APIProviderEnrolmentDetails data structure, with - “apiProvDomInfo” set to ROBOT_TESTING_MOD |

### Test Case 6: Partially Update API Provider Not Onboarded

| Test ID | capif_api_provider_management-6 |
| --- | --- |
| Description | This test case will check that a Non-Registered API Provider cannot be partially updated |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Partial Update Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider - Partial update information of previously onboarded Provider: - Send PATCH to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - “apiProvDomInfo”: "ROBOT_TESTING_MOD" |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Partial update Provider must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message “Not Found” - detail with message “Not Exist Provider Enrolment Details” - cause with message “Not found registrations to Send THIS api provider details” |

### Test Case 7: Delete Onboarded API Provider

| Test ID | capif_api_provider_management-7 |
| --- | --- |
| Description | This test case will check that a Registered API Provider can be deleted |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Store signed Certificates - Delete Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider - Delete Provider: - Send DELETE to https://{CAPIF_HOSTNAME}/api-provider-management/v1/registrations/{registrationId} - Use AMF certificate |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider -management/{apiVersion}/registrations /{registrationId} - Delete Provider must accomplish: - Status code 204 Not Found |

### Test Case 8: Delete not Onboarded API Provider

| Test ID | capif_api_provider_management-8 |
| --- | --- |
| Description | This test case will check that a Non-Registered API Provider cannot be deleted |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Store signed Certificates - Delete Provider |
| Information of Test | - Create CSR, public and private key for each function. - Onboard Provider: - Delete Provider: a. Send DELETE to https://{CAPIF_HOSTNAME}/api-provider-management/v1/registrations/{registrationId} b. Use AMF certificate |
| Expected Result | - Response to first Onboard request must accomplish: - Response Body must follow APIProviderEnrolmentDetails data structure. - For each “apiProvFuncs”, we must check: - “apiProvFuncId” is set - “apiProvCert” under “regInfo” is set properly - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-provider-management/{apiVersion}/registrations /{registrationId} - Responso to Delete Provider must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message “Not Found” - detail with message “Not Exist Provider Enrolment Details” - cause with message “Not found registrations to Send THIS api provider details” |

# D.4	Test Plan for CAPIF API Publish Service

This section list test cases for the CAPIF_Publish_Service_API.

### Test Case 1: Publish API by Authorised API Publisher

| Test ID | capif_api_publish_service-1 |
| --- | --- |
| Description | This test case will check that an API Publisher can Publish an API |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response |
| Information of Test | - Onboard Provider - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Published Service API is stored in CAPIF Database. |

### Test Case 2: Publish API by NON-Authorised API Publisher

| Test ID | capif_api_publish_service-2 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Publish an API without valid apfId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded |
| Execution Steps | - Onboard Provider at CCF - Publish Service API with invalid APF ID |
| Information of Test | - Onboard Provider - Publish Service API with invalid ID at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfIdNotValid}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “Publisher not existing” - cause with message “Publisher id not found” - Published Service API is NOT stored in CAPIF Database. |

### Test Case 3: Retrieve all APIs Published by Authorised API Publisher

| Test ID | capif_api_publish_service-3 |
| --- | --- |
| Description | This test case will check that an API Publisher can Retrieve all API published |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously At least 2 services APIs are published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API service_1 - Retrieve {apiId1} from body and Location header with new resource created from response - Publish Service API service_2 - Retrieve {apiId2} from body and Location header with new resource created from response - Retrieve All published APIs and check if both are present |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId1 - Use APF Certificate - Publish Other Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_2 - Get apiId2 - Use APF Certificate - Retrieve all published APIs: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Use APF Certificate |
| Expected Result | - Response to service 1 Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId1 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId1} - Response to service 2 Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId2 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId2} - Published Service APIs are stored in CAPIF Database - Response to Retrieve all published APIs - 200 OK - Response body must return an array of ServiceAPIDescription data. - Array must contain all previously published APIs. |

### Test Case 4: Retrieve all APIs Published by NON Authorised API Publisher

| Test ID | capif_api_publish_service-4 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Retrieve API published when apfId is not authorised |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was not onboarded previously At least 2 services APIs are published |
| Execution Steps | - Onboard Provider at CCF - Retrieve All published APIs |
| Information of Test | - Onboard Provider - Retrieve all published APIs: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “Publisher not existing” - cause with message “Publisher id not found” - Published Service API is NOT stored in CAPIF Database. |

### Test Case 5: Retrieve single APIs Published by Authorised API Publisher

| Test ID | capif_api_publish_service-5 |
| --- | --- |
| Description | This test case will check that an API Publisher can Retrieve API published one by one |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously At least 2 services APIs are published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API service_1 - Retrieve {apiId1} from body and Location header with new resource created from response - Publish Service API service_2 - Retrieve {apiId2} from body and Location header with new resource created from response - Retrieve service_1 API Detail - Retrieve service_2 API Detail |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId1 - Use APF Certificate - Publish Other Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_2 - Get apiId2 - Use APF Certificate - Retrieve service_1 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId1} - Use APF Certificate - Retrieve service_2 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId2} - Use APF Certificate |
| Expected Result | - Response to service_1 Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId1 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId1} - Response to service_2 Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId2 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId2} - Published Service APIs are stored in CAPIF Database - Response to Retrieve service_1 published API using apiId1: - 200 OK - Response body must return an array of ServiceAPIDescription data. - Array must contain same information than service_1 published registration response. - Response to Retrieve service_2 published API using apiId2: - 200 OK - Response body must return an array of ServiceAPIDescription data. - Array must contain same information than service_2 published registration response. |

### Test Case 6: Retrieve single APIs NON Published by Authorised API Publisher

| Test ID | capif_api_publish_service-6 |
| --- | --- |
| Description | This test case will check that an API Publisher try to get detail of not published API. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously No published API. |
| Execution Steps | - Onboard Provider at CCF - Retrieve not published API Detail |
| Information of Test | - Onboard Provider - Retrieve not published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{SERVICE_API_ID_NOT_VALID} - Use APF Certificate |
| Expected Result | - Response to Retrieve for NOT published API must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - title with message “Not Found” - detail with message “Service API not found” - cause with message “No Service with specific credentials exists” |

### Test Case 7: Retrieve single APIs Published by NON Authorised API Publisher

| Test ID | capif_api_publish_service-7 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Retrieve detailed API published when apfId is not authorised |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Provider was onboarded Invoker was onboarded |
| Execution Steps | - Onboard Provider at CCF - Publish Service API at CCF - Retrieve {apiId1} from body and Location header with new resource created from response - Onboard Invoker at CCF and store signed Invoker Certificate - Retrieve detailed published API acting as Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId1 - Use APF Certificate - Retrieve service_1 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId1} - Use Invoker Certificate |
| Expected Result | - Response to Retrieve for NOT published API must accomplish: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “User not authorized” - cause with message “Certificate not authorized” |

### Test Case 8: Update API Published by Authorised API Publisher with valid serviceApiId

| Test ID | capif_api_publish_service-8 |
| --- | --- |
| Description | This test case will check that an API Publisher can Update published API with a valid serviceApiId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously A service APIs is published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response - Update published Service API - Retrieve detail of Service API |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Update published API at CCF: - Send PUT to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Body serviceAPIDescription with overrided apiName to service_1_modified - Use APF Certificate - Retrieve service_1 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Use APF Certificate - Check apiName is service_1_modified |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Update Published service request must accomplish: - Status code 200 OK - Response body must follow ServiceAPIDescription data structure with: - apiName service_1_modified - Response to Retrieve detail of Service API: - 200 OK - Response body must follow ServiceAPIDescription data structure with: - apiName service_1_modified |

### Test Case 9: Update API Published by Authorised API Publisher with invalid serviceApiId

| Test ID | capif_api_publish_service-9 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Update published API with a invalid serviceApiId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously A service API is published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response - Update published Service API |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Update published API at CCF: - Send PUT to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{SERVICE_API_ID_NOT_VALID} - Body serviceAPIDescription with overrided apiName to service_1_modified - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Update Published service request must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - title with message “Not Found” - detail with message “Service API not found” |

### Test Case 10: Update API Published by NON Authorised API Publisher

| Test ID | capif_api_publish_service-10 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Update API published when apfId is not authorised |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Provider was onboarded Invoker was onboarded |
| Execution Steps | - Onboard Provider at CCF - Publish Service API at CCF - Retrieve {apiId1} from body and Location header with new resource created from response - Onboard Invoker at CCF and store signed Invoker Certificate - Update published API at CCF as Invoker - Retrieve detail of Service API as Provider |
| Information of Test | - Onboard Provider and Invoker - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Update published API at CCF: - Send PUT to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{SERVICE_API_ID_NOT_VALID} - Body serviceAPIDescription with overrided apiName to service_1_modified - Use Invoker Certificate - Retrieve service_1 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Use APF Certificate - Check apiName is service_1 |
| Expected Result | - Response to Update published API acting as Invoker must accomplish: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “User not authorized” - cause with message “Certificate not authorized” - Response to Retrieve Detail of Service API - 200 OK - Response Body must follow ServiceAPIDescription data structure with: - apiName service_1 |

### Test Case 11: Delete API Published by Authorised API Publisher with valid serviceApiId

| Test ID | capif_api_publish_service-11 |
| --- | --- |
| Description | This test case will check that an API Publisher can Delete published API with a valid serviceApiId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously A service API is published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response - Remove published Service API at CCF - Try to retrieve deleted Service API from CCF |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Remove published API at CCF: - Send DELETE to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Use APF Certificate - Retrieve service_1 published API Details: - Send GET to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Remove Published service request must accomplish: - Status code 204 NoContent - Response to Retrieve detail for DELETED published API must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - title with message “Service API not found” - detail with message “No Service with specific credentials exists” |

### Test Case 12: Delete API Published by Authorised API Publisher with invalid serviceApiId

| Test ID | capif_api_publish_service-12 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Delete with invalid serviceApiId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously A service API is published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response - Remove published Service API at CCF |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Remove published API at CCF: - Send DELETE to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{SERVICE_API_ID_NOT_VALID} - Use APF Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Remove Published service request must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - title with message “Service API not found” - detail with message “Service API id not found” |

### Test Case 13: Delete API Published by NON Authorised API Publisher with valid serviceApiId

| Test ID | capif_api_publish_service-12 |
| --- | --- |
| Description | This test case will check that an API Publisher cannot Delete with invalid serviceApiId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard a Provider. Provider was onboarded previously A service APIs is published |
| Execution Steps | - Onboard Provider at CCF - Publish Service API - Retrieve {apiId} from body and Location header with new resource created from response - Onboard Invoker at CCF and store signed Invoker Certificate - Remove published Service API at CCF with invalid serviced as Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Remove published API at CCF as Invoker: - Send DELETE to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{apiId} - Use Invoker Certificate |
| Expected Result | - Response to Remove Published service request must accomplish: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “User not authorized” - cause with message “Certificate not authorized” |

# D.5	Test Plan for CAPIF Discover Service API

This section list test cases for the CAPIF_Discover_Service_API.

### Test Case 1: Discover Published Service Apis by Authorised Invoker

| Test ID | capif_api_discover _service-1 |
| --- | --- |
| Description | This test case will check if Invoker can discover published service APIs. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Request Discover Published APIs: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Use Invoker Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Discover Request by Invoker: - Status code 200 OK - Response body must follow DiscoveredAPIs data structure: - Check if DiscoveredAPIs contains the API Published previously |

### Test Case 2: Discover Published Service APIs by Non Authorised Invoker

| Test ID | capif_api_discover _service-2 |
| --- | --- |
| Description | This test case will check that an API Publisher can't discover published APIs because is not authorized. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Request Discover Published APIs: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Use not Invoker Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Discover Request by no Invoker: - Status code 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - Status 401 - title with message “Unauthorized” - detail with message “User not authorized” - cause with message “Certificate not authorized” |

### Test Case 3: Discover Published Service APIs by Not Registered API Invoker

| Test ID | capif_api_discover _service-3 |
| --- | --- |
| Description | This test case will check that a not registered invoker is forbidden to discover published APIs. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Request Discover Published API with not valid apiInvokerId: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={NOT_VALID_INVOKER_ID} - Param api-invoker-id is mandatory - Use Invoker Certificate |
| Expected Result | - Response to Publish request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId} - Response to Discover Request by Invoker: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - title with message “API Invoker does not exist” - detail with message “API Invoker id not found” |

### Test Case 4: Discover Published Service APIs by Authorised Invoker with 1 result filtered

| Test ID | capif_api_discover _service-4 |
| --- | --- |
| Description | This test case will check if Invoker can discover published service APIs |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. At least 2 Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API service_1 and service_2 at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker - Discover filtered by api-name service_1 Service APIs by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_2 - Use APF Certificate - Request Discover Published APIs filtering by api-name: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId}&api-name=service_1 - Param api-invoker-id is mandatory - Use Invoker Certificate - Filter by api-name service_1 |
| Expected Result | - Response to Publish service_1 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId1 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId1} - Response to Publish service_2 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId2 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId2} - Response to Discover Request by Invoker: - Status code 200 OK - Response body must follow DiscoveredAPIs data structure: - Check if DiscoveredAPIs contains only the API Published with api-name service_1 |

### Test Case 5: Discover Published Service APIs by Authorised Invoker with no match

| Test ID | capif_api_discover _service-5 |
| --- | --- |
| Description | This test case will check if Invoker can discover published service APIs. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. At least 2 Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API service_1 and service_2 at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker - Discover filtered by api-name not published Service API by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_2 - Use APF Certificate - Request Discover Published APIs filtering by api-name not published: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId}&api-name=NOT_VALID_NAME - Param api-invoker-id is mandatory - Use Invoker Certificate - Filter by api-name NOT_VALID_NAME |
| Expected Result | - Response to Publish service_1 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId1 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId1} - Response to Publish service_2 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId2 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId2} - Response to Discover Request by Invoker: - Status code 200 OK - Response body must follow DiscoveredAPIs data structure: - Check if DiscoveredAPIs contains the previously registered Service API Published. - Response to Discover Request By Invoker: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - title with message “API Invoker{api_invoker_id} has no API Published that accomplish filter conditions” - detail with message “No API message Published accomplish filter conditions” |

### Test Case 6: Discover Published Service APIs by Authorised Invoker not filtered

| Test ID | capif_api_discover _service-6 |
| --- | --- |
| Description | This test case will check if Invoker can discover published service APIs. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. At least 2 Service APIs are published |
| Execution Steps | - Onboard Provider at CCF and publish Service API service_1 and service_2 at CCF. - Onboard Invoker at CCF - Discover Service APIs by Invoker - Discover without filter by Invoker |
| Information of Test | - Onboard Provider and Invoker - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Use APF Certificate - Publish Service API: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_2 - Use APF Certificate - Request Discover Published APIs filtering by api-name not published: - Send GET to https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Use Invoker Certificate |
| Expected Result | - Response to Publish service_1 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId1 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId1} - Response to Publish service_2 request must accomplish: - Status code 201 Created - Response body must follow ServiceAPIDescription data structure with: - apiId2 - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/published-apis/{apiVersion}/{apfId}/service-apis/{serviceApiId2} - Response to Discover Request by Invoker: - Status code 200 OK - Response body must follow DiscoveredAPIs data structure: - Check if DiscoveredAPIs contains the 2 previously registered Service API Published. |

# D.6	Test Plan for CAPIF API Security Service

This section list test cases for the CAPIF_Security_API.

### Test Case 1: Creates a Security Context for an API Invoker

| Test ID | capif_api_security-1 |
| --- | --- |
| Description | This test case will check that an API Invoker can create a Security context |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. |
| Execution Steps | - Onboard Invoker at CCF. - Stored signed Certificate - Create Security Context |
| Information of Test | - Onboard Invoker at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate |
| Expected Result | - Create security context: - 201 Created response. - body returned must accomplish ServiceSecurity data structure. - Location Header must contain the new resource URL {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} |

### Test Case 2: Creates a Security Context for an API Invoker with provider role

| Test ID | capif_api_security-2 |
| --- | --- |
| Description | This test case will check that a Provider cannot create a Security context with valid apiInvokerId. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF - Create Security Context using Provider Certificate |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use AEF Certificate |
| Expected Result | - Create security context using provider certificate: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be invoker". - No context stored at Database |

### Test Case 3: Creates a Security Context for an API Invoker with provider role and invalid apiInvokerId

| Test ID | capif_api_security-3 |
| --- | --- |
| Description | This test case will check that a Provider cannot create a Security context with invalid apiInvokerID. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF - Create Security Context using Provider Certificate and not valid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API _INVOKER_NOT_VALID} - Use AEF Certificate |
| Expected Result | - Create security context using provider certificate: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be invoker". - No context stored at Database |

### Test Case 4: Creates a Security Context for an API Invoker with Invoker role and invalid apiInvokerId

| Test ID | capif_api_security-4 |
| --- | --- |
| Description | This test case will check that an Invoker cannot create a Security context with valid apiInvokerId. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. |
| Execution Steps | - Onboard Invoker at CCF. - Create Security Context using invoker Certificate and not valid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API _INVOKER_NOT_VALID} - Use Invoker Certificate |
| Expected Result | - Create security context using Invoker certificate: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Invoker not found". - cause with message "API Invoker not exists or invalid ID". - No context stored at Database |

### Test Case 5: retrieve the Security Context of an API Invoker

| Test ID | capif_api_security-5 |
| --- | --- |
| Description | This test case will check that a provider can retrieve the Security context of an API Invoker |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Retrieve Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Retrieve Security Context of Invoker by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate |
| Expected Result | - Retrieve security context: - 200 OK response. - body returned must accomplish ServiceSecurity data structure. |

### Test Case 6: retrieve the Security Context of an API Invoker with invalid apiInvokerId

| Test ID | capif_api_security-6 |
| --- | --- |
| Description | This test case will check that a provider can retrieve the Security context of an API Invoker |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Retrieve Security Context by Provider of invalid invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Retrieve Security Context of Invoker by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Using AEF Certificate |
| Expected Result | - Retrieve security context: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Invoker not found". - cause with message "API Invoker not exists or invalid ID". |

### Test Case 7: Retrieve the Security Context of an API Invoker with invalid apfId

| Test ID | capif_api_security-7 |
| --- | --- |
| Description | This test case will check that a provider cannot retrieve the Security context of an API Invoker without valid apfId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Retrieve Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Retrieve Security Context of Invoker by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate |
| Expected Result | - Retrieve security context: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be aef". |

### Test Case 8: Delete the Security Context of an API Invoker

| Test ID | capif_api_security-8 |
| --- | --- |
| Description | This test case will check that a provider can delete a Security context |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Delete Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Delete Security Context of Invoker by Provider: - Send DELETE https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use AEF Certificate - Retrieve Security Context of Invoker by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate |
| Expected Result | - Delete Security Context: - 204 No Content response - Retrieve security context: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Security context not found". - cause with message "API Invoker not exists or invalid ID". |

### Test Case 9: Delete the Security Context of an API Invoker with Invoker entity role

| Test ID | capif_api_security-9 |
| --- | --- |
| Description | This test case will check that an Invoker cannot delete a Security context |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Delete Security Context by Invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Delete Security Context of Invoker by invoker: - Send DELETE https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate |
| Expected Result | - Delete Security Contex by invokert: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be aef". |

### Test Case 10: Delete the Security Context of an API Invoker with Invoker entity role and invalid apiInvokerId

| Test ID | capif_api_security-10 |
| --- | --- |
| Description | This test case will check that an Invoker cannot delete a Security context with invalid apiInvokerId. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Delete Security Context by Invoker with invalid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Delete Security Context of Invoker by invoker: - Send DELETE https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Use Invoker Certificate |
| Expected Result | - Delete Security Contex by invokert: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be aef". |

### Test Case 11: Delete the Security Context of an API Invoker with invalid apiInvokerId

| Test ID | capif_api_security-11 |
| --- | --- |
| Description | This test case will check that an Provider cannot delete a Security context of invalid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Delete Security Context by Provider with invalid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Delete Security Context of Invoker by invoker: - Send DELETE https://{CAPIF_HOSTNAME}/trustedInvokers/{NOT_VALID_API_INVOKER_ID} - Use AEF Certificate |
| Expected Result | - Delete Security Contex by invokert: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Invoker not found". - cause with message "API Invoker not exists or invalid ID". |

### Test Case 12: Update the Security Context of an API Invoker

| Test ID | capif_api_security-12 |
| --- | --- |
| Description | This test case will check that an API Invoker can update a Security context |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Update Security context by invoker - Retrieve Security Context By Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Update Security Context of Invoker: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/update - body service security body but with notification destination modified to http://robot.testing2 - Using Invoker Certificate. - Retrieve Security Context of Invoker by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate. |
| Expected Result | - Update Security Context by invoker: - 200 OK response. - body returned must accomplish ServiceSecurity data structure. - Retrieve security context: - 200 OK response. - body returned must accomplish ServiceSecurity data structure. - Check is this returned object match with modified one. |

### Test Case 13: Update the Security Context of an API Invoker with provider entity role

| Test ID | capif_api_security-13 |
| --- | --- |
| Description | This test case will check that a provider cannot update a Security context |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Update Security context as provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Update Security Context of Invoker: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/update - body service security body but with notification destination modified to http://robot.testing2 - Using AEF Certificate. |
| Expected Result | - Update Security Context by provider: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be aef". |

### Test Case 14: Update the Security Context of an API Invoker with provider entity role and invalid apiInvokerId

| Test ID | capif_api_security-14 |
| --- | --- |
| Description | This test case will check that a provider cannot update a Security context of invalid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Update Security context as provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Update Security Context of Invoker: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/update - body service security body but with notification destination modified to http://robot.testing2 - Using AEF Certificate. |
| Expected Result | - Update Security Context by provider: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be aef". |

### Test Case 15: Update the Security Context of an API Invoker with invalid apiInvokerId

| Test ID | capif_api_security-15 |
| --- | --- |
| Description | This test case will check that an API Invoker cannot update a Security context not valid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Update Security context as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Use Invoker Certificate - Update Security Context of Invoker: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/update - body service security body but with notification destination modified to http://robot.testing2 - Using Invoker Certificate. |
| Expected Result | - Update Security Context: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Invoker not found". - cause with message "API Invoker not exists or invalid ID". |

### Test Case 16: Revoke the authorization of the API Invoker for APIs

| Test ID | capif_api_security-16 |
| --- | --- |
| Description | This test case will check that a Provider can revoke the authorization for APIs |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Revoke Security Context by Provider - Retrieve Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Use Invoker Certificate - Revoke Authorization by Provider: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/delete - Using AEF Certificate. - Retrieve Security Context by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate. |
| Expected Result | - Revoke authorization: - 204 No Content response - Retrieve Security Context: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Security context not found". - cause with message "API Invoker has not security context". |

### Test Case 17: Revoke the authorization of the API Invoker for APIs without valid apfId

| Test ID | capif_api_security-17 |
| --- | --- |
| Description | This test case will check that an Invoker can't revoke the authorization for APIs |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Revoke Security Context by Invoker - Retrieve Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Use Invoker Certificate - Revoke Authorization by invoker: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/delete - Using Invoker Certificate. - Retrieve Security Context by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate. |
| Expected Result | - Revoke authorization: - 401 Unauthorized response. - body returned must accomplish ProblemDetails data structure, with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "User role must be provider". - Retrieve Security Context: - 200 OK response. - body returned must accomplish ServiceSecurity data structure. - Check is this returned object match with created one. |

### Test Case 18: Revoke the authorization of the API Invoker for APIs with invalid apiInvokerId

| Test ID | capif_api_security-18 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot revoke the authorization for APIs for invalid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Invoker at CCF. - Onboard Provider at CCF. - Create Security Context using invoker Certificate - Revoke Security Context by Provider wit invalid apiInvokerId - Retrieve Security Context by Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Create Security Context for this Invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID} - Use Invoker Certificate - Revoke Authorization by provider: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{API_INVOKER_NOT_VALID}/delete - Using Invoker Certificate. - Retrieve Security Context by Provider: - Send GET https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using AEF Certificate. |
| Expected Result | - Revoke authorization: - 404 Not Found response. - body returned must accomplish ProblemDetails data structure, with: - status 404 - title with message "Not Found" - detail with message "Invoker not found". - cause with message "API Invoker not exists or invalid ID". - Retrieve Security Context: - 200 OK response. - body returned must accomplish ServiceSecurity data structure. - Check is this returned object match with created one. |

### Test Case 19: Retrieve access token

| Test ID | capif_api_security-19 |
| --- | --- |
| Description | This test case will check that an API Invoker can retrieve a security access token OAuth 2.0. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{securityId}/token: - body access token req body and example example - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 200 OK - body must follow AccessTokenRsp with: - access_token present - token_type=Bearer |

### Test Case 20: Retrieve access token by provider

| Test ID | capif_api_security-20 |
| --- | --- |
| Description | This test case will check that an API Provider cannot retrieve a security access token OAuth 2.0. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as Provider |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{securityId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using AEF Certificate. |
| Expected Result | - Response to request access token: - 401 Unauthorized response. - body returned must accomplish AccessTokenErr data structure, with: - error unauthorized_client - error_description=Role not authorized for this API route |

### Test Case 21: Retrieve access token by provider with invalid apiInvokerId

| Test ID | capif_api_security-21 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token without valid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as Provider with invalid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{API_INVOKER_ID_NOT_VALID}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using AEF Certificate. |
| Expected Result | - Response to request access token: - 401 Unauthorized response. - body returned must accomplish AccessTokenErr data structure, with: - error unauthorized_client - error_description=Role not authorized for this API route |

### Test Case 22: Retrieve access token with invalid apiInvokerId

| Test ID | capif_api_security-22 |
| --- | --- |
| Description | This test case will check that an API Invoker can't retrieve a security access token without valid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker with invalid apiInvokerId |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{API_INVOKER_ID_NOT_VALID}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 404 Not Found response. - body returned must accomplish ProblemDetails29571 data structure, with: - status 404 - title Not Found - detail Security context not found - cause API Invoker has no security context |

### Test Case 23: Retrieve access token with invalid client_id

| Test ID | capif_api_security-23 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token without valid client_id at body |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{apiInvokerId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Client_id is not valid - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 400 Bad Request response. - body returned must accomplish AccessTokenErr data structure, with: - error invalid_client - error_description=Client Id not found |

### Test Case 24: Retrieve access token with unsupported grant_type

| Test ID | capif_api_security-24 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token with unsupported grant_type |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{apiInvokerId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=not_valid. - Create Scope properly for request: 3gpp#{aef_id}:{api_name} - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 400 Bad Request response. - body returned must accomplish AccessTokenErr data structure, with: - error unsupported_grant_type - error_description=Invalid value for grant_type \(${grant_type}\), must be one of \['client_credentials'\] - 'grant_type' |

### Test Case 25: Retrieve access token with invalid scope

| Test ID | capif_api_security-25 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token with complete invalid scope |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{apiInvokerId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: not-valid-scope - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 400 Bad Request response. - body returned must accomplish AccessTokenErr data structure, with: - error invalid scope - error_description=The first characters must be ‘3gpp’ |

### Test Case 26: Retrieve access token with invalid aefId at scope

| Test ID | capif_api_security-26 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token with invalid aefId at scope |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{apiInvokerId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#1234:*service_1 - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 400 Bad Request response. - body returned must accomplish AccessTokenErr data structure, with: - error invalid scope - error_description=One of aef_id does not belong of your security context |

### Test Case 27: Retrieve access token with invalid apiName at scope

| Test ID | capif_api_security-27 |
| --- | --- |
| Description | This test case will check that an API Exposure Function cannot retrieve a security access token with invalid apiName at scope |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. API Invoker has created a valid Security Context |
| Execution Steps | - Onboard Provider and publish Service API service_1 at CCF. - Onboard Invoker at CCF. - Discover service APIs by Invoker - Create Security Context using invoker Certificate - Request Access token as invoker |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body with apiName service_1 - Use APF Certificate - Request Discover Published APIs not filtered: - Send GET to ccf_discover_url https://{CAPIF_HOSTNAME}/service-apis/v1/allServiceAPIs?api-invoker-id={apiInvokerId} - Param api-invoker-id is mandatory - Using Invoker Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Using Invoker Certificate. - Create Security Information Body with one securityInfo for each aef present at each serviceAPIDescription present at Discover. - Request Access Token by invoker: - Sent POST https://{CAPIF_HOSTNAME}/securities/{apiInvokerId}/token: - body access token req body - securityId is apiInvokerId. - grant_type=client_credentials. - Create Scope properly for request: 3gpp#{aef_id}:not-valid - Using Invoker Certificate. |
| Expected Result | - Response to request access token: - 400 Bad Request response. - body returned must accomplish AccessTokenErr data structure, with: - error invalid scope - error_description=One of the api names does not exist or is not associated with the aef id provided |

# D.7		Test Plan for CAPIF API Access Control Policy Service

This section list test cases for the CAPIF_Access_Control_Policy_API.

### Test Case 1: Retrieve ACL

| Test ID | capif_api_acl -1 |
| --- | --- |
| Description | This test case will check that an API Provider can retrieve ACL from CAPIF |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId registered previously. |

### Test Case 2: Retrieve ACL with 2 Service APIs published

| Test ID | capif_api_acl-2 |
| --- | --- |
| Description | This test case will check that an API Provider can retrieve ACL from CAPIF for 2 different serviceApis published. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Services API published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 and service_2 - Create Security Context - Provider Get ACL information for service_1 - Provider Get ACL information for service_2 |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_2 - Use APF Certificate - Create Security Context for both published APIs - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL for serviceApiId1: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Certificate - Provider Retrieve ACL for serviceApiId2: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId2}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Certificate |
| Expected Result | - Response to ACL Service for service_1 must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId1 registered previously. - Response to ACL Service for service_2 must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId2 registered previously. |

### Test Case 3: Retrieve ACL with security context created by two different Invokers

| Test ID | capif_api_acl-3 |
| --- | --- |
| Description | This test case will check that an API Provider can retrieve ACL from CAPIF containing 2 objects. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service API published in CAPIF Security context created by two API Invokers for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 and service_2 - Create Security Context - Provider Get ACL information for service_1 - Provider Get ACL information for service_2 |
| Information of Test | - Onboard two Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for each invoker: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL for serviceApiId: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain two objects. - One object must match with apiInvokerId1 and the other one with apiInvokerId2. |

### Test Case 4: Retrieve ACL filtered by api-invoker-id

| Test ID | capif_api_acl-4 |
| --- | --- |
| Description | This test case will check that an API Provider can retrieve ACL filtering by apiInvokerId from CAPIF containing 1 objects. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service API published in CAPIF Security context created by two API Invokers for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 and service_2 - Create Security Context - Provider Get ACL information for service_1 indicating first api-invoker-id - Provider Get ACL information for service_2 indicating second api-invoker-id |
| Information of Test | - Onboard two Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Publish another Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_2 - Use APF Certificate - Create Security Context for each invoker for both published APIs: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL for serviceApiId1: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id}&api-invoker-id={apiInvokerId1} - Use serviceApiId, aefId and apiInvokerId1 - Use AEF Certificate - Provider Retrieve ACL for serviceApiId2: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id}&api-invoker-id={apiInvokerId2} - Use serviceApiId, aefId and apiInvokerId2 - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain two objects. - One object must match with apiInvokerId1 - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain two objects. - One object must match with apiInvokerId2 |

### Test Case 5: Retrieve ACL filtered by supported-features

| Test ID | capif_api_acl-5 |
| --- | --- |
| Description | This test case will check that an API Provider can retrieve ACL filtering by supportedFeatures from CAPIF containing 1 objects. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service API published in CAPIF Security context created by two API Invokers for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information indicating first supported-features - Provider Get ACL information indicating second supported-features |
| Information of Test | - Onboard two Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for each invoker for both published APIs: - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL for serviceApiId1: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id}&supported-features={apiInvokerId1} - Use serviceApiId, aefId and apiInvokerId1 - Use AEF Certificate - Provider Retrieve ACL for serviceApiId2: - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId1}?aef-id=${aef_id}& supported-features={apiInvokerId2} - Use serviceApiId, aefId and apiInvokerId2 - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain two objects. - One object must match with supportedFeatures1 - Response to ACL Service must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain two objects. - One object must match with supportedfeatures2 |

### Test Case 6: Retrieve ACL with aef-id not valid

| Test ID | capif_api_acl -6 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF if aef-id is not valid |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${AEF_ID_NOT_VALID} - Use serviceApiId and AEF_ID_NOT_VALID - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {service_api_id}, aef_id: {aef_id}, invoker: {api_invoker_id} and supportedFeatures: {supported_features}". - cause with message "Wrong id". |

### Test Case 7: Retrieve ACL with service-id not valid

| Test ID | capif_api_acl -7 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF if service-api-id is not valid |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${NOT_VALID_SERVICE_API_ID}?aef-id=${aefId} - Use NOT_VALID_SERVICE_API_ID and aef_id - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {service_api_id}, aef_id: {aef_id}, invoker: {api_invoker_id} and supportedFeatures: {supported_features}". - cause with message "Wrong id". |

### Test Case 8: Retrieve ACL with service-id and aef-id not valid

| Test ID | capif_api_acl -8 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF if service-api-id and aef-id are not valid |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${NOT_VALID_SERVICE_API_ID}?aef-id=$AEF_ID_NOT_VALID} - Use NOT_VALID_SERVICE_API_ID and AEF_ID_NOT_VALID - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {service_api_id}, aef_id: {aef_id}, invoker: {api_invoker_id} and supportedFeatures: {supported_features}". - cause with message "Wrong id". |

### Test Case 9: Retrieve ACL without Security Context created previously by Invoker

| Test ID | capif_api_acl -9 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL if no invoker had requested Security Context to CAPIF |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context not created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aefId} - Use serviceApiId and aefId - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {service_api_id}, aef_id: {aef_id}, invoker: {api_invoker_id} and supportedFeatures: {supported_features}". - cause with message "Wrong id". |

### Test Case 10: Retrieve ACL filtered by api-invoker-id not present

| Test ID | capif_api_acl -10 |
| --- | --- |
| Description | This test case will check that an API Provider get not found response if filter by not valid api-invoker-id doesn't match any registered ACL. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId }?aef-id=${aefId}&api-invoker-id={NOT_VALID_API_INVOKER_ID} - Use serviceApiId, aefId and NOT_VALID_API_INVOKER_ID - Use AEF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {service_api_id}, aef_id: {aef_id}, invoker: {api_invoker_id} and supportedFeatures: {supported_features}". - cause with message "Wrong id". |

### Test Case 11: Retrieve ACL with APF Certificate

| Test ID | capif_api_acl -11 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF using APF Certificate |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use APF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "Certificate not authorized". |

### Test Case 12: Retrieve ACL with AMF Certificate

| Test ID | capif_api_acl -12 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF using AMF Certificate |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AMF Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "Certificate not authorized". |

### Test Case 13: Retrieve ACL with Invoker Certificate

| Test ID | capif_api_acl -13 |
| --- | --- |
| Description | This test case will check that an API Provider can't retrieve ACL from CAPIF using Invoker Certificate |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Invoker Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Invoker Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use Invoker Certificate |
| Expected Result | - Response to ACL Service must accomplish: - 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - status 401 - title with message "Unauthorized" - detail with message "Role not authorized for this API route". - cause with message "Certificate not authorized". |

### Test Case 14: No ACL for invoker after being removed

| Test ID | capif_api_acl -14 |
| --- | --- |
| Description | This test case will check that ACLs are removed after invoker is removed. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Security context created for Service API published |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API with name service_1 - Create Security Context - Provider Get ACL information - Remove Invoker from CAPIF - Provider Get ACL information |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Create Security Context for this Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id}&api-invoker-id={api-invoker-id} - Use serviceApiId, aefId and api-invoker-id - Use AEF Certificate - Remove Invoker from CAPIF - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id}&api-invoker-id={api-invoker-id} - Use serviceApiId, aefId and api-invoker-id - Use AEF Certificate |
| Expected Result | - Response to first ACL Service request must accomplish: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match with apiInvokerId registered previously - Response to second ACL Service request must accomplish: - 404 Not Found Response. - body returned must accomplish Problem Details data structure. - apiInvokerPolicies must: - status 404 - title with message "Not Found" - detail with message "No ACLs found for the requested service: {NOT_VALID_SERVICE_API_ID}, aef_id: {AEF_ID_NOT_VALID}, invoker: None and supportedFeatures: None". - cause with message "Wrong id". |

# D.8	Test Plan for CAPIF API Logging Service

This section list test cases for the CAPIF_Logging_API_Invocation_API.

### Test Case 1: Creates a new individual CAPIF Log Entry

| Test ID | capif_api_logging -1 |
| --- | --- |
| Description | This test case will check that a CAPIF AEF can create log entry to Logging Service |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog - Use AEF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 201 Created - Response Body must follow InvocationLog data structure with: - aefId - apiInvokerId - logs - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invocation-logs/v1/{aefId}/logs/{logId} |

### Test Case 2: Creates a new individual CAPIF Log Entry with invalid aefId

| Test ID | capif_api_logging -2 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (AEF) cannot create Log Entry without valid aefId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{not-valid-aefId}/logs - Body InvocationLog - Use AEF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message "Not Found" - detail with message "Exposer not exist". - cause with message "Exposer id not found". |

### Test Case 3: Creates a new individual CAPIF Log Entry with invalid serviceAPI

| Test ID | capif_api_logging-3 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (AEF) cannot create Log Entry without valid aefId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog with serviceAPI apiName apiId not valid] - Use AEF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message "Not Found" - detail with message "Service not exist". - cause with message "Service id not found". |

### Test Case 4: Creates a new individual CAPIF Log Entry with invalid apiInvokerId

| Test ID | capif_api_logging-4 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (AEF) cannot create Log Entry without valid apiInvokerId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body serviceApiDescription with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog with apiInvokerId not valid - Use AEF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message "Not Found" - detail with message "Invoker not exist". - cause with message "Invoker id not found". |

### Test Case 5: Creates a new individual CAPIF Log Entry with invalid aefId in body

| Test ID | capif_api_logging-5 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (AEF) cannot create Log Entry without valid aefId in body |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body serviceApiDescription with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog with aefId not valid - Use AEF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 401 Unauthorized - Error Response Body must accomplish with ProblemDetails data structure with: - status 401 - title with message "Unauthorized" - detail with message "AEF id not matching in request and body". - cause with message “Not identical AEF id". |

# D.9	Test Plan for CAPIF API Events Service

This section list test cases for the CAPIF_Events_API.

### Test Case 1: Creates a New Individual CAPIF Event Subscription

| Test ID | capif_api_events-1 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (Invoker or Provider) can Subscribe to Events |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker or Provider. |
| Execution Steps | - Onboard Invoker at CCF. - Subscribe to Events - Retrieve (subscribeId) and (subscriptionId) from Location Header |
| Information of Test | - Onboard Invoker at CCF. - Event Subscription: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - Use Invoker Certificate |
| Expected Result | - Onboard Invoker. - Response to Event Subscription must accomplish: - Status code 201 Created - The URI of the created resource shall be returned in the “Location” HTTP header, following this structure {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response body must follow EvenstSubscription data structure: - Event subscriptions are stored in CAPIF Database |

### Test Case 2: Creates a New Individual CAPIF Event Subscription with invalid subscriberId

| Test ID | capif_api_events-2 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (Invoker or Publisher) cannot Subscribe to Events without valid SubcriberId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker or Provider. |
| Execution Steps | - Onboard Invoker at CCF. - Subscribe to Events with an invalid subscriberId - Retrieve (subscribeId) and (subscriptionId) from Location Header |
| Information of Test | - Onboard Invoker at CCF. - Event Subscription: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{INVALID_SUBSCRIBER_ID}/subscriptions - Use Invoker Certificate |
| Expected Result | - Onboard Invoker. - Response to Event Subscription must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Invoker or APF or AEF or AMF Not found” - Detail with message “Subscriber Not Found” - Event subscription are not stored in CAPIF Database |

### Test Case 3: Delete an Individual CAPIF Event Subscription

| Test ID | capif_api_events-3 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (Invoker or Publisher) can Delete an Event Subscription |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker or Provider. |
| Execution Steps | - Onboard Invoker at CCF. - Subscribe to Events with an invalid subscriberId - Retrieve (subscribeId) and (subscriptionId) from Location Header - Remove Event Subscription |
| Information of Test | - Onboard Invoker at CCF. - Event Subscription: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - Use Invoker Certificate - Remove Event Subscription: - Send DELETE to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions/{subscriptionId} - Use Invoker Certificate |
| Expected Result | - Onboard Invoker. - Response to Event Subscription must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Invoker or APF or AEF or AMF Not found” - Detail with message “Subscriber Not Found” - Event subscriptions are stored in CAPIF Database - Remove Event Subscription: - 204 No Content - Event Subscription is not present in CAPIF Database |

### Test Case 4: Delete an Individual CAPIF Event Subscription with an Invalid subscriberId

| Test ID | capif_api_events-4 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (Invoker or Publisher) cannot Delete to Events without valid SubcriberId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker or Provider. CAPIF subscriber is subscribed to Events. |
| Execution Steps | - Onboard Invoker at CCF. - Subscribe to Events with an invalid subscriberId - Retrieve (subscribeId) and (subscriptionId) from Location Header - Remove Event Subscription with not valid Subscriber |
| Information of Test | - Onboard Invoker at CCF. - Event Subscription: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - Use Invoker Certificate - Remove Event Subscription with Invalid subscriberId: - Send DELETE to https://{CAPIF_HOSTNAME}/capif-events/v1/{INVALID_SUBSCRIBER_ID}/subscriptions/{subscriptionId} - Use Invoker Certificate |
| Expected Result | - Onboard Invoker. - Response to Event Subscription must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Invoker or APF or AEF or AMF Not found” - Detail with message “Subscriber Not Found” - Event subscriptions are stored in CAPIF Database - Remove Event Subscription with invalid subscriberId: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Invoker or APF or AEF or AMF Not found” - Detail with message “Subscriber Not Found” |

### Test Case 5: Delete an Individual CAPIF Event Subscription with an Invalid subscriptionId

| Test ID | capif_api_events-5 |
| --- | --- |
| Description | This test case will check that a CAPIF subscriber (Invoker or Publisher) cannot Delete an Event Subscription without valid SubscriptionId |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker or Provider. |
| Execution Steps | - Onboard Invoker at CCF. - Subscribe to Events with an invalid subscriberId - Retrieve (subscribeId) and (subscriptionId) from Location Header - Remove Event Subscription with not valid subscriptionId |
| Information of Test | - Onboard Invoker at CCF. - Event Subscription: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - Use Invoker Certificate - Remove Event Subscription with not valid subscriptionId: - Send DELETE to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions/{SUBSCRIPTION_ID_NOT_VALID} - Use Invoker Certificate |
| Expected Result | - Onboard Invoker. - Response to Event Subscription must accomplish: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Invoker or APF or AEF or AMF Not found” - Detail with message “Subscriber Not Found” - Event subscriptions are stored in CAPIF Database - Remove Event Subscription: - Status code 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - Status 404 - Title with message “Not Found” - Title with message “Service API not existing” - Detail with message “Event API subscription Id not found” |

### Test Case 6: Invoker receives Service API invocation Events

| Test ID | capif_api_events-6 |
| --- | --- |
| Description | This test case will check that a CAPIF Invoker subscribed to SERVICE_API_INVOCATION_SUCCESS and SERVICE_API_INVOCATION_FAILURE, receive the notification when AEF Send TO logging service result of invocations to their APIs. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider and publish one API at CCF - Onboard Invoker at CCF. - Discover Published APIs and extract apiIds and apiNames. - Subscribe SERVICE_API_INVOCATION_SUCCESS and SERVICE_API_INVOCATION_FAILURE event filtering by aefId. - Retrieve (subscribeId) and (subscriptionId) from Location Header - Emulate Success and Failure on API invocation of provider by Invoker, using Invocation Logs API. |
| Information of Test | - Onboard Provider - Publish Service API at CCF: - Send POST to https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - Body serviceAPIDescription with apiName service_1 - Get apiId - Use APF Certificate - Onboard Invoker at CCF. - Discover published APIs: - Get Api Ids And Api Names from response. - Event Subscription to SERVICE_API_INVOCATION_SUCCESS and SERVICE_API_INVOCATION_FAILURE of provider previously registered: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - Body of Event Subscription request with: - Events: ['SERVICE_API_INVOCATION_SUCCESS','SERVICE_API_INVOCATION_FAILURE'] - eventFilter: only receive events from provider's aefId. - Use Invoker Certificate - Create a Log Entry emulating provider receive Success and Failure api Invocation from Invoker - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - body log entry request body with: - aefId from provider published. - apiInvokerId from invoker onboarded. - apiId of published API - apiName of published API - 200 and 400 results in two logs. - Use AEF Certificate |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Response to creation of log entry on CCF must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/api-invocation-logs/{apiVersion}/{aefId}/subscriptions/{logId} - Mock Server received messages must accomplish: - Two Events have been received. - Validate received events follow EventNotification data structure, with invocationLog in eventDetail parameter. - One should be SERVICE_API_INVOCATION_SUCCESS related with 200 result at Log. - The other one must be SERVICE_API_INVOCATION_FAILURE related with 400 result at Log. |

### Test Case 7: Invoker subscribe to Service API Available and Unavailable events

| Test ID | capif_api_events-7 |
| --- | --- |
| Description | This test case will check that a CAPIF Invoker subscribed to SERVICE_API_AVAILABLE and SERVICE_API_UNAVAILABLE, receive the notification when AEF publish and remove it. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider and publish one API at CCF - Onboard Invoker at CCF. - Discover Published APIs and extract apiIds and apiNames. - Subscribe SERVICE_API_INVOCATION_SUCCESS and SERVICE_API_INVOCATION_FAILURE event filtering by aefId. - Retrieve (subscribeId) and (subscriptionId) from Location Header - Provider publish new API - Provider remove published API |
| Information of Test | - Perform provider registration - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Store serviceApiId - Use APF Certificate - Perform invoker onboarding - Discover published APIs: - Get Api Ids And Api Names from response. - Event Subscription to SERVICE_API_AVAILABLE and SERVICE_API_UNAVAILABLE of provider previously registered: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: ['SERVICE_API_AVAILABLE','SERVICE_API_UNAVAILABLE'] - eventFilter: only receive events from provider's aefId. - Use Invoker Certificate - Publish new Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_2 - Store serviceApiId - Use APF Certificate - Remove published Service API at CCF: - Send DELETE to resource URL https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{SERVICE_API_ID} - Use APF Certificate |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Mock Server received messages must accomplish: - Two Events have been received. - Validate received events follow EventNotification data structure, with invocationLog in eventDetail parameter. - One should be SERVICE_API_AVAILABLE apiId of service_2 published API. - The other one must be SERVICE_API_UNAVAILABLE apiId of service_1 published API. |

### Test Case 8: Invoker subscribe to Service API Update

| Test ID | capif_api_events-8 |
| --- | --- |
| Description | This test case will check that a CAPIF Invoker subscribed to SERVICE_API_UPDATE, receive the notification when AEF Update some information on API Published. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider and publish one API at CCF - Onboard Invoker at CCF. - Discover Published APIs and extract apiIds and apiNames. - Subscribe to SERVICE_API_UPDATE event filtering by aefId. - Retrieve (subscribeId) and (subscriptionId) from Location Header - Provider update information of Service API Published. |
| Information of Test | - Perform provider registration - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Store serviceApiId - Use APF Certificate - Perform invoker onboarding - Discover published APIs: - Get Api Ids And Api Names from response. - Event Subscription to SERVICE_API_AVAILABLE and SERVICE_API_UNAVAILABLE of provider previously registered: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: [‘SERVICE_API_UPDATE’] - eventFilter: only receive events from provider's aefId. - Use Invoker Certificate - Update published API at CCF: - Send PUT to resource URL https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis/{serivceApiId} - body [service api description] with overrided apiName to service_1_modified** - Use APF Certificate |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Response to Update Published Service API: - 200 OK - Response Body must follow ServiceAPIDescription data structure with: - apiName service_1_modified** - Mock Server received messages must accomplish: - One Events have been received. - Validate received events follow EventNotification data structure, with invocationLog in eventDetail parameter. - Event should be SERVICE_API_UPDATE with eventDetail with modified apiName. |

### Test Case 9: Provider subscribe to API Invoker Events

| Test ID | capif_api_events-9 |
| --- | --- |
| Description | This test case will check that a CAPIF Provider subscribed to API Invoker events (API_INVOKER_ONBOARDED, API_INVOKER_UPDATED and API_INVOKER_OFFBOARDED), receive the notifications when Invoker is onboarded, updated and removed respectively. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider and publish one API at CCF - Subscribe Provider to API_INVOKER_ONBOARDED, API_INVOKER_UPDATED and API_INVOKER_OFFBOARDED events. - Onboard Invoker at CCF. - Update Onboarding Information at CCF with a minor change on "notificationDestination" - Offboard Invoker |
| Information of Test | - Perform provider registration - Event Subscription to API_INVOKER_ONBOARDED, API_INVOKER_UPDATED and API_INVOKER_OFFBOARDED events: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: ['API_INVOKER_ONBOARDED', 'API_INVOKER_UPDATED', 'API_INVOKER_OFFBOARDED'] - Use Provider AMF Certificate - Perform invoker onboarding - Update information of previously onboarded Invoker: - Send PUT to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} - Reference Request Body is: [put invoker onboarding body] - "notificationDestination": "http://host.docker.internal:8086/netapp_new_callback", - Offboard: - Send DELETE to https://{CAPIF_HOSTNAME}/api-invoker-management/v1/onboardedInvokers/{onboardingId} |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Response to Onboard request must accomplish: - 201 Created - Response Body must follow APIInvokerEnrolmentDetails data structure with: - apiInvokerId - onboardingInformation->apiInvokerCertificate must contain the public key signed. - Response Header Location must be received with URI to new resource created, following this structure: {apiRoot}/api-invoker-management/{apiVersion}/onboardedInvokers/{onboardingId} - Response to Update Request (PUT) with minor change must contain: - 200 OK response. - notificationDestination on response must contain the new value - Response to Offboard Request (DELETE) must contain: - 204 No Content - Mock Server received messages must accomplish: - Three Events have been received. - Validate received events follow EventNotification data structure, with apiInvokerIds in eventDetail parameter. - One Event should be API_INVOKER_ONBOARDED with eventDetail with modified apiInvokerId. - One Event should be API_INVOKER_UPDATED with eventDetail with modified apiInvokerId. - One Event should be API_INVOKER_OFFBOARDED with eventDetail with modified apiInvokerId. |

### Test Case 10: Provider subscribe to ACL Update event

| Test ID | capif_api_events-10 |
| --- | --- |
| Description | This test case will check that a CAPIF Provider subscribed to ACCESS_CONTROL_POLICY_UPDATE receive a notification when ACL Changes. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF API Invoker had a Security Context for the Service API published by Provider Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider. - Publish a provider API with name service_1. - Onboard Invoker at CCF. - Subscribe Provider to ACCESS_CONTROL_POLICY_UPDATE event. - Discover APIs filtered by aef_id - Create Security Context for Invoker. - Provider Retrieve ACL |
| Information of Test | - Perform provider registration - Perform invoker onboarding - Event Subscription to ACCESS_CONTROL_POLICY_UPDATE event: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: ['ACCESS_CONTROL_POLICY_UPDATE'] - eventFilters: apiInvokerIds array with apiInvokerId of invoker - Use Provider AMF Certificate - Discover published APIs - Create Security Context for Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Provider Certificate |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Create security context: - body returned must accomplish ServiceSecurity data structure. - Location Header must contain the new resource URL {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} - ACL Response: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId registered previously. - Mock Server received messages must accomplish: - One Event has been received. - Validate received event follow EventNotification data structure, with accCtrlPolListExt in eventDetail parameter. - One Event should be ACCESS_CONTROL_POLICY_UPDATE with eventDetail with accCtrlPolListExt including the apiId and apiInvokerPolicies. |

### Test Case 11: Provider receives an ACL unavailable event when invoker removes Security Context

| Test ID | capif_api_events-11 |
| --- | --- |
| Description | This test case will check that a CAPIF Invoker subscribed to ACCESS_CONTROL_POLICY_UNAVAILABLE will receive the notification when AEF remove Security Context created previously. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF API Invoker had a Security Context for the Service API published by Provider Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider. - Publish a provider API with name service_1. - Onboard Invoker at CCF. - Subscribe Invoker to ACCESS_CONTROL_POLICY_UNAVAILABLE event. - Discover APIs filtered by aef_id - Create Security Context for Invoker. - Provider Retrieve ACL - Remove Security Context for Invoker. |
| Information of Test | - Perform provider registration - Perform invoker onboarding - Event Subscription to ACCESS_CONTROL_POLICY_UNAVAILABLE event: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: ['ACCESS_CONTROL_POLICY_UNAVAILABLE'] - eventFilters: apiInvokerIds array with apiInvokerId of invoker - Use Invoker Certificate - Discover published APIs - Create Security Context for Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Provider Certificate - Delete Security Context of Invoker by Provider: - Send DELETE https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use AEF Certificate |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Create security context: - body returned must accomplish ServiceSecurity data structure. - Location Header must contain the new resource URL {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} - ACL Response: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId registered previously. - Delete security context: - 204 No Content response - Mock Server received messages must accomplish: - One Event has been received. - Validate received event follow EventNotification data structure, with accCtrlPolListExt in eventDetail parameter. - One Event should be ACCESS_CONTROL_POLICY_UNAVAILABLE without eventDetail. |

### Test Case 12: invoker receives an invoker Authorization Revoked and ACL unavailable event when Provider revoke Invoker Authorization

| Test ID | capif_api_events-12 |
| --- | --- |
| Description | This test case will check that a CAPIF Invoker subscribed to API_INVOKER_AUTHORIZATION_REVOKED and ACCESS_CONTROL_POLICY_UNAVAILABLE receive both notification when AEF revoke invoker's authorization. |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker and Provider. Provider had a Service API published on CAPIF API Invoker had a Security Context for the Service API published by Provider Mock Server is up and running to receive requests. Mock Server is clean |
| Execution Steps | - Onboard Provider. - Publish a provider API with name service_1. - Onboard Invoker at CCF. - Invoker to ACCESS_CONTROL_POLICY_UNAVAILABLE and API_INVOKER_AUTHORIZATION_REVOKED events. - Discover APIs filtered by aef_id - Create Security Context for Invoker. - Revoke Authorization by Provider |
| Information of Test | - Perform provider registration - Perform invoker onboarding - Event Subscription to ACCESS_CONTROL_POLICY_UNAVAILABLE and API_INVOKER_AUTHORIZATION_REVOKED event: - Send POST to https://{CAPIF_HOSTNAME}/capif-events/v1/{subscriberId}/subscriptions - body event subscription request body with: - events: ['ACCESS_CONTROL_POLICY_UNAVAILABLE','API_INVOKER_AUTHORIZATION_REVOKED'] - eventFilters: apiInvokerIds array with apiInvokerId of invoker - Use Invoker Certificate - Discover published APIs - Create Security Context for Invoker - Send PUT https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId} - Use Invoker Certificate - Provider Retrieve ACL - Send GET https://{CAPIF_HOSTNAME}/access-control-policy/v1/accessControlPolicyList/${serviceApiId}?aef-id=${aef_id} - Use serviceApiId and aefId - Use AEF Provider Certificate - Revoke Authorization by Provider: - Send POST https://{CAPIF_HOSTNAME}/trustedInvokers/{apiInvokerId}/delete - body security notification body - Using AEF Certificate. |
| Expected Result | - Response to Event Subscription must accomplish: - 201 Created - The URI of the created resource shall be returned in the "Location" HTTP header, following this structure: {apiRoot}/capif-events/{apiVersion}/{subscriberId}/subscriptions/{subscriptionId} - Response Body must follow EventSubscription data structure. - Create security context: - body returned must accomplish ServiceSecurity data structure. - Location Header must contain the new resource URL {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId} - ACL Response: - 200 OK Response. - body returned must accomplish AccessControlPolicyList data structure. - apiInvokerPolicies must: - contain only one object. - apiInvokerId must match apiInvokerId registered previously. - Revoke Authorization: - 204 No Content response - Mock Server received messages must accomplish: - Two Event has been received. - Validate received event follow EventNotification data structure, with accCtrlPolListExt in eventDetail parameter. - One Event should be ACCESS_CONTROL_POLICY_UNAVAILABLE without eventDetail. - One Event should be API_INVOKER_AUTHORIZATION_REVOKED without eventDetail. |

# D.10	Test Plan for CAPIF API Auditing Service

This section list test cases for the CAPIF_Auditing_API.

### Test Case 1: Get CAPIF Log Entry

| Test ID | capif_api_auditing-1 |
| --- | --- |
| Description | This test case will check that a CAPIF AMF can get log entry to Logging Service |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Log Entry exist in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry - Get Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog - Use AEF Certificate - Get Log: - Send GET to https://{CAPIF_HOSTNAME}/logs/v1/apiInvocationLogs?aef-id={aefId}&api-invoker-id={api-invoker-id} - Use AMF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 200 OK - Response Body must follow InvocationLog data structure with: - aefId - apiInvokerId - logs |

### Test Case 2: Get CAPIF Log Entry With no Log entry in CAPIF

| Test ID | capif_api_auditing-2 |
| --- | --- |
| Description | This test case will check that a CAPIF AEF cannot get log entry to Logging Service |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Get Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Get Log: - Send GET to https://{CAPIF_HOSTNAME}/logs/v1/apiInvocationLogs?aef-id={aefId}&api-invoker-id={api-invoker-id} - Use AMF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message "Not Found Log Entry in CAPIF". - cause with message "Not Exist Logs with the filters applied". |

### Test Case 3: Get CAPIF Log Entry without aef-id and api-invoker-id

| Test ID | capif_api_auditing-3 |
| --- | --- |
| Description | This test case will check that a CAPIF AMF cannot get log entry to Logging Service |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Log Entry exist in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry - Get Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog - Use AEF Certificate - Get Log: - Send GET to https://{CAPIF_HOSTNAME}/logs/v1/apiInvocationLogs - Use AMF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 400 Bad Request - Error Response Body must accomplish with ProblemDetails data structure with: - status 400 - title with message "Bad Request" - detail with message "aef_id and api_invoker_id parameters are mandatory". - cause with message "Mandatory parameters missing". |

### Test Case 4: Get CAPIF Log Entry with filter api-version

| Test ID | capif_api_auditing-4 |
| --- | --- |
| Description | This test case will check that a CAPIF AMF can get log entry to Logging Service with filter api-version |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Log Entry exist in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry - Get Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog - Use AEF Certificate - Get Log: - Send GET to https://{CAPIF_HOSTNAME}/logs/v1/apiInvocationLogs?aef-id={aefId}&api-invoker-id={api-invoker-id}&api-version={v1} - Use AMF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 200 OK - Response Body must follow InvocationLog data structure with: - aefId - apiInvokerId - logs |

### Test Case 5: Get CAPIF Log Entry with filter api-version but not exist in log entry

| Test ID | capif_api_auditing-5 |
| --- | --- |
| Description | This test case will check that a CAPIF AMF cannot get log entry to Logging Service with filter api-version |
| Pre-conditions | The Administrator must have previously registered the User The user must have the access token, the ca root certificate and the URLs to onboard an Invoker. Service published in CAPIF Log Entry exist in CAPIF |
| Execution Steps | - Onboard Invoker and Provider at CCF. - Publish Service API - Create Log Entry - Get Log Entry |
| Information of Test | - Onboard Invoker and provider at CCF. - Publish Service API at CCF: - Send POST to ccf_publish_url https://{CAPIF_HOSTNAME}/published-apis/v1/{apfId}/service-apis - body [service api description] with apiName service_1 - Use APF Certificate - Log Entry: - Send POST to https://{CAPIF_HOSTNAME}/api-invocation-logs/v1/{aefId}/logs - Body InvocationLog - Use AEF Certificate - Get Log: - Send GET to https://{CAPIF_HOSTNAME}/logs/v1/apiInvocationLogs?aef-id={aefId}&api-invoker-id={api-invoker-id}&api-version={v58} - Use AMF Certificate |
| Expected Result | - Response to Logging Service must accomplish: - 404 Not Found - Error Response Body must accomplish with ProblemDetails data structure with: - status 404 - title with message "Parameters do not match any log entry". - cause with message "No logs found ". |

Annex E:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | SA6#57 |  |  |  |  | TR Initial Version as per S6-233407 | 0.0.0 |
| 2023-10 | SA6#57 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-233386 | 0.1.0 |
| 2023-11 | SA6#58 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-233904, S6-233908, S6-233909, S6-233910. | 0.2.0 |
| 2024-03 | SA6#59 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-240528, S6-240529, S6-240531, S6-240533, S6-240582. | 0.3.0 |
| 2024-04 | SA6#60 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-241358, S6-241351. | 0.4.0 |
| 2024-05 | SA6#61 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-242323, S6-242703, S6-242704, S6-242705. | 0.5.0 |
| 2024-08 | SA6#62 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-243646, S6-243061, S6-243062, S6-243724, S6-243518, S6-243628, S6-243629, S6-243067, S6-243519, S6-243069, S6-243631, S6-243265, S6-243510, S6-243633. | 0.6.0 |
| 2024-10 | SA6#63 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-244167, S6-244198, S6-244557, S6-244558, S6-244559, S6-244560, S6-244561. | 0.7.0 |
| 2024-11 | SA6#64 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-245431, S6-245434, S6-245645, S6-245665. | 0.8.0 |
| 2024-11 | SA6#64 |  |  |  |  | Implementation of the following pCR approved by SA6 on email approval: S6-245722, restructuring prior to initial publication. | 0.9.0 |
| 2024-12 | SA#106 | SP-241701 |  |  |  | Submitted to SA#106 for information and approval | 1.0.0 |