3GPP TSG-SA WG6 Meeting #63	S6-244064

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0371 | rev |  | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the FD file removal procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | October 5, 2024 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are errors in the FD file removal procedures. - The partner MCDara MC system is defined as a unique identifier representing an MCData system in clause 8.14 in 23.280. It cannot be used to identify a partner MCData content server. - Depends on whcihc segment the message is sent, either the MCData ID or the address of the partner MCData content server shall be present. - The URL generated by the primary MCData content server is not the same as generated and used in the partner MCData content server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify and make corrections to clauses in 7.5.2.1.14 and 7.5.2.1.15. - Void 7.5.2.1.18 which is not used and not needed. - Convert ENs in 7.5.2.8.2 and 7.5.2.8.3 to NOTEs. - Add a NOTE in 7.5.2.8.3 to clarify the relationship of URL generated by the primary MCData content server and by the partner MCData content server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will misguide CT stage 3 development |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1.14, 7.5.2.1.15, 7.5.2.1.18, 7.5.2.8.2, 7.5.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.5.2.1.14	MCData remove file request

Table 7.5.2.1.14-1 describes the information flow for the MCData remove file request sent from the media storage client to the media storage function of the MCData content server, and from the MCData content server to another MCData content server in a partner MCData system.

Table 7.5.2.1.14-1: MCData remove file request

| Information element | Status | Description |  |
| --- | --- | --- | --- |
| MCData ID (see NOTE 1) | O | The identity of the MCData user removing file |  |
| Partner MCData content server address (see NOTE 2) | O | The identity of the partner MCData content server where the file has been downloaded. This address is obtained when the partner MCData content server downloads the shared file. |  |
| Content reference | M | URL of the content to be removed |  |
| NOTE 1: The identity of the MCData user removing the file shall be present when sent from MCData client to MCData content server NOTE 2: The address of the partner MCData content server shall be present when sent from MCData content server to partner MCData content server. |  |  |

##### 7.5.2.1.15	MCData remove file response

Table 7.5.2.1.15-1 describes the information flow for the MCData remove file response sent from the media storage function of the MCData content server to the media storage client, and from the MCData content server to another MCData content server in a partner MCData system.

Table 7.5.2.1.15-1: MCData remove file response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID (see NOTE 1) | O | The identity of the MCData user removing file |
| Primary MCData content server address (see NOTE 2) | O | The address of the primary MCData content server where file removal request is sent |
| Result | M | Indicates the success or failure of the file removal |
| NOTE 1: The identity of the MCData user removing the file shall be present when sent from MCData content server to MCData client NOTE 2: The address of the primary MCData content server shall be present when sent from partner MCData content server to primary MCData content server.. |

# * * * * Next change * * * *

##### 7.5.2.1.18	Void

# * * * * Next change * * * *

#### 7.5.2.8	File removal using HTTP by authorized user

##### 7.5.2.8.1	General

The media storage client uses HTTP to remove a file that was previously uploaded to the MCData content server.

##### 7.5.2.8.2	Procedure for single MCData system

The procedure in figure 7.5.2.8.2-1 describes the case where a MCData user is removing the file that was previously uploaded to the MCData content server.

Pre-conditions:

1.	The MCData user on the media storage client is registered for receiving MCData service.

2.	The file has been successfully uploaded by the MCData user using the procedures defined in subclause 7.5.2.2.

3.	The MCData content server has the ability to verify if the requesting MCData user is authorised to remove.

Figure 7.5.2.8.2-1: File removal using HTTP by authorised user

1.	The user on the media storage client decides to remove a file that was previously uploaded.

2.	The URL of the file to be removed is included in the request sent to the media storage function on the MCData content server.

3.	The MCData content server remove the file indicated by the URL.

4.	The MCData content server informs the media storage client if the file is successfully removed.

NOTE:	It is implementation specific if a notification will be sent to the file URL recipient that hasn’t downloaded the file.

##### 7.5.2.8.3	Procedure for interconnection between MCData systems

The procedure in figure 7.5.2.8.3-1 describes the case where an MCData user removes the file that was previously uploaded to the primary MCData system MCData content server, and where the file has been made available in the partner MCData system MCData content server.

Pre-conditions:

1.	The MCData user on the media storage client is registered for receiving MCData service.

2.	The file has previously been uploaded to the MCData content server in the primary MCData system of MCData client 1.

3.	The file has been successfully transferred to the MCData content server in the partner MCData system.

Figure 7.5.2.8.3-1: File removal using HTTP by authorized user

1.	The user on the media storage client decides to remove a file that was previously uploaded.

2.	The URL of the file to be removed is included in the request sent to the media storage function on the primary MCData content server.

3.	The primary MCData content server removes the file indicated by the URL.

NOTE:	Step 3 may occur at any time following step 2 and before step 6.

4.	As the primary MCData content server has recorded that the file has previously been sent to the partner MCData system, the primary MCData content server sends the MCData remove file request to the partner MCData content server, containing the URL of the file which was stored on the primary MCData content server.

5.	The partner MCData content server removes the file indicated by the URL.

NOTE 1:	The partner MCData content server shall maintain an association of the URL from the primary MCData content server and the local URL where the file is stored locally.

6.	The partner MCData content server informs the primary MCData content server that the file has been successfully removed.

7.	The primary MCData content server informs the media storage client if the file is successfully removed.

NOTE 2:	It is implementation specific if a notification will be sent to the file URL recipient that hasn’t downloaded the file.

# * * * * Next change * * * *