3GPP TSG-SA Meeting #106	SP-241845

Madrid, Spain, 10-13 December, 2024	(revision of SP-240xxx)

Source: 	China Telecom

Title: 	AI/ML related terminology across on 3GPP

Document for: 	Approval

Agenda Item: 	7

Work Item / Release:	FS_AIML_CAL / Rel-19

- Background

TR 22.850 v0.1.0 contains multiple clauses describing the AI/ML related terminology and activities in all 3GPP work groups. This contribution aims to provide additional analysis on AI/ML related terminologies from Release 18 and 19 across 3GPP work groups.

2. Proposal

We propose to discuss and document these terminology analysis in TR 22.850.

3. Reference

[3]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)".

[4]	3GPP TR 23.700-84: “Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (Release 19)”.

[6]	3GPP TS 22.261, “Service requirements for the 5G system (Release 19)”.

[19]	3GPP TR 28.858; AI/ML management – phase 2

[21]	3GPP TR 22.876: "Study on AI/ML Model Transfer; Phase 2".

[34]	3GPP TS 23.482: “Functional architecture and information flows for AIML Enablement Service (Release 19)”.

* * * * First Change * * * *

- Analysis on AI/ML across 3GPP

6.2	AI/ML related terminology

6.2.2	Analysis on Horizontal Federated Learning

The term ‘Horizontal Federated Learning’ has been defined in SA2 and RAN1 defines ‘Federated Learning’, and SA5 also defines ‘Federated Learning’, as illustrated in Table 6.2.2-1. SA1 describes ‘Distributed/Federated learning’ in the service requirement for 5G system. For the term ‘Horizontal Federated Learning’ and ‘Vertical Federated Learning’, in TR 28.858 [19] SA5 uses the same definition as SA2 in TR 23.700-84 [4]. SA6 mentions the term ‘Federated Learning’, “Horizontal federated Learning’ and ‘Vertical Federated Learning’ in TS 23.482 [34], but definitions of these terms are not provided.

Based on the topology and location where the learning tasks take place, SA5 categorizes AI/ML architectures into generative AI, centralized, distributed, and federated learning in TR 28.858 [19]. SA6 uses the term ‘Distributed Learning’ in TS 23.482 [34], but definition of this term is not given. In general, federated learning emphasizes the preservation of local data, ensuring that sensitive information remains within the local area, while distributed learning focuses on the efficient distribution of training workload.

Editor’s note: 	It is FFS whether distributed learning and federated learning should be considered different learning architectures and given separate definitions.

Editor's note:	Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table 6.2.2-1: Definition of Horizontal Federated Learning as defined across 3GPP WGs

| TSG (TS/TR) | Horizontal Federated Learning |
| --- | --- |
| SA WG2 TR 23.700-84 [4] | Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs). |
| RAN WG1 TR 38.843 [3] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g. UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| 3GPP SA5 TR 28.858 [19] | Federated Learning: a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions including one acting as an FL server and multiple acting as FL clients iteratively without exchanging data samples. |
| 3GPP SA1 TS 22.261 [6] | Distributed/Federated Learning over 5G system: The cloud server trains a global model by aggregating local models partially-trained by each end devices. Within each training iteration, a UE performs the training based on the model downloaded from the AI server using the local training data. Then the UE reports the interim training results to the cloud server via 5G UL channels. The server aggregates the interim training results from the UEs and updates the global model. The updated global model is then distributed back to the UEs and the UEs can perform the training for the next iteration. |

* * * * Second Change (all new)* * * *

6.2.X Analysis on Transfer Learning

The term ‘Transfer Learning’ has been defined by SA5, as illustrated in Table X. SA6 mentions the term Transfer Learning’ in TS 23.482 [34], but definition of the term is not given. SA1 also uses the term ‘Transfer Learning’ in TS 22.261 [6] and TR 22.876 [21] without providing definitions of the term.

Editor’s note:	 Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

Table X: Definition of Transfer Learning as defined across 3GPP WGs

| TSG (TS/TR) | Transfer Learning |
| --- | --- |
| 3GPP SA5 TR 28.858 [19] | ML Knowledge-based Transfer Learning: a technique where the knowledge gained from training of one or more ML models is applied or adapted to improve or develop another ML model. |

* * * * End of Change * * *