| 3GPP TR 23.700-09 V1.0.0 (2024-12) |  |
| --- | --- |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Mission critical services support in the Isolated Operation for Public Safety (IOPS) mode of operations (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Key Issues	8

4.1	Key issue #1 evaluate the existing IOPS work	8

4.1.1	Description	8

4.2	Key issue #2: on including NTN in IOPS scenarios	8

4.2.1	General	8

4.2.2	Open issues	9

5.0	Architectural requirements	9

6.0	Solutions	9

6.1	Solution 1: Clause 1 (Scope) of TS 23.180	9

6.1.1	General	9

6.1.2	Solution	9

6.1.3	Other affected clauses	9

6.1.4	Solution evaluation	10

6.2	Solution 2: Clause 3.1 (Definitions) of TS 23.180	10

6.2.1	General	10

6.2.2	Solution	10

6.2.3	Other affected clauses	10

6.2.4	Solution evaluation	10

6.3	Solution 3: Clause 4 (Introduction) of TS 23.180	10

6.3.1	General	10

6.3.2	Solution	11

6.3.3	Other affected clauses	11

6.3.4	Solution evaluation	11

6.4	Solution 4: Clause 5 (Architecture requirement) of TS 23.180	11

6.4.1	General	11

6.4.2	Solution	11

6.4.3	Other affected clauses	13

6.4.4	Solution evaluation	14

6.5	Solution 5: Clause 6 (Involved business relationships)	14

6.5.1	General	14

6.5.2	Solution	14

6.5.3	Other affected clauses	15

6.5.4	Solution evaluation	15

6.6	Solution 6: Clause 7 (Functional model) of TS 23.180	15

6.6.1	General	15

6.6.2	Solution	15

6.6.3	Other affected clauses	20

6.6.4	Solution evaluation	21

6.7	Solution 7: Converting the functional model and deployment section of TS 23.180	21

6.7.1 	General	21

6.7.2	 Addressing the functional model and deployment section of 3GPP TS 23.180	21

6.8	Solution 8: Clause 10.2.2.3 (IOPS discovery request) of TS 23.180	23

6.8.1	General	23

6.8.2	Solution	23

6.8.3	Other affected clauses	24

6.8.4	Solution evaluation	24

6.9	Solution 9: Converting the broadcast/multicast transmission section of TS 23.180	24

6.9.1 	General	24

6.9.2	 Addressing the MBMS section of 3GPP TS 23.180	24

6.9.3	Other affected clauses	28

6.9.4	Solution evaluation	29

6.10	Solution 10: Clause 10.5.1.4 (IOPS emergency group call) of TS 23.180	29

6.10.1	General	29

6.10.2	Solution	29

6.11	Solution 11: Clause 10.5.1.3 (IOPS group call setup) of TS 23.180	31

6.11.1	General	31

6.11.2	Solution	31

6.12	Solution 12: Clause 10.5.3.1 (IOPS floor control - General) of TS 23.180	34

6.12.1	General	34

6.12.2	Solution	34

6.13	Solution 13: Clause 10.5.3.3 (IOPS floor control during silence) of TS 23.180	35

6.13.1	General	35

6.13.2	Solution	35

6.14	Solution 14: Clause 10.6.1.4 (IOPS group standalone SDS using signalling control plane) of TS 23.180	37

6.14.1	General	37

6.14.2	Solution	37

6.15	Solution 15: Clause 10.7 (MC IOPS notification) of TS 23.180	39

6.15.1	General	39

6.15.2	Solution	39

6.16	Solution 16: Clauses 10.2.3, 10.3.3 and 10.6.1.3.2	43

6.16.1	General	43

6.16.2	Solution	43

6.16.4	Other affected clauses	48

6.16.5	Solution evaluation	48

6.17	Solution 17: Clauses A.1, A.2 and A.6 (Involved business relationships)	49

6.17.1	General	49

6.17.2	Solution	49

6.17.3	Other affected clauses	51

6.17.4	Solution evaluation	51

7	Overall evaluation	51

7.1	General	51

7.2	Architecture evaluation	52

8	Conclusions	53

Annex A:	54

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Mission critical services can be provided even in the case of a backhaul failure, limited backhaul connectivity, or a nomadic 5G system deployment based on the availability of generic Isolated Operations for Public Safety (IOPS). IOPS provides local connectivity to the public safety users that are within the communication range of the radio base station(s) that supports the IOPS mode of operation, as described in 3GPP TS 23.401 Annex K [8] and based on requirements in 3GPP TS 22.346 [5], 3GPP TS 22.179 [2], 3GPP TS 22.280 [3], and in 3GPP TS 22.261 [4].

This study focuses on identifying application architecture solutions needed to support mission critical services during the IOPS mode of operation.

# 1	Scope

This technical report is a study of mission critical service in the Isolated Operation for Public Safety (IOPS) mode of operation. The present document identifies architecture enhancements needed to support mission critical services delivered over IOPS, based on the stage 1 requirements, including 3GPP TS 22.179 [2], 3GPP TS 22.280 [3], 3GPP TS 22.261 [4], and 3GPP TS 22.346 [5].

A previous technical report focused on EUTRAN in 3GPP TR23.778 [6] and associated normative work in 3GPP TS23.180 [7] have also been completed.

This document provides recommendations for normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.280: "Mission Critical Services Common Requirements".

[4]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”.

[5]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[6]	3GPP TR 23.778: “Study on Mission Critical services access aspects”.

[7]	3GPP TS 23.180: “Mission critical support in the Isolated Operation for Public Safety (IOPS) mode of operation, functional architecture and information flows”.

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Definitions

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Key Issues

## 4.1	Key issue #1 evaluate the existing IOPS work

### 4.1.1	Description

3GPP completed the initial IOPS work based on EUTRAN as part of release 17 in 3GPP TR 23.778 [6] and 3GPP TS 23.180 [7]. The lessons learned and work done in these documents should be studied and considered for the generic IOPS functional model and procedures that will be agnostic to the network access.

Further study is needed to:

-	Review and evaluate existing work in the current IOPS technical specification 3GPP TS 23.180 [7] and determine if the existing baseline in TS 23.180 [7] can be used in the generic IOPS solution.

-	Provide a summary of the solutions from 3GPP TS 23.180 [7] that are suitable for a generic IOPS model and can be used with the required changes and which cannot be re-used for generic IOPS and must be re-written.

## 4.2	Key issue #2: on including NTN in IOPS scenarios

### 4.2.1	General

3GPP TS 22.346 [5] and 3GPP TS 22.261 [4] have described the main isolated operation scenarios with respect to the backhaul connectivity:

- Isolated operation, i.e., no backhaul, as the event of having an interruption to the normal backhaul connectivity to the core.

- Limited backhaul connectivity with two options:

- Limited backhaul connectivity as the event of having only limited signalling plane of MC service UE reachable to the core.

- Limited backhaul connectivity as the event of having limited signalling plane and limited media traffic of MC service UE.

### 4.2.2	Open issues

There are several aspects that need to be studied to utilize NTN for the isolated deployment scenarios mentioned above. This includes but not limited to:

- Understand how to utilize NTN to provide reliable MC services during the IOPS operations mentioned above.

- Understand the role of NTN in building a generic IOPS functional model and procedures in order to maintain an acceptable level of operations.

- Study some specific challenges and possible solutions in the application layer when using satellite backhaul in IOPS operations, including:

  - The temporal variations of the achievable capacity and the QoS due to the transient nature of the connection with LEO satellite constellations.

# 5.0	Architectural requirements

No architectural requirements were identified

# 6.0	Solutions

## 6.1	Solution 1: Clause 1 (Scope) of TS 23.180

### 6.1.1	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.1.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

1	Scope

This document specifies the functional architecture, procedures and information flows needed to support mission critical services in the IOPS mode of operation. IOPS mode of operation allows an isolated 3GPP network to maintain a level of MC services in case the backhaul connection is lost. For example, for EPS the IOPS mode of operation is defined in 3GPP TS 23.401 [2].

The corresponding service requirements are defined in 3GPP TS 22.346 [9] and 3GPP TS 22.280 [11].

The present document is applicable to support mission critical services in the IOPS mode of operation for all supported access types including E-UTRAN, 5G or 5GA.

The functional architecture to support mission critical services in the IOPS mode of operation can be used for public safety applications and also for general commercial applications, e.g. utility companies and railways, for the case of a backhaul failure.

### 6.1.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.1.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.2	Solution 2: Clause 3.1 (Definitions) of TS 23.180

### 6.2.1	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7].

### 6.2.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: Provides the ability to maintain a level of communications for MC users in case the backhaul connection is lost.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

### 6.2.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.2.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.3	Solution 3: Clause 4 (Introduction) of TS 23.180

### 6.3.1	General

This clause is related to KI#1 addresses clause 4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make it applicable for all supported 3GPP access methods.

### 6.3.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

4	Introduction

For the case of a backhaul failure, mission critical (MC) services (e.g. MCPTT defined in 3GPP TS 23.379 [5] and MCData defined in 3GPP TS 23.282 [6]) can also be supported based on the availability of an isolated operation for public safety (IOPS) MC system. The IOPS MC system provides MC services support to the MC service UE for example via a single public land mobile network (PLMN) dedicated to the IOPS mode of operation, for example,  as defined in 3GPP TS 23.401 [2].

A common functional architecture, including application plane and signalling plane, to support MC services in the IOPS mode of operation is specified in this document.

### 6.3.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.3.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.4	Solution 4: Clause 5 (Architecture requirement) of TS 23.180

### 6.4.1	General

This clause is related to KI#1 addresses clause 5 of 3GPP TS 23.180 [7]. It describes which changes are needed to make it applicable for all supported 3GPP access methods.

### 6.4.2	Solution

### 5.1.2	Requirements

[AR-5.1.2-a] For the case of a backhaul failure between the radio access network and the macro 3GPP core network, the IOPS MC system shall provide support for MC services in the IOPS mode of operation until the failure is recovered.

[AR-5.1.2-b] MC service UEs within the coverage of the local IOPS 3GPP system  shall be able to provide MC services over the IOPS MC system.

[AR-5.1.2-c] The IOPS MC system shall provide mechanisms to verify the availability of MC service UEs within the IOPS MC system.

## 5.2	MC service requirements

### 5.2.1	Description

This clause specifies the MC service requirements in the IOPS mode of operation.

### 5.2.2	Requirements

[AR-5.2.2-a] The following MCPTT service features shall be supported in the IOPS mode of operation

-	MCPTT group call

-	MCPTT emergency group call

-	MCPTT private call

-	MCPTT emergency private call

[AR-5.2.2-b] The following MCData service features shall be supported in the IOPS mode of operation

-	MCData short data service

## 5.3	Network broadcast/multicast session requirements

### 5.3.1	Description

This clause specifies the network bearer requirements in the IOPS mode of operation.

### 5.3.2	Requirements

[AR-5.3.2-a] The IOPS MC system shall provide support for unicast  session establishment and modification to support MC services via unicast transmissions in the IOPS mode of operation.

[AR-5.3.2-b] The IOPS MC system shall provide support for multicast/broadcast session establishment and modification to support MC services via multicast/broadcast transmissions in the IOPS mode of operation.

[AR-5.3.2-c] The IOPS MC system shall support announcement of multicast/broadcast sessions  to the MC service UEs.

## 5.4	IP connectivity functionality

### 5.4.1	Description

The support of MC services in the IOPS mode of operation can be based on the support of the IP connectivity functionality. The IP connectivity functionality enables that MC services are provided by the MC service clients via the IOPS MC connectivity function.

An IOPS MC connectivity function supporting the IP connectivity functionality does not provide MC services. Instead, the IOPS MC connectivity function enables discovering MC service users and provides IP connectivity for the communication among the discovered MC service users, i.e. it distributes the related IP traffic to the MC service UEs over IP unicast and/or multicast transmissions. A MC service UE supporting the IP connectivity functionality in the IOPS mode of operation enables transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

This clause specifies requirements for the support of the IP connectivity functionality in the IOPS mode of operation.

### 5.4.2	General requirements

[AR-5.4.2-a] The IOPS MC connectivity function supporting the IP connectivity functionality shall support discovering MC service users within the coverage of the local IOPS 3GPP system.

[AR-5.4.2-b] The IOPS MC connectivity function supporting the IP connectivity functionality shall support announcement, subscription and notification mechanisms for discovered MC service users supporting the IP connectivity functionality.

[AR-5.4.2-c] The IOPS MC connectivity function supporting the IP connectivity functionality shall provide IP connectivity for communication among MC service users requesting the support of the IP connectivity functionality.

[AR-5.4.2-d] The MC service UEs supporting the IP connectivity functionality in the IOPS mode of operation shall enable transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

### 5.4.3	Bearer management requirements

[AR-5.4.3-a] The MC service UEs shall use specific APNs or DNNs for the support of the IP connectivity functionality, as follows:

-	An APN/DNN for the IOPS related application signalling related to the IP connectivity functionality support, e.g. for IOPS discovery, subscription and notification.

-	An MC service APN/DNN for the communication with another MC user or group via the IOPS MC connectivity function based on the IP connectivity functionality, e.g. for the signalling and media.

[AR-5.4.3-b] The APNs or DNNs for the IP connectivity functionality support shall be made available to the MC service UEs either via UE (pre)configuration or via initial UE configuration, as described in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

### 6.4.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

#### 6.4.3.1	Clause 2

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.289]	3GPP TS 23.289: "Mission Critical Services over 5G System; Stage 2".

#### 6.4.3.2	Clause 3

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

Local IOPS 3GPP system: It is a group of 3GPP core entities that are either co-sited with or co-located in an IOPS capable base station.

Unicast session: A unicast session is either a unicast bearer in EPS, or a unicast PDU session in 5GS.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APN	Access Point Name

DNN	Data Network Name

IOPS	Isolated Operation for Public Safety

EPC	Evolved Packet Core

EPS	Evolved Packet System

MBMS	Multimedia Broadcast and Multicast Service

MC	Mission Critical

PLMN	Public Land Mobile Network

### 6.4.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.5	Solution 5: Clause 6 (Involved business relationships)

### 6.5.1	General

This solution addresses key issue #1 with the focus on clause 6 to be written in a generic way to be a generic IOPS solution.

### 6.5.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

6	Involved business relationships

The description of the involved business relationships for the support of MC services in the IOPS mode of operation is contained in clause 6 of 3GPP TS 23.280 [3], and clause 6.4 of 3GPP TS 23.289 [23.289].

### 6.5.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.5.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.6	Solution 6: Clause 7 (Functional model) of TS 23.180

### 6.6.1	General

This solution addresses key issue #1 with the focus on clause 7 to be written in a generic way to be a generic IOPS solution.

### 6.6.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

# 7	Functional model

## 7.1	General

The functional model to support MC services in the IOPS mode of operation in case of a backhaul failure consists of a set of functions and reference points.

NOTE:	Terminology such as client and server are not meant to imply specific physical implementation of a functional entity.

## 7.2	Description of the planes

The functional model for the support of MC services in the IOPS mode of operation is defined as a series of planes to allow for the breakdown of the architectural description.

The application plane supports all the necessary functions to establish and provide MC services in the IOPS mode of operation. The application plane uses the services of the signalling control plane to establish the association of the users involved in an MC service. The signalling control plane also offers access to and control of services across MC services.

## 7.3	Functional model description

### 7.3.1	General

The application plane and signalling plane supporting the functional model for the IOPS mode of operation are described in this clause.

### 7.3.2	Functional model for the signalling control plane in the IOPS mode of operation

Figure 7.3.2-1 shows the common functional model for the signalling control plane in the IOPS mode of operation.

Figure 7.3.2-1 Functional model for the signalling control plane in the IOPS mode of operation

NOTE:	The functional model for the signalling plane in the IOPS mode of operation is described as a common functional model across MC services.

### 7.3.3	MCPTT functional model for the application plane in the IOPS mode of operation

Figure 7.3.3-1 shows the MCPTT functional model for the application plane in the IOPS mode of operation.

Figure 7.3.3-1 MCPTT functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.3-1, the following apply:

-	MCPTT-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, or 5G MBS session announcement, between the IOPS MC connectivity function and the MCPTT client.

-	MCPTT-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

-	MCPTT-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

### 7.3.4	MCData functional model for the application plane in the IOPS mode of operation

Figure 7.3.4-1 shows the MCData functional model for the application plane in the IOPS mode of operation.

Figure 7.3.4-1 MCData functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.4-1, the following apply:

-	MCData-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, 5G MBS session announcement, between the IOPS MC connectivity function and the MCData client.

-	MCData-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

-	MCData-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

## 7.4	Functional entities description

### 7.4.1	Application plane

#### 7.4.1.1	General

Entities within the application plane of an IOPS MC system provide application specific functions to support MC services in the IOPS mode of operation.

#### 7.4.1.2	IOPS connectivity client

The IOPS connectivity client functional entity provides support for enabling that a user at the MC service client is registered and discovered by the IOPS MC system in the IOPS mode of operation. This functional entity is located in the MC service UE.

#### 7.4.1.3	IOPS MC connectivity function

The IOPS MC connectivity function entity provides supporting MC services in the IOPS mode of operation. It includes registering and discovering users within the IOPS MC system. The IOPS MC connectivity functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [4] for EPS, or Application Function (AF) for 5GS to manage unicast and multicast transmissions for group communication related sessions.

The IOPS MC connectivity function entity is supported by the signalling application server functional entity of the signalling control plane.

#### 7.4.1.4	IOPS distribution function

The IOPS distribution function entity provides support for distributing the IP packets containing the MC service application data received from a MC service UE in the IOPS mode of operation.

By means of information provided by the IOPS MC connectivity function (e.g. IP addresses, transport layer ports), it will provide the following functionality:

-	receiving uplink IP transmissions from the MC service UE by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP unicast transmissions to MC service UEs by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP multicast transmissions to MC service UEs by means of the MCPTT-IOPS-4 reference point for MCPTT and the MCData-IOPS-4 reference point for MCData.

#### 7.4.1.5	MC service client

The MC service client functional entity, e.g. a MCPTT client and a MCData client, supporting the IOPS mode of operation acts as the user agent for the corresponding MC service transactions as well as all IOPS related application transactions.

### 7.4.2	Signalling control plane

#### 7.4.2.1	Signalling user agent client

The signalling user agent client functional entity supports at the MC service client all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

#### 7.4.2.2	Signalling application server

The signalling application server functional entity supports at the IOPS MC system all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

## 7.5	Reference points

### 7.5.1	Application plane

#### 7.5.1.1	Reference point MC-IOPS-1 (between the IOPS MC connectivity function and the IOPS connectivity client)

The MC-IOPS-1 reference point, which exists between the IOPS MC connectivity function and the IOPS connectivity client, is used for IOPS related application signalling (e.g. registration, publication, subscription and notification events). The MC-IOPS-1 reference point utilizes the MC-IOPS-X1 reference point for transport and routing of the IOPS related signalling.

#### 7.5.1.2	Reference point MCPTT-IOPS-2 (between the IOPS MC connectivity function and the MCPTT client)

The MCPTT-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCPTT client, is used to communicate signalling related to the broadcast/multicast sessions  utilized for broadcast/multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.3	Reference point MCData-IOPS-2 (between the IOPS MC connectivity function and the MCData client)

The MCData-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCData client, is used to communicate signalling related to the broadcast/multicast sessions  utilized for broadcast/multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.4	Reference point MCPTT-IOPS-3 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-3 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP unicast transmissions in uplink and downlink. The MCPTT-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.5	Reference point MCData-IOPS-3 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-3 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP unicast transmissions in uplink and downlink. The MCData-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.6	Reference point MCPTT-IOPS-4 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-4 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP multicast transmissions in downlink. The MCPTT-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/ Nmb6 reference point defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.7	Reference point MCData-IOPS-4 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-4 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP multicast transmissions in downlink. The MCData-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/Nmb6 reference points as defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.8	Reference point MC-IOPS-5 (between the IOPS MC connectivity function and the local IOPS 3GPP system)

The MC-IOPS-5 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system, is used to obtain unicast sessions with appropriate QoS from the local IOPS 3GPP system. The MC-IOPS-5 reference point utilizes the Rx interface of the EPS, or N5 interface of 5G system according to 3GPP TS 23.203 [8] or 3GPP TS 23.503[23.503], respectively.

#### 7.5.1.9	Reference point MC-IOPS-6 (between the IOPS MC connectivity function and the local IOPS 3GPP system )

The MC-IOPS-6 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system, is used to request the allocation and activation of multicast/broadcast transport resources, i.e. MBMS bearers or 5GMBS sessions, during the IOPS mode of operation. The MC-IOPS-6 reference point uses the MB2-C interface as defined in 3GPP TS 23.468 [4], Nmb10, Nmb13, or N33, as defined in 3GPP TS 23.247 [23.247].

### 7.5.2	Signalling control plane

#### 7.5.2.1	Reference point MC-IOPS-X1 (between the signalling application server and the signalling user agent client)

The MC-IOPS-X1 reference point, which exists between the signalling application server and the signalling user agent client, is used to support the IOPS related application signalling (e.g. registration, publication, subscription and notification events).

NOTE:	The IOPS related signalling supporting, e.g., the IOPS discovery, subscription and notification events can be SIP-based. In such a case, the signalling application server acts as a SIP application server (AS) or application function (AF) and the signalling user agent client acts as the SIP user agent for all SIP related transactions.

### 6.6.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.501]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[23.247]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[23.503]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

### 6.6.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.7	Solution 7: Converting the functional model and deployment section of TS 23.180

### 6.7.1 	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

NOTE:	in clause 6.aa.2 and 6.aa.3, the original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

### 6.7.2		Addressing the functional model and deployment section of 3GPP TS 23.180

# 9	Application of functional model to deployments

## 9.1	Overview

The IOPS architectural model includes application functions at the IOPS MC system and UEs to support MC services in the IOPS mode of operation during a backhaul failure. The IOPS MC system provides MC services support to the MC service UE via a single PLMN dedicated to the IOPS mode of operation.

The architectural model to support MC services in the IOPS mode of operation consists of a signalling control plane and an application plane. The signalling control plane provides the necessary signalling support for the related IOPS application layer transactions, e.g. the registration and discovery of UEs on the IOPS MC system. The IOPS application plane provides the necessary support for the transport of the IOPS operation related application data as well as the IP packets containing the MC service application data to be distributed via the IOPS MC system.

The MC service application data includes all signalling control data and application data (control and media) required to provide MC services between MC service clients. The IP related transmissions are established over the IOPS MC system via IP unicast and multicast transmissions.

## 9.2	IOPS architectural model diagram

Figure 9.2-1 shows the IOPS architectural model for the IOPS MC system solution in case of a backhaul failure.

Figure 9.2-1 Architectural model in the IOPS mode of operation

### 9.2.1	IOPS application services support layer

The IOPS MC system is composed of the following functional entities for the support of MC services in the IOPS mode of operation:

-	an IOPS MC connectivity function as described in clause 7.4.1.3, an IOPS distribution function as described in clause 7.4.1.4 and a signalling application server as described in clause 7.4.2.2.

### 9.2.2	Local IOPS 3GPP system

A local IOPS 3GPP system can comprise either:

-	a local core network and a single isolated IOPS-capable base station, which may be co-located or have connectivity to the local core network; or

-	a local core network and two or more IOPS-capable base station, which have connectivity to a single local core network.

NOTE:	For 4G IOPS, the implementation and deployment guidelines using a Local EPC approach is described in 3GPP TS 23.401 [2].

The local IOPS 3GPP system provides point-to-point and/or point-to-multipoint transmission services with QoS in the IOPS mode of operation, e.g., as described in 3GPP TS 23.401 [2] for EPS.

### 9.2.3	UE 1

The UE 1 is an MC service UE supporting the IOPS mode of operation. It supports transmission services and applications to provide MC services over the IOPS MC system. The UE 1 is composed of the following functional entities:

-	for MC services over the IOPS MC system, MC service client(s) as described in clause 7.4.1.5;

-	for IOPS application plane, an IOPS connectivity client as described in clause 7.4.1.2; and

-	for IOPS signalling control, a signalling user agent as described in subclause 7.4.2.1.

### 9.2.4	UE 2

## The UE 2 represents one or more UEs with the same functionality as UE 1.6.8	Solution 8: Clause 10.2.2.3 (IOPS discovery request) of TS 23.180

### 6.8.1	General

This clause is related to KI#1 and addresses clause 10.2.2.3 of 3GPP TS 23.180 [7].

### 6.8.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.2.2.3	IOPS discovery request

Table 10.2.2.3-1 describes the information flow for the IOPS discovery request from the IOPS connectivity client to the IOPS MC connectivity function.

Table 10.2.2.3-1: IOPS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service ID | M | The IOPS MC service ID of the requesting MC user |
| IP connectivity information | M | It includes the requesting MC service UE's IP address (NOTE 1) |
| List of IOPS MC service group ID (NOTE 2) | M | It includes the IOPS group (IOPS MC service group ID and its associated IOPS group IP multicast address) or list of IOPS groups the requesting MC user is configured with |
| Availability of connectivity information | O | It indicates if the connectivity information of the requesting MC user can be made available to other users on the system |
| List of IOPS MC service IDs (NOTE 2 and NOTE 3) | O | The specific list of IOPS MC service IDs that the requesting MC user's connectivity information can be shared with |
| Priority state | M | Indicates whether the requesting MC user is requesting higher priority or not |
| NOTE 1: The MC service UE's IP address is assigned by the local IOPS 3GPP system based on the PDN connection or PDU session established by the MC user for the IOPS IP connectivity functionality. NOTE 2: The list of IOPS MC service IDs or IOPS MC service group IDs shall be of the same MC service as the MC user requesting to be discovered. NOTE 3: If present, the user's connectivity information availability can be only made available to the provided specific list of IOPS MC service IDs. |  |  |

### 6.8.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.8.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.9	Solution 9: Converting the broadcast/multicast transmission section of TS 23.180

### 6.9.1 	General

This clause is related to KI#1 addresses the MBMS section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

NOTE:	in clause 6.8.2 and 6.8.3, the original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

### 6.9.2		Addressing the MBMS section of 3GPP TS 23.180

## 10.4	Use of broadcast/multicast transmissions

### 10.4.1	General

An IOPS MC connectivity function can provide support for group communications based on broadcast/multicast transmissions. Broadcast/multicast transmissions can be used for the downlink delivery of MC service application signalling and media to the MC service UEs over broadcast/multicast session (e.g., MBMS bearers in EPS, MBS session in 5GS).

The use of broadcast/multicast transmissions in the IOPS mode of operation are based on the related information flows and procedures specified in 3GPP TS 23.280 [3], 3GPP TS 23.379 [5], 3GPP TS 23.282 [6], and 3GPP TS 23.289 [23289].

### 10.4.2	Information flows

#### 10.4.2.1	MapIOPSGroupToNetworkSession (IP connectivity functionality)

Table 10.4.2.1-1 describes the information flow to associate a group session based on the IP connectivity functionality to a broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.1-1: MapIOPSGroupToNetworkSession (IP connectivity functionality)

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group session (e.g. MBMS subchannel). |
| Identifier of broadcast/multicast session (NOTE) | O | The identifier of the network broadcast/multicast session (e.g., TMGI) if the media of the group session is not sent on the same broadcast/multicast session as this MapIOPSGroupToNetwork message. |
| Session acknowledgement indicator | O | Indication that the group session requires acknowledgement from receiving MC service clients. |
| NOTE: “Identifier of broadcast/multicast session” IE shall be present if this message is sent over a different broadcast/multicast session than the media of the group session; This IE may be present if this message is sent over the same broadcast/multicast session as the media of the group session. |  |  |

#### 10.4.2.2	UnmapIOPSGroupFromNetworkSession (IP connectivity functionality)

Table 10.4.2.2-1 describes the information flow to disconnect a group session based on the IP connectivity functionality from an broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.2-1: UnmapIOPSGroupFromNetworkSession

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |

### 10.4.3	Use of pre-established broadcast/multicast session

The MC services in the IOPS mode of operation shall support the procedure for using pre-established broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use pre-established broadcast/multicast sessions for distributing the media associated to group sessions. Depending on the capacity of the broadcast/multicast session, the broadcast/multicast sessioncan be used to broadcast one or more group sessions in parallel.

### 10.4.4	Use of dynamic broadcast/multicast session establishment

The MC services in the IOPS mode of operation shall support the procedure for using dynamic broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use dynamic MBMS bearer establishment for distributing the media associated to group sessions.

### 10.4.5	Group session connect and disconnect over broadcast/multicast session (IP connectivity functionality)

#### 10.4.5.1	General

MBMS bearers can be used for group sessions based on the IP connectivity functionality. One broadcast/multicast session can be associated to one or more specific groups or group sessions. Before sending packets related to a group session, i.e. signalling and media, over an broadcast/multicast session, the IOPS MC connectivity function transmits the association information between the group session and the broadcast/multicast session. When the group session over the broadcast/multicast session is finished, this temporary association information of a group session to specific resources on a broadcast/multicast session is undone. Prior to the association of a group session to a broadcast/multicast session the broadcast/multicast session is activated and announced to the MC service clients.

#### 10.4.5.2	Procedure

##### 10.4.5.2.1	Group Session connect over broadcast/multicast session

In figure 10.4.5.2.1-1 the MC service client 1 is the client that initiates a group session, e.g. an MCPTT client initiating a group call. The MC service client 1 may be within the broadcast/multicast  service area. The MC service clients 2 … n represent MC service clients, e.g. MCPTT clients, receiving the group session packets over an MBMS bearer. There may be other receiving MC service clients both over unicast sessions and over this or other broadcast/multicast session(s), however, they are not illustrated in this figure.

Pre-conditions:

-	The MC service user profile used for the IOPS mode of operation is pre-provisioned in the MC service UEs

-	The IOPS MC service group ID and its associated IOPS group IP multicast address are pre-configured in the MC service clients

-	The MC service users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The MC service users are discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality

Figure 10.4.5.2.1-1: Group session connect on broadcast/multicast session (IP connectivity functionality)

1.	Activation and announcement of broadcast/multicast session availability.

2.	MC user at the MC service client 1 wants to initiate an MC service group session with a specific IOPS MC service group based on the IP connectivity functionality.

3.	The MC service client 1 transmits the related group session packets via the IOPS MC connectivity function targeting the IOPS MC service group (i.e. using the corresponding IOPS group IP multicast address)

4.	The IOPS MC connectivity function determines that the received packets correspond to a group session. Therefore, the IOPS MC connectivity function transmits a MapIOPSGroupToNetwork message over a previously established broadcast/multicast session to all users monitoring the broadcast/multicast session. The MapIOPSGroupToNetwork message includes association information between the group session and the broadcast/multicast session. It includes the corresponding IOPS MC service group ID associated to the IOPS group IP multicast address related to the group session.

5.	The IOPS MC connectivity function distributes the received group session packets from the MC service client 1 to other MC service clients (e.g. MC service clients 2 … n) over the broadcast/multicast session.

NOTE:	MC service clients receiving the group session packets over the broadcast/multicast session only decode the packets that are addressed to their pre-configured IOPS group IP multicast address(es). Otherwise, the MC service clients discard the packets.

##### 10.4.5.2.2	Group session disconnect over broadcast/multicast session

Figure 10.4.5.2.2-1 shows the high-level procedure where an UnmapIOPSGroupFromNetworkSession message is transmitted from the IOPS MC connectivity function to MC service clients to indicate that a group session is being disconnected or dissociated from the broadcast/multicast session.

Figure 10.4.5.2.2-1: Group session disconnect on broadcast/multicast session (IP connectivity functionality)

1.	A group session based on the IP connectivity functionality is ongoing; the group session packets received from the MC service client 1 are distributed over broadcast/multicast session to other MC service clients within the system.

2.	The IOPS MC connectivity function has determined to disconnect the group session from the broadcast/multicast session for the MC service clients within the system.

NOTE:	The IOPS MC connectivity function may decide to disconnect a group session from a broadcast/multicast session if group session packets are not anymore received addressing the corresponding IOPS MC service group (or IOPS group IP multicast address).

3.	An UnmapGroupFromBearer message is transmitted from the IOPS MC connectivity function to the MC service clients and possibly to the MC service client 1 (if it is within the broadcast/multicast coverage area) on broadcast/multicast session(s).

### 6.9.3	Other affected clauses

#### 6.9.3.1	definitions update

The generic definitions are required.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

Broadcast/multicast session: is either the MBMS bearer in EPS or the MBS session in 5GS.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

### 6.9.4	Solution evaluation

The above proposal converts the current MBMS section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.10	Solution 10: Clause 10.5.1.4 (IOPS emergency group call) of TS 23.180

### 6.10.1	General

This clause is related to KI#1 addresses section 10.5.1.4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section IOPS emergency group call of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.10.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.1.4	IOPS emergency group call

The procedure in figure 10.5.1.4-1 illustrates the procedure for an IOPS MCPTT emergency group call establishment based on the IP connectivity functionality. The IOPS emergency group call is a special case of the IOPS group call setup procedure described in clause 10.5.1.3, wherein the IOPS group call announcement contains an indication that the IOPS group call is an IOPS emergency group call. The group call participants can become aware of the IOPS MCPTT group's in-progress emergency state based on the emergency indicator.

When an MCPTT client intends to initiate an IOPS emergency group call, the MCPTT client can request higher priority from the IOPS MC connectivity function via the IOPS discovery request.

An IOPS group call in-progress can be upgraded to an IOPS emergency group call by including the emergency indicator within the periodic IOPS group call announcement. An IOPS group call in-progress can also be upgraded by a participating MCPTT client by sending an IOPS emergency group call upgrade to the IOPS group.

The MCPTT user who initiated an IOPS emergency group call or upgraded an IOPS group call to an emergency group call, or an authorized user can cancel the emergency state of the group call by sending an IOPS emergency group call state cancel to the IOPS MCPTT group. The emergency state of the IOPS group call remains active until the emergency group call ends or the in-progress emergency state is cancelled.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs;

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients;

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality;

-	MCPTT users affiliated to the target IOPS MCPTT group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality;

-	The IOPC MC connectivity function may have established an broadcast/multicast session and announced it to the MCPTT clients;

-	MCPTT client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCPTT group;

-	MCPTT clients 1, 2 … n are configured within the same IOPS MCPTT group.

Figure 10.5.1.4-1: IOPS emergency group call setup based on the IP connectivity functionality

1.	The MCPTT user at MCPTT client 1 would like to initiate an IOPS emergency group call with a specific IOPS MCPTT group based on the IP connectivity functionality.

NOTE 1:	The MCPTT client 1 may have previously requested higher priority from the IOPS MC connectivity function using the IOPS discovery request.

2.	The MCPTT client 1 sends an IOPS group call announcement to the target IOPS MCPTT group. The announcement contains an indication that the call is an IOPS emergency group call. The MCPTT client 1 transmits the group session packets carrying the IOPS group call announcement to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides to distribute the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions. If the MCPTT client 1 requested a priority state from the IOPS MC connectivity function, the IOPS MC connectivity function distributes the group session packets with higher priority.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call announcement to the MCPTT clients from the target IOPS MCPTT group.

5.	The MCPTT clients receiving the IOPS group call announcement with an emergency indicator join the IOPS emergency group call and notify the target MCPTT users about the IOPS emergency group call. The IOPS MCPTT group's emergency state is defined.

NOTE 2:	Whilst the emergency state of the IOPS group call remains active, other participating MCPTT clients of the group call may also request higher priority from the IOPS MC connectivity function using the IOPS discovery request.

6.	If confirm mode indication is included in the IOPS group call announcement, the receiving MCPTT clients respond to the IOPS MCPTT group indicating the result of the establishment of the announced IOPS emergency group call. The receiving MCPTT clients transmit the group session packets carrying the IOPS group call response to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

NOTE 3:	Step 6 can also occur prior to step 5.

7.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides to distribute the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions. If any participating MCPTT client of the group call requested a priority state from the IOPS MC connectivity function, the IOPS MC connectivity function distributes the group session packets with higher priority.

8.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call response to the MCPTT clients from the target IOPS MCPTT group. The MCPTT clients recognize the IOPS emergency group call originator through the IOPS group call announcement and can check the participants of the IOPS group call through the received response message.

9.	The MCPTT clients have successfully established the IOPS emergency group call based on the IP connectivity functionality.

NOTE 4:	Due to the movement of the participants (in and out of the local IOPS 3GPP system coverage) during the IOPS emergency group call, the IOPS group call announcement is periodically sent by the MCPTT client 1.

NOTE 5:	The participating MCPTT clients do not need to respond to the periodic IOPS group call announcement.

## 6.11	Solution 11: Clause 10.5.1.3 (IOPS group call setup) of TS 23.180

### 6.11.1	General

This clause is related to KI#1 addresses section 10.5.1.3 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section 10.5.1.3 (IOPS group call setup) of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.11.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.1.3	IOPS group call setup

The procedure in figure 10.5.1.3-1 illustrates the procedure for an IOPS MCPTT group call establishment based on the IP connectivity functionality. The procedure describes how an MCPTT client initiates and establishes an IOPS MCPTT group call with other MCPTT clients.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs;

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients;

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality;

-	MCPTT users affiliated to the target IOPS MCPTT group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality;

-	The IOPC MC connectivity function may have established an broadcast/multicast session and announced it to the MCPTT clients;

-	MCPTT client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCPTT group;

-	MCPTT clients 1, 2 … n are configured within the same IOPS MCPTT group.

Figure 10.5.1.3-1: IOPS group call setup based on the IP connectivity functionality

1.	The MCPTT user at MCPTT client 1 would like to initiate an IOPS group call with a specific IOPS MCPTT group based on the IP connectivity functionality.

2.	The MCPTT client 1 sends an IOPS group call announcement to the target IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS group call announcement to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call announcement to the MCPTT clients from the target IOPS MCPTT group.

5.	The MCPTT clients receiving the IOPS group call announcement join the IOPS group call and notify the target MCPTT users about the IOPS group call.

6.	If confirm mode indication is included in the IOPS group call announcement, the receiving MCPTT clients respond to the IOPS MCPTT group indicating the result of the establishment of the announced IOPS group call. The receiving MCPTT clients transmit the group session packets carrying the IOPS group call response to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

NOTE 1:	Step 6 can also occur prior to step 5.

7.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions.

8.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call response to the MCPTT clients from the target IOPS MCPTT group. The MCPTT clients recognize the IOPS group call originator through the IOPS group call announcement and can check the participants of the IOPS group call through the received response message.

9.	The MCPTT clients have successfully established the IOPS group call with floor control based on the IP connectivity functionality.

NOTE 2:	Due to the movement of the participants (in and out of the local IOPS 3GPP system coverage) during the IOPS group call, the IOPS group call announcement is periodically sent by the MCPTT client 1.

NOTE 3:	The participating MCPTT clients do not need to respond to the periodic IOPS group call announcement.

## 6.12	Solution 12: Clause 10.5.3.1 (IOPS floor control - General) of TS 23.180

### 6.12.1	General

This clause is related to KI#1 addresses section 10.5.3.1 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the General section of IOPS floor control of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.12.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.3.1	General

For MCPTT calls based on the IP connectivity functionality in the IOPS mode of operation, floor control is performed by using floor control messages among the MCPTT clients without a centralized MCPTT server. The MCPTT client can transmit voice packets over the IOPS MC connectivity function once it is granted the right to speak, either locally in the UE or by the reception of a floor granted message from another MCPTT client.

The MCPTT client currently speaking performs the temporary floor arbitrator during speaking since there is no centralized MCPTT floor control server. The floor arbitrator controls the floor whether or not queue is supported, and when floor is requested with override. If queue is supported, the MCPTT client performing floor arbitrator grants the right to speak to the next speaker and transfers the floor arbitrator role after completing the voice transfer and releasing the floor. For IOPS group calls, the floor arbitrator also transfers the floor control queue when granting the floor. The next MCPTT client receiving the right to speak becomes the new floor arbitrator and, for IOPS group calls, has the floor control queue.

For IOPS group calls, the group session packets carrying the floor control messages can be transmitted by the IOPS MC connectivity function over broadcast/multicast transmissions and can be monitored by all the members from the target IOPS MCPTT group.

The following clauses specify the floor control procedures and information flows for IOPS private calls and IOPS group calls based on the IP connectivity functionality in the IOPS mode of operation.

## 6.13	Solution 13: Clause 10.5.3.3 (IOPS floor control during silence) of TS 23.180

### 6.13.1	General

This clause is related to KI#1 addresses section 10.5.3.3 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section 10.5.3.3 (IOPS floor control during silence) of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.13.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.3.3	IOPS floor control during silence

If a floor arbitrator does not exist, figure 10.5.3.3-1 shows the successful high level floor control procedure during periods when there is no detectable talker in an IOPS group call based on the IP connectivity functionality.

NOTE 1:	The description also applies to IOPS private calls.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs.

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients (for the case of an IOPS group call)

-	The IOPC MC connectivity function may have established a broadcast/multicast session and announced it to the MCPTT clients

-	The MCPTT users are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	MCPTT clients has retrieved connectivity information from the target MCPTT user (for the case of an IOPS private call).

-	An IOPS private call or IOPS group call based on the IP connectivity functionality has been established. No participant is currently talking (i.e. the floor is idle) and no floor arbitrator is identified.

Figure 10.5.3.3-1: Successful floor taken flow in an IOPS group call based on the IP connectivity functionality (no floor contention)

1.	The MCPTT client 1 sends the IOPS floor request message to the target IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS floor request message to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

2.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions.

3.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS floor request to the MCPTT clients from the target IOPS MCPTT group.

4.	The MCPTT client 1 does not detect any floor contention. Floor contention occurs when multiple floor requests may exist simultaneously.

NOTE 2:	The mechanism for detecting floor contention in the IOPS mode of operation is out of scope of the present document.

5.	The MCPTT client 1 sends the IOPS floor taken message to the IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS floor taken message to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

6.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over broadcast/multicast and/or unicast transmissions.

7.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS floor taken message to the MCPTT clients from the target IOPS MCPTT group.

8.	The MC user at MCPTT client 1 gets a notification that the IOPS floor request was successful (the floor has been granted).

NOTE 3:	Step 8 can also occur prior to steps 6 and 7.

9.	The MCPTT client 1 begins voice transmission with the target IOPS MCPTT group based on the IP connectivity functionality.

## 6.14	Solution 14: Clause 10.6.1.4 (IOPS group standalone SDS using signalling control plane) of TS 23.180

### 6.14.1	General

This clause is related to KI#1 addresses section 10.6.1.4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.14.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.6.1.4	IOPS group standalone SDS using signalling control plane

##### 10.6.1.4.1	General

IOPS group standalone SDS using signalling control plane based on the IP connectivity functionality can use pre-configured information provided to MCData clients prior to initiating the data service. When an MCData client initiates an IOPS group standalone SDS based on the IP connectivity functionality it uses the pre-configured IOPS group IP multicast address associated to the target IOPS MCData group ID. The related group session packets, i.e. signalling messages, carrying the data are transmitted to the IOPS MC connectivity function for distribution to the corresponding discovered MC users of the target IOPS MCData group.

The IOPS MC connectivity function can distribute the group session packets to the discovered MC users over broadcast/multicast sessions as described in clause 10.4.5.

The IOPS MC connectivity function can also replicate and distribute the group session packets over unicast transmissions to MCData UEs associated to the target IOPS MCData group. MCData UEs receiving the group session packets are associated to discovered MC users that included the target IOPS MCData group ID within the IOPS discovery request, as described in clause 10.5.2.3.

##### 10.6.1.4.2	Procedure

The procedure in figure 10.6.1.4.2-1 describes the case where an MCData user is initiating an IOPS group MCData communication for sending standalone SDS data over signalling control plane to an IOPS MCData group, with or without disposition request. Standalone refers to sending unidirectional data in one transaction.

Pre-conditions:

-	MCData user profile used for the IOPS mode of operation is pre-provisioned in the MCData UEs.

-	The IOPS MCData group ID and its associated IOPS group IP multicast address are pre-configured in the MCData clients.

-	MCData users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality.

-	MCData users affiliated to the target IOPS MCData group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	The IOPS MC connectivity function may have established a broadcast/multicast session and announced it to the MCData clients.

-	MCData client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCData group.

-	MCData clients 1, 2 … n are configured within the same IOPS MCData group.

Figure 10.6.1.4.2-1: IOPS group standalone SDS using signalling control plane based on the IP connectivity functionality

1.	The MCData user at MCData client 1 would like to initiate an IOPS SDS data transfer with a specific IOPS MCData group based on the IP connectivity functionality. The MCData client 1 checks whether the MCData user 1 is authorized to send an IOPS MCData group standalone data request.

2.	The MCData client 1 sends an IOPS MCData group standalone data request to the target IOPS MCData group. The MCData client 1 transmits the group session packets carrying the IOPS MCData group standalone data request to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

The IOPS MCData group standalone data request contains the data payload, i.e. the SDS content. The request also contains a conversation identifier for message thread indication and may contain a disposition request if indicated by the user at MCData client 1.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCData group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCData clients over broadcast/multicast session and/or unicast transmissions.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS MCData group standalone data request to the discovered MCData clients from the target IOPS MCData group.

5.	The MCData clients receiving the IOPS MCData group standalone data request check whether any policy is to be asserted to limit certain types of messages or content to certain members due to, for example, location or user privilege. If the policy assertion is positive and the payload is for MCData user consumption (e.g. it is not application data, not command instructions, etc.) then the MCData clients notify the target MCData users.

The actions taken when the payload contains application data or command instructions are based on the payload content. Payload content received by an MCData client which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	If the policy assertion was negative, the corresponding MCData client sends an appropriate notification to MCData client 1.

6.	If MCData data disposition was indicated (for delivery, read or both) within the request sent by the MCData client 1, then the receiving MCData clients initiate the corresponding IOPS MCData data disposition notification(s) towards the MCData client 1, i.e. addressing the MCData UE 1's IP address.

7.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 1's IP address. The IOPS MC connectivity function checks if the MCData UE 1's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

8.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData data disposition notification to the MCData client 1.

## 6.15	Solution 15: Clause 10.7 (MC IOPS notification) of TS 23.180

### 6.15.1	General

This clause is related to KI#1 addresses section 10.7 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.15.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

## 10.7	MC IOPS notification

### 10.7.1	General

In the IOPS mode of operation, it is assumed that the IOPS MC system does not have connectivity to the primary MC system due to the backhaul failure. Therefore, the primary MC system cannot be aware of the initiation of the IOPS operation and the corresponding activation of an IOPS MC connectivity function within the primary MC system coverage.

When an IOPS MC system is active, MC service UEs can move around and may enter and leave the IOPS MC system coverage, i.e. the MC service users may switch from the active IOPS MC connectivity function to the MC service server of the primary MC system, and vice versa.

In order to notify the primary MC service server about the active IOPS MC connectivity function, when MC service users register to the primary MC service server after being recently registered to the IOPS MC connectivity function, the MC service users can provide information to the primary MC service server about the active IOPS MC connectivity function and optionally include associated dynamic information. The primary MC service server uses the provided information to become aware of the active IOPS MC connectivity function.

NOTE:	Dynamic information can be, e.g., information about other available MC service users or active MC service groups that the MC service user identified while on the IOPS MC connectivity function. The primary MC service server can use the dynamic information to determine that affiliated MC service users might be registered on the active IOPS MC connectivity function and might not be reachable on the system.

Upon the receipt of the notification, the primary MC service server may notify other MC service users in the proximity of the IOPS MC system coverage about the corresponding active IOPS MC connectivity function. This information can be used by the MC service users to optimize the user experience, e.g. to improve the switching time between systems and to obtain information about the potential availability of other registered MC service users or active MC service groups on the IOPS MC connectivity function.

### 10.7.2	Information flows

#### 10.7.2.1	MC IOPS notification

Table 10.7.2.1-1 describes the information flow MC IOPS notification from the MC service client to the primary MC service server.

Table 10.7.2.1-1: MC IOPS notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user providing the notification |
| IOPS MC system information | M | Information related to the identified active IOPS MC connectivity function (see NOTE) |
| List of MC service group IDs | O | The list of groups identified by the MC service user as active on the IOPS MC connectivity function |
| List of MC service IDs | O | The list of other users identified by the MC service user as available on the IOPS MC connectivity function |
| NOTE: The IOPS MC system information consists of the following elements: IOPS PLMN ID, server URI of the IOPS MC connectivity function, and location information (set of coordinates including altitude, longitude and latitude, and time of measurement and optional accuracy) related to the MC service user registration on the IOPS MC connectivity function. |  |  |

Table 10.7.2.1-2 describes the information flow MC IOPS notification from the primary MC service server to the MC service client.

Table 10.7.2.1-2: MC IOPS notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID (see NOTE 1) | M | The identity of the MC service user receiving the notification |
| IOPS MC system information | M | Information related to the identified active IOPS MC connectivity function (see NOTE 2) |
| List of MC service group IDs (see NOTE 3) | O | The list of MC service groups identified as active on the IOPS MC connectivity function |
| List of MC service IDs (see NOTE 3) | O | The list of MC service users identified as available on the IOPS MC connectivity function |
| NOTE 1: This information element is not included if the notification is transmitted over broadcast/multicast session. NOTE 2: The IOPS MC system information consists of the following elements: server URI of the IOPS MC connectivity function, and location information (set of coordinates including altitude, longitude and latitude) where the IOPS MC connectivity function is identified as active. NOTE 3: The MC service server may provide information about identified active MC service groups or available MC service users on the IOPS MC connectivity function. This information is only included if the MC service user receiving the notification is authorized, e.g. if the MC service user is a member of the corresponding MC service groups. This information element is not included if the notification is transmitted over broadcast/multicast session. |  |  |

### 10.7.3	MC IOPS notification procedure

Figure 10.7.3-1 describes the IOPS MC notification procedure when an MC service user has left an active IOPS MC system and enters the primary MC system.

Pre-conditions:

-	There is an active IOPS MC connectivity function and the neighbouring cells of the IOPS MC system are part of the primary MC system.

-	The MC service user 1 is initially registered to the IOPS MC connectivity function for the support of MC services in the IOPS mode of operation. The MC service user 1 is authorized to provide MC IOPS notifications to the primary MC service server.

-	The MC service user 2 is registered to the primary MC service server and is in the proximity of the IOPS MC system coverage.

-	MC server users 1 and 2 are members of the same MC service group or are authorized to have a one-to-one MC service communication.

Figure 10.7.3-1: MC IOPS notification procedure

1.	An IOPS mode of operation is active, and the MC service communication is handled by the IOPS MC connectivity function. The MC service client 1 is registered to the IOPS MC connectivity function.

2.	The MC service client 1 moves out of the coverage of the IOPS MC system and registers to the primary MC service server.

3.	The MC service client 1 sends a MC IOPS notification to the primary MC service server to provide information about an active IOPS MC connectivity function in the area. This notification includes information about the active IOPS MC connectivity function such as server URI, associated IOPS PLMN ID, and location information. Also, the MC service client 1 may indicate which MC service groups and MC service users were identified as active and available on the IOPS MC connectivity function.

4.	The primary MC service server becomes aware of the active IOPS MC connectivity function and can use the received information to determine that affiliated MC service users might be registered on the active IOPS MC connectivity function and might not be reachable on the system. If the primary MC service server determines that the IOPS MC connectivity function is active and identifies that affiliated MC service users are in the proximity of the IOPS MC system, the primary MC service server may notify the corresponding MC service users about the IOPS MC connectivity function.

NOTE 1:	The primary MC service server can use received information from different MC IOPS notifications (e.g. location information including the time of measurement) and the information obtained from location information subscriptions (as described in 3GPP TS 23.280 [3]) to determine if the IOPS MC connectivity function might be still active. For instance, if information received from location information subscriptions indicates that MC service users are located within the notified active IOPS MC system coverage, the primary MC service server can determine that the IOPS MC connectivity function is no longer active.

5.	If the primary MC service server determines that the IOPS MC connectivity function is active, it sends a MC IOPS notification to the MC service client 2 in proximity of the active IOPS MC system coverage. This information can be used by the MC service user to become aware of the active IOPS MC connectivity function. Hence, the MC service user might decide to not move into the IOPS MC system coverage or to improve the switching time between the systems. Also, the MC service user can be aware of the potential availability of MC service users or active MC service groups on the IOPS MC connectivity function.

NOTE 2:	The MC IOPS notification can be sent on a broadcast/multicast session configured within the proximity of the active IOPS MC system to target multiple MC service users. In this case, information about the active MC service IDs and MC service group IDs is not included in the MC IOPS notification.

## 6.16	Solution 16: Clauses 10.2.3, 10.3.3 and 10.6.1.3.2

### 6.16.1	General

This solution addresses key issue #1 with the focus on clauses 10.2.3, 10.3.3 and 10.6.1.3.2 to be written in a generic way to be a generic IOPS solution.

### 6.16.2	Solution

#### Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. 6.16.2.1	clause 10.2.3

### 10.2.3	Procedure

After an MC service user is authenticated on the IOPS MC connectivity function, the IOPS discovery procedure is initiated by the MC users requesting support of the IP connectivity functionality to the IOPS MC connectivity function. If the IOPS MC connectivity function indicates the support of the IP connectivity functionality, the MC service users can send an IOPS discovery request.

The procedure for requesting support of the IP connectivity functionality by the IOPS connectivity client to the IOPS MC connectivity function is described in figure 10.2.3-1.

NOTE 1:	The procedure for requesting support of the IP connectivity functionality is only required when the IOPS connectivity client does not contain information about the support of this functionality by the serving IOPS MC connectivity function.

Pre-conditions:

-	The MC service user is authenticated on the IOPS MC connectivity function.

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The IOPS connectivity client does not contain information about the support of the IP connectivity functionality by the serving IOPS MC connectivity function

Figure 10.2.3-1 IP connectivity functionality request in the IOPS mode of operation

1.	The IOPS connectivity client requests to the IOPS MC connectivity function the support of the IP connectivity functionality.

2.	The IOPS MC connectivity function indicates to the IOPS connectivity client if the IP connectivity functionality is supported or not for the MC user.

The procedure for the discovery of MC users in the IOPS mode of operation is described in figure 10.2.3-2. The IOPS discovery is initiated by the MC users to support MC services based on the IP connectivity functionality.

As part of the IOPS discovery procedure, an MC user can request higher priority from the IOPS MC connectivity function by including a priority state indication within the IOPS discovery request. Based on this indication, the IOPS MC connectivity function can decide to handle with higher priority the transactions related to the MC user, e.g. the distribution of packets related to any MC communication from or to the corresponding MC user, subscriptions, and notifications. When an MC user intends to establish an IOPS emergency (group or private) call, the MC user can request a priority state from the IOPS MC connectivity function.

The MC user's priority state remains unchanged until the MC user explicitly changes it (i.e. by sending a new IOPS discovery request) or the discovery status of the MC user changes to not-discovered on the IOPS MC connectivity function.

Pre-conditions:

-	The MC service user is authenticated on the IOPS MC connectivity function.

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The IOPS MC connectivity function has indicated to the IOPS connectivity client the support of the IP connectivity functionality.

Figure 10.2.3-2 User discovery in the IOPS mode of operation

1.	The MC user sends an IOPS discovery request to the IOPS MC connectivity function. The request includes providing connectivity information for the support of MC services based on the IP connectivity functionality.

2.	The IOPS MC connectivity function stores the information received from the MC user and registers the user's connectivity status as discovered.

3.	The IOPS MC connectivity function provides a response to the IOPS connectivity client indicating the success or failure of the discovery of the requesting MC user.

NOTE 2:	The MC user shall send a new IOPS discovery request to update or modify information elements.

NOTE 3:	The IOPS MC connectivity function shall send a new IOPS discovery response to the IOPS connectivity client for the case that the periodic IOPS discovery request has not been received yet or when the IOPS discovery request periodicity needs to be changed. The IOPS MC connectivity function can verify the availability and IP connectivity information of the discovered MC user within the IOPS MC system based on the reception of periodic IOPS discovery requests.

#### 6.16.2.2	Clause 10.3.3

### 10.3.3	Procedures

The procedure for the IP connectivity subscription of MC users in the IOPS mode of operation is described in figure 10.3.3-1.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user is discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality.

-	The MC service user has decided to subscribe to receive connectivity information of other discovered MC users and/or IOPS groups on the IOPS MC connectivity function.

Figure 10.3.3-1 IP connectivity subscription in the IOPS mode of operation

1.	The IOPS connectivity client subscribes to receive connectivity information of other discovered MC users and/or groups on the IOPS MC connectivity function.

2.	The IOPS MC connectivity function checks if a subscription for the MC service ID(s) and/or IOPS group(s) included in the request can be activated for the requesting MC user.

NOTE:	A subscription cannot be activated if an MC user associated to the requested MC service ID has indicated in its IOPS discovery request that its connectivity information is not available to other users on the IOPS MC connectivity function. Besides, a subscription may not be activated if a requested IOPS group does not belong to the list of IOPS groups the requesting MC user has included in its IOPS discovery request.

3.	The IOPS MC connectivity function indicates to the IOPS connectivity client the subscription status associated to the request.

The procedure for the IP connectivity notification of MC users in the IOPS mode of operation is described in figure 10.3.3-2.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user has an active IP connectivity subscription associated to one or multiple MC service IDs.

-	The IOPS MC connectivity function has available information related to an IP connectivity subscription associated to one or multiple MC service IDs.

Figure 10.3.3-2 IP connectivity notification in the IOPS mode of operation

1.	The IOPS MC connectivity function notifies to the IOPS connectivity client the connectivity information related to an active IP connectivity subscription associated to one or multiple MC service IDs. The connectivity information can be retrieved by the MC service client to establish an MC service communication based on the IP connectivity functionality.

2.	The IOPS connectivity client provides an IP connectivity notify response to the IOPS MC connectivity function.

The procedure for the IP connectivity group notification of MC users in the IOPS mode of operation is described in figure 10.3.3-3.

Pre-conditions:

-	The MC service user has an active PDN connection or PDU session to the IOPS MC connectivity function for the specific IP connectivity functionality procedure

-	The MC service user has an active IP connectivity subscription associated to one or multiple IOPS MC service group IDs.

-	The IOPS MC connectivity function has available information related to an IP connectivity subscription associated to one or multiple IOPS MC service group IDs.

Figure 10.3.3-3 IP connectivity group notification in the IOPS mode of operation

1.	The IOPS MC connectivity function notifies to the IOPS connectivity client the group connectivity information related to an active IP connectivity group subscription associated to one or multiple IOPS MC service group IDs. The group connectivity information can be retrieved by the MC service client to establish an MC service group communication based on the IP connectivity functionality.

2.	The IOPS connectivity client provides an IP connectivity group notify response to the IOPS MC connectivity function.

#### 6.16.2.3	Clause 10.6.1.3.2

##### 10.6.1.3.2	Procedure

The procedure in figure 10.6.1.3.2-1 describes the case where an MCData user is initiating an IOPS one-to-one MCData communication for sending standalone SDS data over signalling control plane to another MCData user, with or without disposition request. Standalone refers to sending unidirectional data in one transaction.

Pre-conditions:

-	MCData user profile used for the IOPS mode of operation is pre-provisioned in the MCData UEs.

-	MCData users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality.

-	The MCData users are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	MCData clients have retrieved connectivity information from target MCData users.

Figure 10.6.1.3.2-1: IOPS one-to-one standalone SDS using signalling control plane based on the IP connectivity functionality

1.	The MCData user at MCData client 1 would like to initiate an IOPS SDS data transfer with the MCData user at MCData client 2 based on the IP connectivity functionality. The MCData client 1 checks whether the MCData user 1 is authorized to send an IOPS MCData standalone data request.

2.	The MCData client 1 retrieves the connectivity information of the target MCData user from the IOPS connectivity client 1 (not shown in figure) and sends an IOPS MCData standalone data request towards the MCData client 2. The MCData client 1 transmits the session packets carrying the IOPS MCData standalone data request to the IOPS MC connectivity function for distribution to the corresponding target MCData UE 2's IP address.

The IOPS MCData standalone data request contains the data payload, i.e. the SDS content. The request also contains a conversation identifier for message thread indication and may contain a disposition request if indicated by the user at MCData client 1.

3.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 2's IP address. The IOPS MC connectivity function checks if the MCData UE 2's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

4.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData standalone data request to the MCData client 2.

5.	Upon the receipt of the IOPS MCData standalone data request, the MCData client 2 checks whether any policy is to be asserted to limit certain types of message or content to certain members due to, for example, location or user privilege. If the policy assertion is positive and the payload is for MCData user consumption (e.g. it is not application data, not command instructions, etc.) then the MCData client 2 notifies the target MCData user.

The actions taken when the payload contains application data or command instructions are based on the payload content. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	If the policy assertion was negative, the MCData client 2 sends an appropriate notification to MCData client 1.

6.	If MCData data disposition was indicated (for delivery, read or both) within the request sent by the MCData client 1, then the receiving MCData client 2 initiates the corresponding IOPS MCData data disposition notification(s) towards the MCData client 1, i.e. addressing the MCData UE 1's IP address.

7.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 1's IP address. The IOPS MC connectivity function checks if the MCData UE 1's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

8.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData data disposition notification to the MCData client 1.

### 6.16.4	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.16.5	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

## 6.17	Solution 17: Clauses A.1, A.2 and A.6 (Involved business relationships)

### 6.17.1	General

This solution addresses key issue #1 with the focus on clauses A.1, A.2 and A.6 to be written in a generic way to be a generic IOPS solution.

### 6.17.2	Solution

#### 6.17.2.1	Solution for clause A.1

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

# A.1	General

This Annex provides information about the static configuration data needed for the support of MC services in the IOPS mode of operation.

The configuration data belong to one of the following categories:

-	MC service UE configuration data (see subclause A.2);

-	MC service user profile configuration data (see subclause A.3);

-	MC service group configuration data (see subclause A.4); and

-	MC service configuration data (see subclause A.5).

-	Location user profile configuration data (see subclause A.8)

The configuration data in each configuration category corresponds to a single instance of the category type i.e. the MC service UE, MC service group, MC service user and MC service configuration data refers to the information that will be stored against each MC service UE, MC service group, MC service user and MC service.

NOTE:	The configuration data described in this Annex together with corresponding configuration data provided in 3GPP TS 23.280 [3], 3GPP TS 23.289[23.289], 3GPP TS 23.289 [23.289] 3GPP TS 23.379 [5] and 3GPP TS 23.282 [6] represent the complete set of data for each configuration data category element.

The columns in the tables have the following meanings:

-	Reference: the reference of the corresponding requirement in 3GPP TS 22.346 [9] and 3GPP TS 22.280 [11] or the corresponding clause from either the present document or the referenced document.

-	Parameter description: A short definition of the semantics of the corresponding item of data, including denotation of the level of the parameter in the configuration hierarchy.

-	When it is not clear to which functional entities the parameter is configured, then one or more columns indicating this are provided where the following nomenclature is used:

-	"Y" to denote "Yes" i.e. the parameter denoted for the row needs to be configured to the functional entity denoted for the column.

-	"N" to denote "No" i.e. the parameter denoted for the row does not need to be configured to the functional entity denoted for the column.

Parameters within a set of configuration data have a level within a hierarchy that pertains only to that configuration data. The level of a parameter within the hierarchy of the configuration data is denoted by use of the character ">" in the parameter description field within each table, one per level. Parameters that are at the top-most level within the hierarchy have no ">" character. Parameters that have one or more ">" characters are child parameters of the first parameter above them that has one less ">" character. Parent parameters are parameters that have one or more child parameters. Parent parameters act solely as a "grouping" of their child parameters and therefore do not contain an actual value themselves i.e. they are just containers for their child parameters.

Each parameter that can be configured online shall only be configured through one online reference point. Each parameter that can be configured offline shall only be configured through one offline reference point. The most recent configuration data made available to the MC service UE shall always overwrite previous configuration data, irrespective of whether the configuration data was provided via the online or offline mechanism.

#### 6.17.2.2	Solution for clause A.2

# A.2	MC service UE configuration data

MC service UE configuration data has to be known by an MC service UE after MC service authorization. The CSC-4 reference point, specified in 3GPP TS 23.280 [3], is used for configuration between the configuration management server and the configuration management client on the MC service UE when the MC service UE is on-network.

MC service UE configuration data can be configured offline using the CSC-11 reference point specified in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

Within each MC service, the MC service UE configuration data can be the same or different across MC service UEs.

The MCPTT UE configuration data specified in table A.2-1 in 3GPP TS 23.379 [5] is also used, as needed, in the IOPS mode of operation for the MCPTT service.

The MCData UE configuration data specified in table A.2-1 in 3GPP TS 23.282 [6] is also used, as needed, in the IOPS mode of operation for the MCData service.

#### 6.17.2.2	Solution for A.6

# A.6	Initial MC service UE configuration data

Initial MC service UE configuration data is essential to the MC service UE to successfully connect to the MC system, as described in 3GPP TS 23.280 [3] , and 3GPP TS 23.289 [23.289].

The configuration data defined in table A.6-1 is additionally provided to the MC service UE's clients to successfully connect to the IOPS MC system in the IOPS mode of operation. The MC service UE's clients (e.g. MC service client, IOPS connectivity client) obtain the data during the bootstrap process (described in clause 10.1.1 in 3GPP TS 23.280 [3]), and can be configured on the MC service UE offline using the CSC-11 reference point or via other means e.g. as part of the MC service client's provisioning on the UE, using a device management procedure.

Table A.6-1: Initial MC service UE configuration data (IOPS)

| Reference | Parameter description |
| --- | --- |
| Clause 5.4.3 | PDN connectivity information in IOPS (NOTE 1) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC services PDN in IOPS |
|  | >> APN |
|  | >> PDN access credentials |
|  | PDU session information in IOPS (NOTE 2) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC service PDU in IOPS |
|  | >> DNN |
|  | >> PDU access credentials |
|  | >Default configured slice(s) S-NSSAI(s) |
|  | Application plane server identity information |
|  | > Indication of whether the UE shall use IPv4 or IPv6 for the support of MC services in IOPS |
|  | > IOPS MC connectivity function |
|  | >> Server URI |
| NOTE 1: These parameters shall only be used to access via local IOPS EPS system. NOTE 2: These parameters shall only be used to access via local IOPS 5GS system. |  |

#### 6.17.2.3	solution for location user profile configuration

A location user profile configuration data is needed to be included in 3GPP TS 23.180 to reflect the configurations related to location information reporting. The location user profile configuration in annex A.8 of 3GPP TS 23.280 [3] are applicable as needed in the IOPS mode of operation.

### 6.17.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.17.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

# 7	Overall evaluation

## 7.1	General

The following subclauses contain an overall evaluation of the solutions presented in this technical report, and their applicability to the key issues.

## 7.2	Architecture evaluation

The solutions studied in this technical report are based on the existing IOPS solution in TS 23.180 [7] and new KI’s included in this report.

A summary of the solutions studied in this technical report are listed in table 7.2-1.

Table 7.2-1: Architecture evaluation – Functional model solutions in IOPS

| Solution | Applicable key issues (subclause reference) | Evaluation (subclause reference) | Dependency on other working groups |
| --- | --- | --- | --- |
| Solution 1: Clause 1 (Scope) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.1 | None |
| Solution 2: Clause 3.1 (Definitions) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.2 | None |
| Solution 3: Clause 4 (Introduction) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.3 | None |
| Solution 4: Clause 5 (Architecture requirement) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.4 | None |
| Solution 5: Clause 6 (Involved business relationships) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.5 | None |
| Solution 6: Clause 7 (Functional model) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.6 | None |
| Solution 7: Converting the functional model and deployment section of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.7 | None |
| Solution 8: Clause 10.2.2.3 (IOPS discovery request) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.8 | None |
| Solution 9: Converting the functional model and deployment section of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.9 | None |
| Solution 10: Clause 10.5.1.4 (IOPS emergency group call) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.10 | None |
| Solution 11: Clause 10.5.1.3 (IOPS group call setup) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.11 | None |
| Solution 12: Clause 10.5.3.1 (IOPS floor control – General) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.12 | None |
| Solution 13: Clause 10.5.3.3 (IOPS floor control during silence) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.13 | None |
| Solution 14: Clause 10.6.1.4 (IOPS group standalone SDS using signaling control plane) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.14 | None |
| Solution 15: Clause 10.7 (MC IOPS notification) of TS 23.180 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.15 | None |
| Solution 16: Clauses 10.2.3, 10.3.3 and 10.6.1.3.2 | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.16 | None |
| Solution 17: Clauses A.1, A.2 and A.6 (Involved business relationships) | Key issue specified in subclauses: 4.1. (Key issue 1) | 6.17 | None |

The solutions 1thru 17 are based on changes to the existing TS23.180 [7] to create a generic IOPS solution for backhaul failure.

# 8	Conclusions

This technical report partially fulfills the objectives of the study on mission critical services access aspects (MCSAA), including the following:

1)	Identification of key issues (clause 4) and architecture requirements (clause 5).

2)	Identification of architectural requirements.

3)	Application architecture solutions and individual solutions (clause 6) addressing the key issues (clause 4).

4)	Overall evaluation (clause 7) of the solutions.

The results from the study have identified the following observations:

-	For key issue #1, evaluate the existing IOPS work, specifically defined in 3GPP TS 23.180  [7] has been completed and the solutions are captured in Solution 1 - 17

-	For key issue #2 no solution was agreed.

The results from the solutions for KI#1 will be considered for the follow-up normative work in Release 19.

There are no known dependencies with other groups for the completion of this work.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-07 | #62 adhoc-e | S6a240202 |  |  |  | TR 23.700-09, v0.0.1 | 0.0.1 |
| 2024-07 | #62 adhoc-e | S6a240203 |  |  |  | pCR Scope section for TR | 0.1.0 |
| 2024-07 | #62 adhoc-e | S6a240203 |  |  |  | pCR Introduction section for TR | 0.1.0 |
| 2024-08 | #62 Maastricht | S6-243070 |  |  |  | pCR on new key issue to evaluate the existing IOPS work | 0.2.0 |
| 2024-08 | #62 Maastricht | S6-243071 |  |  |  | pCR new key issue on including NTN in IOPS scenarios | 0.2.0 |
| 2024-10 | #63 Hyderabad | S6-244428 |  |  |  | pCR on Making the IOPS Scope section access generic | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244429 |  |  |  | pCR on making the Definitions and Abbreviations sections access generic | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244430 |  |  |  | pCR on clause 4 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244431 |  |  |  | pCR on clause 5 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244099 |  |  |  | pCR on clause 6 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244432 |  |  |  | pCR on clause 7 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244433 |  |  |  | pCR on new solution of IOPS network deployment aspect to KI#1 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244434 |  |  |  | Pseudo-CR on describing the IOPS discovery request message access generic | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244435 |  |  |  | pCR on new solution of MBMS transmissions to KI#1 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244436 |  |  |  | pCR Generic IOPS section 10.5.1.4 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244437 |  |  |  | pCR Generic IOPS section 10.5.1.3 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244438 |  |  |  | pCR Generic IOPS section 10.5.3.1 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244439 |  |  |  | pCR Generic IOPS section 10.5.3.3 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244440 |  |  |  | pCR on Generic IOPS Solution on TS23.180 Section 10.6.1.4 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244441 |  |  |  | pCR on Generic IOPS Solution on TS23.180 Section 10.7 of 3GPP TS 23.180 | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244442 |  |  |  | pCR New open issues on including NTN in IOPS scenarios | 0.3.0 |
| 2024-10 | #63 Hyderabad | S6-244443 |  |  |  | pCR on Generic IOPS Evaluation and Conclusions | 0.3.0 |
| 2024-11 | #64 Orlando | S6-245051 |  |  |  | Pseudo-CR on clause 4 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245332 |  |  |  | Pseudo-CR on Generic IOPS Section 5 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245333 |  |  |  | Pseudo-CR on Generic IOPS – Cleaning up references in section 6 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245334 |  |  |  | Pseudo-CR on Generic IOPS – Correction to 6.6.2, 6.9.2 (Section 10.4 of TS 23.180) | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245210 |  |  |  | pCR on Solution#9 correction | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245050 |  |  |  | Pseudo-CR on clauses 10.2.3, 10.3.3 and 10.6.1.3.2 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245335 |  |  |  | Pseudo-CR on clauses A.1, A.2 and A.6 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245336 |  |  |  | Pseudo-CR on Generic IOPS – updated Conclusion for KI#2 | 0.4.0 |
| 2024-11 | #64 Orlando | S6-245369 |  |  |  | Pseudo-CR on Generic IOPS – updated Evaluation and Conclusion to include solutions 6.16 and 6.17 for KI#1 | 0.4.0 |
| 2024-12 | SA#106 | SP-241700 |  |  |  | Submitted to SA#106 for information and approval | 1.0.0 |