3GPP TSG-SA WG1 Meeting #108 	S1-244040

Orlando, Florida, USA, 18-22 November 2024

Source:	SK Telecom

pCR Title:	Use case on Network simplification using Evolved SBA

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	SeongJun Lee <seoul.lee@sk.com>

DongJin Lee <dongjin@sk.com>

Abstract: This document provides a new use case on Network simplification using Evolved SBA.

1. Introduction

This contribution proposes a new use case on Network simplification using Evolved SBA.

2. Reason for Change

TR 22.870 v0.0.0 does not yet contain any use case that covers Network simplication using Evolved SBA.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.0.0.

# *** First Changes***

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

5.x	Use case on Network simplification using Evolved SBA

5.x.1	Description

The current Core network architecture uses various types of interfaces and protocols, which may increase complexity with an increased number of Network Functions (NFs). A new network architecture should be simplified by using common interfaces, allowing flexible deployment and scalability in Cloud-native environments. Additionally, this new architecture should support interworking with earlier 3GPP systems through common interfaces to ensure efficient interoperability.

With the current Core network architecture, access requests to and from the UE must be routed exclusively through the AMF. This could potentially create a bottleneck and a single point of failure. Although the AMF is not directly involved in Session Management, it must still handle straightforward transfers to other NFs subsequently, leading to potential inefficiencies.

Adopting an enhanced, unified common interface like evolved SBA-based can significantly improve security, service independence, flexibility, and reliability, as well as handle large data volumes for AI-assisted processing.

5.x.2	Pre-conditions

5.x.3	Service Flows

5.x.4	Post-conditions

5.x.5	Existing features partly or fully covering the use case functionality

5.x.6	Potential New Requirements needed to support the use case

[PR.5.x.6-1] The 6G system shall support a network simplification using common interface.

Note 1: It is FFS how to define common interface, evolved SBA, AI-assisted data processing and how to distinguish it.

Note 2 : It is FFS to analyze the impact on existing system.

# *** End of Changes ***