3GPP TSG-SA WG1 Meeting #108 	S1-244048

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Digital twin for Industrial IoT

Agenda Item:	8.1.6

Source:	ZTE

Contact:	Chen Lin   chen.lin23@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a use case about digital twin for Industrial IoT, which create a digital twin for a factory’s entire production process. Based on the real-time monitoring and adjustment, potential equipment failures can be detected before they occur.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	Hamed Ahmadi, Avishek Nag, Zaheer Khan, Kamran Sayrafian and Susanto Rahardja, Network twins and twins of networks: an overview on the relationship between digital twins and 6G, IEEE Communications Standards magazine, December 2021.

* * * Second Change * * * *

10.x.1	 Description

Digital Twin(DT) tries to create a virtual representation of the physical components and functions of wireless networks. In the context of 6G, digital twins are expected to enable unprecedented levels of monitoring, analysis and optimization of industry IoTs. Meanwhile, the application of digital twin in industry requires the combination of high data rate (e.g. 100GB/s) with millisecond latency and ultra high reliability [x].

To be specific, many sensors and high resolution cameras are installed in the physical twin for network data collection purpose. The digital twin needs to collects the ultra-high-resolution videos, stores the data models and historical data of the physical twin, and provides visualization of the physical twin. In order to realize extremely detailed and accurate digital representation, the digital twin requires higher data rate and reliable data transmission. On the other hand, in order to reflect the physical counterparts instantaneously, the data transmission with low latency is required for real-time synchronization between physical twin and digital twin.

By virtually simulating the industry, the time and cost needed for physical testing and prototyping can be greatly reduced. Moreover, based on the network component monitoring, the 6G system can predict and prevent failures before they occur and thus improve the network performance.

In the following use case, a factory create a digital twin of its entire production process, allowing for real-time adjustments and prediction of equipment failures before they occur.

10.x.2	 Pre-conditions

A manufacturing plant has a critical assembly line. Many sensors and high resolution cameras are installed in the assembly line and can connect to a 6G network within the industrial park. The 6G system is operated by operator A, which created a digital twin for this assembly line.

- x.3	 Service Flows

- Within the physical twin of assembly line, sensors collect and report real-time data on various parameters like temperature, vibration, pressure, and performance metrics while the cameras report the ultra-high-resolution videos to digital twin.

- For the digital twin of assembly line, it integrates data from multiple sources, stores the operational data and historical maintenance record, and provide a detailed and accurate digital representation of the physical twin.

- The digital twin may analyze the collected data to identify patterns and anomalies that may indicate potential issues based on a predictive model.

- When an unusual pattern in the vibration data of a key component is detected, the digital twin system simulates the progression of this anomaly and predicts that it could lead to a failure within the next 72 hours. Digital twin automatically alerts the maintenance team. Meanwhile, digital twin may provide a detailed diagnostic report, identifying the specific part likely to fail.

- Based on the alerts and diagnostic report, the maintenance team use the digital twin to plan and simulate the repair procedure.

- After the maintenance is performed, the digital twin continues to monitor the component to ensure the issue is resolved.

10.x.4	 Post-conditions

Based on the digital twin of the assembly line, unexpected breakdown is prevented, which significantly reduces maintenance costs, increases equipment lifespan, and improve overall production efficiency.

10.x.5	Existing features partly or fully covering the use case functionality

In TS 22.261, clause 6.28.1 on requirements for cyber-physical control applications in vertical domains, clause D.2.0 on Factory automation include the following requirements:

Communication services supporting cyber-physical control applications need to be ultra-reliable, dependable with a high communication service availability, and often require low or (in some cases) very low end-to-end latency.

The related use cases require support of a large number of sensor devices per plant, as well as high communication service availability (99,99%). Furthermore, power consumption is relevant since some sensor devices are battery-powered with a targeted battery lifetime of several years (while providing measurement updates every few seconds). Range also becomes a critical factor due to the low transmit power levels of the sensors, the large size of the plant, and the high-reliability requirements on transport. End-to-end latency requirements typically range between 10 ms and 1 s. User-experienced data rates can be rather low since each transaction typically comprises less than 256 bytes. However, there has been a shift from field buses featuring somewhat modest data rates (~ 2 Mbit/s) to those with higher data rates (~ 10 Mbit/s) due to the increasing number of distributed applications and "data-hungry" applications. An example for the latter is the visual control of production processes. For this application, the user experienced data rate is typically around 10 Mbit/s and the transmitted packets are much larger than what was stated earlier.

In TS 22.104, clause 5.2 on periodic deterministic communication include the following requirements:

Process and asset monitoring using industrial wireless sensors is a special case of periodic deterministic communication with more relaxed requirements on timeliness and availability. These use cases put a slightly different set of requirements on the 5G system due to the specific constraints of industrial wireless sensors. These requirements for individual logical communication links are listed in Table 5.2-2 and additional information on the underlying use cases can be found in Annex A.

In TS 22.104, clause A.2.1 provides the following requirements for factories of the future:

5)	Monitoring and maintenance: Monitoring of certain processes and/or assets in the context of industrial production without an immediate impact on the processes themselves (in contrast to a typical closed-loop control system in factory automation, for example). This particularly includes applications such as condition monitoring and predictive maintenance based on sensor data, but also big data analytics for optimising future parameter sets of a certain process, for instance. For these use cases, the data acquisition process is typically not latency-critical.

10.x.6 Potential New Requirements needed to support the use case

[PR.10.x.6-1]	Subject to operator’s policy , the 6G system shall be able to support the UL data rate 100Gbps with latency lower than 1ms.

[PR.10.x.6-2]	Subject to operator’s policy , the 6G system shall be able to support the real-time data collection (e.g. environment sensing data, ultra-high-resolution videos, etc) from a selected number of UEs to authorized third party.