3GPP TSG SA WG 1 Meeting #108 	S1-244126

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	CATT, ISSDU, China Mobile

pCR Title:	Update on 5.3 Use case on Satellite Communication with Resilient Operation Mode for Public Safety

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to update existing identified requirements and capture additional potential requirements of use case 5.3 to TR22.887.

1. Introduction

The use case about satellite communication with resilient operation mode for public safety was first proposed in SA1 #106 meeting and then captured to TR22.887 v.0.1.0. With further study, the wording of PR1 & PR2 need to be modified to clarify the applicability, and more requirements relevant to this use case can be identified and suggest to be included.

2. Reason for Change

Resilient operation needs a definition to clarify the technical focus behind. Also, the following changes will be included: (1) modify the pre-condition and service flow to clarify the use case and align with PRs;

(2) update the wording of PR1 and PR2 to clarify the role of the operator;

(3) add new potential requirements identified from the use case to make the use case solid and workable.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v0.2.0

* * * First Change * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Resilient Operation: in the context of this study, it refers to a mechanism that the onboard satellite network maintains user services via proper operation without the ground core network involved when the backhaul link to the ground core network is temporarily unavailable.

* * * Next Change * * * *

5.3	Use case on Satellite Communication with Resilient Operation Mode for Public Safety

5.3.1	Description

This use case describes a scenario where the resilient satellite communication system for public safety supports resilient operation mode when either the backhaul link between the LEO satellite and the ground station is temporarily unavailable due to a natural disaster or the core newtork of LEO satellite was temporally unavailable due to cyberwarfare attack.

In order to preserve service continuality, the resilient operation could eliminate service interruption period before re-establish the backhaul link or restore the core network service.

Figure 5.2.1-1: Scenario for Satellite Communication with Resilient Operation Mode for Public Safety

5.3.2	Pre-conditions

The Coast Guard has deployed a smart boat named Virginia for river patrol. Officers Jason and Alice can navigate the boat while reporting their status periodically to the Coast Guard Administration Centre.

The LEO satellite can connect to GEO/MEO on-board 5G core network via ISLs and the ground 5G core network via the feeder link.

The LEO/MEO/GEO satellite system is authorized to use resilient operation mode when detect backhaul link got lost.

The officers’ smartphones and the smart boat have subscription for the resilient operation.

5.3.3	Service Flows

1. Connectivity to land 5GC:

- Both officers’ smartphones and their smart boat are connected to the 5G network via a LEO satellite link.

- They periodically report their current status to the Coast Guard Administration Centre.

2. Interruption of backhaul link:

- An EF5 tornado develops and moves toward the satellite ground station.

- The backhaul link between the LEO satellite and the ground station becomes temporarily unavailable because the ground station's antennas are damaged by the EF5 tornado.

3. Switch to resilient operation mode:

- The resilient satellite communication system automatically switches to resilient operation mode after the LEO satellite loses the backhaul link connection.

- Officer Jason and Alice can still report their status to the Coast Guard Administration Centre, and communicate with each other via UE-Satellite-UE under the resilient operation mode.

4. Re-establish the connectivity to land 5GC:

- The resilient satellite communication system continually searches for an available communication path and reconnects to the ground station via multi-hop inter-satellite links over LEO/MEO/GEO satellites.

5.3.4	Post-conditions

During the river patrol,

Officers Jason and Alice can report status to the Coast Guard Administration Centre as expected.

5.3.5	Existing features partly or fully covering the use case functionality

Currently, 5G network lack mechanisms for Resilient Satellite Communication with Resilient Operation Mode.

5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-001] Subject to operator’s policy, the 5G network shall provide means to support resilient operation over different orbit satellites when the backhaul connection to the ground core network is unavailable in order to preserve service continuity.

[PR 5.3.6-002] Subject to operator’s policy, the 5G network shall be able to provide alternative backhaul connection with the same service level agreement (SLA) via different orbit satellites to resume the communication between the satellite to the ground station when the network is in resilient operation.

[PR 5.3.6-003] Subject to operator’s policy, the 5G network shall provide means to support authorized UEs to obtain services when the network is in resilient operation.

[PR 5.3.6-004] Subject to regulatory requirements and operator’s policy, the 5G network shall support UE-Satellite-UE communication when the network is in resilient operation.

[PR 5.3.6-005] The 5G system with satellite access shall be able to collect charging information for traffic exchanged via resilient operation over different orbit satellites.

* * * End of Changes * * * *