3GPP TSG-SA1 Meeting # 108	S1-244212

# Orlando, Florida, USA, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0816 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements on multiple access technologies supporting different IMS services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MA_IMS |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The types of IMS services nowadays are becoming increasingly diverse, with bunches of data services via IMS Data Channel in addition to audio and video, which may have differentiated requirements on QoS and data transmission. When network conditions change, some user traffic related to dedicated IMS services can be offloaded from 3GPP access to non-3GPP access path, while other types of IMS services remain on the 3GPP access path. For instance, traffic of video and IMS data channel services can be offloaded to the Wi-Fi path, leaving audio traffic on 3GPP access network. Another similar case is also applicable to IMS data channel services (e.g. offloading IMS DC file transfer service to the Wi-Fi path with keeping the IMS DC real-time translation service on the 3GPP access path). The existing requirement on multiple access technologies supporting traffic offloading does not explicitly cover different types of IMS services and/or different types of services over IMS Data Channel, e.g., audio, video, text and IMS DC file transfer, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the requirement for supporting traffic offloading among different access technologies based on different types of IMS services and/or different types of services over IMS Data Channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambigous whether multiple access technologies based on the types of IMS services and different types of services over IMS Data Channel are supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

6.3.2.1	General

Based on operator policy, the 5G system shall enable the UE to select, manage, and efficiently provision services over the 3GPP or non-3GPP access.

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s).

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

Based on operator policy, the 5G system shall be able to support dynamically offload the traffic of different types of IMS services and/or different types of services over IMS Data Channel (e.g. from 3GPP RAT to non-3GPP access technology).

Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies (e.g. NR, E-UTRA, non-3GPP), to access one or more 3GPP services.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over the access technologies in use, taking into account e.g. service, traffic characteristics, radio characteristics, and UE's moving speed.

The 5G system shall be able to support data transmissions optimized for different access technologies (e.g. 3GPP, non-3GPP) and accesses to local data networks (e.g. local traffic routing) for UEs that are simultaneously connected to the network via different accesses.

NOTE:	This applies to the scenario with simultaneous 3GPP and non-3GPP accesses.

Based on operator policy, the 5G system shall be able to add or drop the various access connections for a UE during a session.

The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network).

The 5G system shall support UEs with multiple radio and single radio capabilities.

The 5G system shall support dynamic and static network address allocation of a common network address to the UE over all supported access types.

The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

The 5G system shall support the capability to operate in licensed and/or unlicensed bands.

End of Change