3GPP TSG-SA WG1 Meeting #108 	S1-244218

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use case on active WPT for IoT in unmanned factory

Agenda Item:	8.1.5

Source:	ZTE Corporation

Contact: 		xu.ling@zte.com.cn,  si.yuan@zte.com.cn, dai.bo@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a new use case for TR22.870, which is about 6G active wireless power transfer (A-WPT) for IoT devices in unmanned factory.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- N. Ayir, T. Riihonen, M. Allén and M. F. Trujillo Fierro, "Waveforms and End-to-End Efficiency in RF Wireless Power Transfer Using Digital Radio Transmitter," in IEEE Transactions on Microwave Theory and Techniques, vol. 69, no. 3, pp. 1917-1931, March 2021, doi: 10.1109/TMTT.2020.3047654s

* * * Next Change * * * *

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3GPP Wireless power transfer (WPT) service: 6G networks transmit energy through wireless signals based on UE request, and UE obtains energy from the wireless signals sent by the 6G network.

Active wireless power transfer (A-WPT): 3GPP IoT device or UE actively request energy, then 3GPP network delivery wireless power through wireless signals to response this request.

* * * Next Change * * * *

Followings are all new texts

9	Massive Communication

9.x	Use case on Active WPT for IoT device in unmanned factory

9.x.1	Description

In factory, getting stable power supply is important for IoT devices, and a brief offline time may lead to serious consequences, e.g., IoT sensors for detecting temperature, toxic gases, pressure, etc. If the sensors cannot report environmental information in a timely manner, it may result in major production accidents and significant economic losses. Currently, IoT terminals are mainly powered by batteries, but due to limited battery capacity, it is not possible to ensure that IoT terminals are in normal working condition for a long time. Replacing batteries brings significant maintenance costs and environmental pollution.

Ambient energy harvesting is considered to address the issue, e.g., solar-powered, wind-powered and powered by P-WPT (Passive wireless power transfer, UE passively harvests energy based on existing wireless signals in the environment and does not interact with the network.).  But, powered by ambient energy harvesting requires the IoT device located near an energy source, such as heat, wind and base station. In some cases, there may be no energy source in the environment and IoT devices can not harvest energy (e.g., solar-powered devices can not work at night). In other cases, IoT devices may not be able to harvest enough energy from the environment. For example, when using P-WPT,  IoT devices passively receive wireless signals (which may be  a communication signal sent to other users) for charging. The power of passively received signals (e.g., IoT devices and communication target UEs are located in different beam directions) is often low and may not meet the charging power required by IoT devices. In a word, ambient energy harvesting is dependent on the external environment which can not provide a steady supply of energy.

In order to deal with the situation described above, more active power delivery for low-power IoT devices is being developed which is using radio frequency wireless power transfer (RF-WPT) technology [2]. The charging station transmit RF energy signals to low-power devices, which receive them using a rectifying antenna and recharge the battery. When it comes to 3GPP, base stations can actively transmit energy signals to the low-power devices to response its request, i.e. A-WPT. The 6G network can provide 3GPP WPT service (e.g., Inform base station/charging station to transmit energy signals) after receiving the charging request from the low-power IoT device. The IoT device can also exchange related charging status information to network to improve charging efficiency (e.g. report received signal strength indicator).

With the provided A-WPT service, factory IoT devices can request and obtain a stable supply of energy and can keep working continuously for a long time, which can reduce factory maintenance costs and save labor expenses. For 3GPP operators, providing A-WPT services can make full use of spectrum resources and improve network utilization. In addition, A-WPT can reduce battery consumption and minimize environmental pollution caused by waste batteries, improve the flexibility of energy use, and thus contribute to sustainable social development and green low-carbon initiatives.

Following give an introduction of basic A-WPT service flow for low power IoT device in factory.

9.x.2	Pre-Conditions

Factory A is mainly engaged in the production of dangerous goods, and has subscribed 6G-based IoT communication service and 3GPP WPT service to ensure production safety.

IoT sensors (e.g., temperature sensor) in Factory A, monitor the production environment and report the real-time status of the production environment to the environmental monitoring platform through a 6G network.

9.x.3	Service Flows

- 6G network broadcast system message and indicates that 3GPP WPT service are available.

- IoT sensor A sends the identification information to the 6G network and completes the authentication procedure.

- IoT sensor A measures the environmental parameters (e.g. temperature, pressure and humidity) and reports them periodically to the 6G network.

- IoT sensor A send charge request to 6G network when its battery level is below working threshold. The charge request may include message identifiers, Charge duration, etc. Assuming that the charging time is quantified in milliseconds, seconds, minutes, and hours, it would require approximately 31 bits. The number of message identifier bits is approximately 8 bits or more. The reliability and latency requirements for charging requests are similar to the requirements specified in TS 22.261 Table 7.5.2-1[X].

- After receiving the charge request and certification, 6G network select one base station B to provide charging service for IoT sensor A. Then, base station B  begins to send power signals to the sensor A.

- IoT sensor A send charge termination request to 6G network when power is full.

- 6G network notifies base station B  to stop transmitting power, and base station B  stops sending power signals to IoT sensor A.

- IoT sensor A remains normal work during wireless charging and continues to monitor environmental data.

9.x.4	Post-Conditions

By using A-WPT, sensors can work stably for a long time without battery replacement.

9.x.5	Existing features partly or fully covering the use case functionality

In TS22.261, Table 7.5.2-1, Performance requirements for highly reliable machine type communication can be reused.

9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6 -1] Subject to operator’s policy, the 6G network shall be able to broadcast that 3GPP WPT service is available.

[PR 9.x.6 -2] Subject to operator’s policy, the 6G system shall be able to support to start, and stop 3GPP WPT service.

[PR 9.x.6 -3] The 6G network shall be able to transmit RF power signals to provide 3GPP WPT service.

[PR 9.x.6 -4] The 6G IoT devices shall be able to request 3GPP WPT service from 6G network with 3GPP WPT service.

[PR 9.x.6 -4] The 6G IoT devices shall be able to receive RF power signals for energy harvesting.

[PR 9.x.6 -5] Subject to operator’s policy, the 6G network shall be able to authenticate 6G IoT device who request 3GPP WPT services.

[PR 9.x.6 -6] Subject to operator’s policy, the 6G system shall be able to provide 3GPP WPT service for 6G IoT device  with performance specified in Table 9.x.6-1.

Table 9.x.6-1: KPIs for 3GPP WPT for IoT device

| Scenario | Charging Duration | Charging Service Availability | Charging service Reliability | Charging Range | IoT Devices speed |
| --- | --- | --- | --- | --- | --- |
| WPT for IoT devices in factory | TBD | TBD | TBD | ≤100m indoor | Stationary |
| Note: Charging Duration: The amount of time an IoT device takes from the time it initiates a charge request to the time it completes the charge Charging Service Availability: The proportion of successfully completed charging service times to the total number of services. Charging Service Reliability:The proportion of the time for receiving signal power to reach the charging threshold to the charging duration |  |  |  |  |  |