3GPP TSG-WG SA1 Meeting #108	S1-244219

Orlando, US, 18th Nov – 22nd Nov, 2024

Source:	Huawei

pCR Title:	Discussion on consolidation of functional requirements for TR22.887

Draft Spec:	3GPP TR 22.887

Agenda item:	7.3

Document for:	Discussion

Contact:	Fangyuan Zhu<zhufangyuan@huawei.com>

Abstract: This document discusses the use cases and the associated potential requirements captured in TR 22.887 so far, and proposes the way forward for consolidation.

1. Introduction

The phase 4 study on satellite access has made good progress with 20 use cases and associated potential requirements captured in the TR. While new use cases and potential requirements may be proposed, this document discusses and proposes the way forward for initial consolidation.

2. Discussion

The captured use cases can be summarised into 4 category as list in the table below.

| Num | Category | Use cases |
| --- | --- | --- |
| 1 | Multi-orbits satellite access for multiple services | 5.4 Use case on service continuity through multi-orbit satellite access 5.5 Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks 5.6 Use case on switching between multi-orbits satellite networks in remote area applications 5.7 Use case on assisting vehicular communications via multi-orbits satellite access 5.8 Use case on traffic over different orbit satellites 5.12 Use case on supporting different services with multi-orbit satellites 5.13 Use case on wide-area disaster control support using multi-orbit satellite networks 5.14 Use case on broadband services through multi-orbit satellite access 5.15 Use case on multi-orbit access for cross-hybrid service 5.16 Use case on using GEO for initial network entry and service initialization 5.17 Use case on resource and energy efficiency in a multi-orbit Satellite Access System 5.18 Use case on using GEO to land data in a desirable geographical location 5.19 Use case on using GEO for sleep/power management 5.20 Use case on Multi-orbit satellite backhauling |
| 2 | Resilient notification service | 5.2 Use case on Resilient Notification |
| 3 | Emergency communications and mission critical services using satellite access | 5.3 Use case on Satellite Communication with Resilient Operation Mode for Public Safety 5.9 Use case on Public Warning Service 5.10 Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay 5.11 Use case on emergency communication using satellite access |
| 4 | IMS voice calls using GEO satellite access | 5.1 Use case of IMS Voice Call Using GEO Access |

The table below lists the use cases and the associated potential requirements in each category.

| Category: Multi-orbits satellite access for multiple services |  |
| --- | --- |
| Use cases | Potential requirements |
| 5.4 Use case on service continuity through multi-orbit satellite access | [PR 5.4.6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbits (due to the movement of the UE and/or the satellites). NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits). [PR 5.4.6-002] A 5G network with satellite access shall be able to support charging mechanisms for communication services using satellite access in different orbits based on the type of satellites (e.g., LEO, MEO, GEO). |
| 5.5 Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks | [PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbits satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g., availability of the coverage, required QoS. |
| 5.6 Use case on switching between multi-orbits satellite networks in remote area applications | [PR 5.6.6-001] A 5G system with satellite access shall support minimum service interruption when a relay UE is switching between multiple orbit satellite connections (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] [PR 5.6.6-002] The 5G system shall allow the network operator to configure policies to enable switching relay UE between multi orbits satellites (e.g., policies considering QoS, availability of the coverage if discontinuous). Editor’s note: This requirement is FFS. |
| 5.7 Use case on assisting vehicular communications via multi-orbits satellite access | [PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with multi-orbits satellite access shall support mechanisms to allow a UE connecting one of satellite access networks in different orbits considering e.g., QoS, satellite availability. [PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbits satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN. [PR 5.7.6-003] A 5G network with multi-orbits satellite access shall be able to collect charging information for a UE served by different orbit types of satellite access networks. |
| 5.12 Use case on supporting different services with multi-orbit satellites | [PR 5.12.6-001] Based on regulatory requirements and operators’ policy, the 5G system shall support Service Hosting Environment (e.g., edge application server) on multi-orbit satellites. [PR 5.12.6-002] Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide most suitable service hosting environment via multi-orbit satellites. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits. |
| 5.13 Use case on wide-area disaster control support using multi-orbit satellite networks | [PR 5.13.6-001] Subject to regulatory requirements and operator policies, the 5G network with multi-orbit satellite network shall support switching between satellite access depending on the latency requirements. NOTE: The switching between satellite access of a multi-orbit satellite network can be triggered by latency requirements. |
| 5.14 Use case on broadband services through multi-orbit satellite access | [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission. |
| 5.15 Use case on multi-orbit access for cross-hybrid service | [PR 5.15.6-001] A 5G system with multi-orbit satellite access shall be capable of selecting communication paths across different satellite access (GEO, MEO, LEO) as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service. |
| 5.16 Use case on using GEO for initial network entry and service initialization | [PR 5.16.6-001] A 5G system with multi orbit satellite access shall be able to support network entry initiation via one satellite access type and network entry completion via a different satellite access type. Editor’s note: this requirement is FFS [PR 5.16.6-002] A 5G System supporting multi orbit satellite access shall enable the UE to select the relevant GSO for network entry initiation. Editor’s note: this requirement is FFS |
| 5.17 Use case on resource and energy efficiency in a multi-orbit Satellite Access System | [PR 5.17.6-001] A 5G network supporting satellite access with multi-orbit operation shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. [PR 5.17.6-002] A 5G system supporting satellite access with multi-orbit operation shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand. |
| 5.18 Use case on using GEO to land data in a desirable geographical location | [PR 5.18.6-001] A 5G system with multi orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope |
| 5.19 Use case on using GEO for sleep/power management | [PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite system. Editor’s note: this requirement is FFS |
| 5.20 Use case on Multi-orbit satellite backhauling | [PR 5.20.6-001] The 5G system using multi orbit satellite network for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS. [PR 5.20.6-002] The 5G system shall be able to collect and distinguish charging information related to backhaul traffic over different orbits of the satellite network. |
| Category: Resilient notification service |  |
| 5.2 Use case on Resilient Notification | [PR 5.2.6-001] Subject to operator’s policies, a 5G system with satellite access shall be able to support Resilient Notification service to notify users, with valid subscription to the notification service, about a missed mobile terminated service when the user is unreachable via satellite access. NOTE: The Resilient Notification service can provide the user with service information e.g., caller’s information and service type. |
| Category: Emergency communications and mission critical services using satellite access |  |
| 5.3 Use case on Satellite Communication with Resilient Operation Mode for Public Safety | [PR 5.3.6-001] The 5G Network shall provide means to support resilient operation over different orbit satellites when backhaul-link is unavailable in order to preserve service continuity. [PR 5.3.6-002] The 5G Network shall provide alternative backhaul-link communication with the same service level agreement (SLA) via multi-orbits satellite access for resilient satellite communication. |
| 5.9 Use case on Public Warning Service | [PR 5.9.6-001] If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via Non-Terrestrial Networks. |
| 5.10 Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay | [PR 5.10.6-001] A 5G system with satellite access, shall support delivery of a cached content from a content caching application via a broadcast/multicast service for mission critical services. [PR 5.10.6-002] A 5G system with satellite access shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE for mission critical services. [PR 5.10.6-003] A 5G network with satellite access shall be able to support the communication with Mobile Base Station Relays having a backhaul link via satellite for optimizing multicast services. Editor's Note: the KPIs for this use case are FFS. |
| 5.11 Use case on emergency communication using satellite access | [PR 5.11.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system should be able to support means to provide smaller granularity location information (compared with original cell information in serving satellite) of emergency communication user to the trusted 3rd party when the user triggers the emergency communication via satellite access. |
| Category: IMS voice calls using GEO satellite access |  |
| 5.1 Use case of IMS Voice Call Using GEO Access | [PR 5.1.6-001] The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3]. [PR 5.1.6-002] The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) considering the transmission data rate, latency and packet size. [PR 5.1.6-003] The 5G system with GEO satellite access shall be able to support a codec suitable for the transfer of voice considering the transmission data rate and latency provided by the 5G system with GEO satellite access. |

3. Proposal

Following this 2-step approach, please consider the consolidation of the potential requirements without Editor’s Note in each category summarised in the tables below. This is intended as the base for the consolidation discussion, and no much merging or revising of the original PRs is done.

Table 1 – Consolidated Requirements for Multi-orbits satellite access for multiple services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.1-1 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbits (due to the movement of the UE and/or the satellites). NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits) | [PR 5.4.6-001] | service continuity |
| CPR 1.1-2 | A 5G network with satellite access shall be able to support charging mechanisms for communication services using satellite access in different orbits based on the type of satellites (e.g., LEO, MEO, GEO) A 5G network with multi-orbits satellite access shall be able to collect charging information for a UE served by different orbit types of satellite access networks. | [PR 5.4.6-002] [PR 5.7.6-003] | Merge of these two PRs, applying to charging aspects. It may need to be moved to a separate clause |
| CPR 1.1-3 | Subject to operator’s policy, the 5G network with multi-orbits satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g., availability of the coverage, required QoS | [PR 5.5.6-001] | Multi-orbits satellite access, applying to mobile base station relay |
| CPR 1.1-4 | A 5G system with satellite access shall support minimum service interruption when a relay UE is switching between multiple orbit satellite connections (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] | [PR 5.6.6-001] | Relay UE |
| CPR 1.1-5 | Subject to regulatory requirement and operator’s policy, a 5G network with multi-orbits satellite access shall support mechanisms to allow a UE connecting one of satellite access networks in different orbits considering e.g., QoS, satellite availability A 5G system with multi-orbit satellite access shall be capable of selecting communication paths across different satellite access (GEO, MEO, LEO) as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service | [PR 5.7.6-001] [PR 5.15.6-001] | Merge of these two PRs, applying to satellite access selection |
| CPR 1.1-6 | Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbits satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN | [PR 5.7.6-002] | service interruption minimization |
| CPR 1.1-7 | Based on regulatory requirements and operators’ policy, the 5G system shall support Service Hosting Environment (e.g., edge application server) on multi-orbit satellites | [PR 5.12.6-001] | Service Hosting Environment |
| CPR 1.1-8 | Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide most suitable service hosting environment via multi-orbit satellites. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits | [PR 5.12.6-002] | Service Hosting Environment |
| CPR 1.1-9 | Subject to regulatory requirements and operator policies, the 5G network with multi-orbit satellite network shall support switching between satellite access depending on the latency requirements. NOTE: The switching between satellite access of a multi-orbit satellite network can be triggered by latency requirements. | [PR 5.13.6-001] | switching between satellite access |
| CPR 1.1-10 | Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission | [PR 5.14.6-001] | data transmission |
| CPR 1.1-11 | A 5G network supporting satellite access with multi-orbit operation shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand | [PR 5.17.6-001] | energy efficiency |
| CPR 1.1-12 | A 5G system supporting satellite access with multi-orbit operation shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand | [PR 5.17.6-002] | satellite access type selection |
| CPR 1.1-13 | A 5G system with multi orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope | [PR 5.18.6-001] | preferred satellite ground station location |
| CPR 1.1-14 | The 5G system using multi orbit satellite network for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS | [PR 5.20.6-001] | Wording improvement are needed |
| CPR 1.1-15 | The 5G system shall be able to collect and distinguish charging information related to backhaul traffic over different orbits of the satellite network | [PR 5.20.6-002] | Wording improvement are needed |
|  | The 5G system shall allow the network operator to configure policies to enable switching relay UE between multi orbits satellites (e.g., policies considering QoS, availability of the coverage if discontinuous). Editor’s note: This requirement is FFS. | [PR 5.6.6-002] | TBC |
|  | A 5G system with multi orbit satellite access shall be able to support network entry initiation via one satellite access type and network entry completion via a different satellite access type. Editor’s note: this requirement is FFS | [PR 5.16.6-001] | TBC |
|  | A 5G System supporting multi orbit satellite access shall enable the UE to select the relevant GSO for network entry initiation. Editor’s note: this requirement is FFS | [PR 5.16.6-002] | TBC |
|  | Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite system. Editor’s note: this requirement is FFS | [PR 5.19.6-001] | TBC |

Table 2 – Consolidated Requirements for Resilient notification service

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.2-1 | Subject to operator’s policies, a 5G system with satellite access shall be able to support Resilient Notification service to notify users, with valid subscription to the notification service, about a missed mobile terminated service when the user is unreachable via satellite access. NOTE: The Resilient Notification service can provide the user with service information e.g., caller’s information and service type | [PR 5.2.6-001] |  |

Table 3 – Consolidated Requirements for Emergency communications and mission critical services using satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.3-1 | The 5G Network shall provide means to support resilient operation over different orbit satellites when backhaul-link is unavailable in order to preserve service continuity | [PR 5.3.6-001] |  |
| CPR 1.3-2 | The 5G Network shall provide alternative backhaul-link communication with the same service level agreement (SLA) via multi-orbits satellite access for resilient satellite communication | [PR 5.3.6-002] |  |
| CPR 1.3-3 | If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via Non-Terrestrial Networks | [PR 5.9.6-001] |  |
| CPR 1.3-4 | A 5G system with satellite access, shall support delivery of a cached content from a content caching application via a broadcast/multicast service for mission critical services | [PR 5.10.6-001] |  |
| CPR 1.3-5 | A 5G system with satellite access shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE for mission critical services | [PR 5.10.6-002] |  |
| CPR 1.3-6 | Subject to the regulatory requirements and operator’s policy, the 5G system should be able to support means to provide smaller granularity location information (compared with original cell information in serving satellite) of emergency communication user to the trusted 3rd party when the user triggers the emergency communication via satellite access | [PR 5.11.6-001] |  |
|  | A 5G network with satellite access shall be able to support the communication with Mobile Base Station Relays having a backhaul link via satellite for optimizing multicast services. Editor's Note: the KPIs for this use case are FFS | [PR 5.10.6-003] | TBC |

Table 4 – Consolidated Requirements for IMS voice calls using GEO satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.4-1 | The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3]. | [PR 5.1.6-001] |  |
| CPR 1.4-2 | The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) considering the transmission data rate, latency and packet size. | [PR 5.1.6-002] |  |
| CPR 1.4-3 | The 5G system with GEO satellite access shall be able to support a codec suitable for the transfer of voice considering the transmission data rate and latency provided by the 5G system with GEO satellite access. | [PR 5.1.6-003] |  |