3GPP TSG-SA1 Meeting #108	S1-244256

18-22 Nov 2024, Orlando, US	(revision of xx-yyxxxx)

Source:	China Telecom, ZTE

pCR Title:	Use case on improved network resilience in a distributed network

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.1

Document for:	Approval

Contact:	Yinglin Chen, chenyl37@chinatelecom.cn; Lijuan Chen, chen.lijuan@zte.com.cn

Abstract: This use case introduces a failure scenario where a distributed network experiences core network failure while its connection to the public network is functional.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change (All new text)* * * *

5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.x	Improved network resilience in a distributed network

### 5.x.1	Description

Distributed network is a network customized with the minimum set of services and capabilities required by market scenarios. It can be public or non-public. The public network operator operating one public network providing basic service (e.g. connectivity) and multiple customized distributed networks providing differentiated service holds a network of distributed networks.

Within the network of distributed networks, when a distributed network experiences network failure, other distributed networks or the public network within the network of distributed networks can collaborate to provide service to the affected UEs.

This use case introduces a failure scenario where a distributed network experiences core network failure while its connection to the public network is functional.

### 5.x.2	Pre-conditions

PLMN A and Distributed network B are both operated by Mobile Operator A, and the two networks have shared-RAN.

PLMN A is a public network.

Distributed network B is a non-public network providing service to a university, and it has locally deployed user plane in the campus. Based on business agreement, the subscription data of Distributed network B’s subscribers is stored and managed in PLMN A. Distributed network B retrieves subscription data from PLMN A when a subscriber registers on it and stores the data locally for a period of time. Distributed network B can also update and manage the subscription data. The two copies of subscription data in Distributed network B and PLMN A are synchronised. This mechanism ensures that subscription is owned by Operator A and can be managed by the university.  Except for the retrieval of subscription data, Distributed network B can operate independently from PLMN A.

During abnormal conditions, PLMN A is utilized by Operator A to minimize the service interruption in Distributed network B.

UE C, a student’s phone, has a subscription to Operator A. The subscription data of UE C is available in PLMN A. It is authorized to access Distributed network B. Therefore, its subscription data is also available in Distributed network B.

### 5.x.3	Service Flows

- UE C initiates registration to Distributed network B when in campus.

- UE C is authorised to access Distributed network B with the subscription data retrieved from PLMN A.

- UE C starts to download a tutorial video from the local server.

- The control plane of Distributed network B fails due to a software bug, while the locally deployed user plane of Distributed network B remains functional.

- PLMN A determines that Distributed network B is unable to provide service. Based on PLMN A’s instruction or local configuration, the shared-RAN routes the signalling traffic of UE C to the core network of PLMN A instead of the failed Distributed network B. Since the locally deployed user plane of Distributed network B remains functional, the user plane path remains unchanged (i.e. UE—shared RAN—user plane of Distributed network B—local server).

Figure X.1. The control plane traffic of Distributed network B is routed to PLMN A due to the failure of Distributed network B’s control plane, and the user plane traffic of Distributed network B remains unchanged and routed locally.

### 5.x.4	Post-conditions

UE C is unconscious of the failure of Distributed network B. The video is successfully downloaded.

### 5.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Based on operator policy, the 6G network shall enable a network operator to enforce control for data transmission in a distributed network utilizing the control plane resource in another network (PLMN or distributed network). The two networks are operated by the same network operator.

NOTE 1:	The above requirement applies to the scenario that the control plane resource of the distributed network is not available, while the user plane resource of the distributed network is available.

[PR 5.x.6-2] The 6G network shall minimize service interruption when the serving network is changed to a different network operated by the same network operator due to the failure of the previous serving network.

[PR 5.x.6-3] The 6G network shall support the synchronisation of subscription data for distributed network’s subscribers between the distributed network and another network (PLMN or distributed network). The two networks are operated by the same network operator.

[PR 5.x.6-4] For a UE authorized to access a distributed network, the 6G network shall provide a suitable API by which an authorized third-party shall be able to manage the UE subscription.

* * *End of Change * * * *