3GPP TSG SA WG 1 Meeting #108 	S1-244514

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244131)

Source:	CATT

pCR Title:	Use Case on Efficient Data Exchange for Cross-domain AI/ML Inference

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution introduces one use case about the AI/ML relevant data exchange within 6G system for cross-domain AI/ML inference.

1. Introduction

6G system is expected to support built-in intelligence in the initial architecture to address the applications with ML training/inference’s need as well as improve the network operation in a more intelligent manner, which could be supported in form of dedicated entity (e.g. NDWAF) or distributed AI embedded in several entities. This trend will bring out a challenge for the 6G system to manage and execute the in-network data exchange among various AI/ML endpoints efficiently. This contribution proposes a use case about the data relevant to AI/ML inference is exchanged efficiently within 6G system for cross-domain AI/ML inference.

2. Reason for Change

Introduce a use case about efficient data exchange within 6G system for cross-domain AI/ML inference.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v0.0.1

* * * First Change * * * *

W	Other Use Cases

W. x	Use case on efficient data exchange for cross-domain AI/ML inference

W.x.1	Description

Different from traditional network operation, AI is data-driven and will introduce vast amount of data exchanged among various types of AI/ML endpoints (e.g. data provider, network entities with AI built-in, data consumer). A typical case, AI/ML model training and inference will include several operations that require data exchange between AI/ML endpoints, such as collecting AI datasets for training, transmission of AI/ML model and intermediate data, transmission of training data and inference result, etc. Such operations raise big challenge to the mobile communication system in transmission cost and complexity.

In addition, some inference requirements of the application cannot be fulfilled with single model or one-time inference due to the data security across the AI/ML endpoints, or the complexity of the request itself, so that there is additional data exchange across the AI/ML endpoints belonging to different inference tasks. For example, the trajectory planning for the vehicle like autonomous driving car, UAV need to consider the regulatory framework, efficiency and security of the road or flight zone. However when the vehicle moves with the need of continuous communication service, the situation becomes complicated in that the communication quality along the planned path will be prioritized to be an important factor to evaluate overall user experience. The inference result based on the AI/ML model used for the transportation management can only be treated as a local optimal result unless verified by the inference result based on network AI/ML model. Thus, the overall optimal inference result may request the collaboration of multiple AI/ML models, and multiple inference tasks.as the figure shows.

# This use case describes the cross-domain AI/ML inference applied in tragectory planning with the collabration of cloud AI/ML model and network AI/ML model for different inference tasks . One-time trajectory planning will include the data transmission between UE/UAV and the application server (e.g. UTM) as line ①) and associated data transmission for service intermediate data e.g. AI input data, and cloud AI inference result as line ② and ③.  Compared with local AI inference on the vehicle, multi-tasks AI/ML inference will have additional data transmission to collect AI input data,  exchange intermediate data and feedback to the task host, which will introduce more transmission latency and processing delay, so it’s challenging for 6G system to provide efficient data transmission with required end-to-end QoS of AI based application (e.g. UAV, autonomous ship) .

Figure W.x.1-1: Efficient Data Exchange for Cross-domain AI/ML Inference

W.x.2	Pre-conditions

The network entities of 6G core network have be deployed network related AI/ML models, e.g. NW Entity#1.

The application server is deployed transportation related AI/ML models, and is mutual trusted with 6G network.

Various UEs are the subscribers of 6G network and authorized to obtain AI service from the network. UEs are also capable of 3GPP sensing and non-3GPP sensing.

W.x.3	Service Flows

- UE sends a request to the Application Server asking for trajectory planning for UAV to the target locations, and work with the UAV to execute reliable video surveillance along the path via communication path ①.

- The application server analyzes the request and identifies two consecutive AI/ML inference tasks. The inference will be executed firstly in the application server locally based on the information and data provided by UE. Then the inference result will be fed to the network AI/ML model via transmission path ② for the following inference

- During the second inference, UAV and/or base stations will provide the information about the signal quality around UAV, detected objects in planned route, network connection status, etc to NW Entity#1 via transmission paths ③, for the inference.

- The NW Entity#1 will transmit the inference result to the Application Server via transmission path ② or control plane path (e.g. NEF)

- UAV gets the planning route from Application Server via communication path ①. And the Application Server responds UE about the task result.

W.x.4	Post-conditions

UAV can fly along the planning route as well as upload the video smoothly.

W.x.5	Existing features partly or fully covering the use case functionality

Referring to TS22.261, efficient user plane is requested with the following requirements.

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path between UEs attached to the same network, modifying the path as needed when the UE moves during an active communication.

Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a Service Hosting Environment for specific services, modifying the path as needed when the UE moves during an active communication.

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network; or

-	an application server located in a customer premises network or personal IoT network.

However, the data transmission mentioned here will have UE or an application in a Service Hosting Network as one end while the data is service data consuming by UE or the application. But AI data exchange breaks such paradigm in that sometimes the two ends will not have UE or the application involved, and the data is user-irrelevant data.

W.x.6	Potential New Requirements needed to support the use case

[PR-W.x.6-1] Subject to application needs and user consent, the 6G system shall support efficient data transmission for AI relevant data between the entities(e.g. UE, RAN, CN, trusted 3rd party) considering the requested QoS.

[PR-W.x.6-2] Subject to operator’s policy, the 6G network shall support mechanisms to route AI relevant data for a specific service considering  the characteristics of data (e.g. volume, type, security level), and modify the path as needed when AI/ML endpoints changes.