3GPP TSG SA WG 1 Meeting #108 	S1-244564

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244099)

Source:	Novamint, Viasat, Inmarsat, Nokia, Eutelsat Group

pCR Title:	Pseudo-CR on terms for TR22887

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes updates and new definitions in relation to multi orbit access in TR.22887

1. Introduction

The terms related to multi orbit access needs to be updated to remove FFS and clarify some definitions.

2. Reason for Change

To provide definitions for satellite access, for multi orbit satellite access and for network entry as well as to update the related existing definitions which are FFS.

In addition, the clause 3.2 which is not used in the TR is proposed to be removed.

| 5.4 Use case on service continuity through multi-orbit satellite access | [PR 5.4.6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbits (due to the movement of the UE and/or the satellites). NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits). [PR 5.4.6-002] A 5G network with satellite access shall be able to support charging mechanisms for communication services using satellite access in different orbits based on the type of satellites (e.g., LEO, MEO, GEO). |
| --- | --- |
| 5.5 Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks | [PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbits satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g., availability of the coverage, required QoS. |
| 5.6 Use case on switching between multi-orbits satellite networks in remote area applications | [PR 5.6.6-001] A 5G system with satellite access shall support minimum service interruption when a relay UE is switching between multiple orbit satellite connections (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] [PR 5.6.6-002] The 5G system shall allow the network operator to configure policies to enable switching relay UE between multi orbits satellites (e.g., policies considering QoS, availability of the coverage if discontinuous). Editor’s note: This requirement is FFS. |
| 5.7 Use case on assisting vehicular communications via multi-orbits satellite access | [PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with multi-orbits satellite access shall support mechanisms to allow a UE connecting one of satellite access networks in different orbits considering e.g., QoS, satellite availability. [PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbits satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN. [PR 5.7.6-003] A 5G network with multi-orbits satellite access shall be able to collect charging information for a UE served by different orbit types of satellite access networks. |
| 5.12 Use case on supporting different services with multi-orbit satellites | [PR 5.12.6-001] Based on regulatory requirements and operators’ policy, the 5G system shall support Service Hosting Environment (e.g., edge application server) on multi-orbit satellites. [PR 5.12.6-002] Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide most suitable service hosting environment via multi-orbit satellites. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits. |
| 5.13 Use case on wide-area disaster control support using multi-orbit satellite networks | [PR 5.13.6-001] Subject to regulatory requirements and operator policies, the 5G network with multi-orbit satellite network shall support switching between satellite access depending on the latency requirements. NOTE: The switching between satellite access of a multi-orbit satellite network can be triggered by latency requirements. |
| 5.14 Use case on broadband services through multi-orbit satellite access | [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission. |
| 5.15 Use case on multi-orbit access for cross-hybrid service | [PR 5.15.6-001] A 5G system with multi-orbit satellite access shall be capable of selecting communication paths across different satellite access (GEO, MEO, LEO) as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service. |
| 5.16 Use case on using GEO for initial network entry and service initialization | [PR 5.16.6-001] A 5G system with multi orbit satellite access shall be able to support network entry initiation via one satellite access type and network entry completion via a different satellite access type. Editor’s note: this requirement is FFS [PR 5.16.6-002] A 5G System supporting multi orbit satellite access shall enable the UE to select the relevant GSO for network entry initiation. Editor’s note: this requirement is FFS |
| 5.17 Use case on resource and energy efficiency in a multi-orbit Satellite Access System | [PR 5.17.6-001] A 5G network supporting satellite access with multi-orbit operation shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. [PR 5.17.6-002] A 5G system supporting satellite access with multi-orbit operation shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand. |
| 5.18 Use case on using GEO to land data in a desirable geographical location | [PR 5.18.6-001] A 5G system with multi orbit satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope |
| 5.19 Use case on using GEO for sleep/power management | [PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite system. Editor’s note: this requirement is FFS |
| 5.20 Use case on Multi-orbit satellite backhauling | [PR 5.20.6-001] The 5G system using multi orbit satellite network for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS. [PR 5.20.6-002] The 5G system shall be able to collect and distinguish charging information related to backhaul traffic over different orbits of the satellite network. |

= > To introduce definition for satellite orbit type and to use it in the requirements when relevant

| 5.4 Use case on service continuity through multi-orbit satellite access | [PR 5.4.6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellite orbit types (due to the movement of the UE and/or the satellites). NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits). [PR 5.4.6-002] A 5G network with satellite access shall be able to support charging mechanisms for communication services using different satellite orbit types (e.g., LEO, MEO, GEO). |
| --- | --- |
| 5.5 Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks | [PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbits satellite access shall be able to support mobile base station relays to access different satellite orbit types considering e.g., availability of the coverage, required QoS. |
| 5.6 Use case on switching between multi-orbits satellite networks in remote area applications | [PR 5.6.6-001] A 5G system with satellite access shall support minimum service interruption when a relay UE is switching between multiple satellite orbit types (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] [PR 5.6.6-002] The 5G system shall allow the network operator to configure policies to enable switching relay UE between multiple satellite orbit types (e.g., policies considering QoS, availability of the coverage if discontinuous). Editor’s note: This requirement is FFS. |
| 5.7 Use case on assisting vehicular communications via multi-orbits satellite access | [PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with satellite access supporting multiple satellite orbit types shall support mechanisms to allow a UE connecting to one of the multiple satellite orbit types considering e.g., QoS, satellite availability. [PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with multi-orbits satellite access shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN. [PR 5.7.6-003] A 5G network with satellite access supporting multiple satellite orbit types shall be able to collect charging information for a UE served by different satellite orbit types networks. |
| 5.12 Use case on supporting different services with multi-orbit satellites | [PR 5.12.6-001] Based on regulatory requirements and operators’ policy, the 5G system shall support Service Hosting Environment (e.g., edge application server) on different satellite orbit types [PR 5.12.6-002] Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system shall support a mechanism to provide most suitable service hosting environment via different satellite orbit types. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits. |
| 5.13 Use case on wide-area disaster control support using multi-orbit satellite networks | [PR 5.13.6-001] Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types shall support switching between satellite orbit types depending on the latency requirements. NOTE: The switching between satellite access of a multi-orbit satellite network can be triggered by latency requirements. |
| 5.14 Use case on broadband services through multi-orbit satellite access | [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access supporting multiple satellite orbit types shall be able to support a UE to connect with satellite orbit type for data transmission, but be served via a different satellite orbit type of the same network before and after the data transmission. |
| 5.15 Use case on multi-orbit access for cross-hybrid service | [PR 5.15.6-001] A 5G system with satellite access supporting multiple satellite orbit types shall be capable of selecting communication paths across different satellite orbit types (GEO, MEO, LEO) as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service. |
| 5.16 Use case on using GEO for initial network entry and service initialization | [PR 5.16.6-001] A 5G system with satellite access supporting multiple satellite orbit types shall be able to support connecting initially to the network through one satellite orbit type and then having its data traffic through a different satellite orbit type. Editor’s note: this requirement is FFS [PR 5.16.6-002] A 5G System with satellite access supporting multiple satellite orbit types shall enable the UE to select the relevant satellite orbit type for the initial connection to the network. Editor’s note: this requirement is FFS |
| 5.17 Use case on resource and energy efficiency in a multi-orbit Satellite Access System | [PR 5.17.6-001] A 5G network s with satellite access supporting multiple satellite orbit types shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. [PR 5.17.6-002] A 5G system with satellite access supporting multiple satellite orbit types shall be able to select the satellite orbit type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand. |
| 5.18 Use case on using GEO to land data in a desirable geographical location | [PR 5.18.6-001] A 5G system with satellite access supporting multiple satellite orbit types shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope |
| 5.19 Use case on using GEO for sleep/power management | [PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types shall be able to support a mechanism for the network to inform a UE when coverage is available and in which satellite orbit type Editor’s note: this requirement is FFS |
| 5.20 Use case on Multi-orbit satellite backhauling | [PR 5.20.6-001] The 5G system with satellite access supporting multiple satellite orbit types for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS. [PR 5.20.6-002] The 5G system shall be able to collect and distinguish charging information related to backhaul traffic over different satellite orbit types of the satellite network. |

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v0.2.0.

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

GEO/GSO: the terms GEO (geostationary) and GSO (geosynchronous) are interchangeable in the context of the present Technical Report, unless indicated otherwise.

Resilient Notification: in the context of this study, it refers to a highly reliable mechanism that delivers notifications directly to user devices, such as smartphones, when in poor conditions in satellite access.

satellite access: direct connectivity between the UE and the satellite.

NOTE: 	This definition is taken from TS 22.261 [2].

satellite orbit type: refers to the type of orbit (LEO, MEO, GEO) including specific characteristics (altitude, orbital characteristics, satellite capabilities, IoT or NR, …)

* * Next Change * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GEO	Geostationary satellite Earth Orbit

GSO	Geosynchronous Orbit

LEO	Low-Earth Orbit

MEO	Medium-Earth Orbit

NGSO	Non-Geostationary Satellite Orbit