3GPP TSG-SA WG1 Meeting #108 	S1-244750

Orlando, Florida, USA, 18-22 November, 2024	(revision of S1-244270, S1-244576)

Title:	22.870 pCR: Introduction of System and Service Operational Resilience

Agenda Item:	8.1.1 (6G study - System and Operation Aspects)

Source:	Samsung, KDDI, NTT DOCOMO, SyncTechno Inc., Thales, Toyota,
	Softbank

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This contribution discusses Resiliency, a key topic in the SA1 6G study.

Discussion

changes in S1-244750

- add supporting companies [KDDI, NTT DOCOMO, Softbank, Toyota]

- remove changes on changes

- modified the title "system and service operational resilience" to "resilience" and added clarifying text instead [Rapporteur]

changes in S1-244576

- attempt to merge in KDDI, NTT DoCoMo

- correct 'expectations for high availability go beyond this number' [Nokia]

- remove IoT, V2X [Mediatek]

- add support from SynchTechno Inc., Thales

Resiliency is a key topic in 6G. Resiliency is often used synonymously with 'robustness' - that is, the system becomes more resilient when it has higher availability. Resiliency can be used this way, but the term has a broader meaning.

Resiliency according to the UN Sustainable Development Group is defined as:

Resilience is the ability of individuals, households, communities, cities, institutions, systems and societies to prevent, resist, absorb, adapt, respond and recover positively, efficiently and effectively when faced with a wide range of risks, while maintaining an acceptable level of functioning without compromising long-term prospects for sustainable development, peace and security, human rights and well-being for all. [1]

There are at least three aspects to resilience which we should consider in 6G:

a)	the capability to respond to risks and incidents which cannot be foreseen. This goes beyond 'hardening', that is, features that respond to common issues faced by operations such as increased demand for capacity.

b)	we need to consider use cases in which incidents that cause lack of availability can be dealt with by service users before, during the incident and after the incident, so as to avoid, get through and adapt to the consequences of  interruptions in availability.

c)	existing capabilities to expose information and coping mechanisms can be reviewed and possibly expanded in 6G.

The 5G system identifies quite a few high availability performance requirements, while the means to achieve this metric is generally left 'to implementation', which is to say, outside of the scope of standardization. Using best effort mobile data service, 99.99% service availability is generally not achieved. There is clearly a gap between aspirations for 99.99% and greater service availability and what is delivered to vertical sector stakeholders.

This observation was made in the FS_5GSEI study, especially in use case 5.7 Remote DSO management of connectivity for Smart Energy and  5.18 Coordination for Energy Recovery Use Case. In the NSOEU feature and some NEF capabilities of information offered by NWDAF, a third party can identify a decline in performance and take proactive steps to reduce reliance on communication services that see at increased risk of outage. Further, in NSOEU, a capability was added to coordinate energy service recovery between energy service utility operators and mobile network operators. These is an example of 'proactive' and 'reactive' response. Additional SEI requirements for 'reporting performance from the customer to the operator' were never implemented, but could be part of standardized support for resiliency in the form of adaptation 'after an incident.'

Other features besides SEI and NSOEU that could be considered as part of a 'toolbox for resilient response to incidents' include network recovery procedures, congestion response (such as UAC, congestion control, etc.) and MINT.

Resilience Engineering is a field that brings together safety experts from fields of healthcare, nuclear power, aviation and other areas, concerned with how to 'reframe an organization's model of how safety is created before accidents occur by developing measures and indicators of contributors to resilience such as the properties of buffers, flexibility, precariousness and tolerance and patters of interactions across scales such as responsibility-authority double binds.' [2] There are limits to the degree to which automated optimization and recovery mechanisms succeed in crises, and diverse means that can be introduced to avoid incidents, identify the conditions of an incident to enable rapid recovery, and to learn from incidents.

The applicability of Resilience Engineering principles to the 6G system should be studied. This pCR introduces the term resilience as well as the general concept. Future input will provide use cases and assessment of existing capabilities of the 3GPP system that support resiliency.

References in discussion section

[1]	United Nations Sustainable Development Group, "Executive Summary, UN Common Guidance on Helping Build Resilient Societies", dhttps://unsdg.un.org/sites/default/files/2021-09/UN-Resilience-Guidance-Exec-Summ-Sept.pdf

[2] Hollnagel, E., Woods, D., Leveson, N., "Resilience Engineering: Cocnepts and Precepts", CBC Press, 2006.

Proposal

It is proposed to make the change listed below to TR 22.870.

Begin changes

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

resilience: The ability for a service or system to prevent, resist, absorb, adapt, respond and recover positively, efficiently and effectively when faced with a wide range of risks, while maintaining an acceptable level of functioning.

.

Next change

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.X	Resilience

### 5.X.1	General

Availability as a performance criteria identifies the fraction of time that a service operates, fulfilling all its performance requirements. Especially for 'verticals', expectations for high availability go beyond service availability common in PLMNs for commercial mobile data subscribers. For critical communications, e.g. for MCX and energy utilities, operations should be resilient as well. To the extent possible, incidents where service is not available, need to be avoided proactively.

To some extent, such incidents cannot be entirely avoided, as conditions emerge either through natural disasters, technical oversight, dependencies on other services, or unforeseen operational problems. In this case, it is important to support operational enablers so that network operations can rapidly restore service. Finally, some functionality to support a network operator or service provider to adapt as a result of such incidents would be beneficial.

Clause 5.X of the present document investigates use cases and potential requirements for resilience in system operation. Existing functionality is also considered. Any opportunity for resilience that is identified is evaluated to determine whether such functionality is outside 3GPP scope (and is rather in the scope of 'implementation' or 'organizational competence', etc.) or can be beneficially added to the 6G standard.

The scope of sub-clause 5.X extends beyond the functional aspects of the 3GPP system to service, regulatory and operational aspects of resilience, to the extent that these are in 3GPP scope.

Next change (FROM S1-244029, S1-244176, S1-244033, S1-244270)

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[m]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[n]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[o]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[p]	3GPP TS 22.173: " Multimedia Telephony Service and supplementary services; Stage 1".

[q]	3GPP TS 23.380: "IMS Restoration Procedures".

Next change (FROM S1-244176)

### 5.X.Y	Use case on network congestion control

#### 5.X.Y.1	Description

This 6G use case is motivated by experience with service outages in the 5GS and EPS. 5GS and EPS mechanisms include back-off timers, cause values and other mechanisms to reduce UE actions that cause or perpetuate congestion. It has been observed, despite these existing NAS level congestion control functions, problems remain.

-	UEs need to be switched OFF/ON after receiving a Back-off Timer IE from a network (incl, IMS). So, the network using a Back off Timer does not solve the issue of the manual OFF/ON caused by the variety of UE behaviors in practice. Clarifying or expanding back off timer specification could be beneficial.

-	The flexibility allowed by 3GPP standards for vendors to decide on signalling behavior after receiving cause values can lead to inconsistent UE behavior, some of which a mobile operator does not expect.

For the 6G system, it would be beneficial to:

-	Develop more robust and standardized congestion control mechanisms that account for the diverse range of devices and their capabilities.

-	Implement more intelligent and adaptive network management techniques to handle congestion dynamically and efficiently.

#### 5.X.Y.2	Pre-conditions

Operator A and UE A deploys a mechanism for network congestion control.

UE A subscribes to Operator A.

#### 5.X.Y.3	Service Flows

1.	Service interruption such as system failures and network congestion hit the Operator A’s network.

2.	The Operator A’s network provides a network status indicating the failure condition (e.g., system failures, network congestion) to the UEs.

3.	UE A receives the network status and triggers configurations (e.g., maximum allowed number of signalling events, timing of signalling) based on the network status information.

4.	Operator A’s network recovers from the failure and provides a network status indicating the return to normal operation to the UEs.

5.	UE A receives the network status and triggers configurations (e.g., returning to normal mode) based on the network status information.

#### 5.X.Y.4	Post-conditions

User A is not heavily impacted and continue to consume the service from Operator A.

Operator A avoids a large-scale network failure, recovers the network in a short period.

#### 5.X.Y.5	Existing features partly or fully covering the use case functionality

General NAS level congestion control and the values for Back-off Timer and cause have been specified in TS 23.501 [m] and TS 24.501 [n], respectively. However, how the UEs apply to the Back-off Timer depend on vendor’s implementation.

#### 5.X.Y.6	Potential New Requirements needed to support the use case

[PR.5.x.y.6-1] Subject to operator policy, the 6G system shall be able to support mechanisms to reduce congestion in the range of common operating scenarios and device capabilities.

NOTE 1:	The specific scenarios and device capabilities are not defined in the stage 1 specification. These are left to downstream analysis on the basis of actual experience with network congestion and existing congestion control mechanisms. The scope of this requirement is intentionally broad: it includes all logical components of the 6G system.

Next change (FROM S1-244166, S1-244033)

### 5.X.M	Use case on mitigating IMS failure

#### X.Y.M.1	Description

When IMS related service outages occur, experience shows that the current standard specified behavior leads to massive subscriber de-registration and re-registration, as a cyclical pattern. This can cause overload in the core network. For example, IMS authorization of massive numbers of subscribers can cause substantial signalling with the HSS/UDM. Current IMS failure conditions are not articulated: different failure conditions are signalled. There are insufficient functions in the standard to allow for a controlled or coordinated recovery from this failure scenario.

This use case concerns rapid recovery (to achieve a minimal service interruption) for IMS-related failures.

#### 5.X.M.2	Pre-conditions

User A and User B subscribe the communication and the data services of Operator A.

Operator A’s network deploys a restoration mechanism for IMS failure.

#### 5.X.M.3	Service Flows

1.	User A calls User B.

2.	One or more network functions providing IMS service suffer a service outage. These functions have partial or full inability to serve as specified and required.

3.	User A's call with User B is interrupted.

4.	Coordinated recovery operations occur: The UE of User A and the UE of User B, as well as the networks serving them, are part of this recovery.

#### 5.X.M.4	Post-conditions

User A and User B's call resumes. There is no cyclical attempt at reregistration or other failures resulting from lack of coordination of recovery of the IMS service.

#### 5.X.M.5	Existing features partly or fully covering the use case functionality

The Minimization of Service Interruption requirements have been specified in clause 6.31 of TS 22.261 [o]. However, it focuses on enabling a UE of a given PLMN to obtain connectivity service from another PLMN in areas affected by a disaster condition.

The IMS Multimedia Telephony Service and supplementary services requirements have been specified in TS 22.173 [p]. However, it has not any consideration on IMS-related / IMS-affected failure.

The IMS restoration procedures for the P-CSCF/S-CSCF failure scenario have been specified in TS 23.380 [q]. However, it has not any consideration on the UE’s behaviours based on various IMS-related / IMS-affected failures and the failures of core network providing IMS service.

#### 5.X.M.6	Potential New Requirements needed to support the use case

[PR.5.x.m.6-1] Subject to operator policy, the 6G system shall support a mechanism to prevent a UE from invoking re-attach or re-registration (including re-authorization) in cycles, if the IMS or 6G network providing IMS service is under failure condition.

[PR.5.x.m.6-2] Subject to operator policy, the 6G system shall support a mechanism to avoid unnecessary re-attach/re-registration (including re-authorization) to 6G network and minimize the load on 6G network if the IMS or 6G network providing IMS service is under failure condition.

[PR.5.x.m.6-3] Subject to operator policy, the 6G system shall support a mechanism to provide appropriate error handling between IMS and UEs to enable coordinated recovery of IMS service.

[PR.5.x.m.6-4] Subject to operator policy, the 6G system shall be able to support mechanisms that enable the 6G network to coordinate recovery of IMS service, such that security aspects are considered (e.g. preventing fraud from exploiting the status information) based on the operator’s policy.

Next change (FROM S1-244020. Highlighted text is new.)

### 5.X.Z	Service Independence for Satellite Access

Editor's Note:	There is no need for a 'use case' description, the requirements are based on the principles described in the description below.

#### 5.X.Z.1	Description

Given that telecommunication services are essential for the various activities of the population, the 6G system shall be designed to ensure high resiliency and therefore suitable to provide communication services to mitigate terrestrial communication service outages, resulting from natural and man-made disasters. These service outages can lead to temporary and local failure of the system.

To increase the availability of 3GPP satellite access, dependencies on other services need to be reduced. In this way, the 6G system can operate independently of non-3GPP technology. This removes the risk of service outage of non-3GPP technology resulting in reduced availability of the 3GPP satellite access.

For example, the Global Navigation Satellite System (GNSS), which is highly used in 5G system, is not exempt of vulnerabilities, such as jamming or spoofing. Hence when GNSS is degraded or unavailable, this impacts the availability of the 5G services. This may have dramatic consequences in case of:

-	connectivity to public transportation (aeronautic, railway, land, maritime)

-	emergency call or messaging,

-	tracking/management of dangerous goods.

#### 5.X.Z.2	Service	Requirements

[P.R.-5.X.Z.1]	Subject to the regulatory requirements and operator’s policy, the 6G system with satellite access shall be able to provide services to an authorized UE independently of a 3rd party system (e.g. Global Navigation Satellite Systems).

End of changes