3GPP TSG SA WG 1 Meeting #108 	S1-244803

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244797, S1-244100)

Source:	Novamint, Viasat, Inmarsat

pCR Title:	Pseudo-CR on update 5.16

Draft Spec:	3GPP TR 22.887 v0.2.0

Agenda item:	7.3

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes to update the use case 5.16  of TR.22887 to remove the FFS and to fix some text issues

1. Introduction

In TR 22.887, the use case 5.16  has still some FFS to be addressed

2. Reason for Change

To remove the FFS for the use case 5.16 of  TR 22.887 based on the following:

- To remove FFS for [PR 5.16.6-001] as the definitions including the new definitions provided in another contribution are making the requirement clear

- To remove FFS for [PR 5.16.6-002] as the definitions including the new definitions provided in another contribution are making the requirement clear

+ additional changes for replacing GEO par GSO and replacing handover by mobility plus to update the texts to use the proper terms

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v0.2.0.

* * * First Change * * * *

## 5.16	Use case on using GEO for initial network entry and service initialization

### 5.16.1	Description

GEO satellites have a much wider field of view than NGSO satellites and typically have longer active duty cycles than NGSO systems.  They can be used to transmit cell broadcast signals and system information on a very large coverage area very efficiently, where a GEO satellite cell coverage area may be of a size that is equivalent to one or several NGSO satellite footprints areas (and thus to several NGSO cells). Cell search and network entry can be a simpler process using GEO satellites, while cell search and network entry can be an extended process for a UE served by NGSO satellites exclusively.

Furthermore, satellites often present significant payload power budget constraints, which can be more severe for NGSO satellites. In such scenarios, an NGSO satellite’s active coverage footprint might be selectively reduced in order to conserve power.  This has however the effect of creating coverage holes under the footprint of the satellite, in which cells may not be activated.

The presence of a GEO satellite coverage overlay on top of NGSO allows to bridge coverage gaps by providing broadcast signal coverage and initial access for the network.  In this mode, we assume that upon power-on, an UE will perform initial cell search, initial access and network registration (also known as “network entry”)  into a GEO satellite.  During the initial access and registration procedure the network may choose to serve  the UE via an available NGSO satellite.  The NGSO satellite may be instructed by the network to activate a dormant beam or cell with a specific set of resources in the targeted coverage area, or the UE may be instructed to join an existing NGSO beam or cell.

This concept can be generalized from GEO and NGSO to multiple satellite access types but the concept is most easily understood and justified when considering GEO and NGSO.

### 5.16.2	Pre-conditions

- The network supports multiple satellite access types

- The network supports mobility  between multiple satellite access types

- A network plan that deconflicts frequency allocations between the component satellite access types.

- The network supports network entry initiation on one satellite access of the network and network entry completion on a different satellite access.

### 5.16.3	Service Flows

- UE is turned on and wants to establish a connection

- UE does initial network entry on a GEO satellite (or more generally a satellite of the appropriate satellite access type)

- The system provides the UE enough information to enable network entry completion on a satellite access of the NGSO (or more generally a satellite access of the appropriate satellite access type)

- UE does network entry completion on an NGSO (or more generally a satellite access of the appropriate satellite access type)

### 5.16.4	Post-conditions

The UE is registered on the preferred satellite from the network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) and carries out communication on that satellite network.

### 5.16.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-001] A 5G network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics.

Editor’s note: this requirement is FFS

[PR 5.16.6-002] A 5G network with satellite access supporting multiple satellite orbits types with different characteritics (e.g.; altitude, orbital characteristics, satellite capabilities) shall enable the UE to select the relevant satellite with a given characteristics for the initial connection to the network.

Editor’s note: this requirement is FFS