3GPP TSG-SA WG1 Meeting #108 	S1-244825

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244509)

Title:	AI/ML model life cycle management

Agenda Item:	8.1.7

Source:	ZTE, China Mobile, KPN

Contact:	Chen Lin   (chen.lin23@zte.com.cn)

Xiaonan Shi (shixiaonan@chinamobile.com)

Norp Toon (Toon.norp@tno.nl)

Abstract: <provide a short description of the content>

This contribution proposes a use case about AI model life cycle management, which enables the 6G system to provide dynamic and adaptive AI/ML model management.

************************ New Use Case ***********************************

W.y	Use case on AI/ML model training and AI data transfer

W.x.1	 Description

With the introduction of AI and computing, 6G communication network needs to address the following two scenarios:

1) AI for Network: It is called AI enabled network, which improves the performance, efficiency and user experience of the network itself through AI. AI for network mainly includes the use of AI to optimize traditional algorithms, network functions, as well as network operation and maintenance management.

2) Network for AI: It is called network enabled AI, which provides multiple support capabilities(e.g. connection, computing power, data) for AI applications through the network, making AI training/inference more efficient and complying with the local governance of data requirements and data privacy.

With network for AI as an example, the third party may request the 6G system to train AI models considering that third party may lack the computing resources and expertise to train the complex AI models efficiently. Upon receiving such  request, the 6G system may select suitable network entities to perform the AI model training based on the third party’s requirement. Upon the completion of the AI mode training, the 6G system will deliver the well-trained AI/ML model to the third party which runs the AI/ML model to make inference or derive prediction. On the other hand, the performance of AI/ML model can be monitored by 6G network. When the performance of AI/ML model drop, the AI/ML model can be retrained by 6G network to reflect the environment change. Moreover, when multiple AI/ML models are available, more suitable AI/ML can be selected and transferred to third party based on current situation. In addition to third party, the AI/ML model used by subscribers can also be trained and managed by the 6G network.

Although traditional cloud providers may offer powerful centralized AI training and management capabilities, the 6G operator can provide more suitable infrastructure for efficient AI model training and management. To be specific, AI/ML models can be updated timely at the network edge of 6G system and subscribers can benefit from immediate feedback for mission-critical scenarios. Moreover, the AI/ML model can be trained or updated closer to where the training data is generated in 6G system. As we know, the UEs of subscribers may collect data about their environment and this data can be transmitted to 6G network for AI/ML model training and management. This localized processing minimizes the need to send massive amounts of data to distant cloud servers, leading to reduced bandwidth usage and improved energy efficiency.

When the subscriber or third party request the 6G network to train AI/ML models, the 6G network may select suitable network entities (e.g. core network, edge server, etc) to perform the AI/ML model training. During this procedure, AI training data need to be transferred between subscriber/third party and the selected network entity. Upon the completion of the AI/ML mode training, the well-trained AI/ML model needs to be transferred to the subscriber/third party. As we can see, efficient AI data(e.g. AI training data, AI/ML model) transfer should be supported. And the security and data privacy should be guaranteed as well.

On the other hand, the QoS requirements on priority, delay, reliability and data rate vary greatly for the AI data of different subscribers/third party. For example, some AI/ML model transfer requires low latency and high reliability while some other AI/ML model transmission requires large data volume and higher latency. Therefore, the 6G system should be able to provide the required QoS (throughput, reliability, latency) for AI data transfer.

W.x.2	 Pre-conditions

A third-party agricultural technology company develops customized AI models for precision farming applications, such as crop disease detection, yield prediction, and irrigation optimization. These models need to be trained on data specific to each farm, including soil conditions, weather patterns, and crop varieties.

The 6G system is operated by operator A, which owns sufficient computing power and support AI service (e.g. AI/ML model training and management) for third party. The agricultural technology company subscribe the AI service from operator A which helps farm to train and manage customized AI models..

W.x.3	 Service Flows

- Farm utilizes various sensors and IoT devices to collect data related to soil conditions, weather, crop growth, and other relevant parameters. The collected data can be used as AI training data and is stored locally on farm.

- The farm requests a customized AI model from the agricultural technology company. The request specifies the type of model needed (e.g., disease detection, yield prediction) and other requirements specific for this farm.

- The agricultural technology company ask for the 6G system to train the customized AI model. The requirements for the AI model are also delivered to the 6G system. Upon receiving such request, the 6G system selects suitable network entities (3GPP core network or edge server) to perform the AI model training.

- The farm’s AI training data is delivered to selected network entities for AI model training, ensuring the model is tailored to the farm’s unique conditions.

- Upon the completion of the AI mode training, the well-trained AI model is delivered securely from the 6G system to the farm.

- The farm can then use the deployed AI model to perform real-time analysis and make decisions regarding crop management, irrigation scheduling, and disease control.

W.x.4	 Post-conditions

Based on the AI model training service provided by 6G system, each farm receives AI model specifically trained based on its own data, leading to more accurate predictions and optimized performance. Farms do not need to invest expensive computing hardware or AI expertise for model training. The farm infrastructure costs can be greatly reduced..

W.x.5	Existing features partly or fully covering the use case functionality

In TS 28.105, the ML model life cycle is captured in clause 4a.0, where the ML model training, ML model testing, AI/ML inference emulation, ML model deployment, AI/ML inference are described. However, only the ML models used in 5GC, NG-RAN and management system are taken into account in TS 28.105. The life cycle of ML model used in UE is not covered in TS 28.105. Moreover, how to manage the AI/ML model based on the request of authorized third parties is not covered in TS 28.105 as well.

In TS 22.261, clause 6.5.2 on efficient user plane include the following requirements:

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path between UEs attached to the same network, modifying the path as needed when the UE moves during an active communication.

The 5G network shall enable a Service Hosting Environment provided by operator.

Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a Service Hosting Environment for specific services, modifying the path as needed when the UE moves during an active communication.

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network; or

-	an application server located in a customer premises network or personal IoT network.

The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to:

-	another location served by a different Service Hosting Environment; or

-	another location served by an application server located outside the operator’s network; or

-	another location served by an application server located in a customer premises network or personal IoT network, and vice versa.

The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows:

-	within a Service Hosting Environment; or

-	from a Service Hosting Environment to another Service Hosting Environment; or

-	from a Service Hosting Environment to an application server located place outside the operator’s network; or

-	from a Service Hosting Environment to an application server located in a customer premises network or personal IoT network, and vice versa.

In TS 22.261, clause 6.40.1 on AI/ML model transfer include the following requirements:

Artificial Intelligence (AI)/Machine Learning (ML) is being used in a range of application domains across industry sectors. In mobile communications systems, mobile devices (e.g. smartphones, automotive, robots) are increasingly replacing conventional algorithms (e.g. speech recognition, image recognition, video processing) with AI/ML models to enable applications. The 5G system can at least support three types of AI/ML operations:

-	AI/ML operation splitting between AI/ML endpoints

The AI/ML operation/model is split into multiple parts according to the current task and environment. The intention is to offload the computation-intensive, energy-intensive parts to network endpoints, whereas leave the privacy-sensitive and delay-sensitive parts at the end device. The device executes the operation/model up to a specific part/layer and then sends the intermediate data to the network endpoint. The network endpoint executes the remaining parts/layers and feeds the inference results back to the device.

-	AI/ML model/data distribution and sharing over 5G system

Multi-functional mobile terminals might need to switch the AI/ML model in response to task and environment variations. The condition of adaptive model selection is that the models to be selected are available for the mobile device. However, given the fact that the AI/ML models are becoming increasingly diverse, and with the limited storage resource in a UE, it can be determined to not pre-load all candidate AI/ML models on-board. Online model distribution (i.e. new model downloading) is needed, in which an AI/ML model can be distributed from a NW endpoint to the devices when they need it to adapt to the changed AI/ML tasks and environments. For this purpose, the model performance at the UE needs to be monitored constantly.

-	Distributed/Federated Learning over 5G system

The cloud server trains a global model by aggregating local models partially-trained by each end devices. Within each training iteration, a UE performs the training based on the model downloaded from the AI server using the local training data. Then the UE reports the interim training results to the cloud server via 5G UL channels. The server aggregates the interim training results from the UEs and updates the global model. The updated global model is then distributed back to the UEs and the UEs can perform the training for the next iteration.

As we can see, user traffic in 5G system is delivered between UE and applications. Efficient user plane is supported to address diverse user’s traffic requirement on reliability, latency, and bandwidth. The user traffic is transparent to the network entities involved in the user plane data transfer. On the other hand, the AI/ML splitting operation is used for the application domain (e.g. speech recognition, image recognition, video processing) enhancement of mobile devices (e.g. smartphones, automotive, robots) in TS 22.261. To be specific, the mobile device executes the operation/model up to a specific part/layer and then sends the intermediate data to the network endpoint. The network endpoint executes the remaining parts/layers and feeds the inference results back to the device. The intermediate data and the inference result for the AI/ML splitting operation in 5G  is essentially transferred via traditional user plane.

W.x.6 Potential New Requirements needed to support the use case

[PR.w.x.6-1]	Subject to operator’s policy and user consent, the 6G network shall be able to support AI/ML model training in edge server or core network, and AI/ML model management for authorized third parties.

[PR.w.x.6-2]	Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall be able to securely transfer data (e.g. AI model, AI training data) between multiple end-points within the edge or core network or 3rd party, and protect the data privacy.

[PR.w.x.6-3]	Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall be able to provide the required QoS (e.g. user experienced data rate, reliability, end to end latency) for AI data transfer.