SA WG2 Meeting #164 	S2-2408210

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0550 |  | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support AI/ML model performance monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To specify the Procedure for Direct AI/ML based Positioning based on the conclusion in clause 8.1 of TR 23.700-84 as following: Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.1 The result of model performance monitoring may trigger the LMF to change the positioning method, e.g. from AI/ML based positioning to the legacy positioning, or vice versa. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. - Principle #4.3 The detailed procedure and service for this model performance monitoring will be decided in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new clause for the procedures for Direct AI/ML based Positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)6.X; (new) 6.X.1; (new) 6.X.2; (new) 6.X.3; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.X	Procedure for Direct AI/ML based Positioning

6.X.1 procedure for Direct AI/ML based Positioning

Editor’s note: the procedure for Direct AI/ML based Positioning is to be added.

6.X.2 procedure for AI/ML model performance monitoring

Figure 6.X.2-1 shows a procedure for AI/ML model performance monitoring during Direct AI/ML based Positioning. The PRU/UE with known location information is reported to LMF to compare with the final location that is calculated by Direct AI/ML based Positioning based on the measurements from PRU/UE or gNB. The result of AI/ML model performance monitoring may trigger LMF to change the positioning method, e.g., from Direct AI/ML based positioning to the legacy positioning, or may trigger AI/ML model retraining.

Figure 6.X.2-1 procedure of AI/ML model performance monitoring

- AMF sends the location request to the selected LMF, when receiving LCS service request from UE (i.e., MO-LR) as specified in clause 6.2, or from LCS Client (i.e., MT-LR) as specified in clause 6.1, or AMF as specified in clause 6.10.

- LMF determines to select Direct AI/ML based Positioning method to calculate the UE location by performing AI/ML model inference based on the measurements reported from PRU/UE or gNB.

- LMF determines to perform AI/ML performance monitoring, e.g., based on the request. Then LMF selects the PRU/UE to report the known location information to LMF.

- PRU/UE reports the known location information to LMF.

- LMF evaluates the AI/ML model performance by comparing the reported known location information of PRU/UE with the calculated location information in step 2 to obtain the AI/ML model performance monitoring result.

The result trigger the LMF to change the positioning method, e.g., from AI/ML based positioning to the legacy positioning, and also may trigger the AI/ML model retraining in the training entity.

- LMF continues the positioning procedure to send the positioning result to LCS service consumer, e.g., to UE (i.e., MO-LR) as specified in clause 6.2, or to LCS Client (i.e., MT-LR) as specified in clause 6.1, or to AMF as specified in clause 6.10

6.X.3 procedure for AI/ML model training

Editor’s note: the procedure for AI/ML model training is to be added.

* * * End of Changes * * *