3GPP TSG-SA2 Meeting #166	S2-2411336

Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5740 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional Description of Energy Efficiency Control Functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, OPPO?, Samsung?, China Mobile?, Rakuten Mobile?, NEC?, ZTE? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | r04 (S2-2411073) New WID on Energy Efficieny and Energy Saving and KI#1 conclusion have been approved in SP-241388, and S2-2409296 respectively. A functional description of Energy Information Function (EIF) should be provided to support new functionality of collection and calculation of energy related information. r05 As highlighted in S3-244606, in some regulations, energy usage may be legally equivalent to data consumption and as such, be subject to user consent for its collection, usage and/or exposure. There is a need to cover user consent based on local regulations, which would require the communication with the UDM to reuse existing user consent mechanisms specified in TS 33.501 Annex V, User consent requirements and mechanisms |  |  |  |  |  |  |  |  |
| Summary of change: |  | r04: Summary of change: Functional description of EIF. r05: Add description regarding user consent for data collection and exposure based on user consent based on output of SA3#119 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | r04: Cannot support this new functionality r05: User consent cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change  *****

### 6.2.x	EIF

The Energy Information Function (EIF) includes support for the following functionalities:

- Collect data from OAM and 5GC NF(s) to assist the calculation of energy related information.

- Calculate the energy related information (including energy consumption information and renewable energy information) of user plane communication.

- Expose the calculated energy related information to authorized consumers.

NF(s) running the feature(set) of energy information collection/exposure (e.g. EIF, NEF) support user consent for collection and exposure of energy-related information depending on local regulations. For support of user consent, the NF communicates with the UDM as specified in Annex V of TS 33.501 [29].

Editor’s Note: It is FFS whether this NF performs other functionalities.

Editor's Note: It is FFS whether or not EIF will use network data analytics framework as defined in TS 23.288 [86].

***** End of Changes *****