3GPP TSG-WG SA2 Meeting #166	S2-2411411

# 18 - 22 November, 2024, Orlando, USA								(revision of S2-2409655)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0254 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF (re)selection considering N6 delay for EAS discovery with Local DNS Server/Resolver |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UPF (re)selection considering N6 delay for option A or B has been supported in current TS 23.548, as clause 6.2.3.2.2. However, how to support UPF (re)selection considering N6 delay under Option C or Option D is still unclear, which also needs to be supported. For option A or B, the EASDF reports EAS IP to the SMF. For option C or D, due to no EASDF involved, , the DNS messages are directly forwarded to and received from L-DNS Server/Resolver by L-PSA UPF, so it is proposed that the L-PSA UPF detects and reports the DNS Response. The L-PSA can be indicated by the SMF to report the DNS Response with FQDN for the EAS(s) that are (re)selected by considering N6 delay measurements. Then the SMF performs UPF (re)selection considering N6 delay based on the EAS IP address of the DNS Response reported by the L-PSA UPF. If edge relocation is required, SMF may perform the Change of additional PDU Session Anchor for IPv6 multi-homing or UL CL procedure as clause 4.3.5.6 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For EAS discovery with Local DNS Server/Resolver, SMF indicates L-PSA UPF to report DNS Response with FQDN for the EAS(s) that are (re)selected by considering N6 delay measurements, and performs L-PSA UPF (re)selection considering N6 delay measurement results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPF (re)selection considering N6 delay cannot be supported for EAS discovery with Local DNS Server/Resolver. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3	Edge Relocation

6.3.1	General

Edge Relocation refers to the procedures supporting EAS changes and/or PSA UPF relocation.

Edge Relocation may be triggered by an AF request (e.g. due to the load balance between EAS instances in the EHE) or by the network (e.g. due to the UE mobility).

With Edge Relocation, the user plane path may be re-configured to keep it optimized. This may be done by PDU Session re-establishment using SSC mode 2/3 mechanisms or Local PSA UPF relocation using UL CL and BP mechanisms. The corresponding procedures are defined in TS 23.501 [2] and TS 23.502 [3].

Due to Edge Relocation, the UE may need to re-discover a new EAS and establish the connectivity to the new EAS to continue the service. The re-discovery of EAS is specified in clause 6.2.

Edge Relocation may result in AF relocation, for example, as part of initial PDU Session Establishment, a central AF may be involved. However, due to Edge Relocation another AF serving the Edge Applications is selected.

The trigger of Edge Relocation by the network is specified in clause 4.3.6.3 of TS 23.502 [3]. Some EAS (re-)Discovery procedures in clause 6.2 may also trigger Edge Relocation.

This clause further describes the following procedures:

-	Edge Relocation involving AF change.

-	Edge Relocation using EAS IP replacement.

-	AF request for simultaneous connectivity for source and target PSA.

-	Packet buffering for low Packet Loss.

-	Edge Relocation considering User Plane Latency Requirements.

-	Edge Relocation triggered by AF

-	Edge Relocation for a set of UEs for common DNAI.

-	Edge Relocation considering N6 delay for EAS discovery with Local DNS Server/Resolver.

Annex F describes example procedure for EAS Relocation on Release 16 capabilities.

For non-roaming PDU Session, the 5GC functions in the following clauses are located in the HPLMN.

For LBO roaming PDU Session, the 5GC functions in the following clauses are located in the serving VPLMN.

For HR-SBO PDU Sessions specified in clause 6.7, the AF may send to V-NEF an AF request to influence traffic routing as described in clause 4.3.6 of TS 23.502 [3] for supporting Edge Relocation (e.g. for the purpose of subscription to UP path management events, especially for the change of local PSA UPF in VPLMN). In this case, the steps involving PCF in the following clauses are skipped.

# * * * * Second change (all new text) * * * *

6.3.X	UPF (re)selection considering N6 delay for EAS Discovery Procedure with Local DNS Server/Resolver

The following procedure is for UPF (re)selection considering N6 delay for EAS discovery with Local DNS Server/Resolver. SMF indicates L-PSA UPF to report DNS Response with FQDN for the EAS(s) that are (re)selected by considering N6 delay measurements, and performs L-PSA UPF (re)selection considering N6 delay measurement results. If edge relocation is required, SMF performs the Change of additional PDU Session Anchor for IPv6 multi-homing or UL CL procedure as clause 4.3.5.6 of TS 23.502 [3].

UE

SMF

ULCL/BP

PCF

NEF

4. DNS Response

3. DNS Query

5. N4 Session Report

L-PSA1

L-PSA2

AF

1. Processing AF requests to influence traffic routing procedure, clause 4.3.6.2 of TS 23.502

2. Steps 0~3 of EAS discovery procedure with Local DNS Server/Resolver, clause 6.2.3.2.3

L-DNS server

6. DNS Response

8. Change of additional PDU Session Anchor for IPv6 multi-homing or UL CL procedure, clause 4.3.5.6 of TS 23.502

7. N6 delay measurement, clause 5.2.8.X of TS 23.501

Figure 6.3.X-1: UPF (re)selection considering N6 delay for EAS discovery with Local DNS server/resolver

1.	Processing AF requests to influence traffic routing procedure is performed, as described in clause 4.3.6.2 of TS 23.502 [3].

2.	Steps 0-3 of EAS discovery procedure with Local DNS Server/Resolver is performed, as described in clause 6.2.3.2.3, with the following change:

-	If Indication of N6 delay measurement is received in PCC rule in step 1, the SMF determines which FQDN(s) and EAS(s) IP address are impacted. The UPF(s) that routing the traffic to target EAS/FQDN may be(re)selected by considering N6 delay measurements. The SMF sends the FQDN(s) to the L-PSA UPF, and indicates the L-PSA UPF to report any DNS Response that matches the FQDN(s).

3.	UE sends DNS Query to the Local DNS Server/Resolver.

4.	The L-PSA UPF receives the DNS Response from Local DNS Server/Resolver.

If the FQDN of the DNS Response matches the FQDN(s) configured by the SMF in step 2, the following steps are performed, otherwise only step 7 is performed.

5.	The L-PSA UPF reports the DNS Response to the SMF via N4 reporting as described in clause 4.4.2.2 of TS 23.502 [3].

6.	The L-PSA UPF forwards the DNS Response to UE.

7.	If received DNS Response reported by the L-PSA UPF, the SMF may trigger the measurement of N6 delay between the L-PSA UPF and the EAS IP included in the DNS Response, as described in clause 5.8.2.X of TS 23.501 [2].

8.	The SMF performs L-PSA UPF (re)selection considering the N6 delay. If SMF determines the L-PSA UPF needs to be changed, Change of additional PDU Session Anchor for IPv6 multi-homing or UL CL procedure is performed as described in clause 4.3.5.6 of TS 23.502 [3].

# * * * * End of changes * * * *