3GPP SA WG2 Meeting #166                                                                           S2-2411437

Orlando, USA, 18 – 22 November 2024

Source:		vivo

Title:		Discussion on LP-WUS Assistance

Agenda item:		19.50

Work Item / Release:		TEI19

Document for:		DISCUSSION

Abstract of the contribution: it introduces LP-WUS subgrouping and the corresponding CN impacts.

- Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. In order to improve UE energy efficiency, RAN WGs had a study on low-power wake up signal and receiver for NR (LP-WUS/WUR) in Rel-18 and observed that significant UE power saving gain and paging latency reduction is obtained by using LP-WUS/WUR to trigger UE paging monitoring compared with existing UE power saving techniques. Based on the SID outcome, RAN has created a new work item “NR_LPWUS” in Rel-19, see RP-241824.

RAN2 has sent an LS to SA2 (C1-24xxxx / R2-2409225) on LP-WUS subgrouping wherein RAN2 has concluded that the Paging Early Indication with Paging Subgrouping (PEIPS) method is taken as baseline for LP-WUS subgrouping and assumes similar procedure for PEIPS will be used for LP-WUS subgrouping.

This paper tries to discusses potential SA2 work to support LP-WUS subgrouping based on the RAN2 agreements and PEIPS method.

2. Background of PEIPS

PEIPS is used to reduce power consumption in the UE due to false paging alarms (i.e. when the UE receives a paging message on PDSCH on its paging occasion, which is not intended for that UE). This is done by further dividing the UEs within a PO into multiple subgroups. A UE will decode the paging message in its PO only if it receives an indication that its subgroup is being paged.

There are two ways for the UE to determine the paging subgroup including CN assigned subgrouping and UE ID based subgrouping. Only CN assigned subgrouping is within the remit of CN and paging subgroup ID is provided to the UE using NAS signalling.

The below figure shows the implementation of PEIPS captured by TS 23.501 and TS 24.501:

Main contents are as follow:

- Applicable in 5GS to UEs in 5GMM-IDLE and 5GMM-CONNECTED mode with RRC inactive indication.

- The PEIPS assistance information is used to determine the paging subgroup for the UE.

  - The UE indicates the supports PEIPS assistance information and UE paging probability information during the registration procedure except for emergency case.

  - The CN evaluates UE capability and provides the PEIPS assistance information (i.e., Paging subgroup ID) to the UE during registration procedure, and provides it to the RAN upon CN paging.

  - The CN can update the PEIPS assistance information to the UE using UCU procedure.

3 Discussion

In the incoming LS, RAN2 informed about the conclusions reached by RAN2 about LP-WUS subgrouping as following:

1)	The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

2)	RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

3)	RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

4)	For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/SA2/RAN3 discussion.

5)	For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

Some key information and clarification are highlighted as below:

- The PEIPS is taken as a baseline for LP-WUS subgrouping including CN assigned subgrouping and UE_ID based subgrouping, but only CN assigned subgrouping is within the remit of CN;

- Similar procedure for PEIPS is expected to be used for LP-WUS subgrouping;

- The maximum number of subgroups depends on the RAN WGs progress. For your information, the latest RAN1 working assumption has specified that the maximum number of subgroups per PO supported in R19 is 32. In addition, the RAN WGs will keep SA2 updated of any further relevant developments on LP-WUS subgrouping.

Observation 1: From RAN2 perspective, the PEIPS is taken as a baseline for LP-WUS subgrouping and the similar procedure is expected to be used for LP-WUS subgrouping.

Main contents of LP-WUS subgrouping can be observed from the study of RAN WGs:

- Applicable in 5GS to UEs in 5GMM-IDLE and 5GMM-CONNECTED mode with RRC inactive indication.

- The assistance information (e.g., LP-WUS subgroup ID) needs to provided by the CN to enable the UE to determine the paging subgroup triggered by LP-WUS.

The requirement of LP-WUS subgrouping is very similar with PEIPS, the PEIPS method can be re-used for LP-WUS subgrouping.

Proposal 1: From SA2 perspective, the PEIPS can be taken as a baseline for LP-WUS subgrouping and the similar procedure can be used for LP-WUS subgrouping.

However, the below difference can also be observed between LP-WUS subgrouping and PEIPS:

-	LP-WUS subgrouping is a new feature, because:

  - LP-WUS  is a new WID and will be captured in a new “Feature group” different form PEI, in addition, LP-WUS subgrouping is the subset of LP-WUS and PEIPS is the subset of PEI.

  - LP-WUS subgrouping not only further divides the UEs in a PO into more subgroups than PEIPS, but also applies different techniques (e.g. RRM measurement). So it can obtain more significant UE power saving gain.

-	The maximum number of LP-WUS subgroups is larger than PEI subgroups, because LP-WUS adopts a finer grouping method.

Though the PEIPS is a baseline for LP-WUS subgrouping, legacy UE capability for PEIPS can not be re-used for LP-WUS subgrouping because they are different features.

Proposal 2: LP-WUS subgrouping and PEIPS are different features and legacy UE capability for PEIPS can not be re-used for LP-WUS subgrouping.

According to proposal 1 and proposal 2, some new requirement is expected to reflect in the SA2 spec, main including:

- A new 5GMM capability is needed to enable the AMF to evaluate the UE capability;

- A new procedure is needed to provide the LP-WUS subgrouping assistance information (i.e., LP-WUS subgroup ID) to the UE.

Proposal 3: A new UE capability similar with NR paging subgrouping is expected for LP-WUS subgrouping. A new procedure similar with provision of PEIPS assistance information (i.e., Paging subgroup ID) is expected to support LP-WUS subgrouping assistance information (e.g., LP-WUS subgroup ID).

4. Proposal

Based on the above discussion, following observations and proposals were provided:

Observation 1: From RAN2 perspective, the PEIPS is taken as a baseline for LP-WUS subgrouping and the similar procedure is expected to be used for LP-WUS subgrouping.

Proposal 1: From SA2 perspective, the PEIPS can be taken as a baseline for LP-WUS subgrouping and the similar procedure can be used for LP-WUS subgrouping.

Proposal 2: LP-WUS subgrouping and PEIPS are different features and legacy UE capability for PEIPS can not be re-used for LP-WUS subgrouping.

Proposal 3: A new UE capability similar with NR paging subgrouping is expected for LP-WUS subgrouping. A new procedure similar with provision of PEIPS assistance information (i.e., Paging subgroup ID) is expected to support LP-WUS subgrouping assistance information (e.g., LP-WUS subgroup ID).