3GPP TSG-SA WG2#166	S2-2411460

Orlando, US, Nov 18 – 22, 2024	(was S2-241xxxx)

Source:	Nokia

Title:	Discussion paper on User Plane Address in PDU Session Procedures

Document for:	Discussion / Information

Agenda Item:	19.8.2

Work Item / Release:	Release 19

Abstract of the contribution: This paper explains on carrying non-3GPP device User Plane Address in the PDU Session Procedures

# 1.	Background

Agreed S2-2411269 with CR 5750 has the following

For the traffic of non-3GPP devices requiring differentiated QoS:

-	For Ethernet PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

-	For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send the IP Address and/port ranges associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

-	For IPv6 PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send the IPv6 Address/prefix(sub) that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

Editor’s note: 	whether the UE sends the Non-3GPP Device Identifier and user plane information (e.g. IP Address) in PDU session establishment is FFS.

Note: In this discussion paper, all references of UE are to be considered and read as applicable to 5G-RG.

# 2	Discussion

There are two scenarios that are considered as part of conclusions to handle non-3GPP devices behind a UE; in one scenario each of the non-3GPP devices may be using individual PDU Session while in the other scenario multiple non-3GPP devices may share a single PDU Session but with individual QoS flows.

It is also to be noted that the network (SMF, UPF) provides the user plane address to the UE as part of the PDU Session Establishment procedures, which clarifies that the routable user plane address is not known before the completion of PDU Session Establishment procedure.

So, the UE may provide sub prefixes to the non-3GPP devices behind it based on the shorter prefix it receives as part of the PDU Session Establishment Response.

Similar such approach is applicable for the IPV4 PDU session type.

Additionally, for the Ethernet PDU session type the UPF learns the MAC addresses of the UEs and the devices behind the UE. And SMF may optionally subscribe to get notified of the MAC address that the UPF learns. So, in general, MAC address details are not sent on control plane signaling.

# 3.	Proposal

The proposal is to

- Non-3GPP Device Identifier may be part of PDU Session Establishment Request.

- User plane address information is not required to be part of PDU Session Establishment Request.

- Non-3GPP Device Identifier may be part of PDU Session Modification Request.

- User plane address information may be part of PDU Session Modification Request.

- Ethernet PDU session type can be considered only for first scenario (described in above section), i.e., each of the non-3GPP devices has individual Ethernet type PDU Session.

PDU Session procedures in 23.502 to consider accordingly, as per above.

The corresponding CRs include S2-2411465 (23.501 CR 5750) and S2-2411463 (23.502 CR 4844).