3GPP TSG-WG SA2 Meeting # 166	S2-2411496

# Orlando, US, 18 - 22 November 2024										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5830 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetShare |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolve the following EN and addressing incoming LS S2-2411497. Update the description for PDU session establishment procedure handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description in 5.18.1 to remove the EN. Udpate the PDU session establishment procedure handling. Two alternatives are proposed for final conclusion/selection of one of the options during the meeting. -alt 1: with extra impact on the service operation between AMF and SMF -alt 2: no service operation impact, but support of participting operator slice is required in hosting network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear handling of the slice handling in Indirect Network Sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1, 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements.

In this Release of the specification, the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. The 5G System may also support Indirect Network Sharing deployment between hosting operator (i.e. shared network operator) and participating operator (see clause 6.21 of TS 22.261 [2], Figure 5.18.1-2 and Annex R), in which the RAN is shared. The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN.

5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID (i.e. with same or different country code (MCC) some of which is specified in TS 23.122 [17] and different network codes (MNC)) or combinations of PLMN ID and NID. 5G MOCN supports NG-RAN Sharing with or without multiple Cell Identity broadcast as described in TS 38.300 [27]. Indirect Network Sharing for 5G system, including UE, shared RAN and CP NFs of hosting operator, shall support each participating operator to use more than one PLMN ID.

For Indirect Network Sharing, the shared RAN is broadcasting multiple PLMN IDs, including the PLMN ID which represents the hosting operator and the PLMN IDs which represent participating operators. Multiple PLMN IDs are supported by the serving AMF (i.e. the AMF in the core network of the hosting operator). A UE from a participating operator can select the PLMN ID representing the participating operator in the shared RAN area based on existing procedures specified in TS 23.122 [17]. The serving AMF selects core network functions in the PLMN of the participating operator for the UE, based on home routed roaming architecture principle as specified in clause 4.2.4. In addition, the serving AMF selects the SMF of participating operator possibly considering UE location information and also selects a V-SMF in its own network during the PDU session establishment procedure. The serving AMF determines the serving PLMN ID as follows:

-	For procedures involving the Network Function in the participating operator network (e.g. AUSF), the AMF sets the serving PLMN ID to the selected PLMN ID.

-	For procedures involving the Network Function only in the hosting operator network (e.g. PCF), the AMF sets the serving PLMN ID to the PLMN ID of the hosting operator.

NOTE 0:	The representation of the serving PLMN ID is detailed in stage 3 and can have different forms for different service operations.

NOTE 1:	In this Release of specification, the Indirect Network Sharing is only applicable for NR with 5GC of hosting operator and 5GC of participating operator. There are maximum of two SMFs (i.e. V-SMF of the hosting operator and H-SMF of the participating operator) controlling a PDU session.

5G MOCN also supports the following sharing scenarios involving non-public networks, i.e.NG-RAN can be shared by any combination of PLMNs, PNI-NPNs (with CAG), and SNPNs (each identified by PLMN ID and NID).

NOTE 2:	PNI-NPNs (without CAG) are not explicitly listed above as it does not require additional NG-RAN sharing functionality compared to sharing by one or multiple PLMNs.

In all non-public network sharing scenarios, each Cell Identity as specified in TS 38.331 [28] is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG); or

-	one or multiple PLMNs only.

NOTE 3:	This allows the assignment of multiple cell identities to a cell and also allows the cell identities to be independently assigned, i.e. without need for coordination, by the network sharing partners, between PLMNs and/or non-public networks.

NOTE 4:	Different PLMN IDs (or combinations of PLMN ID and NID) can also point to the same 5GC. When same 5GC supports multiple SNPNs (identified by PLMN ID and NID), it is up to the operator's policy whether they are used as equivalent SNPNs for a UE.

NOTE 5:	There is no standardized mechanism to avoid paging collisions if the same 5G-S-TMSI is allocated to different UEs by different PLMNs or SNPNs of the shared network, as the risk of paging collision is assumed to be very low. If such risk is to be eliminated then PLMNs and SNPNs of the shared network needs to coordinate the value space of the 5G-S-TMSI to differentiate the PLMNs and SNPNs of the shared network.

Figure 5.18.1-1: A 5G Multi-Operator Core Network (5G MOCN) in which multiple CNs are 
connected to the same NG-RAN

Figure 5.18.1-2: Indirect Network Sharing in which multiple participating operators' CNs connect to hosting operator's CN to share NR

NOTE 6:	Not all interfaces between the hosting operator and the participating operator are depicted in the Figure 5.18.1-2 for simplicity.

NOTE 7:	For the sake of clarity, SEPPs are not depicted in the Figure 5.18.1-2.

# * * * * Next change * * * *

### Alt 1:

### 5.18.5	Network Sharing and Network Slicing

As defined in clause 5.15.1, a Network Slice is defined within a PLMN or SNPN. Network sharing is performed among different PLMNs and/or SNPNs.

In the case of 5G MOCN, each PLMN or SNPN sharing the NG-RAN defines and supports its PLMN- or SNPN- specific set of slices that are supported by the common NG-RAN.

In the case of Indirect Network Sharing, the differences of Network Slicing handling is performed as the follows:

-	If, in an Indirect Network Sharing area, the selected PLMN ID which is broadcasted by shared RAN is the HPLMN ID or EHPLMN ID (which is not derived from UE's SUPI) of the UE, from the network side, the network slicing handling shall be performed as defined in clause 5.15.4 for Registration following non-roaming aspect and clause 5.15.6 for PDU session management following roaming aspect. The changes compared to baseline operations are as follows:

-	During Registration procedure, the Requested NSSAI (optionally with the mapping of Requested NSSAI) only including the S-NSSAI(s) of participating operator from the UE is as described in TS 24.501 [47]. After receiving the Registration Request including the above information, the serving AMF determines the corresponding S-NSSAIs of hosting operator based on the Subscribed S-NSSAIs and slice mapping information from the HPLMN S-NSSAI(s) to the VPLMN S-NSSAI(s) locally or acquires the slice mapping information from the NSSF of hosting operator's network. The serving AMF further checks whether the corresponding S-NSSAIs of hosting operator is supported, by the shared RAN. If none of the corresponding S-NSSAIs of hosting operator are supported the serving AMF rejects the Registration Request. Otherwise the serving AMF continues the Registration procedure and sends to the UE Allowed NSSAI, Partially Allowed NSSAI, Rejected S-NSSAIs and Configured NSSAI only containing the S-NSSAI(s) of participating operator, i.e. HPLMN S-NSSAI(s) without mapping information (i.e. when the selected PLMN ID is HPLMN ID) or EHPLMN S-NSSAI(s) with mapping information to HPLMN S-NSSAI(s) (i.e. when the selected PLMN ID is EHPLMN ID which is not derived from SUPI and different S-NSSAI values from the HPLMN S-NSSAI(s) are used as specified in clause 5.15.4). The serving AMF creates a list of S-NSSAIs of the hosting operator corresponding to the S-NSSAIs included in the Allowed NSSAI sent to the UE and includes these S-NSSAIs in the Allowed NSSAI in the signalling to the shared RAN and to the PCF during the AM policy association establishment. The UE Context in the serving AMF shall retain the slice mapping information. The above AMF functionality is applied also during UE Configuration Update procedure.

NOTE 1:	In the case of Indirect Network Sharing, to support EHPLMN ID broadcast option, as deployment requirement, the S-NSSAI value supported by the EHPLMN is same as HPLMN.

-	During PDU Session Establishment procedure, the UE includes the S-NSSAI(s) of participating operator in the PDU Session Establishment Request as described in TS 24.501 [47]. After receiving the S-NSSAI of participating operator from UE, the serving AMF uses the slice mapping information stored in the UE Context to determine the S-NSSAI of hosting operator (i.e. VPLMN S-NSSAI) to be used as "the S-NSSAI belonging to the Allowed NSSAI" as defined in clause 5.15.6. The serving AMF provides VPLMN S-NSSAI and HPLMN S-NSSAI to the V-SMF of hosting operator for the following PDU Session Establishment Procedure. In addition, the serving AMF provides the selected PLMN (e.g. HPLMN or EHPLMN) S-NSSAI info (with or without mapping information between S-NSSAI of the EHPLMN and HPLMN) to the V-SMF to formulate the PDU Session Establishment Accept message. The V-SMF uses the VPLMN S-NSSAI for the user plane handling (NG-RAN, V-UPF) in hosting network.

NOTE 2:	If NSSF is involved in the above cases, the AMF will send HPLMN S-NSSAI(s) to the NSSF to retrieve the slice mapping information from the HPLMN S-NSSAI(s) to the VPLMN S-NSSAI(s).

NOTE 3:	In the case of Indirect Network Sharing, HPLMN/EHPLMN S-NSSAI is corresponding to S-NSSAI of participating operator and VPLMN S-NSSAI is corresponding to S-NSSAI of hosting operator.

NOTE 4:	In the case of Indirect Network Sharing, the shared RAN only needs to support the S-NSSAI of hosting operator and select the serving AMF as described in clause 5.15.5.2.1. Then, the shared RAN does not need to be provisioned with the slice mapping information. The serving AMF with the above functionality will be selected in this case.

-	For other cases, i.e. the broadcast PLMN ID which represents the participating operator is different from HPLMN ID or EHPLMN IDs of UE, the network slicing handling including UE and network shall comply with the roaming case as defined in clause 5.15.6. The applied S-NSSAI values for the PLMN ID selected by UE (i.e. the broadcast PLMN ID) are the S-NSSAI values of hosting operator.

NOTE 5:	Which broadcast option to be used and whether to broadcast one or more PLMN ID(s) for each participating operator is determined based on the agreement between the hosting operator and participating operator.

### Alt 2:

### 5.18.5	Network Sharing and Network Slicing

As defined in clause 5.15.1, a Network Slice is defined within a PLMN or SNPN. Network sharing is performed among different PLMNs and/or SNPNs.

In the case of 5G MOCN, each PLMN or SNPN sharing the NG-RAN defines and supports its PLMN- or SNPN- specific set of slices that are supported by the common NG-RAN.

In the case of Indirect Network Sharing, the differences of Network Slicing handling is performed as the follows:

-	If, in an Indirect Network Sharing area, the selected PLMN ID which is broadcasted by shared RAN is the HPLMN ID or EHPLMN ID (which is not derived from UE's SUPI) of the UE, from the network side, the network slicing handling shall be performed as defined in clause 5.15.4 for Registration aspect and clause 5.15.6 for PDU Session handling. The changes compared to baseline operations are as follows:

-	During Registration procedure, the Requested NSSAI (optionally with the mapping of Requested NSSAI) only including the S-NSSAI(s) of participating operator from the UE is as described in TS 24.501 [47]. After receiving the Registration Request including the above information, the serving AMF determines the corresponding S-NSSAIs of the selected PLMN based on the Subscribed S-NSSAIs and slice mapping information from the HPLMN S-NSSAI(s) to the selected PLMN S-NSSAI(s) as specified in clause 5.15.4 if needed. The serving AMF continues the Registration procedure and sends to the UE Allowed NSSAI, Partially Allowed NSSAI, Rejected S-NSSAIs and Configured NSSAI only containing the S-NSSAI(s) of the selected PLMN..

NOTE 1:	In the case of Indirect Network Sharing, to support EHPLMN ID broadcast option, as deployment requirement, the S-NSSAI value supported by the EHPLMN is same as HPLMN.

-	During PDU Session Establishment procedure, the UE includes the S-NSSAI(s) of the selected PLMN (representing the participating operator) in the PDU Session Establishment Request as described in TS 24.501 [47]. After receiving the S-NSSAI of the selected PLMN (representing the participating operator) from UE, the serving AMF determines the S-NSSAI of the selected PLMN (i.e. VPLMN S-NSSAI) to be used as "the S-NSSAI belonging to the Allowed NSSAI" as defined in clause 5.15.6. The serving AMF uses VPLMN S-NSSAI and HPLMN S-NSSAI for the following PDU Session Establishment Procedure.

NOTE 2:	If NSSF is involved in the above cases, the AMF will send HPLMN S-NSSAI(s) to the NSSF to retrieve the slice mapping information from the HPLMN S-NSSAI(s) to the VPLMN S-NSSAI(s).

NOTE 3:	In the case of Indirect Network Sharing, HPLMN/EHPLMN S-NSSAI is corresponding to S-NSSAI of participating operator.

NOTE 4:	In the case of Indirect Network Sharing, the shared RAN supports the S-NSSAI(s) of hosting operator and also the SNSSAI(s) in the PLMNs representing the participating operator and select the serving AMF as described in clause 5.15.5.2.1.

-	For other cases, i.e. the broadcast PLMN ID which represents the participating operator is different from HPLMN ID or EHPLMN IDs of UE, the network slicing handling including UE and network shall comply with the roaming case as defined in clause 5.15.6. The applied S-NSSAI values for the PLMN ID selected by UE (i.e. the broadcast PLMN ID) are the S-NSSAI values of hosting operator.

NOTE 5:	Which broadcast option to be used and whether to broadcast one or more PLMN ID(s) for each participating operator is determined based on the agreement between the hosting operator and participating operator.

# * * * * End of changes * * * *