3GPP SA WG2 Meeting #166	S2-2411558

Orlando, FL,USA, 18-22 November 2024

# Title:	[Draft] Reply LS on AIML data collection

# Response to:	LS on AIML data collection (RP-242389/S2-2411455)

# Release:	Release 19

# Work Item:	NR_AIML_Air

Source:	SA2

To:	TSG SA, TSG RAN

Cc:	SA3, SA5, RAN1, RAN2, RAN3

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 thanks TSG RAN for the LS on AIML data collection in S2-2411455, SA2 discussed the analysis of the different data collection options for UE-side model training described in Table 7.2.1.3.2-1, SA2 performed an initial evaluation of the feasibility of option 1b, option 2 and option 3 based on the RAN#105 way forward and the requirements mentioned in LS S2-2411455 and agreed the following feedback:

Option 1b:

It is possible to reuse the Data collection from the UE Application defined in clause 6.2.8 in TS 23.288 with further enhancements to support option 1b. It is feasible to support the MNO full controllability and full visibility for option 1b based on this solution. SA2 needs to study further how to tailor and enhance the existing solution defined in clause 6.2.8 in TS 23.288 based on RAN’s requirement.

Option 2:

If the first termination entity is the LMF, both CP and UP solutions are feasible to support UE data collection for AI/ML based positioning. The existing security and user data privacy procedure in TS 23.273 can be reused for option 2. The MNO controllability and visibility for both CP and UP solutions are feasible. The LPP protocol is used for data collection for both CP and UP solutions. SA2 needs to study the enhancements to support the AI/ML based positioning feature based on the existing procedure defined in TS 23.273.

However, for CSI and beamforming use cases, there is no existing network entity in 5GC to support the UE data collection termination and there is no existing procedure to support UE data collection via either CP or UP connection. SA2 needs to study which network entity is the first termination entity in 5GC and how to support the UE data collection procedure via either CP or UP solution for option 2 to fulfil RAN’s requirement.

Option 3:

There is no existing UP solutions for option 3, the discussion of UP solution for option 3 requires SA2’s involvement. The feasibility on how to support the CP solution for option 3 should be discussed in SA5.

2. Actions:

To TSG SA, TSG RAN

ACTION: 	SA2 asks TSG SA and TSG RAN to take above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #AH		24-28 January 2025	Electronic meeting

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece