SA WG2 Meeting #166	S2-2411683

November 18 – 22, 2024, Orlando, US

Source:	NTT DOCOMO

Title:	Discussion on clock capability of Ambient IoT devices

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Proposing discussion around available clocks in an Ambient IoT device.

# Introduction

During SA2#165 Hyderabad, some interim conclusions were reached and documented in TR 23.700-13.

Clause 8.3.2 comprises an Editor’s note regarding support of enabling temporary disabled AIoT devices.

“

### 8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

“

This paper aims at providing discussion on how temporary disable/enable can be supported in this release instead of potentially shifting the functionality to a future release.

# Discussion

#### Available clocks in an Ambient IoT device

TR 38.769 describes clock functionality available in an Ambient IoT Device in Table 5.2.3-1. Purpose #3 of the clock is time counting.

Observation 1: The Ambient IoT device can count time.

TR 23.700-13 contains an Interim conclusion to support write operation for Ambient IoT devices.

### 8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

Observation 2: Based on observation 1, the Ambient IoT device could support temporary disable based on receiving a timer value in a write command and start a clock once the timer expires, the Ambient IoT device is enabled again.

# Proposal

Support disable/enable of Ambient IoT devices in Release-19 using a write operation with a timer value. In addition, if necessary, send an LS to RAN1 confirming the clock in Table 5.2.3-1 of TR 38.769 is suitable for the functionality of enable/disable. A draft LS is available in S2-2411687.