SA WG2 Meeting SA2#166	S2-2411699

18-22, November 2024, Orlando, USA     	(Revision of S2-2410198)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5741 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UIA support of maximum number of Device IDs and traffic association |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One of the objectives of the UIA_ARC WID is to support the optional restriction within the 5GS on maximum number of simultaneously active Device ID per UE/5G-RG for devices whose traffic need to be differentiated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to support this featrue, description about the following aspects are added in TS 23.501, including - General about non-3GPP device identifier - Association between non-3GPP device traffic and non-3GPP device identifier - Restriciton of the maximum number of simultaneously active Device ID per UE/5G-RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction of max number of simultaneously active Device ID can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.A (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

## 5.x	Support for Non-3GPP Device Identifier

### 5.x.1	General

This clause specifies the scenario of a non-3GPP device connecting through the UE. In this scenario QoS differentiation of traffic is applied to the traffic that originates from or is directed to the non-3GPP device. The non-3GPP device does not use NAS and is not authenticated by 5GC.

The support of identification of traffic for non-3GPP devices connecting behind a 5G-RG is specified in TS 23.316 [84].

The Non-3GPP Device Identifier is unique within the scope of the UE’s SUPI.

### 5.x.2 Traffic identification

When a non-3GPP device is connecting to the UE, the UE may bind the Non-3GPP Device Identifier to a non-3GPP device, for the traffic of non-3GPP devices that require differentiated QoS. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1: How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific.

NOTE 2: At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information.

### 5.x.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS:

-	For Ethernet PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

-	For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send the IP Address and/port ranges associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

-	For IPv6 PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send the IPv6 Address/prefix(sub) that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

Editor’s note: 	whether the the UE sends the Non-3GPP Device Identifier and user plane information (e.g. IP Address) in PDU session establishment is FFS.

### 5.x.4	QoS differentiation

QoS differentiation and policy control is defined in TS 23.503 [45] clause 6.1.3.xx

#### 5.x.A	Maximum Number of Simultaneously Active Non-3GPP Device Identifier

The UE or 5G-RG, with the support of Non-3GPP Device Identifier, receives a list of provisioned non-3GPP Device Identifiers and the maximum number of simultaneously active Non-3GPP Device Identifiers, based on the UE or 5G-RG’s non-3GPP device information stored in UDR. When supported, the UE or 5G-RG can bind a specific Non-3GPP Device Identifier to a particular non-3GPP device.

The UE or 5G-RG shall enforce that a Non-3GPP Device Identifier is only actively used by one Non-3GPP device at any given time. Upon connection of a Non-3GPP device intended to be associated with a Non-3GPP Device Identifier, the UE or 5G-RG determines if any available Non-3GPP Device Identifiers can be assigned to the newly connected non-3GPP device.

If the maximum number of simultaneously active Non-3GPP Device Identifiers is reached, or the UE/5G-RG will not have any available Non-3GPP Device Identifiers to assign, the UE or 5G-RG will either deny further non-3GPP device connections requesting Non-3GPP Device Identifier or routes the traffic through the PDU Session that matches the default URSP rule based on the policy to support backward compatibility.

***** End of Changes *****