3GPP SA WG2 Meeting #166	 S2-2411719

# Orlando, FL,USA, 18-22 November 2024								                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0495 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to remove EN for multi-hop UE-to-UE Relay of IP PDU type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several Editor’s Notes for the multi-hop UE-to-UE Relay operations. They need to be resolved for the completion of the work item. In clause 5.8.6.2, the following editor’s note can be removed, as there was no additional information identified for the multi-hop UE-to-UE Relay discovery operation: Editor's note: Additional information or identifiers for MANET operation assistance may be added. In clause 6.4.3.10.1, there is an editor’s note: Editor's note: It is FFS if the Layer-2 link management messages need to be enhanced to support efficient MANET operations. Given that the layer-2 link management procedures already provides reliable link status monitoring for the connection between the UE-to-UE Relays, the periodical HELLO messages is not strictly required. Therefore, it is suggeted that the correponding timers, e.g. HELLO_INTERVAL, VALIDITY_TIME and REFRESH_INTERVAL, should be set with larger values to reduce the periodical HELLO messages. The responsive HELLO message triggered by a change event would not be affected. Thus the UE-to-UE Relays can still be updated timely. Additionally, it is also clarified that each of the layer-2 link can be treated as a MANET interface for the relay, and that can clarify the HELLO message generation logic. In clause 6.7.5.2.1, there is an editor’s note: Editor's note: It is FFS if other enhancements needs to be introduced to support the efficient propagation of the User Info ID and IP address of the 5G ProSe End UE. Currently, there is already PC5-S signalling defined for layer-2 link modification. It is possible to be reused for DNS entry propagation. Additional, it was also agreed in study conclusion that a MANET message for this purpose can be supported if defined by stage 3. Therefore, a NOTE is added to replace the editor’s note to clarify the DNS propagation aspect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the ENs related to the multi-hop UE-to-UE Relay of IP PDU type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for multi-hop UE-to-UE Relay operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.2; 6.4.3.10.1; 6.7.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.8.6.2	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

The Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is sent by the relay and contains the following information:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID. It is used for the transmitting the message at AS layer, and included in the AS layer headers only.

-	Destination Layer-2 ID: the Destination Layer-2 ID for Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: provides information about the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay provides to the 5G ProSe End UEs.

The 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE or a 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, and includes the following information:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE or 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

-	(optional) User Info ID of discoverer: this may be used by the receiver of the message to determine whether to respond to the solicitation.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s).

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoverer: the User Info ID from the received Solicitation message, to ensure the response can be matched at the receiver.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe Multi-hop UE-to-UE Relay.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

# >>>> NEXT CHANGES<<<<

##### 6.4.3.10.1	Layer-2 Link establishment and management for IP PDU type

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.5.2:

-	The Direct Communication Request message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User info (i.e. Application Layer ID) of source 5G ProSe End UE or 5G ProSe Multi-hop UE-to-UE Relay: the identity of the source 5G ProSe End UE or 5G ProSe Multi-hop UE-to-UE Relay requesting the connection establishmet.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the 5G ProSe Multi-hop UE-to-UE Relay provided during the 5G ProSe Multi-hop UE-to-UE Relay Discovery procedure.

-	(optional)ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe Multi-hop UE-to-UE Relay as requested.

-	Security Information: the information for the establishment of security for the PC5 link establishment (see TS 33.503 [29]).

The Direct Communication Accept message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User Info ID of 5G ProSe UE-to-UE Relay.

Other link management procedures as defined in clause 6.4.3 (i.e. Link identifier update in 6.4.3.2, Layer-2 link release in clause 6.4.3.3, Layer-2 link modification in clause 6.4.3.4, Layer-2 link maintenance in 6.4.3.5) can be used for the layer-2 link without enhancements.

When a new layer-2 link is established, or an existing layer-2 link is released, the 5G ProSe Multi-hop UE-to-UE Relay need to inform the status change internally to the MANET router stack.

Each of the established layer-2 link with another 5G ProSe UE-to-UE Relay is treated as a MANET interface as defined in RFC 6130 [36]. The layer-2 links of the same 5G ProSe UE-to-UE Relay may share the same IP address, as they are isolated, i.e. each of the layer-2 link only allows communication with one peer 5G ProSe UE-to-UE Relay.

The 5G ProSe UE-to-UE Relay may use a large value for HELLO_INTERVAL, VALIDITY_TIME and REFRESH_INTERVAL as defined in RFC 6130 [36] to reduce the periodic HELLO messages. The 5G ProSe UE-to-UE Relay can rely on the 5G ProSe link management procedures defined in clause 6.4.3 for maintaining the information in Neighbor Information Base.

# >>>> NEXT CHANGES<<<<

##### 6.7.5.2.1	Layer-2 link establishment via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays (IP type)

Figure 6.7.5.2.1-1 shows the Layer-2 link establshment procedure via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays. This procedure is used by an 5G ProSe End UE to connect to the relay cloud, or for a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay to connect to other 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays.

Figure 6.7.5.2.1-1: 5G ProSe Communication via IP type of 5G ProSe Multi-hop UE-to-UE Relay

0.	The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and the other 5G ProSe End UEs have already established IP type of connections. The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and 5G ProSe End UEs may have used the discovery procedure as defined in clause 6.3.2.6.2, and established the Layer-2 link as defined in clause 6.4.3.10.1.

1.	The 5G ProSe End UE performs the 5G Multi-hop UE-to-UE Relay discovery procedure as described in clause 6.3.2.6.2, and finds the 5G Multi-hop UE-to-UE Relay-1.

2.	The 5G ProSe End UE establishes an IP based Layer-2 link with the 5G ProSe Multi-hop UE-to-UE Relay-1 as described in clause 6.4.3.10.1.

Depends on the configuration, the 5G ProSe End UE may obtain an IP address/Prefix from the 5G ProSe Multi-hop UE-to-UE Relay-1. Otherwise, the 5G ProSe End UE informs the 5G ProSe Multi-hop UE-to-UE Relay-1 of its pre-configured IP address/prefix.

The 5G ProSe End UE may also configure the corresponding services and PC5 QoS Flows over the Layer-2 like with the 5G ProSe Multi-hop UE-to-UE Relay-1. The 5G ProSe End UE may use the Layer-2 link modification procedure as defined in clause 6.4.3.4 to add/modify/remove PC5 QoS Flows corresponding to the services and link status.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 updates its records of the 5G ProSe End UEs and may trigger the MANET signalling, e.g. OLSRv2 signalling [35], to update other connected 5G ProSe Multi-hop UE-to-UE Relays. This will result in a routing table on the relays that allows other UEs to reach the newly connected 5G ProSe End UE.

The 5G ProSe Multi-hop UE-to-UE Relay may also trigger signalling to update the DNS entries in other connected 5G ProSe Multi-hop UE-to-UE Relays for the 5G ProSe End UE.

NOTE :	The signalling for DNS entry update can use either the layer-2 link modification procedure with additional container of DNS entries, or the enhanced MANET message described in clause 4.2.8A.

4.	The 5G ProSe End UE may perform a DNS query for a target 5G ProSe End UE, if it does not know the IP address/prefix of the target. The DNS query may stop at the 5G ProSe Multi-hop UE-to-UE Relays that has the entry for the target 5G ProSe End UE.

5G ProSe End UE starts to communicate with the target 5G ProSe End UE using IP based communications. The 5G ProSe Multi-hop UE-to-UE Relays forwards the packets based on the routing table established with the MANET protocols.

# >>>>END OF CHANGES<<<<