3GPP TSG-WG SA2 Meeting #166 	S2-2411804

18 - 22 November, 2024, Orlando, USA	(revision of S2-24xxxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: I-SMF selection comparation and way forward

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract: there are two proposals on I-SMF insertion in S2#164 meeting, this document compares the two approaches and proposes a way forward for I-SMF selection.

1. Introduction

For KI#1 on I-SMF selection two types of solution were proposed in previous two meeting:

- AMF triggered I-SMF selection (S2-2409762, S2-2410294). AMF triggers I-SMF selection based on SMF selection subscription data and UE location.

- SMF triggered I-SMF selection (S2-2410604). SMF triggers AMF to select I-SMF based on SM subscription data and the target DNAI received in PCC rule.

This paper compares the impact of these two solutions and give a suggestion on which solution is to be selected.

2. Discussion

2.1 Solution Observation

Option 1: AMF based solution:

Figure 1: AMF triggered I-SMF insertion during PDU session establishment(S2-2410294)

As shown in Figure 1, when the AMF receives Local Offloading Allowed Indicator from subscription data, the AMF can trigger local I-SMF insertion per UE location.  After the local I-SMF insertion, the interaction between local I-SMF and SMF is similar as the existing HR-SBO mechanism. The existing HR-SBO solution can be used as a reference to design the I-SMF based Local Offloading Management solution.

[Observation 1]:   AMF based approaches is similar as the existing HR-SBO solution. Based on the subscription data and UE location AMF determine whether the local I-SMF is to be inserted.

Option 2: SMF based solution:

Figure 2: SMF triggered I-SMF insertion during PDU session establishment(S2-2410600)

As shown in Figure 2, AMF selects the SMF first. When the SMF receives the Local Offloading Allowed Indicator from SM subscription data, the SMF indicate this to PCF. And PCF provides a list of DNAI to the SMF. Then SMF determines a target DNAI and notify it and additional local offloading indication to the AMF. Based on that information, AMF triggers local I-SMF insertion. After the I-SMF insertion, the interaction between I-SMF and SMF is similar as the AMF based solution.

[Observation 2]:  SMF based approach is based on SM subscription data and DNAI list received from PCC rule. Per UE location, PCC rule information, SMF determine a target DNAI and trigger AMF to insert a local I-SMF.

2.2 Solution Analysis

From above observation, it can be seen the difference between two mechanism is on which NF triggers I-SMF insertion, i.e., AMF trigger or SMF trigger.

Option 1: AMF trigger I-SMF selection

During the discussion at S2#165 meeting, one question related to AMF based mechanism is raised:

Q1: Why the I-SMF is needed as the SMF can serve current UE location?

A1: The intention of KI#1 is to reduce the impact of central SMF. Hence in the network deployment, the central SMF may not support all the DNAI in its service area, i.e. the DNAI support by the UPF connected to this SMF service area may be not supported by the SMF.

For example, SMF1 service area includes TA1/TA2/TA3/TA4. The UPF service area and DNAI relation can be: UPF1(TA1/TA2/DNAI1), UPF2(TA2/TA3/DNAI1), UPF3(TA3/DNAI2), UPF4(TA4/ DNAI3). So all the DNAI supported in the SMF service area, i.e. TA1/TA2/TA3/TA4, are:  DNAI1/DNAI2/DNAI3.  However, if the SMF does not support to control UPF3, then even SMF support TA3 as part of its service area but DNAI2 is not include in its NF profile. Hence SMF1 does not support all DNAI in its service area.

There is another SMF2, its service area is TA3/TA4. It supports to control UPF2/UPF3/UPF4. Hence the SMF2 support DNAI1/DNA2/DNAI3 in its NF profile and support all the DNAI in its service area, i.e. TA3/TA4.

During the PDU session establishment procedure, the SMF1 is selected as the anchor SMF and UE camps in TA3, SMF2 acting as I-SMF is also inserted into the path. This is to support potential traffic offloading via DNAI2.

So even the UE is within the SMF service area but SMF not support all DNAI within its service area, an I-SMF, which serves the UE location and support all DNAI within its service area, is selected for Local Offloading Management.

By doing this, it can be seen if the UE is within the I-SMF service area, as all DNAIs within this service area are supported by the I-SMF, regardless traffic is to be offloaded on which DNAI, it is always possible for I-SMF to handle it. Hence the SMF can offload the EC handling to the I-SMF and reduce the load of central SMF, e.g., SMF1 offload EC handling to SMF2.

From the discussion last meeting, it seems this part is unclear. It is proposed to clarify this in the updated paper.

[Propoasl 1] If the UE is within the SMF service area but the selected SMF does not support all the DNAI within its service area, or the UE is outside the SMF service area, AMF select/re-select an I-SMF to support Local Offloading Management in case Local Offloading Management is allowed in the SMF selection subscription data.

Option 2: SMF trigger I-SMF selection

For the SMF based solution, the I-SMF is inserted due to the target DNAI is not supported by the SMF.

The SMF determine one target DNAI and notify it to AMF. It can be assumed that when the SMF notifies the Target DNAI to AMF, the target DNAI need be associated with the UE current location, i.e. based on this target DNAI, it can select one I-SMF which can serve both this DNAI and UE current location. Otherwise, this notification to the AMF is useless, e.g. the I-SMF support the target DNAI but not serves UE current location.

If we want to use this mechanism at least below issue need be considered:

- SMF may receive a list of target DNAI from PCF. Currently the DNAI parameter within PCC rule are only related to Application Function influence on traffic routing Enforcement Control (refer to table 6.3.1 of TS23.503).

If this information is injected by AF, then for local offloading deployment does operator need AF always interact with PCF first? How about if this information is provisioned via the EDI, i.e. no PCF notification?

Whether this information can be preconfigured at the PCF? Normally the preconfiguration is only for static information. For one Application the related information can be changed dynamically, e.g. location of EAS due to DC on/off. So the preconfiguration should not be regarded as the only way, i.e. other provisioning mechanism always need be considered. The information at the PCF should not only relied on the preconfiguration.

- How the SMF determine the target DNAI? As mentioned above the target DNAI need be UE location related. Hence it can be assumed that SMF at least need be aware two information: a) the mapping informatoin between the location (e.g. TA/cell) and DNAI; b) the UE current location.

If the SMF maintain the mapping information, the SMF need be always keep updated with the information of mapping beween location and DNAI, e.g. an DNAI is added to (or removed from) the network. Also the SMF need be aware the UE specific location information, e.g. the subscription of AOI information. Due to the different DNAI may be associated with different AOI, the SMF also need determine whether the subscribed AOI need be updated and if yes update the UE location subscripton information.

- Mobility handling. When the UE moves from one I-SMF to another I-SMF area, the target DNAI may be updated, e.g. from DNAI-1 to DNAI-2.  There are two cases:

  - UE is still in the SMF service area. In this case does the new I-SMF-2 can only be inserted after the notification of DNAI-2 from SMF? Similar can also happen if the UE move from no I-SMF service area to location with I-SMF service area.

  - UE is out of the SMF service area. In this case if per existing ETSUN procedure the inserted I-SMF2 to cover the UE current location can not serve the DNAI-2 as requesed by SMF, how to handle it? AMF triggers I-SMF rellocation?

[Observation 3]: For the SMF based solution, above three issue need be considered/clarified: 1) whether the DNAI list can only be generated by PCF; 2) how the SMF determines target DNAI.  3) how to support UE mobility.

The motivation of the KI#1 is to reduce the impact of the central SMF, i.e., the anchor SMF. The impact on the anchor SMF on two solutions can be listed as following:

- For AMF based solution, as shown in Figure 1, the information corresponding to I-SMF service area can be learnt/stored by AMF same as before, and the selection/insertion of I-SMF are controlled by AMF. The anchor SMF only needs authorize the request for local offloading per subscription data. The impact to anchor SMF is minimum.

For AMF based approaches, the mapping between location and DNAI is maintained by the I-SMF. AMF only need discover the related I-SMF. And the UE current location is aware by the AMF. Hence there are no impact on the anchor SMF on this aspect.

For UE mobility handling, with the additional parameter for I-SMF discovery, it will always be found by AMF directly during the normal mobility handling. I-SMF reallocation is not needed.

- For SMF based solution as analzed above, the anchor SMF needs to determine when to notify the AMF to do the I-SMF insertion. This requires the subscription of AOI information and mapping table maintained by the SMF. Also some new procedure for mobility handling, e.g. AMF triggers I-SMF reallocation, retrieval of the DNAI list besides PCF, etc are also need be considered.

[Oberservation 4]: Comparing above two solutions, the AMF based approaches has less impact to the central SMF, i.e., anchor SMF, and system impact.

3. Proposal

Based on the above-mentioned analysis, it is clear that the AMF based approach show clear advantage comparing to SMF based approach and does reduce the EC handling burden of central SMF, i.e., the target of KI#1. Hence we suggest to go with AMF based approaches on how to establish PDU session.

The related updated paper, S2-2411592/11593/11594 are also provided.

[Conclusion]: For the I-SMF insertion, it is suggested to go with AMF based mechanism. After the I-SMF insertion, the HR-SBO based mechanism can be reused for local offloading handling.