3GPP SA WG2 Meeting #166	S2-2411975

Orlando, USA, November 18 – 22, 2024	(revision of S2-24xxxxx)

Source: 	LG Electronics

Title: 	New solution for AIML UE data collection

Document for: 	Approval

Agenda Item: 	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This paper proposes to add a new solution for AIML UE data collection in TR 23.700-84.

1	Discussion

RAN LS (S2-2411455/RP-242389) lists various data collection options for UE-side model training (Options 1a, 1b, 2 and 3). It is considered that SA2 needs to investigate these options in detail from system perspective to provide inputs on options 1b, 2, and 3 as complete as possible requested by TSG RAN.

We would like to propose to use TR 23.700-84 that has not yet approved to capture solutions related to the above options.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-84v2.0.0.

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[8]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[9]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[10]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[14]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[x1]	RP-242389, "LS on AIML data collection, RAN to SA", SA2, SA3 and SA5, RAN#105, September 2024.

[x2]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[x3]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

* * * * Start of Next Change * * * *

!! All New Texts !!

Annex Z:
Candidate Solutions for AIML data collection for UE-side model training

Z.1	General

This Annex provides candidate solutions related to [x1].

Z.A	Solution #A: AIML UE data collection corresponding to Option 1b

Z.A.1	Description

ANNEX - TP agreed in RAN2 of [x1] describes Option 1b as below:

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

This solution proposes a solution for AIML UE data collection corresponding to Option 1b by leveraging mechanism that an AF (i.e. Data Collection Application Function (DCAF)) collects data from UE Application specified in clause 6.2.8 of TS 23.288 [5].

Details related to DCAF including the following are described in TS 26.531 [x2]:

4.1 General … The Data Collection AF is responsible for receiving data reports from UEs pertaining to a particular UE data domain, for processing the UE data contained in those reports, and for exposing them to subscribed event consumers in the form of events. Typically, one type of exposed event is based on UE data pertaining to a single UE data domain, but more complex relationships where events are synthesised from several UE data domains are not precluded. … B.2 Baseline UE data domains Table B.1-1 specifies ownership of the baseline UE data domains defined in TS 23.288 [4]. Table B.2-1: Ownership of baseline UE data domains UE data domain; Owner. Service Experience; 5G System (MNO). UE Location; 5G System (MNO). Communication; 5G System (MNO). Performance; 5G System (MNO). Planned Trips; 5G System (MNO). Application-specific UE data domains shall be owned by the ASP.

In order to achieve controllability of MNO on data transfer, the following is proposed:

1)	The DCAF for AIML UE data collection resides inside the MNO domain.

-	The DCAF controls AIML data collection from the UE including initiation, termination, and full management of data transfer from the UE.

-	The DCAF can store AIML data collected from UEs and expose the AIML UE data to the OTT server (e.g. outside the MNO domain), e.g. per request from the OTT server.

2)	Dedicated S-NSSAI and DNN are used for AIML UE data collection.

-	The UE establishes and uses a PDU Session based on the URSP rule related to AIML UE data collection to transfer AIML UE data to the DCAF.

3)	Address of the DCAF for AIML UE data collection is configured in the UE (e.g. based on an FQDN).

-	UE Application supporting AIML UE data transfer establishes a connection to the DCAF over user plane via the PDU Session established for AIML UE data collection. The DCAF communicates with the UE Application and collects AIML UE data from UE Application.

NOTE:	If HPLMN of the UE requires to control UE data collection, it is assumed that address of the DCAF in the HPLMN is configured in the UE. If roaming for UE-side data collection is supported and VPLMN requires to control UE data collection for inbound roaming UE, it is assumed that address of the DCAF in the VPLMN is configured in the UE.

In order to achieve full visibility for standardized data contents in the MNO domain, the following is proposed:

a)	For AIML UE data contents that should be visible in the MNO domain, their encoding and format are defined as an extension of DataReport type specified in TS 26.532 [x3] or similar.

Editor's Note:	The encoding and format for AIML UE data contents that should be visible in the MNO domain require coordination with RAN WGs.

Z.A.2	Procedure

Figure Z.A.2-1: Procedure for AIML UE data collection by DCAF inside the MNO domain

1.	The UE establishes a PDU Session for AIML UE data collection based on the URSP rule.

2.	The UE establishes a connection for AIML UE data collection with the DCAF over user plane via the PDU Session.

3.	The DCAF collects the AIML UE data from UE Application.

4.	The DCAF exposes the AIML UE data to the OTT server, e.g. per request from the OTT server.

Z.A.3	Impacts to Services, Entities and Interfaces

DCAF (in the MNO domain)

-	Collects AIML UE data over user plane.

-	Exposes AIML UE data collected from UEs to OTT server.

UE

-	Establishes a connection for AIML UE data collection with the DCAF over user plane.

-	Transfer AIML UE data to the DCAF.

* * * * End of Changes * * * *