SA WG2 Meeting #S2-166	S2-2411981

# 18 November - 22 November 2024, Orlando, USA

Title:	[Draft] Response LS to RAN3 on FS_XRM Ph2

Response to:	R3-244844/S2-2411282

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	[CATT]3GPP SA2

To:	3GPP RAN3

Cc:

Contact person:	Chunshan Xiong

chushan.xiong@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TS23.501 CR on available data rate]

- Overall description

SA2 thanks RAN3 for the Response LS on FS_XRM Ph2(R3-244844/S2-2411282) and kindly asks RAN3 to consider the following SA2 responses.

- From RAN3 perspective, the control plane based solution providing periodicity per QoS flow as part of the TSCAI is sufficient for semi-static periodicity case. Other potential cases (e.g. dynamic case) need to be clarified by SA2, if any.

Feedback from SA2:

The in-band signaling of the periodicity is not supported in Rel-19.

- For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate.

RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting.

Feedback from SA2:

It is assumed that the available data rate is determined by the available radio resources in the NG-RAN for the GBR QoS Flow. It is assumed that the NG-RAN uses a similar mechanism as the alternative QoS profile to determine the data rate.

How the NG-RAN provides the concrete value of the available data rate is NG-RAN implementation, the available data rate can be any value e.g. below the GFBR (e.g.as the case of "GFBR can no longer be guaranteed" QoS notification), or between GFBR and MFBR, or even above MBFR (e.g., based on the available data rate, the AS/AF can initiate a higher data rate coding or initiate addition XR stream e.g. the haptic stream). How frequently the NG-RAN reports the available data rate is NG-RAN implementation. The available data rate is assumed to be valid until a new data rate is provided by the NG-RAN.

2	Actions

To RAN3:

ACTION: SA2 kindly asks RAN3 to take the above answers into account and provide feedback if any.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #166-Ad Hoc-E	20 January – 24 January 2025                Online

TSG-SA2 Meeting #167		17 February - 21 February 2025         	Athens, GR