3GPP TSG SA WG2#166	S2-2412033

Orlando, USA, Nov 18 – 22, 2024	(was S2-241xxxx)

Source:	Lenovo

Title:	Way Forwards on pending issues for VFL

Document for:	Discussion and Agreement

Agenda Item:	19.15.2

Work Item / Release:	Release 19

Abstract of the contribution:

# 1	Discussion

## 1.1	Introduction

This DP provides way forwards on pending issues for VFL based on discussions and agreements made in SA2#165.

## 1.2	VFL server discovery of VFL clients via NRF

Open Issues:

-	Issue 1: How the VFL server ensures that VFL clients can participate in the VFL process for an Analytics ID are interoperable.

-	Issue 2: Whether Feature information per VFL client is available at NRF

-	Issue 3: Whether VFL server selects VFL clients from the same vendor (i.e. Vendor information available at NRF).

Discussion

For Issue 1: The current "stable" agreement is that the VFL server discovers VFL clients based on VFL capability type which is stored in the NF profile of a VFL client on per Analytics ID granularity. The main issue is that a VFL server cannot use only the VFL capability type and Analytics ID to select VFL clients as the VFL server does not know what models are available at a VFL client and does not know whether the features supported by each VFL clients can be mapped to a label at the VFL server. As such it is proposed to have a two-step approach where at a first step the VFL server first discovers and selects compatible VFL clients via NRF and at a second step the VFL server identifies the Features supported by the selected VFL clients during the VFL preparation phase.

Proposed way forwards for issue 1:

-	NF profile includes an additional identifier "VFL interoperability identifier" on per Analytics ID that is used by the VFL server to select compatible VFL clients. How the VFL server/clients are aware/configured of the VFL interoperability identifier is up to implementation.

-	A two-step approach is proposed where at a first step the VFL server first discovers and selects compatible VFL clients via NRF and at a second step the VFL server identifies the Features supported by the selected VFL clients during the VFL preparation phase

For Issue 2: While Feature information can be also available at NRF to simplify the NRF implementation it is proposed to include within the NF profile, only VFL interoperability information

Proposed way forwards for issue 2:

-	Feature information is not included in NF profile but exchanged during VFL preparation phase.

For Issue 3: Vendor information is an alternative method to ensure the VFL server selects compatible VFL clients. However such approach is too restrictive and prefer to have a general "multi-vendor" solutions

Proposed way forwards for issue 3:

-	Vendor information is implicitly supported by supporting a VFL interoperability identifier within the NF profile

## 1.3	VFL server selection of VFL clients during VFL preparation phase

Open Issues:

-	Issue 4: Whether VFL server takes into account Feature information that is exchanged during preparation phase and what information is exchanged as part of Feature information.

-	Issue 5: Whether interoperability information is exchanged during preparation phase (and how)

-	Issue 6: Whether VFL server provides initial ML model to VFL clients

Discussion

For Issue 4: Feature information is essential for the VFL server as the VFL server needs to be aware of the Feature(s) supported by each VFL client to determine how to map intermediate results provided by the VFL client to a label. Given that Features may correspond to VFL specific functionality it is proposed not to standardise the contents of the features but allow VFL server and clients to exchange metadata via a Feature Information container or via defining a Feature ID (which is not standardised).

Proposed Way Forwards for Issue 4:

-	VFL server discovers the Feature(s) supported by each VFL client during VFL preparation phase

-	VFL server/clients exchange feature information as metadata via a Feature information container or using a Feature ID. The metadata/Feature ID are not standardised.

For Issue 5: As per section 1.1, Interoperability Information is information needed by the VFL server that the models/features available at the VFL client are compatible to be used for the VFL training. It is preferred that such information is available at the NRF so as the VFL server to select compatible VFL clients.

Proposed Way Forwards for Issue 5:

-	Interoperability information (i.e. information for the VFL server to select compatible VFL clients) is available at NF profile and it is not needed for VFL preparation phase

For Issue 6: The main assumption for Vertical Federated Learning as that VFL server and clients are not aware of the ML models available. If the VFL server provides the initial ML model at each VFL client it is not clear how the VFL server determines what initial ML model should be sent to each VFL client given that each VFL client will support different Feature(s) and may have access to different data. The use case for such approach need to be clarified.

Proposed Way Forwards for Issue 6:

-	Do not support initial ML model exchange given the complexity of the VFL server to determine what initial ML model is needed at each VFL client.

## 1.4	Role of NEF during sample alignment

Open Issues:

-	Issue 7: Whether NEF can assist in VFL client selection when AF is VFL server

-	Issue 8: Whether NEF can assist in sample alignment when AF is VFL server

Discussion

For Issue 7: This issue depends on what information is available at NF profile which depends on the conclusion of Issue 1 (VFL interoperability). If VFL interoperability information is available at the NF profile, the NEF can assist in the selection of VFL clients ensuring that the VFL server receives a list of interoperable VFL clients only and thus avoids sending unnecessary/sensitive core network information to 3rd party AFs. For this to be supported the consumer includes in the Discovery Request VFL interoperability information and the NEF responds with a list of clients supporting the same interoperability information.

NOTE: If NF profile includes VFL interoperability identifier no additional NEF involvement is needed.

Proposed Way Forwards for Issue 7:

-	If NF profile includes interoperability information NEF assists VFL client selection by responding to the VFL server with a list of interoperable VFL clients.

-	If NF profile includes VFL interoperability identifier within VFL interoperability information no impacts at NEF are expected.

For Issue 8: During VFL preparation the VFL server and VFL clients exchange their supported samples via the NEF. During discussions in SA2#165 it was also proposed the NEF to aggregate all responses from VFL client and provide a single response to the VFL server. In such scenario, the NEF can additionally ensure that the VFL server receives in the single response, a sample list supported by all NWDAF VFL clients. An advantage of such approach is that it allows to avoid sending unnecessary/sensitive core network information to 3rd party AFs.

Proposed Way Forwards for Issue 8:

-	During sample alignment between AF acting as VFL Server and NWDAF VFL clients, if the NEF aggregates all information from VFL clients and provides a single response to the VFL server, the NEF can additionally ensure that the VFL server receives in the single response, a sample list supported by all NWDAF VFL clients.

## 1.5	Procedure of VFL server/VFL client during training

Open Issues:

-	Issue 9: VFL server computation of backward local ML model training information (e.g. gradient information or loss information) based on intermediate result provided by each VFL client.

-	Issue 10: Selection of VFL server for model training

Discussion

For Issue 9: No details are included on how the VFL server computes backward local ML model training information given that, up to now, the VFL server does not know what features are supported by each VFL clients and as such does not know how to map an intermediate result provided by each VFL client to a Label.

Proposed Way Forwards for Issue 9:

-	VFL server takes into account Features supported by each VFL client to compute backwards local model training information.

For issue 10: When the NWDAF AnLF selects an NWDAF MTLF for model training and the NWDAF MTLF determines that the model requires training using VFL. If the NWDAF MTLF does not support VFL then the NWDAF MTLF discovers an NWDAF MTLF capable to carry out VFL (similar procedure as per HFL). The NWDAF MTLF then needs to determine whether the MTLF can train the ML model (i.e. the MTLF has the labels for the Analytics request) or the labels are available at the AF. If it is the latter the MTLF discover an AF operating as VFL server.

After training is complete the MTLF indicates to the AnLF a reference to the distributed ML model by providing the VFL correlation ID.

Proposed Way Forwards for Issue 10:

-	For Model Training if NWDAF MTLF does not support VFL, MTLF discovers an MTLF with VFL capability. If the MTLF with VFL capability determines that the Model needs training by AF, MTLF discovers an AF with VFL server capability.

-	When the ML model is trained the MTLF provides to the AnLF reference to the distributed ML model by providing the VFL correlation ID.

## 1.6	Procedure of VFL server/VFL client during inference

Open Issues:

-	Issue 11: Selection of VFL server during inference

For Issue 11: As per Issue 10, an AnLF when requesting an ML model from the MTLF is not aware of how the ML model is trained. The issue arises is whether the NWDAF AnLF is able to generate inference based on an “distributed” ML model that was trained using VFL as such AnLF will not receive any trained ML model (in contrast to HFL) but will receive an indication that an ML model is available and potentially a VFL correlation ID corresponding to the VFL training process. An NWDAF supporting only AnLF will not be able to use such correlation under the assumption that AnLF is unaware of the training process but will need to discover the VFL Server that is able to carry out inference using the distributed model. For simplicity it is proposed that the MTLF (acting as VFL Server) that trained the ML model is also acting as an AnLF and as such the AnLF triggers the MTLF to initiate inference using the VFL correlation ID.

Proposed Way Forwards for Issue 11:

-	For Inference the MTLF that trained the ML Model is also an AnLF. The AnLF triggers the MTLF that trained the ML model to provide inference using the VFL correlation ID corresponding to the VFL training process.

# 2.	Proposal

Following way forwards proposals are suggested:

During Discovery (addressed in CR1198 CR rev 10 – S2-2412036)

-	NF profile includes an additional identifier "VFL interoperability identifier" on per Analytics ID that is used by the VFL server to select compatible VFL clients. How the VFL server is aware/configured of the VFL interoperability identifier is up to implementation.

-	A two-step approach is proposed where at a first step the VFL server first discovers and selects compatible VFL clients via NRF and at a second step the VFL server identifies the Features supported by the selected VFL clients during the VFL preparation phase

-	For the case where AF is the VFL server: The are no impacts to NEF, VFL client selection is based on VFL interoperability identifier.

During VFL preparation phase (addressed in CR1246 CR rev 5 – S2-2412037):

-	VFL server discovers the Feature(s) supported by each VFL client during VFL preparation phase

-	VFL server/clients exchange feature information as metadata via a Feature information container. The metadata are not standardised.

-	ML Models are not shared between VFL server and clients.

-	For the case where AF is the VFL server, if the NEF aggregates all information from VFL clients and provides a single response to the VFL server, the NEF can additionally ensure that the VFL server receives in the single response, a sample list supported by all NWDAF VFL clients.

During VFL training (addressed in CR1134 CR rev 17 – S2-2412039)

-	VFL server takes into account Features supported by each VFL client to compute backwards local model training information.

-	For Model Training if NWDAF MTLF does not support VFL, MTLF discovers an MTLF with VFL capability. If the MTLF with VFL capability determines that the Model needs training by AF, MTLF discovers an AF with VFL server capability.

During VFL inference (addressed in CR1208 CR rev 9– S2-2412050)

-	The MTLF that trained the ML Model is also an AnLF. The AnLF triggers the MTLF (where AnLF is collocated) that trained the ML model to provide inference using the VFL correlation ID corresponding to the VFL training process.