3GPP TSG-WG SA2 Meeting #166	S2-2412046

Orlando, Nov 18 – 22, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#1 on the general aspects

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 on the general aspects are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1 on the general aspects.

- With S2-2412047 and S2-2412048, the EN about architecture options can be removed.

- An EN about Device info managed by AIOTF and another EN about UDM involvement are resolved by stating that the binding info is stored in AIOTF in bullet 1-a.

- An EN about NF profile of AIOTF and another EN about gNB aggregation are converted to a NOTE.

- In bullet 1-d, it is clarified that the AIOTF will select the topology(ies) for the requested AIoT services.

- An EN about UL addressing is resolved by stating that AIOTF will create and use the AIoT session ID to address the target NFs to forward the UL traffic.

- In bullet f, it is also captured what information needs to be provided by the CN to the AIoT RAN.

- An EN about aggregation detail is removed as the aspect should be addressed by KI#3.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.1.0.

FIRST CHANGE

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

Editor's note:	Information needed for radio resource allocation to readers is FFS and requires cooperation with RAN WG2/RAN WG3.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIOTF performs the following functionalities.

a.	The AIOTF manages and stores the device related information such as the binding information with the last known Reader ID.

b.	The AIOTF registers itself in the NRF with its NF profile.

Note:	The details of the NF profile will be discussed during the normative phase.

c.	The AIOTF receives an AIoT service request from the AF, selects the appropriate topology(ies) for the requested AIoT services based on the operator policy, and triggers the BS/UE readers to perform AIoT service operations towards the AIoT Device(s).

d.	The AIOTF may aggregate the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends them to the AF.

Note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN will be determined by RAN WGs and SA2 will align with the decision during the normative phase.

e.	The AIOTF creates an AIoT session ID for each AF service request, which is used to address target NFs when forwarding uplink traffic.

f.	The AIOTF provides additional information (e.g., list of AIoT device IDs, session ID, estimated number of devices, etc.)  necessary for the AIoT RAN to perform its logic.

Note:	What information can be provided to the AIoT RAN will be determined during the normative phase together with RAN3.

2.	A Permanent AIoT Device IDis stored in the AIoT Device and the UDM or a Credential Holder's AAA server.

Editor's note:	Whether and how the validation of the Ambient IoT Device ID is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

Editor's note:	Whether an AIoT NAS protocol is supported between the AIoT Device and the AIoTF, and what is the supported functionality, is FFS.

END OF CHANGES