SA WG2 Meeting #S2-165	S2-2412075

14 - 18 October, 2024, Hyderabad, India	(revision of S2-2410303)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5084 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1, Energy information collection procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the KI#1 conclusion of 23.700-66 and EnergySys WID, this CR proposes to add procedure on the energy consumption collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedure on energy consumption data collection and calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EnergySys feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.X	Energy Consumption information collection and calculation

### 4.X.1	General

Energy consumption information collection and calculation is supported in the following granularities in this release.

-	Specific UE;

-	PDU session of a UE;

-	per PDU session per application.

-	per PDU session per QoS flow.

### 4.X.2	Procedure of Energy Consumption collection and calculation for a PDU session of a UE

Figure 4.X.2-1: Per PDU session Energy consumption information collection and calculation

1.	The UE has established a PDU session.

2.	The EIF receives a subscription for Energy Consumption of the PDU session of the UE. This subscription may be from a 5GC NF, or AF. The request includes the UE ID (SUPI/GPSI), condition (e.g. time window, etc), IP address of PDU session, DNN and S-NSSAI of the PDU session.

3.	The Energy Consumption subscription triggers the EIF to perform Energy Consumption information collection and calculation for the PDU session of the UE.

4.	The EIF retrieves the SMF serving the PDU session from UDM.

Editor's note:	Whether using the DNN/S-NSSAI to retrieve the SMF is FFS.

5.	The EIF send the Energy information collection request to SMF using Nsmf_EventExposure_Subscribe (SUPI, PDU session ID, condition).

6.	The SMF send the Energy information collection request to UPF. It request the UPF to report the data volume of the PDU session. The request also includes the condition, e.g. in a time window.

If there is I-SMF and I-UPF, the A-SMF Energy information collection request to I-SMF. The I-SMF send the Energy information collection request to I-UPF.

7.	UPF report the data volume of PDU session to SMF.

If there is I-SMF and I-UPF, the I-UPF report data volume in the I-UPF to I-SMF. And I-SMF report the I-UPF id and data volume in the I-UPF to A-SMF.

8.	SMF report the data volume of PDU session to EIF. It may include: UPF IDs and data volume in each UPF, RAN IDs and data volume in each RAN node.

NOTE 1:	The SMF can derive RAN IDs and data volume in each RAN node.

9.	The EIF retrieves the Energy related information of involved User Plane nodes (i.e. UPFs and RAN nodes) from OAM. For each user plane node, Energy related information from OAM includes, (data volume, energy consumption, time window).

10.	The EIF calculates the Energy Consumption of the PDU session.

NOTE 2:	How the SMF calculates the Energy Consumption of PDU session is up to EECF implementation, e.g. according to TS 23.501 [2] Annex XX.

11.	The EIF notifies the UE energy consumption data to NF/AF.

### 4.X.3	Procedure of Energy Consumption collection and calculation for Qos flow of PDU session

The procedure in the clause 4.X.2 is reused with following difference:

-	step 2, 5, 6: The Qos flow ID is provides in addition to PDU session ID.

### 4.X.4	Procedure of Energy Consumption collection and calculation for a PDU session of a UE

The procedure in the clause 4.X.2 is reused with following difference:

-	step 2: Only UE ID is provided (there is no PDU session ID).

-	step 4: retrieves the SMFs for the all of PDU session of the UE.

-	step 5-8: will be performed for the all the PDU session of UE.

END OF CHANGES