3GPP TSG-WG SA2 Meeting #166	S2-2412124

Orlando, USA, 18th Nov – 22th Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, Update interim conclusion to solve ENs

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: The interim conclusion for Identifier and Identification Management is updated to solve the ENs and changed to conclusion.

1. Introduction/Discussion

There are several Editor’s Notes in the conclusion of KI#2 about Identifier and Identification Management. This pCR would like to solve the Editor’s Notes and update the conclusion correspondingly.

Editor's note:	Whether the operational entity is always needed is FFS.

For third party allocated ID, since different third party may have different service requirements which are various. Therefore, it would be difficult and not flexible to further define the components for this type of Ambient IoT Device ID. From operator network point of view, as long as the Ambient IoT Device ID can include the information used to locate the subscription-like data, the network can perform the service operation properly. Based on current interim conclusion, the third party allocated ID includes third party information which can be used to locate credentials holder or subscription-like data. Therefore, no more components are needed.

Proposal 1:	The Editor’s Note can be deleted and the clarification of the third party information can be updated accordingly.

Editor's note:	For operator allocated Identifier, it is FFS whether further information (e.g. the credential holder ID) in Part 1 is needed.

For operation allocated Identifier, it is similar with the consideration mentioned above. Therefore, it would be difficult and not flexible to further define the components for this type of Ambient IoT Device ID. With the help of network identifier, the corresponding PLMN storing the subscription-like data can be discovered. Then how the concrete Credentials Holder can be found within that PLMN can be left to operator defined as part of the Part 2 of the Ambient IoT Device ID information. Hence, no further information is needed in the Part 1.

Proposal 2: 	The Editor’s Note can be deleted.

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

If the length of Identifier is fixed, the length would be set to large enough to consider the maximum number of devices within a network. However, for a network deployed in a local campus, there may not be a large number of Ambient IoT Devices. Therefore, long Identifier length would cause larger communication latency and low system performance. Hence, variable length is suggested.

Proposal 3: 	The Editor’s Note is deleted and a NOTE is added to clarify that the length of Identifier is variable depending on service requirements.

Editor's note:	The uniqueness of Identifier is FFS.

For the two types of Ambient IoT Device ID, how the Operator and 3rd Party ensure uniqueness is operator implementation dependent.

Proposal 4:	The Editor’s Note is deleted and a NOTE is added to clarify that for operator allocated Identifier, the uniqueness is operator implementation dependent and for third party allocated ID, the uniqueness is third party implementation dependent.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

Since SA3 is till under discussion about whether and how to solve the privacy protection and whether the authentication is mandatory or not, the alignment in SA2 will take place during normative phase.

Proposal 5:	The Editor’s Note is deleted and a NOTE is added to clarify that whether the temporary ID allocated by AIOTF is required for the privacy protection will be coordinated with SA3 during normative phase.

Proposal 6:	Reword the usage of part 1 information in the Device ID to locate the subscription-like data instead of authentication server.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.2	Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, which is used to indicate the identifier is assigned by an operator or by a third party.

-	The operational entity, which is the entity managing and allocating the Identifier:

-	For operator allocated ID, it is a network identifier (i.e. MCC+MNC and/or NID).

-	For third party allocated ID, it is information used to identify a 3rd party.

2)	Part2 information:

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

NOTE 2:	If the information used to identify the 3rd party is the scheme and the company prefix of the EPC, Part2 information is not necessary to be the whole EPC but can be the leftover part of the EPC.

NOTE 3:	The coding for the above information is left to stage 3.

Figure 8.2.1-1: The Operator allocated ID

For operator allocated Identifier, the network identifier is mandatory and can be used to index the subscription like data.

Figure 8.2.1-2 The third party allocated ID

For third party allocated Identifier, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the subscription like data.

NOTE X:	The length of Identifier is variable depending on service requirements.

NOTE Y:	For operator allocated Identifier, the uniqueness is operator implementation dependent. For third party allocated ID, the uniqueness is third party implementation dependent.

NOTE Z:	Whether the temporary ID allocated by AIOTF is required for the privacy protection will be coordinated with SA3 during normative phase.

# * * * * End of changes * * * *