SA WG2 Meeting #S2-166	S2-2412174

18 - 22 November, 2024, Orlando, USA

Title:	[draft] Reply LS on UE's user plane availability status for LCS-UPP

Response to:	LS (C1-246023/S2-2411289) on UE's user plane availability status for LCS-UPP

Release:	Release 18

Work Item:	5G_eLCS_Ph3

Source:	[CATT to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	Yunjing Hou

Tel. Number:	-

E-mail Address:	houyunjing@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.273 CR 0629, CR 0630

1. Overall Description:

SA2 thanks CT1 LS on UE’s user plane availability status for LCS-UPP. Regarding the following issue caused by user plane not available,

In 3GPP TS 24.572, the cause value "user plane not available" was introduced for UE to reject the network initiated user plane connection establishment procedure when the user plane is not available or release the established user plane connection when the user plane availability is changed to “not available”, e.g. due to the activation of the PS data off for the UE.

However, currently there is no dedicated procedure specified to inform the LMF when the UE's user plane availability changes from "unavailable" to "available". What currently is supported:

-	The UE can initiate UE requested user plane connection establishment procedure, which may not be appropriate unless the UE has UL positioning signalling, i.e. either such notification is delayed or user plane positioning resources are unnecessarily established;

-	The LMF can attempt reactivation of user plane positioning using the network initiated user plane connection establishment procedure. This leads to extra unsuccessful user plane connection establishment procedure attempts over the air interface until the user plane is "available".

SA2 has discussed the questions and agrees the following answers:

- Would the above issue need to be resolved?

SA2 answer: yes.

- If the answer to the question 1 is yes, CT1 would like to ask for SA2 guidance on resolving the issue.

SA2 answer: for UE supporting user plane positioning capabilities, if the user plane connection becomes unavailable due to e.g. the activation of PS data off, the use user plane positioning does not work for the UE. In this case, the UE can be considered as the UE not supporting user plane positioning capabilities. So the UE shall update the user plane positioning capabilities to not support by initiating the Registration procedure. When the user plane connection becomes available, e.g. due to deactivation of PS data off, the UE updates the user plane positioning capabilities to support by initiating the Registration procedure. In this mechanism, the AMF and LMF are not impacted. The attached CR is agreed by SA2 to support the mechanism.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into account.

3. Date of Next SA2 Meetings:

SA2#166-Ad Hoc-e                           20th January- 24th  January 2025                          Online

SA2#167                                           17th February- 21th  February 2025                       Athens, GR